3GPP TSG RAN WG1 Meeting #92	R1-1803402

Athens, Greece, February 26th – March 2nd, 2018

Source:              ZTE, Sanechips

Title:                   Offline summary for Al 7.1.6.1 NR UL power control in non-CA aspects

Agenda Item:     7.1.6.1

Document for:   Discussion and Decision

# Offline summary

Offline agreement:

For UE specific pucch-PowerControl and pusch-PowerControl configuration, at least including UL power control parameter set, DL RS for path loss estimate and closed-loop power control process:

- Individual parameters are configured per BWP

Offline agreement:

For the case of PUSCH with grant and no SRI field in uplink grant, at least the following is supported

- DL RS for PL estimation is given by, the RS corresponding to pusch-pathlossreference-index=0  of pusch-pathloss-Reference-rs (i.e., q_d =0), where only one DL RS for path loss is configured

- P0 and alpha are given by, the values corresponding p0alphasetindex =0 of p0-push-alpha-setconfig (i.e., j=2), where only one entry of p0-push-alpha-setconfig is configured for schedule based transmission;

- Closed loop index l=0

Notes that the agreement in RAN1#91 “for PRACH, PUSCH, PUCCH, and SRS, all power control parameters are configured per serving cell/uplink”.

Notes that depending on the further agreement on the MIMO section, additional mapping rules for {j, q_d,l} for the grant PUSCH without SRI can be considered.

Offline agreement:

For the case of PUCCH without PUCCH-Spatial-relation-info, at least the following is supported

- DL RS for PL estimation is given by, the RS corresponding to pucch-pathlossreference-index=0  of pucch-pathloss-Reference-rs (i.e., q_d =0), where only one DL RS for path loss is configured

- P0 is given by, the values corresponding p0setindex =0 of p0-pucch-set (i.e., q_u=0), where only one entry of p0-pucch-set is configured;

- FFS: Closed loop index l=0

Notes that the agreement in RAN1#91 “for PRACH, PUSCH, PUCCH, and SRS, all power control parameters are configured per serving cell/uplink”.

Notes that depending on the further agreement on the MIMO section, additional mapping rules for {q_u, q_d,l} for the grant PUSCH without SRI can be considered

Offline agreement:

Clarification that  is one value for all PUCCH resource

Offline working assumption

| deltaF-pucch-f0 | deltaF values configurable for PUCCH format 0 | INTEGER (-16..15) with 1dB step size |
| --- | --- | --- |
| deltaF-pucch-f1 | deltaF values configurable for PUCCH format 1 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f2 | deltaF values configurable for PUCCH format 2 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f3 | deltaF values configurable for PUCCH format 3 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f4 | deltaF values configurable for PUCCH format 4 | INTEGER (-16..15) with 1dB step size |

Clarification that deltaF-pucch-f2, deltaF-pucch-f3, deltaF-pucch-f4 are included in higher layer PUCCH-config instead of PUCCH-configCommon.