3GPP TSG RAN WG1 Meeting #92     	                                                                      R1-1803405

Athens, Greece, February 26th – March 2nd, 2018 	                                  Revision of R1-1803315

Source:	CATT

Title:	Summary of Offline Discussion on RMSI

Agenda Item:	7.1.1.2.2

Document for:	Discussion and Decision

# Introduction

This purpose of the document is to provide a summary on the remaining key issues and the proposed solutions on AI 7.1.1.2.2 “Remaining minimum system information” for facilitating the continue discussion in RAN1#92 based on the contributions submitted under this AI [1-14].

In this document, the discussion of the remaining issues is organized as follows:

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 1

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 2

RMSI CORESET Configurations for 10 MHz min CH BW with 15 kHz SS

Other Proposals on RMSI CORESETs

RMSI PDCCH

RMSI PDSCH

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 1

Discussion Status: Closed (see Chairman’s Note for the agreement)

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 2

Background

For the SS/PBCH block and control resource set multiplexing pattern 2, a UE monitors PDCCH in the Type0-PDCCH common search space over one slot with Type0-PDCCH common search space periodicity equal to the periodicity of SS/PBCH block. For a SS/PBCH block with index , the UE determines the slot index  and  based on parameter provided by Tables 13-13 through 13-14 in TS 38.213.

| SSB | 120 kHz | 0 | 1 | 2 | 3 | SSB#0 |  | SSB#1 |  | 12 | 13 | 0 | 1 | SSB#2 |  | SSB#3 |  | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMSI | 60 kHz | 0 |  | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 |  | 8 | 9 | 10 | 11 | 12 |  | 13 |  |

- RMSI PDCCH has only 1 OFDM symbol

| SSB | 120 kHz | 0 | 1 | 2 | 3 | SSB#0 |  |  | SSB#1 |  |  |  | 12 | 13 | 0 | 1 | SSB#2 |  |  | SSB#3 |  |  |  | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMSI | 60 kHz | 0 |  | 1 |  | 2 | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  |
|  |  |  |  | 1 |  | 2 |  |  |  |  |  |  |  |  | 7 |  | 8 |  |  |  |  |  |  |  |  |  |  |

- RMSI PDCCH has 2 OFDM symbols

Figure 3-1 Illustration of the RMSI CORESET monitoring window for Pattern 2

When RMSI CORESET configuration has the duration of 1 OFDM symbols for Patter 2, there will be no overlapping of the RMSI CORESET with each other . However, when RMSI CORESET configuration uses the duration of 2 OFDM symbols for Patter 2, there will be overlapping of the RMSI CORESET with each other as shown in above Figure 3-1.

In addition, in order to guarantee UL control transmission at every slot, RAN1 agreed that SS/PBCH block(s) are not allocated at the OFDM symbols of the index of 12 and 13. However, in PDCCH monitoring occasion of pattern 2 for {SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz, the configuration to set the monitoring occasion of PDCCH at the OFDM symbols of the index of 12 and 13 is defined, as shown in Figure 3-2.

Submitted Proposals:

- Proposal from R1-1801329:

To allow overlapping RMSI CORESETs in Pattern 2, UE Rx spatial operations need to be addressed. Adding new first symbol index entries into Table 13-13 and Table 13-14 of TS 38.213 (for Option 2) or specifying UE’s assumption on RMSI CORESET monitoring (for Option 3) can be for further considerations..

Figure 3-3. New configuration of RMSI CORESET and associated SSB (R1-1801329)

- Proposal from R1-1801406:

NR should support RMSI CORESETs for two continuous SS/PBCH blocks overlapping with each other.

- Proposal from R1-1801508

For collision between SS block and RMSI CORESET, following alternatives can be taken into account and alt.1 is preferred:

Alt.1: UE assumes resources within RMSI CORESET that partially or fully overlapped with candidate SS block resources are not used for RMSI PDCCH transmission.

Alt.2: gNB’s indication of transmitted SS block that partially or fully overlap with RMSI CORESET is considered an error in configuration and no UE behavior needs to be specified.

- Proposal from R1-1801642:

Option 1 by removing the configurations with =2 in Table 13-5 and Table 13-8 of TS 38.213 is adopted:

- Proposal from R1-1801709:

For the SS/PBCH block and CORESET multiplexing pattern 2, down-select one of the following options for dealing with the potential collision between SSBs and the RMSI CORESET:

Option 1: Remove the configurations with =2 for Pattern 2 in Table 13-7 and Table 13-10 of TS 38.213;

Option 2: Modify RMSI CORESET configurations in Tables 13-13 and 13-14 of TS 38.213 in order to support =2 for Pattern 2 as shown in Figure 3-2.

| SSB | 120 kHz | 0 | 1 | 2 | 3 | SSB#0 |  |  |  | SSB#1 |  |  |  | 12 | 13 | 0 | 1 | SSB#2 |  |  |  | SSB#3 |  |  |  | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMSI | 60 kHz | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  |
|  |  | 0 |  | 1 |  |  |  |  |  |  |  |  |  | 6 |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 3-2: Illustration of the supporting RMSI CORESET monitoring windows with 1 and 2 OFDM symbols for Pattern 2 (R1-1801709)

- Proposal from R1-1802142:

For multiplexing pattern 2, it is proposed to support the collision cases and left gNB’s choice.

- Proposal from R1-1802191:

RAN1 allows the additional entries RMSI PDCCH monitoring window occasion for pattern 2 as follows:

Limit the number of SS/PBCH block

Entry #1: 0, 1, 2, 3, 0, 1 in i=8k, i=8k+1, i=8k+2, i=8k+3, i=8k+4, i=8k+5 (nc = nSSB,i)

Allow the mapping of the multiple SS/PBCH index to the same OFDM symbol

Entry #2: 0, 2, 0 in i={8k, 8k+1}, i={8k+2, 8k+3}, i={8k+4, 8k+5} (nc = nSSB,i)

Entry #3: 0, 1, 0, 1 in i={8k, 8k+1}, i={8k+2, 8k+3}, i={8k+4, 8k+5}, i={8k+6, 8k+7} (nc = nSSB,i)

- Text from R1-1802383:

“In case of SSB and RMSI SCS of {240kHz, 120kHz}, it would not be possible to support 2 OFDM symbol duration CORESET configuration. Therefore, entries need to be removed.”

- Proposal from R1-1802891:

Support =2 for pattern 2 with SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz by allowing the search spaces corresponding to consecutive SS/PBCH blocks to overlap. For SS/PBCH block, PDCCH} subcarrier spacing {120, 60} kHz omit the =2 and support only =1 for pattern 2.

- Proposal from  R1-1802462:

Regarding the collision issue between SS/PBCH block location and RMSI CORESET monitoring window, there is no need to modify the current configuration of RMSI CORESET monitoring window.

Discussion Status

In offline discussion in Tuesday afternoon, the issue on whether to supporting configurations with =2 for both {SSB SCS, PDCCH SCS} = {120, 60} kHz and {SSB SCS, PDCCH SCS} = {240, 120} kHz were discussed. The following offline agreements were suggested. Some companies wanted to have a further check on the suggested offline agreements. Based on the comments in an email from LGE on Wednesday, we will have further offline discussion on how to handle the collision issue of Pattern 2 before presenting the suggested offline agreements to online meeting.

Agreements (RAN1#92):

  - For {SSB SCS, PDCCH SCS} = {120, 60} kHz, remove the configurations with =2 in Table 13-7.

Offline proposals:

- For {SSB SCS, PDCCH SCS} = {240, 120} kHz

Option 1: Remove the configurations with =2 in Table 13-10 for {SSB SCS, PDCCH SCS} = {240, 120} kHz

Supported by:

  - Option 2:  Modify Table 13-14 to support =2 for {SSB SCS, PDCCH SCS} = {240, 120} kHz

      - Supported by:

      - Note: Different ways of the modifications are proposed by different companies

  - Option 3: No change in the current RMSI CORESET configurations for Pattern 2. It will be up to the network on how to deal with the overlapping issue.

    - Supported by:

- For {SSB SCS, PDCCH SCS} = {120, 60} kHz

  - Option 1: keep the agreement

    - Supported by:

Option 2: revise the agreement to support the configurations with =2 in Table 13-7 for {SSB SCS, PDCCH SCS} = {120, 60} kHz

Supported by:

Suggested Offline agreements

For {SSB SCS, PDCCH SCS} = {240, 120} kHz, modify the configuration in Table 13-14 to support =2 as shown in the following text proposal

========== START of Text Proposal (TS 38.213) =========

Table 13-14: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 2 and {SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, …, 7) |
| --- | --- | --- |
| 0 | or | For =1, 0, 1, 2, 3, 0, 1 in , , , , , () 12, 13 in, , , () |
|  |  | For =2, 0, 0, 2, 2, 0, 0 in , , , , , () 12, 12 in, , , () |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

========== END of Text Proposal (TS 38.213) =========

Suggested Offline agreements

- Revise the following online agreements:

Agreements (RAN1#92):

  - For {SSB SCS, PDCCH SCS} = {120, 60} kHz, remove the configurations with =2 in Table 13-7.

- To the following:

For {SSB SCS, PDCCH SCS} = {120, 60} kHz, modify the configuration in Table 13-13 to support =2 as shown in the following text proposal for TS 38.213

========== START of Text Proposal (TS 38.213) =========

Table 13-13: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 2 and {SS/PBCH block, PDCCH} subcarrier spacing {120, 60} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 |  | For =1, 0, 1, 6, 7 for , , , |
|  |  | For =2, 0, 0, 6, 6 for , , , |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

.

========== END of Text Proposal (TS 38.213) =========

RMSI CORESET configurations for SSB SCS 15kHz with minimum channel bandwidth 10MHz

## Support of RMSI CORESET configurations for “SSB SCS 15kHz + min CH BW 10MHz”

Background

RAN1 has defined RMSI CORESET configurations supporting for all combinations of {SSB SCS, RMSI SCS, minimum channel BW} defined in RAN4, except the following:

- {15kHz, 15kHz, 10MHz}

- {15kHz, 30kHz, 10MHz}

In RAN1#AH18_01, it was discussed on whether there was a need to define the RMSI CORESET configuration tables for 15kHz SCS for SS/PBCH block with minimum channel bandwidth of 10MHz w/o conclusion.

From RAN4 point of view, it is our understanding that it is still an open issue on whether to support 15kHz SCS for SS/PBCH block with minimum channel bandwidth of 10MHz, as well as and if 15kHz SCS for SS/PBCH block with minimum channel bandwidth of 10MHz is supported, what the SS Raster definition (i.e., whether the down-selection factor is 1 or 3).

Submitted Proposals:

- Proposal from R1-1802191:

RAN1 defines a new RMSI CORESET configuration table for the following cases.

{SS/PBCH block, PDCCH} subcarrier spacing is {15,15}kHz with minimum channel bandwidth 10MHz

{SS/PBCH block, PDCCH} subcarrier spacing is {15,30}kHz with minimum channel bandwidth 10MHz

- Proposal from R1-1801947:

If RAN4 supports 15 kHz SS SCS for band n41, add two new configuration tables for the following cases:

{SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for with minimum channel bandwidth 10MHz

{SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for with minimum channel bandwidth 10MHz

Based on the online discussion, we will wait for the decision from RAN4 on whether there is a need to support 15 kHz SS SCS with minimum channel bandwidth 10MHz.

Discussions

The following proposal was presented in online session in Monday:

- If RAN4 decides to support 15 kHz SS SCS with minimum channel bandwidth 10MHz, RAN1 will define new RMSI CORESET configuration tables for supporting the following cases:

  - {SS/PBCH block, PDCCH} subcarrier spacing is {15,15}kHz with minimum channel bandwidth 10MHz

  - {SS/PBCH block, PDCCH} subcarrier spacing is {15,30}kHz with minimum channel bandwidth 10MHz

At that time, there was no objection for the proposal, although there was no formal online decision/conclusion. The issue now is that it is unclear when RAN4 will make the decision. There is a suggestion from a company that RAN1 should define the configuration tables without waiting for RAN4’s decision.

Offline Proposals:

- Introduce new RMSI CORESET configuration tables (in this meeting) to support 15 kHz SS SCS with minimum channel bandwidth 10MHz instead of waiting for RAN4’s decision. RAN1 will remove the RMSI CORESET configuration tables if RAN4 later makes the decision that does not need to support 15 kHz SS SCS with minimum channel bandwidth 10MHz or does not need the  new RMSI CORESET configuration tables to support it;

  - Supported by:

- RAN1 should wait for RAN4’s decision on whether and how to introduce new RMSI CORESET configuration tables in this meeting to support 15 kHz SS SCS with minimum channel bandwidth 10MHz.

  - Supported by:

Offline Agreements:

- RAN1 should send an LS to RAN4 that the current PBCH content design makes it difficult to support any new SSB SCS and minimum channel bandwidth combination.

## RMSI CORESET configuration tables for “SSB SCS 15kHz + min CH BW 10MHz”

Background

- If it reaches the consensus to define new RMSI CORESET configuration tables for the following cases SSB SCS=15kHz with minimum channel bandwidth 10MHz, then the next issue is how to define the RMSI CORESET configuration tables for above two cases.

Proposal from R1-1802191:

Alt.1: Two tables with 4bits configuration

Alt.2: Single table with 5bits configuration

Proposal from R1-1801947:

Details of the tables should depend on RAN4’s decision on the step size of sync raster for band n41, and examples in R1-1801071 can be used as a starting point.

Suggested Offline Conclusion

- Based on the conclusion in Section 4.1, no further discussion on the issue in this meeting.

# Other Proposals on RMSI CORESETs

## Modification of RMSI PDCCH monitoring occasions for Pattern 1

Background

In R1-1802135 it is proposed to optimize the DL/UL resource configuration in NR system. The starting slot should locate on the same half frame index with SSBs.

Submitted Proposals

The determination of the index of slot should be changed in TS 38.213 should be changed.

- satisfying  if  or satisfying  if .

where, HRF is half frame bit indicated in PBCH.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | It is unclear to us why the starting slot should be located on the same half frame index with SSBs. In addition, with the proposed n0: it has n0O=n0O+5, when O=0,2,2.5, since 5∙2μ=12Nslotframe,μ. For example, the configuration index 0 (with O=0) and index 4 (with O=5) in Table 13-11 are actually the same configuration. |
| OPPO | For CATT’s question above, the intention is that trying to allocate both the SSBs and RMSIs in either the 1st half frame or within the 2nd half frame so that slots in another half frame can be configured as UL slots. This is to solve the problem that if the offset of SSBs and RMSI is 5 or 7, there would be no or few UL slots within one frame based on the current formulation. Please note that the SSBs and RMSIs may be in the same frame or in different frame based on the SSB index . For the determination of , if index of is in a span of half frame, is even. If index of is beyond the span of half frame, is odd. satisfying if or in a frame with SFN satisfying if . |

Suggested Offline Conclusion

The proposal was discussed in offline discussion. The proposed changes seem not essential, interested companies are encouraged to take a further look at the proposal to see if the proposed change is essential.

## Change the definition of the initial active DL BWP for RMSI multiplexing pattern 3 case

Background

Initial active DL BWP is defined to the bandwidth of the RMSI CORESET. UE does not perform RLM nor measurements outside the active DL BWP for the serving cell. In R1-1802383 it was discussed that this poses problem to measurement and RLM, and thus it is proposed to “change the definition of the initial active DL BWP for RMSI multiplexing pattern 3 case, such that the initial active DL BWP is the total bandwidth encompassing the RMSI CORESET and SSB bandwidth”.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

Submitted Proposals [R1-1802383, R1-1802894]

- Change the definition of the initial active DL BWP for RMSI multiplexing pattern [2] and/or [3], such that the initial active DL BWP is the total bandwidth encompassing the RMSI CORESET and SSB bandwidth.

Offline Discussion

The above proposal was discussed in offline discussion. Some companies requested more time to check the proposal.

## Clarification of e common resource block boundary of RMSI SCS

Background (R1-1802699)

For SS/PBCH type A, the subcarrier spacing of NCRBSSB is 15KHz. Therefore, after decoding the subcarrier offset in PBCH, UE finds the subcarrier 0 of NCRBSSB, and thus it finds the CRB grid of 15 KHz. If subcarrier spacing of RMSI is 30KHz, UE still has ambiguity on the CRB grid of RMSI, as illustrated in the figure below. UE has ambiguity between case 1 and case 2, since both cases satisfy the nested grid. In fact, the number of bits for subcarrier offset in PBCH was agreed to be increased from 4 to 5 bits to resolve this ambiguity. However, it is not fully captured in the current TS 38.211.

Proposal from R1-1802699

### ========= Text Proposal for 38.211 ===========

#### 7.4.3.1	Time-frequency structure of an SS/PBCH block

In the time domain, an SS/PBCH block consists of 4 OFDM symbols, numbered in increasing order from 0 to 3 within the SS/PBCH block, where PSS, SSS, and PBCH with associated DM-RS are mapped to symbols as given by Table 7.4.3.1-1.

In the frequency domain, an SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block. The quantities  and  represent the frequency and time indices, respectively, within one SS/PBCH block. The UE may assume that the complex values corresponding to resource elements denoted as 'Set to 0' in Table 7.4.3.1-1 are set to zero.  The quantity  in Table 7.4.3.1-1 is given by . The quantity  is the subcarrier offset from subcarrier 0 in common resource block  to subcarrier 0 of the SS/PBCH block, where  is obtained from the higher-layer parameter offset-ref-low-scs-ref-PRB and where the 4 least significant bits of  are given by the higher-layer parameter ssb-subcarrierOffset and for SS/PBCH block type A the most significant bits of  are given by  in the PBCH payload as defined in subclause 7.1.1 of [4, TS 38.212].

Any common resource block partially or fully overlapping with an SS/PBCH block shall be viewed as occupied and not used for transmission of PDSCH or PDCCH. The UE may assume that the complex values corresponding to resource elements that are part of such a partially overlapping common resource block, and are not used for SS/PBCH transmission are set to zero.

For an SS/PBCH block, the UE shall assume

-	antenna port  is used for transmission of PSS, SSS and PBCH,

-	the same cyclic prefix length and subcarrier spacing for the PSS, SSS, and PBCH,

-	for SS/PBCH block type A,  and  with the quantities , and  expressed in terms of 15 kHz subcarrier spacing, and

-	for SS/PBCH block type B,  and  with the quantity expressed in terms of the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and  is expressed in terms of 60 kHz subcarrier spacing.

-	the center of subcarrier 0 of the common resource block coincides with the center of subcarrier 0 of a common resource block of the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon.

### ========= End of Text Proposal for 38.211 ===========

RMSI PDCCH

It was agreed in RAN1#AH18_01 that the details of RMSI PDCCH format design would be covered under control channel session after the coordination between the feature leads.

RMSI PDSCH

## RMSI PDSCH modulation scheme

Discussion Status: Continue discussion until next meeting

## Maximum TBS for RMSI PDSCH

Discussion Status: Closed (see Chairman’s Note for the agreement)

References

- R1-1801329	Remaining  on RMSI configurations	Huawei, HiSilicon

- R1-1801406	Remaining details of RMSI	ZTE, Sanechips

- R1-1801508	Discussion on Remaining Minimum System Information	vivo

- R1-1801642	Remaining issues for RMSI	MediaTek Inc.

- R1-1801709	Remaining Issues on RMSI	CATT

- R1-1801947	Corrections on Remaining Minimum System Information	Samsung

- R1-1802135	RMSI PDCCH monitoring window configuration	Guangdong OPPO Mobile Telecom

- R1-1802142	Remaining issues on RMSI delivery	ETRI

- R1-1802191	RMSI delivery and CORESET configuration	LG Electronics

- R1-1802383	Remaining details of RMSI	Intel Corporation

- R1-1802462	Remaining issues on RMSI	NTT DOCOMO, INC.

- R1-1802811	Remaining system information delivery consideration	Qualcomm Incorporated

- R1-1802891	Remaining open issues on RMSI CORESET configuration	Nokia, Nokia Shanghai Bell

- R1-1802942	Remaining details of remaining minimum system information	Ericsson

- Draft Report of 3GPP TSG RAN WG1 #AH_1801 v0.1.0, ETSI

- 3GPP TS 38.213, V15.0.0 (2018-02)

- R1-1721643	Reply LS on Minimum Bandwidth		RAN4

- R1-1800008	LS on channel raster and SS raster 		RAN4