3GPP TSG RAN WG1 Meeting #92	                        	R1-1803433

Athens, Greece, February 26th – March 2nd, 2018

Source:	Ericsson

Title:	Summary of 7.1.1.2.1 Remaining details on NR-PBCH

Agenda Item:	7.1.1.2.1

Document for:	Discussion and Decision

# Introduction

This is the summary for the 7.1.2.1 providing a summary of the topics being discussed on the remaining details on NR-PBCH based on the views expressed by companies in the contributions listed in the appendix.

# Indication of cell-defining SS/PBCH block in PBCH

In RAN1 NR adhoc 1801, the following was decided for the indication of cell-defining SS/PBCH block in PBCH:

Regarding signaling the valid search raster:

- No additional signaling entries in the PBCH (for example cellBarred, intraFreqReselection) are to be used to inform the UE of the valid raster

- The indication is only for the same frequency band

The three initial issues to resolve are:

- Absolute or relative signaling

- Whether for FR1 to include the value of M

- What to indicate if there is no cell defining SS/PBCH block within the range.

It is recommended to make a quick decision is made so that the final signaling scheme can be put together during the week.

Offline Conclusion:

- The indication should be confined within a contiguous spectrum allocation of a same operator in which the SSB is detected.

Proposals

- Signal an offset from a reference GSCN to the closest GSCN that UE should search for cell-defining SSB

- The reference GSCN is down-selected from the following alternatives

  - Alt. 1: The reference GSCN is the GSCN that SSB is transmitted (or the middle in the cluster for sub 2.7 GHz)

    - Supported by: Qualcomm, Samsung, OPPO, MediaTek, CATT

  - Alt. 2: The reference GSCN is the first GSCN of the frequency band in which SSB is transmitted

    - Supported by: Nokia, LG Electronics, ZTE, Huawei, HiSilicon, NTT DOCOMO

  - Alt. 3: The GSCN is divided into groups of 256. The indication is within the group or one of the adjacent groups

    - Supported by: LG Electronics, Intel,

Proposed agreement

- Use reserved codeword(s) to indicate that there is no cell-defining SS/PBCH around the reference GSCN (Gr) in the range [Gr-∆Glow,Gr+∆Ghigh]

  - Signal ∆Glow by multiplication of the step size, X, and 4 MSBs of RMSI-PDCCH-Config corresponding to parameter ssb-subcarrierOffset set to 15 and  aA+5=0(for FR1)

  - Signal ∆Ghigh by multiplication of the step size, X, and 4 LSBs of RMSI-PDCCH-Config corresponding to parameter ssb-subcarrierOffset set to 15 and  aA+5=0(for FR1)

  - FFS: The step size X is either in sync raster step or frequency and may be different for FR1 and FR2

## Absolute or relative signaling

The support for absolute vs relative signaling are as follows.

- Signaling is absolute/relative to the frequency band edge

  - Supported by: ZTE, Sanechips, CATT, Lenovo, Motorola Mobility, ETRI, LG Electronics, Intel, InterDigital, Nokia, Ericsson

- Signaling is relative to the detected SS/PBCH block

  - Supported by: MediaTek, Qualcomm

OPPO and Nokia additionally proposed that the indication should be within a limited frequency range (confined to one operator) and not the full band.

Proposal:

- Agree to absolute signaling of sync raster point within frequency band

Several companies proposed to use additional bits from PBCH to indicate the sync position and possibly also outside the band, but this is not according to the agreement listed above.

## Indication of the ±5 kHz offset in FR1

The following views were expressed when it comes to the indication of the additional ±5 kHz raster points for sub 2.7 GHz:

- Yes: CATT, Samsung, ETRI, Intel, Qualcomm, Nokia

- No: MediaTek, Lenovo, Motorola Mobility, LG Electronics, InterDigital

Further offline discussion is needed.

## Indication when no cell-defining SS/PBCH block within the range

There are several proposals on what to indicate if there is no valid SS/PBCH in the band:

- Use one reserved codeword to indicate that there is no information on the presence of cell-defining SS/PBCH

  - Supported by: ZTE, Sanechips, OPPO, ETRI, ETRI, LG Electronics, Qualcomm, Ericsson

- Indication last non-cell-defining SS/PBCH block

  - Supported by: MediaTek

- Indicate frequency range where there is no SS/PBCH block

  - Supported by: Nokia

Proposal:

- Use one reserved codeword to indicate that there is no information on the presence of cell-defining SS/PBCH

# Higher layer parameter SSB-index-explicit in 36.213

Ericsson brought up that after moving the SSB index from the MIB to L1 and 38.212, the higher layer parameter SSB-index-explicit should be removed.

Proposal:

- Agreed to the TP to section 4.1 of 38.213 below.

# <Start of Text proposal for section 4.1 of 38.213>

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to . A UE shall determine the 2 LSB bits, for , or the 3 LSB bits, for , of a SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH. For , the UE shall determine the 3 MSB bits of the SS/PBCH block index per half frame by  in the PBCH payload as defined in Subclause 7.1.1 of [4, TS 38.212].

# <End of Text proposal for 38.213>

# Other

These are issues typically raised by single companies. Each if these should be discussed to understand the need for including them in the specifications at this point.

## Scrambling sequence of NR-PBCH

ZTE raised the issue of needing to specify the mapping from bit values on PBCH to the integer value of .

Feature lead comment: 3GPP has a common mapping from integers to bits and vice versa, hence no additional specification is needed for

# Appendix: Contributions used as basis for the summary

Additionally. some companies’ views have been taken from contributions in AI 7.1.1.

| R1-1801328 | Remaining details of NR PBCH design | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1801405 | Remaining details of NR-PBCH Design | ZTE, Sanechips |
| R1-1801507 | Remaining aspects on NR-PBCH | vivo |
| R1-1801641 | Remaining issues for NR-PBCH | MediaTek Inc. |
| R1-1801708 | Remaining Issues on NR PBCH | CATT |
| R1-1801821 | Remaining details on NR-PBCH | Lenovo, Motorola Mobility |
| R1-1801946 | Corrections on NR-PBCH | Samsung |
| R1-1802134 | Discussion on synchronization raster indication | Guangdong OPPO Mobile Telecom |
| R1-1802141 | Indication of cell-defining SS/PBCH block in PBCH | ETRI |
| R1-1802188 | Remaining Details on PBCH design and contents | LG Electronics |
| R1-1802382 | Remaining details of NR-PBCH | Intel Corporation |
| R1-1802461 | Remaining issues on NR-PBCH | NTT DOCOMO, INC. |
| R1-1802614 | Remaining Details on NR-PBCH | InterDigital, Inc. |
| R1-1802810 | Remaining details on NR-PBCH | Qualcomm Incorporated |
| R1-1802890 | Remaining details related to SS/PBCH block | Nokia, Nokia Shanghai Bell |
| R1-1802892 | On indication of valid locations of SS/PBCH with RMSI | Nokia, Nokia Shanghai Bell |
| R1-1802941 | Remaining details of PBCH | Ericsson |