3GPP TSG RAN WG1 #92	R1-1803497

Feb 26th – March 2nd, 2018

Athens, Greece

Agenda item:	7.6

Source: 	Qualcomm Incorporated

Title: 	Offline discussion summary on NR-U

Document for:	Discussion and Decision

# 1	Background

This paper is the summary of offline discussion for NR-unlicensed.

# 2	Previous agreements

Agreement:

Study the additional functionality needed beyond the specifications for operation in licensed spectrum in the following deployment scenarios.

- Carrier aggregation between licensed band NR (PCell) and NR-U (SCell)

  - NR-U SCeall may have both DL and UL, or DL-only.

- Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell)

- Stand-alone NR-U

- An NR cell with DL in unlicensed band and UL in licensed band

- Dual connectivity between licensed band NR (PCell) and NR-U (PSCell)

Note (to be included in the minutes): Some companies believe that a prioritization among the agreed deployment scenarios may be necessary.

Agreement:

- 5GCM in 38.802 is used for NR-U simulation evaluation

- NR-unlicensed simulation evaluation considers the following scenarios

  - Indoor sub-7GHz, 2 operators

  - Outdoor Sub-7 GHz, 2 operators

  - Indoor mmW, 2 Operators

  - Outdoor mmW, 2 operators

  - Stadium scenario for sub-7GHz, 2 operators, can be optionally considered by interested companies.

  - Note: RAN1 prioritizes the simulation for sub-7 GHz band. It does not preclude evaluation for above 7 GHz.

- Deployment scenarios to simulate

  - CA between NR licensed cell and NR unlicensed cell

  - DC (with LTE and with NR)

  - SA

  - An NR cell with DL in unlicensed band and UL in licensed band

  - Note: A single set of evaluations may be applicable to multiple scenarios

  - Note: Only unlicensed cell(s) is simulated.

  - Note: The licensed cell may not be explicitly modeled in the simulation. Necessary assumptions regarding the presence of the licensed carriers can be made and provided.

- Coexistence with other networks (e.g. WiFi, LAA LTE, NR-U)

  - When coexistence with WiFi is evaluated, only consider deployed WiFi systems (e.g. 11ac for 5 GHz)

    - Fairness criterion for coexistence with 11ax can be further discussed at plenary level

    - The coexistence evaluation applies to 5GHz band (11ac) and 60GHz (11ad)

    - From SID: NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier

  - For sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (eg. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations

    - Note: The study assumes regulation will provide the framework concerning the protection for the techonologies not using unlicensed access in those bands

Note (to be included in the minutes): Some companies believe that a prioritization among the agreed simulation scenarios may be necessary.

Agreement:

The following network topologies are included in the evaluations:

- Indoor sub7GHz, choose one of the following options

  - Option 1: Reuse 38.802 indoor hotspot topology and allocating half of the gNBs to each operator (6+6)

  - Option 2: Reuse 38.802 indoor hotspot topology but further reduce gNB density (3+3)

  - Option 3: Based on IEEE indoor enterprise model with modifications

- Outdoor sub7GHz

  - NR dense urban scenario with two layers, but only consider the micro layer

  - Randomly drop one micro layer per operator

- Indoor mmW

  - Reuse indoor sub7GHz topology

  - Parameter changes may be needed and submitted together with simulation results

- Outdoor mmW

  - Reuse outdoor sub7GHz topology

  - Parameter changes may be needed and submitted together with simulation results

# 3	Discussion points

Offline agreement (for AI 7.6):

- The study targets identification of additional functionality needed for a PHY layer design (except channel access procedures) for operation in unlicensed spectrum that may be applicable over a particular frequency range (e.g., sub-7 GHz, 7-52.6 GHz, > 52.6 GHz).

  - FFS: The definition of the frequency ranges

  - Note: Optimizations for a particular frequency band may be necessary.

  - Note: Channel bandwidths below 5 MHz are not targeted

- The study targets the design of channel access procedures for frequency bands based on coexistence and regulatory considerations applicable to the band.

- Note: The study includes identification of procedures for technology neutral channel access for frequency bands that may become available subject to regulations.

  - The study assumes regulation will provide the framework concerning the protection for the techonologies not using unlicensed access in those bands.

Offline agreement (for AI 7.6.3)

From RAN1 design perspective, the study is not limited to a particular unlicensed band.