3GPP TSG-RAN WG1 Meeting AH 1801	R1-1803501

February 26th – March 2nd  January 2018

Athens, Greece

Agenda item:	7.1.3.3.2

Source: 	Qualcomm

Title: 	Draft text proposal to 38.214 subclause 5.3 on UE PDSCH processing time in the case of short PDSCH type A

Document for:	Discussion/Decision

# Introduction

This document provides a draft text proposal fixing issues in the subclause 5.3 UE PDSCH processing procedure time and of TS38.214 based on the meeting week’s discussion, focusing on the following working assumption.

Working assumption:

- For Capability #1 of UE processing time, the following adjustments to N1 are made:

  - If the last symbol of the PDSCH Type A allocation ends d symbols before the 7th symbol in the slot, then N1 is increased by d.

For clarity, the incremental change relative to another draft text proposal is given below.

## 5.3	UE PDSCH processing procedure time

If the first uplink symbol to carry the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after Tproc,1=N1+d1,1+d1,22048+144κ2-μDL∙TC after the last symbol of the PDSCH carrying the TB being acknowledged.

-	N1 is based on µ of tables 5.3-1 or 5.3-2 depending on the UE capability, that corresponds to the one of {µDL, µUL} with the larger N1, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted

-	If HARQ-ACK is transmitted on PUCCH, then d1=0,

-	If HARQ-ACK is transmitted on PUSCH, then d1=1.

-	If the PDSCH is mapping type A as given in [38.211, subclause 7.4.1.1], and the last symbol of PDSCH is on the ith symbol of the slot where i<7, then d1,2=7-i.

-	Otherwise d1,2=0

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH. The value of Tproc,1 is used both in the case of normal cyclic prefix and extended cyclic prefix.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | No additional PDSCH DM-RS configured | Additional PDSCH DM-RS configured |
| 0 | 8 | 13 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | No additional PDSCH DM-RS configured | Additional PDSCH DM-RS configured |
| 0 | [2.5 - 4] | [12] |
| 1 | [2.5 - 6] | [12] |
| 2 | N/A | N/A |
| 3 | N/A | N/A |