3GPP TSG RAN WG1 Meeting #92	R1-1803504

Athens, Greece, February 26th March 2nd, 2018

Agenda item:		7.1.3.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Draft time domain resource allocation proposal

Document for:		Discussion and Decision

# 1	Introduction

This contribution suggest basis for further discussion for time domain resource allocation.

# 2	Time domain resource allocation

Possible offline consensus

- Define separate fixed time domain resource allocation values for each RMSI CORESET multiplexing pattern 1/2/3 for PDSCH scheduled by 0/0a/2 CSS

  - Total of 3 tables, one per pattern

- Define at least one set of fixed time domain resource allocation values for other PDSCH scheduled by 1/3 CSS or USS, until RRC-configured table is received

- Define at least one set of fixed time domain resource allocation values for PUSCH until RRC-configured table is received

- FFS: Request RAN2 to introduce possibility for providing the RRC-confiugred table in SIB1

Agreements:

- Request RAN2 to introduce possibility for providing the RRC-configured table in RMSI to configure PDSCH and PUSCH symbol allocation for PDSCH/PUSCH scheduling after RMSI, where the RRC-configurable table via dedicated signaling was previously agreed in RAN1

  - Draft LS in R1-1803474 (Karri, Nokia)

Possible tables

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 1

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type A | 0 | 2 | 12 |
| 1 | Type A | 0 | 2 | 10 |
| 2 | Type A | 0 | 2 | 9 |
| 3 | Type A | 0 | 2 | 8 |
| 4 | Type B | 0 | 4 | 7 |
| 5 | Type B | 0 | 4 | 4 |
| 6 | Type B | 0 | 9 | 4 |
| 7 | Type A | 0 | 3 | 11 |
| 8 | Type A | 0 | 3 | 9 |
| 9 | Type A | 0 | 3 | 8 |
| 10 | Type A | 0 | 3 | 7 |
| 11 | Type B | 0 | 3 | 4 |
| 12 | Type B | 0 | 10 | 4 |
| 13 | Type B | 0 | 5 | 2 |
| 14 | Type B | 0 | 9 | 2 |
| 15 | Type B | 0 | 12 | 2 |

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 2

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 2 | 2 |
| 1 | Type B | 0 | 4 | 2 |
| 2 | Type B | 0 | 6 | 2 |
| 3 | Type B | 0 | 8 | 2 |
| 4 | Type B | 0 | 10 | 2 |
| 5 | Type B | 1 | 2 | 2 |
| 6 | Type B | 1 | 4 | 2 |

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 3

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 4 | 2 |
| 1 | Type B | 0 | 6 | 2 |
| 2 | Type B | 0 | 8 | 2 |
| 3 | Type B | 0 | 10 | 2 |

Table 5.1.2.1-x: Default resource allocation for PDSCH scheduled using other than CORESET#0

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type A | 0 | 0 | 14 |
| 1 | Type A | 0 | 0 | 12 |
| 2 | Type A | 0 | 0 | 11 |
| 3 | Type A | 0 | 0 | 10 |
| 4 | Type A | 0 | 2 | 12 |
| 5 | Type A | 0 | 2 | 10 |
| 6 | Type A | 0 | 2 | 9 |
| 7 | Type A | 0 | 2 | 8 |
| 8* | Type A | 0 | 0 | 14 |
| 9* | Type A | 0 | 0 | 12 |
| 10* | Type A | 0 | 0 | 11 |
| 11* | Type A | 0 | 0 | 10 |
| 12 | Type A | 0 | 3 | 11 |
| 13 | Type A | 0 | 3 | 9 |
| 14 | Type A | 0 | 3 | 8 |
| 15 | Type A | 0 | 3 | 7 |
| 8** | Type B | 0 | 4 | 7 |
| 9** | Type B | 0 | 5 | 2 |
| 10** | Type B | 0 | 9 | 2 |
| 11** | Type B | 0 | 12 | 2 |

* Applicable only if scheduled with a CORESET residing within the first two symbols in the slot

**not applicable if CORESET not in the first two symbols of the slot

Table 6.1.2.1-x: Default resource allocation for PUSCH

| i | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type A | j | 0 | 14 |
| 1 | Type A | j | 0 | 12 |
| 2 | Type A | j | 0 | 10 |
| 3 | Type B | j | 2 | 12 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 2 | 8 |
| 6 | Type B | j | 4 | 10 |
| 7 | Type B | j | 4 | 8 |
| 8 | Type B | j | 4 | 6 |
| 9 | Type A | j+1 | 0 | 14 |
| 10 | Type A | j+1 | 0 | 12 |
| 11 | Type A | j+1 | 0 | 10 |
| 12 | Type A | j+2 | 0 | 14 |
| 13 | Type A | j+2 | 0 | 12 |
| 14 | Type A | j+2 | 0 | 10 |

Table 6.1.2.1-x: Definition of value j in determination for K2

| µ | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |