3GPP TSG-RAN WG1 Meeting AH 1801	R1-1803533

February 26th – March 2nd  2018

Athens, Greece

Agenda item:	7.1.3.3.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Simultaneous PDSCH Reception

Document for:	Discussion/Decision

# Simultaneous PDSCH Reception

It was raised in [2] that according to TS 38.202, UE in RRC_CONNECTED can receive at most 4 PDSCHs simultaneously. See below illustration. These 4 PDSCHs are indicated by PDCCHs with CRCs scrambled by C-RNTI, P-RNTI, SI-RNTI, and RA-RNTI. This means UE needs to reserve hardware capability for 4 simultaneous PDSCHs to take care of the worst case.

Figure 4-1. Multiple PDSCHs received simultaneously by UE (from [2])

Both [2],[13] note that in LTE TS 36.213, the following statement exists to reduce cost on UE implementation: When RA-RNTI and either C-RNTI or SPS C-RNTI are assigned in the same subframe, the UE is not required to decode a PDSCH on the primary cell indicated by a PDCCH/EPDCCH with a CRC scrambled by C-RNTI or SPS C-RNTI. Therefore [2],[13] proposes to keep the same UE behaviours in NR.

Additionally, [13] goes further to indicate that similar behaviour should be extended to P-RNTI and SI-RNTI. Reasons for extending beyond LTE’s restriction are due the flexible scheduling and DMRS patterns in NR (particularly between broadcast and unicast), as well as the notion that this limitation will anyway manifest in multi-beam systems where it may not be able to ensure same simultaneous transmission/reception across the broadcast and unicast beams. Therefore, we extend this UE behaviour limitation to P-RNTI and SI-RNTI. Note that the overall impact to the system is expected to be minimal due to the infrequency of paging occasions and system information updates.

Offline discussion on simultaneous Downlink PDSCH

Table 4-1: Downlink PDSCH Combinations

| State | RNTI that can be monitored on PCell |
| --- | --- |
| IDLE | Broadcast PDSCH: - SI-RNTI, P-RNTI, RA-RNTI, TC-RNTI |
| RRC_INACTIVE | Broadcast PDSCH: - SI-RNTI, P-RNTI, RA-RNTI, TC-RNTI |
| RRC_CONNECTED | Broadcast PDSCH: - SI-RNTI, P-RNTI, RA-RNTI, TC-RNTI Unicast PDSCH: - C-RNTI, CS-RNTI |

The following represents the status of the proposals as of last night.

Working assumption:

- For broadcast PDSCH, MCS is limited to QPSK, rank is limited to 1.

Agreements:

- The UE is not expected to receive both C-RNTI PDSCH and CS-RNTI PDSCH from the primary cell if they are overlapped with at least one symbol.

- If both RA-RNTI PDSCH and C-RNTI (or CS-RNTI) PDSCH for a given UE from the primary cell are overlapped with at least one symbol, the UE may skip decoding C-RNTI (or CS-RNTI) PDSCH.

For context, the following was reference agreement

Agreements (RAN1 #91):

- NR supports sending of short paging messages e.g. systemInfoModification, cmas-Indication, and etws-Indication if supported in NR, in the Paging DCI.

Working assumption:

- P-RNTI PDSCH is not required to be decoded by UE in RRC_CONNECTED mode

  - Note: P-RNTI PDCCH may not schedule a P-RNTI PDSCH but rather carries the message by itself, which may be used for e.g. systemInfoModification, cmas-Indication, and etws-Indication

  - Send an LS to RAN2 asking for feedback (R1-1803508)

Proposals 4.2: (working assumption)

- While UE acquires SI upon being triggered by Paging DCI

  - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol

- In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH

- The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2216 for FR1, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH

- The first two bullets always apply in FR2

- Send an LS to RAN2 asking for feedback

# References

- R1-1801540	Remaining issues on HARQ management	vivo

- R1-1801663	Remaining issues of DLUL scheduling and HARQ management	MediaTek Inc.

- R1-1801736	On NR HARQ timing and management	CATT

- R1-1801827	Fallback operation for semi-static HARQ-ACK codebook determination	Lenovo, Motorola Mobility

- R1-1801983	Corrections on HARQ Management and Feedback	Samsung

- R1-1802016	Remaining issues on HARQ-ACK codebook	Spreadtrum Communications

- R1-1802044	Discussion on HARQ timing for DL SPS and multi-slots scheduling	CMCC

- R1-1802214	Remaining issues on DL/UL scheduling and HARQ	LG Electronics

- R1-1802415	Remaining details on UE processing times and HARQ operation	Intel Corporation

- R1-1802487	DL/UL scheduling and HARQ management	NTT DOCOMO, INC.

- R1-1802600	Nested HARQ bundling and Multiplexing For Peak Data rates in NR	AT&T

- R1-1802697	Remaining issues on HARQ management	Huawei, HiSilicon

- R1-1802842	Remaining Issues on DL/UL Scheduling, Processing Time and HARQ management	Qualcomm Incorporated

- R1-1802911	On DL/UL scheduling and HARQ management	Ericsson

- RAN1 AH 1801 Chairman’s Notes

- R1-1802415	Remaining details on UE processing times and HARQ operation	Intel Corporation

# Appendix of Proposals

The following provides a summary of the proposals concepts though text was not included for brevity.

## VIVO

Proposal 1:

- 3bits HARQ-Timing indicator field in DCI format 1_1 is removed if the 8 possible timing offset values configured by higher layer parameter DL-data-DL-acknowledgement are same.

Proposal 2:

- For dynamic scheduled PDSCH using DCI format 1_1 without 0bit HARQ-ACK timing indicator field, or DL SPS, UE upon detection of PDSCH in slot n, shall transmit the HARQ-ACK at the first available slot starting from slot n+k,

  - Where k is

    - Configured by higher layer parameter DL-data-DL-acknowledgement, for dynamic scheduled PDSCH, or

    - Indicated by the activation DCI for the DL SPS process

  - Where the available slot is defined such that

    - The slot at least contains unknown or UL symbols given by semi-static TDD configuration

    - The determined PUCCH resource for HARQ-ACK transmission is within the indicated unknown or UL symbols given by semi-static TDD configuration (if configured), and these symbols are not indicated as DL symbols by dynamic SFI (if configured).

## MediaTek, Inc.

Proposal #1: For aggressive N2 operation, if FDMing DMRS and PUSCH is allowed, one additional OFDM symbol is added to N2.

Proposal #2: For aggressive N1, remove the condition of “Front-loaded + additional DMRS” from the table

Proposal #3: To adopt the below Table 2 for aggressive UE processing time in NR Release 15 for slot-based scheduling, including CA case, with single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing

Table 2. UE Processing Time and HARQ Timing (Capability #2)

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | 4 | 5 |
| Frequency-first RE-mapping | N2 | Symbols | 5 | 6 |

- If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table.

Proposal #4: When reporting the support of aggressive UE capability, UE can report the support of N1 and support of N2 separately. In addition, one additional parameter can be carried to indicate the support of 0%, 50% or 100% of max PRB number for DL and UL, separately.

Proposal #5: When reporting the support of aggressive UE capability, one additional parameter can be carried to indicate the support of either 1, 2, or 4CC.

Proposal #6: Clarify that“PDCCH condition Case 1-1 & PUSCH type B” belongs to slot-based scheduling

Proposal #7: In case of CA operation, the extra UE processing time shall be introduced based on the max timing difference among CCs defined in RAN4 spec

- Note : The value may be SCS dependent, depending on RAN4’s discussion

Proposal #8: For cases of PUSCH and PUSCH overlapping in time domain

- “Dynamic grant” vs “configured grant”

  - The dynamic grant overrides the configured grant

  - Note : It was already agreed in RAN2

- “Dynamic grant” vs “dynamic grant”

  - This is error case at least in Rel-15. UE’s behavior is not specified.

- “Configured grant” vs “configured grant”

  - This is error case. UE’s behavior is not specified.

Proposal #9:

- When PDSCH indicated by RA-RNTI and PDSCH indicated by either C-RNTI or C-RNTI(s) for SPS/grant-free are assigned in common OFDM symbol(s), the UE is not required to decode a PDSCH indicated by a PDCCH with a CRC scrambled by C-RNTI or C-RNTI(s) for SPS/grant-free.

- When two PDSCHs indicated by either C-RNTI or C-RNTI(s) for SPS/grant-free are assigned in common OFDM symbol(s), the UE is not required to decode the PDSCH with lower priority.

  - When two PDSCHs have different starting OFDM symbols, the one with later starting symbol has lower priority

  - When two PDSCHs have the same starting OFDM symbol, the one with smaller TBS has lower priority

## CATT

Proposal 1：Semi-static codebook should be determined based on either a union of the sets of K1 values for DCI formats 1_0 and 1_1 or the applicable K1 values when scheduled by DCI format 1_0 are limited to the subset of {1, …, 8} that are configured for DCI format 1_1.

Proposal 2: The HARQ-ACK timing indicator field in DCI format 1_1 is not present if all entries in the set of configured HARQ-ACK timing values are the same.

Proposal 3: There should be no conflict of SPS PDSCH occasion and the corresponding HARQ-ACK with either the semi-static DL/UL configuration or the SFI indication.

Proposal 4: 1-bit SPS HARQ-ACK is added into the dynamic HARQ-ACK codebook in slot n when SPS PDSCH transmission is activated for a UE and the UE is configured to receive SPS PDSCH in a slot n-K1sps, where K1sps is the HARQ-ACK timing indicated in the SPS activation PDCCH.

## Lenovo, Motorola Mobility

Proposal 1: Confirm the following working assumption:

- In case a UE is configured for semi-static HARQ-ACK codebook determination, when the UE detects to receive only one PDSCH on the PCell, the UE reports HARQ-ACK only for the one PDSCH

  - FFS whether additional constraints are necessary for the above operation

Proposal 2: DCI indicates whether to use fallback HARQ-ACK transmission.

Proposal 3: Current two bits in DL assignment for PUCCH resource indication is reused for fallback PUCCH resource indication in case of fallback HARQ-ACK transmission.

Proposal 4: Fallback HARQ-ACK codebook size is set to 1 or 2 depending on RRC configured single-codeword transmission or two-codeword transmission.

## Samsung

Proposal 1: For both slot-based and non-slot-based scheduling, UE processing capability #2 uses the following values.

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | 3 | 5 |
| Front-loaded + additional DMRS | N1 | Symbols | 12 | 12 |
| Frequency-first RE-mapping | N21 | Symbols | 5.5 | 5.5 |

- If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table.

Proposal 2: NR Release-15 UE should support 16 maximum number of HARQ processes.

Proposal 3: In case of 0-bit HARQ-ACK timing for DCI format 1_1, UE shall transmit HARQ-ACK in slot n+k, where k is indicated by higher-layer parameter DL-data-DL-acknowledgement.

Proposal 4: HARQ-ACK resource for DL SPS PDSCH is determined by PUCCH resource indicator field in the activation DCI.

Proposal 5: When HARQ-ACK feedback in slot n+k is conflicted with semi-static DL/UL configuration or dynamic SFI, UE shall cancel HARQ-ACK transmission.

## Spreadtrum

Proposal 1: NR DL SPS activating DCI allocates the time domain resource for DL SPS PDSCHs, DL SPS PDSCHs use the same time domain resource as the DL SPS activating DCI allocates.

Proposal 2: Limit the agreements “Place HARQ-ACK bit(s) in response to SPS PDSCH after ones in response to dynamic PDSCH in the HARQ-ACK codebook” to dynamic HARQ-ACK codebook.

Proposal 3: For dynamic HARQ-ACK codebook determination, first sub-codebook should only contain cell with higher layer parameter CBG-DL = OFF.

## CMCC

Proposal 1: When UL direction implied by DL SPS HARQ-ACK is conflict with the DL direction implied by semi-static DL/UL configuration or DL/unknown indicated by dynamic SFI, the UL direction implied by DL SPS HARQ-ACK is with lower priority.

Proposal 2: Configure multiple HARQ-ACK resources for a DL SPS transmission by RRC signalling, and UE use the first valid HARQ-ACK resource for DL SPS ACK/NACK transmission, where the valid HARQ-ACK resource means the HARQ-ACK resource is not earlier than n+k and not indicated as “DL” by semi-static DL/UL configuration or as DL/unknown by dynamic SFI.

Proposal 3: The PUCCH frequency resource assignment for DL SPS ACK/NACK can be configured in a periodic manner.

## LG

Proposal 1: The UE processing time between PDSCH with SCS u1 and PUCCH with SCS u2 (or between PDCCH with u1 and PUSCH with u2) should be defined as max(T1, T2) based on the agreement in RAN1#91, where T1 or T2 is the absolute time of N1 in case with single SCS u1 or single SCS u2 for both PDSCH and PUCCH (or for both PDCCH and PUSCH).

Proposal 2: Default maximum number of DL HARQ processes before RRC configuration is predefined to 2 which is the minimum among the configurable values for DL HARQ process number.

Proposal 3: Default set of K1 timing values {1, 2, 3, 4, 5, 6, 7, 8} is applied only for the PDSCH transmitted in Pcell scheduled by fallback DCI while the set of K1 timing values is configurable for the PDSCH transmitted in Scell scheduled by fallback DCI.

Proposal 4: Following two cases of cross-CC scheduling between the CCs with different numerology are to be supported with consideration of UE processing complexity.

- Case 1: Cell 1 with smaller SCS (or longer slot) schedules Cell 2 with larger SCS (or shorter slot) where the maximum number of DL/UL scheduling DCIs for a same UE detected via single CORESET or search space is limited.

- Case 2: Cell 1 with larger SCS (or shorter slot) schedules Cell 2 with smaller SCS (or longer slot) where the slot location allowable for the DCI transmission (relative to the corresponding DL data transmission) is restricted.

## Intel

Proposal 1:

  - Aggressive UE processing times (Capability 2) are specified for non-slot-based scheduling for SCS values of 15kHz and 30kHz.

  - The same numbers for slot-based scheduling are adopted when the front-loaded DMRS is in the first symbol of the PDSCH/PUSCH.

Proposal 2:

- For both baseline (Capability 1) and aggressive (Capability 2) UE processing times, for PDSCH mapping type B with 2- or 4-symbol duration, the minimum UE processing time, N1, is increased by d symbols if the scheduling PDCCH ends d symbols after first PDSCH symbol.

Proposal 2A:

- Adopt the following values as the UE minimum processing times for Capability 2:

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | 4 | 4 |
| Front-loaded + additional DMRS | N1 | Symbols | 12 | 12 |
| Frequency-first RE-mapping | N2 | Symbols | 5 | 5 |

Proposal 2B:

- Adopt the respective text proposals in Section 2 of this document to Subclauses 5.3 and 6.4 of TS38.214.

Proposal 3:

- The UE is not expected to receive anything on a symbol if it is within T us from an UL transmission by that UE on unpaired spectrum for a given serving cell.

Proposal 4:

- For any two HARQ process IDs A and B, if scheduled unicast PDSCH transmission for A comes before the scheduled unicast PDSCH transmission for B then the UE is not expected to send the HARQ-ACK for A after the HARQ-ACK for B.

Proposal 5:

- For HARQ-ACK feedback in response to DL SPS:

  - In cases of conflicts with semi-static UL-DL configuration in slot n+k,

    - For HARQ-ACK feedback in response to first PDSCH triggered by the activation DCI,

      - HARQ-ACK transmission is postponed to the next available slot that has no conflicts with semi-static configuration in the symbol(s) originally intended for the HARQ-ACK transmission.

    - For HARQ-ACK in response to subsequent PDSCHs,

      - HARQ-ACK transmission is postponed to the next available slot that has no conflicts with semi-static configuration or with dynamic SFI in the symbol(s) originally intended for the HARQ-ACK transmission.

  - In case of conflicts with dynamic SFI in slot n+k,

    - For HARQ-ACK feedback in response to first PDSCH triggered by the activation DCI,

      - UE behavior is unspecified as the UE is not expected to find such conflicts.

    - For HARQ-ACK in response to subsequent PDSCHs,

HARQ-ACK transmission is postponed to the next available slot that has no conflicts with semi-static configuration or with dynamic SFI in the symbol(s) originally intended for the HARQ-ACK transmission.

## NTT DOCOMO

Proposal 1:

- For Type-1 HARQ-ACK codebook determination, if the UE is capable of receiving up to one PDSCH per slot, the number of HARQ-ACK bit over the PDSCH candidates in a slot is one.

## AT&T

Proposal:  Enhanced HARQ multiplexing and bundling techniques should be supported to reduce the impact of peak throughput when HARQ-ACK is repeated over multiple slots

## Huawei, HiSilicon

In this contribution, we show our views on remaining HARQ issues. On HARQ processing time, the following proposals are made:

Proposal 2-1: Take the following change on the processing capability 2.

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | No additional PDSCH DM-RS configured | Additional PDSCH DM-RS configured |
| 0 | [2.5-4]3 | [12]13 |
| 1 | [2.5-6]5 | [12]13 |

Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | [2.5-4]5 |
| 1 | [2.5-6]6 |

Proposal 2-2: UE processing capability (N1, N2) for slot-based scheduling and non-slot based scheduling are reported separately.

Proposal 2-3: For retransmission, gNB should assume the processing time to be the maximum values in absolute time among all conditions.

Proposal 2-4: For any two continuous unicast PDSCH transmissions from one cell to the same UE, where the order in time of expected HARQ feedbacks for both PDSCHs is different from the order in time of the two PDSCHs, the UE may not provide an valid HARQ-ACK for the earlier PDSCH if without leaving sufficient time for processing both PDSCH transmissions.

On HARQ codebook determination, the following proposals are made:

Proposal 3-1: A UE should determine a number of PDCCH monitoring occasion(s) for PDCCH with DCI format 1_0 or DCI format 1_1 based on a number of possible slot timing values provided to a UE by higher layer parameter for every configured BWP.

Proposal 3-2: A UE determines the PDCCH monitoring occasion(s), which is used to determine semi-static HARQ codebook, based on the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern for each search space in each configured CORESET within a slot.

Proposal 3-3: For type-1 HARQ-ACK codebook determination, if SPS PDSCH transmission is activated for a UE and the UE is configured to receive SPS PDSCH in a slot within the monitoring occasions, an additional bit is attached to HARQ-ACK codebook for HARQ-ACK bit associated with the SPS PDSCH reception.

Proposal 3-4: Jointly use C-DAI, T-DAI and UL-DAI to determine the dynamic HARQ-ACK codebook for HARQ-ACK transmission on PUSCH.

## Qualcomm

Proposal 1: Capture agreement that fallback DCI (i.e., DCI Format 1_0) HARQ-ACK timing values are fixed in specification as given by the text proposal in Appendix 7.1.

Proposal 2: When RRC configures the same value k of HARQ-ACK timing for all possible 3-bit values which are allowed in the DCI Format 1_1 field, then 0 bits are actually allocated for this field in the DCI. In this case, when the UE receives a PDSCH on slot n, the UE is expected to send the corresponding HARQ-ACK on slot n+k. (See Appendix 7.2 for Text Proposal.)

Proposal 3: The following UE behavior should be captured in TS38.213

- In the case of DL SPS PDSCH, in the case when the HARQ-ACK timing indicator field value conflicts with the semi-static slot configuration at slot n+k, the UE is not expected to send the HARQ-ACK.

- In the case of dynamic SFI, the HARQ-ACK for the DL SPS PDSCH is taken to be of higher priority and the UE is expected to still send the HARQ-ACK when there is conflict.

Proposal 4: The UE processing times in Table 3.1-1 in R1-1801124 apply to the case of “slot-based scheduling”, i.e., corresponding to the following conditions

  - Single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing, and Non-CA case

    - FFS: CA case

  - Case 1-1: PDCCH monitoring periodicity of 14 or more symbols

    - PDCCH monitoring on up to three OFDM symbols at the beginning of a slot

  - PDSCH Type A, and PUSCH Type A or B

    - PDSCH duration of at least 7 symbols

    - For C-RNTI only

    - FFS also applicable to the cases when C-RNTI and with other broadcast RNTIs are processed simultaneously by the UE

  - FFS if UE can indicate support for this processing time only when the number of scheduled RBs is less than some fraction X of the active BWP.

Table 3.1-1 UE Processing Time (Capability #2) for Slot-Based Scheduling

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | 2.5 | 2.5 |
| Front-loaded + additional DMRS | N1 | Symbols | 12 | 12 |
| Frequency-first RE-mapping | N21 | Symbols | 2.5 | 2.5 |

- If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table.

Proposal 5: The aggressive UE processing time capability (Capability #2) in NR Release 15 should consider possibility of using smaller processing times than those given by Table 3.1-1 at least when operating under the following conditions

- Single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing

- Case 2: PDCCH monitoring periodicity of less than 14 symbols

  - First symbol of PDSCH assumed to be no earlier than the first symbol of PDCCH scheduling this PDSCH

- PDSCH Type B and PUSCH type B time-domain allocation

  - FFS: increase N1 by d symbols if front-loaded DMRS comes d symbols after first PDSCH symbol

- Note: if PDSCH Type A with less than 7 symbols is supported, the capability needs to be further discussed

- One unicast PDSCH received and/or one unicast PUSCH transmitted within the same slot

  - FFS: More than one PDSCH and/or PUSCH case

- Non-CA (FFS: CA)

- For C-RNTI only

Proposal 6: For CA with cross-carrier scheduling, if the carrier with PDCCH is delayed by is symbols (which may be fractional value) relative to the PDSCH carrier, then the processing time requirement for the UE is increased to N1+ .

Proposal 7: The MAC CE latency, defined as the time between the ACK transmission for the PDSCH carrying the MAC-CE message and the time that the UE applies the MAC-CE message, should be uniformly [2 or 2.5] ms across all cases.

Proposal 8: For the baseline UE capability, the UE is not expected to handle more than 1 uplink unicast data grant and 1 downlink unicast data grant decoded within the CORESETs located in the first 2 or 3 symbols of a slot.

Proposal 9: The following UE behaviour should be captured in TS38.213 and TS38.214.

- For 38.213

  - When the UE decodes PDCCH associated with RA-RNTI on a given slot, the UE is not required to decode any PDCCH for any other RNTI on that slot

- For 38.214

  - When the UE decodes PDSCH associated with RA-RNTI on a given slot, the UE is not required to decode any PDSCH for any other RNTI on that slot

  - When the UE decodes PDSCH associated with SI-RNTI on a given slot, the UE is not required to decode any PDSCH for any other RNTI on that slot

  - When the UE decodes PDSCH associated with P-RNTI on a given slot, the UE is not required to decode any PDSCH for any other RNTI on that slot

Proposal 10: For any two HARQ process IDs A and B, if scheduled unicast PDSCH transmission for A comes before the scheduled unicast PDSCH transmission for B then the UE is not expected to send the HARQ-ACK for A after the HARQ-ACK for B at least under the following conditions

- The PDSCH transmissions for both HARQ processes are scheduled from DCI monitored within the first 3 symbols of the slot and with a 14-symbol PDCCH monitoring periodicity (Case 1-1)

Proposal 11: For each HARQ process ID, the UE is not expected to receive a scheduling DCI (grant) for a unicast PUSCH transmission with the same HARQ process ID until

- The time after the end of the expected transmission of the PUSCH for an earlier transmission on the same HARQ process ID

Proposal 12: For any two HARQ process IDs A and B, if the scheduling DCI for unicast PUSCH transmission A comes before the scheduling DCI for unicast PUSCH transmission B, then the UE is not expected to send the PUSCH for A after the PUSCH for B at least under the following conditions

- The PUSCH transmissions for both HARQ processes are scheduled from DCI monitored within the first 3 symbols of the slot and with a 14-symbol PDCCH monitoring periodicity (Case 1-1)

## Ericsson

- For processing capability #2 adopt a processing requirement according to the table below (or lower)

Table 2.2-1 Proposed Range of UE Processing Time (Capability #2) from RAN1 #90bis

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | 2.5 | 2.5 |
| Front-loaded + additional DMRS | N1 | Symbols | 12 | 12 |
| Frequency-first RE-mapping | N21 | Symbols | 2.5 | 2.5 |

- Clarify when the different processing capabilities applies according to text proposals

- Clarify that the formula calculating the number of symbols between PDSCH and HARQ-ACK and UL grant and PUSCH does not take into account varying CP length between different symbols

- Discuss the processing requirement for SPS release

- Discuss whether there is need to set a limit on the number of the number of PDSCH/PUSCH that can be TDM within a slot, if a limit is introduced an appropriate one is around 8 PDSCH/PUSCHs

- Introduce a capability that indicates that the UE supports simultaneous reception of two PDSCHs on the same carrier scrambled with C-RNTI/SPS C-RNTI

  - Note: The PDSCHs are FDM:ed in frequency

- If the UE is scheduled with multiple PUSCHs that are FDM the PUSCH that corresponds to the latest received PDCCH is the valid PUSCH. The UE should hence drop the other PUSCH transmissions.

- If the UE is scheduled with multiple PDSCHs that are FDM and the UE does not have processing capability to support that, the UE should process the PDSCH that correspond to the latest received PDCCH

- The default values for DL-data-DL-acknowledgement are as follows (To be indicated to RAN2):

| PDSCH-to-HARQ_feedback timing indicator | Number of slots |
| --- | --- |
| '000' | 0 |
| '001' | 1 |
| '010' | 2 |
| '011' | 3 |
| '100' | 4 |
| '101' | 5 |
| '110' | 6 |
| '111' | 7 |

- The default value for the number of DL HARQ processes is 8