3GPP TSG RAN WG1 Meeting #92			R1-1801311

Athens, Greece, February 26th – March 2nd, 2018

3GPP TSG-RAN WG4 Meeting #85                                                                     R4-1714180

Reno, US, 27 Nov - 1 Dec 2017

Title:	LS reply on resource selection for Mode-4 sidelink CA

Reply to:	R1-1719159

Release:	Rel-15

Study Item:

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name:	Ye Liu

E-mail Address:	leo.liuye@huawei.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on resource selection for Mode-4 sidelink CA. After discussion in RAN4, RAN4 would like to provide following technical information to RAN1 questions for further study.

- Question 1: Switching time for intra-band and inter-band due to TX switching and interruption time at the receiver

  - Intra-band PC5 CA

    - If all the TX carriers are configured and activated simultaneously, then switching between two TX carriers requires no additional time and no interruption at RX.

    - If only part of TX carriers configured and activated simultaneously (e.g. UE supports less number of TX carriers than RX carriers), then TX RF LO needs to be retuned to support transmission at other carriers and up to 200us TX RF retuning time is needed. RX chain interruption time depends on UE implementation:

      - Option 1: In case of separate TX/RX chains architecture for each carrier, the RX chain operation may not be interrupted due to TX RF retuning

      - Option 2: In case of shared TX/RX chains architecture for carriers, the RX chain operation may be interrupted for up to 200us

  - Inter-band PC5 CA:

    - Since there is only one band specified for PC5 which is Band 47 so inter-band PC5 CA is not available from RAN4 point of view. However, the switching time for inter-band CA depends on UE implementation and can take 0us, 30us, 100us, 200us, 300us, 500us, 900us.  Inter-band interruption time needs more discussion.

- Question 2: Feasibility of simultaneous transmission on intra-band, non-contiguous carriers. RAN1 requests feedback of impact of MPR and maximum PSD imbalance between carriers.

  - Except intra-band contiguous multi-carrier scenario of 10MHz+10MHz in Rel-14, only intra-band contiguous multi-carrier scenario of 10MHz+20MHz for TX is introduced in Rel-15, so there is no intra-band non-contiguous scenario till now in RAN4. That is because very large MPR is expected if the PA is shared between non-contiguous carriers based on analysis results in legacy intra-band non-contiguous CA in LTE..

2. Actions:

To RAN WG1

ACTION: RAN4 would like RAN1 to take above information into consideration for further study.

3. Date of Next TSG-RAN WG4 Meetings for NR:

TSG-RAN WG4 Meeting #86	26 Feb– 3 Mar Nov 2018	                            Athens, GR

TSG-RAN WG4 Meeting #86bis	16 - 20 Apr 2018	                       Melbourne, AU