3GPP TSG RAN WG1 Meeting #92																	R1-1803349

Athens, Greece, February 26th – March 2nd, 2018

Title:	LS on Power Transition and Uplink Channel and Reference Signal Combinations

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Intel, RAN1

To:	RAN4

Contact Person:

Name: 		Yushu Zhang

E-mail: 		yushu dot zhang at intel dot com

# Overall Description

RAN1 would like to inform RAN4 that RAN1 has identified the following combinations of the physical channels and the reference signals that can be transmitted by the UE on different OFDM symbols in the same slot with different transmit power:

- PUSCH+PUSCH

- SRS+SRS

- PUCCH+PUCCH

- PUSCH+SRS

- PUSCH+PUCCH

- PUSCH+PRACH

- SRS+PUSCH

- SRS+PUCCH

- SRS+PRACH

- PUCCH+PUSCH

- PUCCH+SRS

- PUCCH+PRACH

- PUSCH+PUCCH+PUCCH

- PUSCH+SRS+PUCCH

RAN1 would like to inform RAN4 that RAN1 has identified the following combinations of the physical channels and the reference signals that can be transmitted by the UE on two consecutive slots with different transmit power and the PUSCH DMRS occupies the first OFDM symbol:

- PUSCH+PUSCH

- PUCCH+PUSCH

- SRS+PUSCH

RAN1 would like to inform RAN4 that for each combination different physical channels or reference signals may have different durations in the same slot.

- PUSCH: 2/4/5/6/7/8/9/10/11/12/13/14 symbols

- SRS: 1/2/3/4/5/6 symbols

- PUCCH: 1/2/4/5/6/7/8/9/10/11/12/13/14 symbols

- PRACH: 2/4/6/12 symbols

The transmission priority may be different depending on the channel, content that physical channel is carrying and reference signal. The list above might not be completed.

2. Actions:

To: RAN WG4

ACTION: RAN1 respectfully requests RAN4 to consider the above information for their future work.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN WG1 Meeting #92Bis	16 Apr – 20 Apr 2018	Sanya, China

TSG-RAN WG1 Meeting #93 	21 May – 25 May 2018	Busan, S. Korea