3GPP TSG RAN WG1 Meeting #92			R1-1803350

Athens, Greece, February 26th – March 2nd, 2018

Title:	Reply LS on Clarifications in MAC (DELTA_PREAMBLE)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Contact Person:

#### Name:	Ravikiran Nory

E-mail Address:	ravikiran.nory@ericsson.com

1. Overall Description:

Regarding the question related to values for DELTA_PREAMBLE in the LS [R1-1801318], RAN1 requests RAN2 to take note of the below agreements made in RAN1 NR AH1801

Agreements:

- For long sequence preamble formats, the parameter DELTA_PREAMBLE is given by the following table:

| Preamble Format | DELTA_PREAMBLE values |
| --- | --- |
| 0 | 0 dB |
| 1 | -3 dB |
| 2 | -6 dB |
| 3 | 0 dB |

Agreements:

DELTA_PREAMBLE for short sequence preambles is given by:

| A1 | 8 + 3∙μ dB |
| --- | --- |
| A2 | 5 + 3∙μ dB |
| A3 | 3 + 3∙μ dB |
| B1 | 8 + 3∙μ dB |
| B2 | 5 + 3∙μ dB |
| B3 | 3 + 3∙μ dB |
| B4 | 3∙μ dB |
| C0 | 11 + 3∙μ dB |
| C2 | 5 + 3∙μ dB |

Where μ is the sub-carrier spacing configuration.

2. Actions:

ACTIONS:	RAN1 asks RAN2 to take the above agreements into account in their future work.

3. Date of Next RAN2 Meetings:

RAN1#92-Bis	16th – 20th April 2018	Sanya, China

RAN1#93	21st – 25 May 2018	Busan, Korea