3GPP TSG RAN WG1 Meeting #92bis		R1-1803733

Sanya, China, April 16th – 20th, 2018

Source:	CATT

Title:	Further details on NR 4-step RA Procedure

Agenda Item:	7.1.1.4.2

Document for:	Discussion and Decision

# Introduction

The following agreements were made on PRACH procedure in RAN1#92 [1]:

Agreements: - If a received Msg2 does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after the duration of N1 + new + L2. - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - new>=0, FFS - L2=500 usec refers to the MAC layer processing time Agreement - For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is - N2+BWPSwitching+ Delay, - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4 - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - Delay includes at least MAC layer delay in initializing PRACH - Value of Delay is FFS (to be decided in RAN1) Agreements: - In RA-RNTI equation (RA-RNTI= 1 + s_id + 14*t_id + 14*X*f_id + 14*X*Y*ul_carrier_id)), - Value of X and Y are 80 and 8 respectively. - LS to RAN2 regarding the above decision – Asbjorn (Ericsson) R1-1803335 [Draft] Input values for calculating the RA-RNTI Ericsson Decision: The document is endorsed, and final LS is approved in R1-1803372. Working assumption: - Time period for SSB to RO association is multiple of PRACH configuration periods - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4} such that the number is ≥Actually transmitted SSBsSSBs that can be mapped to one PRACH config period - The time period starts from frame 0 Agreements: - PDCCH order can be transmitted in both common and UE specific search space. - Note: Control session will decide which common search space UE should monitor to receive PDCCH order. Agreements: - UE assumes that the DMRS of both the received PDCCH order and the PDCCH of the corresponding Msg2 are QCLed with the same SSB/CSI-RS. - gNB configures 9 bits to indicate RACH occasion index. - 6 bits are used to indicate an SSB index - Note: This SSB index is just intended to find the RACH occasion to transmit Msg1 - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index - Note: UE follows the SSB CBRA mapping rule to find the specific RACH occasion. - The SSB/CSI-RS that is QCLed with both the DM-RS of PDCCH order and the DM-RS of the PDCCH of the corresponding Msg2 is used for pathloss estimation associated with Msg1.

There are still some remaining issues left for the PRACH procedure [5]. In this contribution, we discuss some of the remaining issues on the PRACH procedure.

# Discussion

The following working assumption was made in RAN1#92:

Working assumption:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4} such that the number is

≥Actually transmitted SSBsSSBs that can be mapped to one PRACH config period

  - The time period starts from frame 0

The above WA requires the support of the mapping of at least L/4 SSBs into one PRACH configuration period, where L is the maximum number of SSBs within an SSB period. An issue was raised in the email discussion on PRACH format [8] on that there may not always be enough ROs to support L/4 SSBs into one PRACH configuration period. An example was provided that for format B4 if L = 64 SSBs, a DL-UL configuration periodicity of 2ms or longer, it may not be enough ROs for PRACH Format within a PRACH configuration period of 10ms, and thus it was proposed to add at least 8 for the number of multiple PRACH configuration periods into the WA. We share the same view that there is a need to support adding 8.

Proposal 1: Add the value of 8 into the following working assumption.

Working assumption:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4, 8} such that the number is

≥Actually transmitted SSBsSSBs that can be mapped to one PRACH config period

  - The time period starts from frame 0

The following working assumption and agreement were made in RAN1#92 [1]:

Working assumption:

- Minimum time gap between the end of PDSCH for Msg4 and the start of HARQ-ACK equals to the duration of N1 + L2.

  - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

  - L2=500 usec refers to the MAC layer processing time

  - Note: the reason for working assumption is due to potential concern over satisfying ITU requirement (to be checked)

Agreements:

- If a received Msg2 does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after the duration of N1 + new + L2.

  - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

  - new>=0, FFS

  - L2=500 usec refers to the MAC layer processing time

where new is left as FFS. We don’t see a particular reason for the minimum time gap between the end of PDSCH for Msg2 and the start of a new preamble sequence needs to be larger then the minimum time gap between the end of PDSCH for Msg4 and the start of HARQ-ACK. Thus, we suggest setting new=0.

Proposal 2: Make the modification on the following agreement:

Agreements:

- If a received Msg2 does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after the duration of N1 + L2.

  - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

  - L2=500 usec refers to the MAC layer processing time

In RAN1#92, there was a discussion on Msg2/Msg3/Msg4 search space [5] and the following proposals were made:

- UE monitors Msg2 search space in the first CORESET in case of PRACH preamble is associated to even SS/PBCH block index and in the second CORESET in case of PRACH preamble is associated to odd SS/PBCH block index in case of two CORESETs within a slot and RMSI CORESET configuration is used for RAR [6]

- NR supports search space of Msg2/3/4 grants to occur only at the beginning of each slot in RAR window [7]

The trigger for the discussion was the following agreements made in previous meeting [2,3]:

Agreements:

- By RMSI, the UE can be configured with at least one CORESET configuration at least for PDCCH for random access.

  - If not configured by RMSI, the CORESET configuration(s) for random access is/are the one(s) configured by PBCH.

Agreements:

- UE is not expected to monitor more than one Msg2/Msg3/Msg4 search space in one slot.

A RMSI CORESET may have two search space sets per slot with different starting symbols that are associated with the SS/PBCH block with index , as shown in the following table from TS 38.213.

Table 13-11: Parameters for PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 1 and frequency range 1

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 2 | 2 | 1 | 1 | 0 |
| 3 | 2 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 6 | 7 | 1 | 1 | 0 |
| 7 | 7 | 2 | 1/2 | {0, if is even}, {, if is odd} |

The concern was that when there are two RMSI CORESETs (spaces) in a slot, it is unclear which one the UE should use for Msg2/Msg3/Msg4 search space.

By looking as the following description provided in TS 38.123:

“For Type1-PDCCH common search space, a UE can be provided a configuration for a control resource set by higher layer parameter rach-coreset-configuration and a configuration for a search space by higher layer parameter ra-SearchSpace. If higher layer parameter rach-coreset-configuration is not provided to the UE, the control resource set for Type1-PDCCH common search space is the same as for Type0-PDCCH common search space.  … If a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space, the association between monitoring occasions for Type1-PDCCH common search space and the SS/PBCH block index is the same as the association of monitoring occasions for Type0-PDCCH common search space as described in Subclause 13.”

it seems quite clear that when a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space, the UE will find the Type1-PDCCH common search space that is associated with the SS/PBCH block index, following the same way as the UE to fine the Type0-PDCCH common search space that is associated with the SS/PBCH block index. By looking at Table 13-11, there is one to one association between one Type0-PDCCH common search space with one SS/PBCH block index, depending the SS/PBCH index is odd or even, although there can be two search spaces in a slot. Thus, we come with the conclusion that there is no ambiguity on how the UE finds the Type1-PDCCH common search space.

Observation: The current specification is clear enough on how the UE finds the Type1-PDCCH common search space when ra-SearchSpace for Type1-PDCCH common search space is not defined and when there are two search spaces in a slot.  There is no need to add further clarification.

# Conclusion

In this contribution, we discussed some remaining issues on the PRACH procedure. Based on the discussion, we made the following proposals:

Proposal 1: Add the value of 8 into the following working assumption and the confirm it.

Working assumption:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4, 8} such that the number is

≥Actually transmitted SSBsSSBs that can be mapped to one PRACH config period

  - The time period starts from frame 0

Proposal 2: Make the modification on the following agreement:

Agreements:

- If a received Msg2 does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after the duration of N1 + L2.

  - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

  - L2=500 usec refers to the MAC layer processing time

# References

- Draft Report of 3GPP TSG RAN WG1 #92 v0.1.0", ETSI

- Draft Report of 3GPP TSG RAN WG1 #91 v0.1.0", ETSI

- Draft Report of 3GPP TSG RAN WG1 #AH_1801 v0.1.0, ETSI

- R1-1801712, “Further details on NR 4-step RA Procedure”, CATT

- R1-1803524, “Summary of Remaining Details on RACH Procedure”, Qualcomm

- R1-1802022, “Remaining details on RACH procedure”, Nokia, Nokia Shanghai Bell

- R1-1802817, “Remaining details on Timing advance granularity and adjustment consideration”, 	Qualcomm Incorporated

- R1-18xxxx, “Summary of Unofficial email discussion on RACH configuration table for FR2”, Convida Wireless