3GPP TSG RAN WG1 Meeting #92bis	R1-1804131

Sanya, China, April 16 – 20, 2018

Agenda Item:	6.2.5.3.2

Source:	Huawei, HiSilicon

Title:	Performance evaluation of transmit diversity for eV2X

Document for:	Discussion and decision

# Introduction

At the RAN1 #92 meeting, the following conclusion for transmit diversity was reached:

- There is not consensus to confirm the working assumption to adopt non-transparent tx diversity, due to concerns on the impact on Rel-14 UEs with IRC receivers

- Can consider further at RAN1#92bis whether the same SD-CDD scheme as PSCCH can be applied to PSSCH.

  - FFS whether there is any spec impact (e.g. depending on choice of delay value(s))

  - Check CDD performance at different UE speeds

  - Evaluations should use practical CFO estimation

At the RAN1 #90bis the following agreement for TxD on PSCCH was reached:

- For PSCCH, small delay CDD can be used on PSCCH

  - FFS whether the cyclic delay value is specified or left for UE implementation

In this contribution, we provide evaluation results for SD-CDD for both PSSCH and PSCCH.

# Evaluation results

The performance of different TxD schemes in AWGN scenario is evaluated in this section. For comparison, the performance of single antenna is also shown in the figures. The details of the simulation setting are listed in the Appendix.

## PSSCH

Figure 1 compares the performance of different TxD schemes and single transmitter scheme with QPSK modulation. SD-CDD provides performance gains over the single antenna case at all speeds. The gain is higher at lower speed than at higher speed. With higher modulations, as Figure 2 and Figure 3 show, SD-CDD outperforms the single antenna case at low speed, but at higher speed, performance is somewhat similar, or can even be slightly worse in some conditions (e.g., 16-QAM at high speed). This suggests that SD-CDD parameters (shift) may need to be adjusted based on the radio and UE conditions.

Observation 1:

- SD-CDD provides significant performance gain, especially for low-order modulation and for low speeds

- At higher speed, the performance gains of SD-CDD vary, depending on the conditions

| (a) 30km/h | (b) 280km/h |
| --- | --- |
| (c) 500km/h |  |

Figure 1. TxD performance of PSSCH with QPSK modulation*

| (a) 30km/h | (b) 280km/h |
| --- | --- |

Figure 2. TxD performance of PSSCH with 16QAM modulation**

30km/h

Figure 3. TxD performance of PSSCH with 64QAM modulation

* Note: QPSK with coding rate 0.25 evaluated for 500km/h

** Note: 16QAM for 500km/h is not evaluated because only low MCSs can be used at 500km/h.

### PSCCH

Figure 4 compares the performance of single transmitter and small delay CDD with 1µs delay. It shows that small delay CDD scheme has performance gain at low speed scenario because of diversity gain in time domain. However, for high to extreme high speed scenarios, small delay CDD does not provide additional gain and has similar performance as single antenna transmission.

Observation 2: For PSCCH, small delay CDD can provide performance gain at low speed scenario.

| (a) 30km/h | (b) 280km/h |
| --- | --- |
| (c) 500km/h |  |

Figure 4. TxD performance of PSCCH

# Conclusions

The performance of various TxD schemes was evaluated. Based on the results, the following observations and proposals are drawn.

Observation 1:

- SD-CDD provides significant performance gain, especially for low-order modulation and for low speeds

- At higher speed, the performance gains of SD-CDD vary, depending on the conditions

Observation 2: For PSCCH, small delay CDD can provide performance gain at low speed scenario.

# Appendix A: Simulation setting

Table 2 Parameter setting of link level simulation

| Parameter | Value |
| --- | --- |
| General parameters: |  |
| Carrier frequency | 6 GHz |
| System bandwidth | 10 MHz |
| Channel Model | ITU-R UMi NLOS |
| Vehicle speed (absolute) | 15 km/h, 60km/h, 140km/h, 250km/h |
| CFO | 1800 Hz |
| Timing error | 2 us |
| Tx-Rx antenna configuration | 2x2, 1x2 |
| TxD schemes | - SFBC - STBC - Small Delay CDD (with 1us delay) - PVST |
| Receiver type | MMSE |
| Frequency offset estimation | Half-sequence correlation in time domain |
| Interference scenarios: |  |
| Interference type | {AWGN, Rel-14 UE, Rel-15 UE} |
| SNR | 25dB |
| SINR | -8:24 dB |
| PSSCH parameters: |  |
| TBS | 300Bytes, 190Bytes, 1620Bytes |
| Modulation and Coding rate | 15km/h: {QPSK 0.5 and 16QAM 0.5, 64QAM 0.6} 140km/h: {QPSK 0.5 and 16QAM 0.5} 250km/h: {QPSK 0.25 and 16QAM 0.25} |
| PSCCH parameters: |  |
| TBS | 48bits (including 16bits CRC) |
| Modulation | QPSK |
| PRB | 2 |
| RSRP measurement parameters: |  |
| Number of PRBs | 12 |

# Appendix B: Simulation results for 190Bytes

| (a) 30km/h | (b) 280km |
| --- | --- |
| (c) 500km/h |  |

Figure 8 TxD performance of PSSCH with QPSK modulation

| (a) 30km | (b) 280km |
| --- | --- |

Figure 9. TxD performance of PSSCH with 16QAM modulation