3GPP TSG RAN WG1 Meeting #92bis                                    R1-1804232

Sanya, China, April 16th – 20th, 2018

Agenda Item:	7.1.2.3.6

Source:	Spreadtrum Communications

Title:	Remaining details on TRS design

Document for:	Discussion and decision

Introduction

This contribution discusses remaining details on TRS design, such as aperiodic TRS and joint use of TRS and SSB.

Aperiodic TRS

Use cases:

There are some uses cases for aperiodic TRS in both periodic event and aperiodic event [1].

Periodic event mainly includes CDRX-ON and IDRX-ON. It was agreed it is up to BS implementation to enable UE to jointly use SSB and periodic TRS before DRX-ON [2].

Conclusion: Periodic SS burst and periodic TRS burst can be arranged such that the UE can utilize one SS burst and one TRS burst for AGC adjustment and T/F tracking before DRX ON No further specification impact may be necessary

However, because the DRX-ON or paging duration are not aligned among UEs [3], aperiodic TRS may be useful.

Aperiodic event mainly includes SCell activation, BWP switching and multi-beam change. As mentioned in [1], aperiodic TRS can reduce the delay between start of “switching” and start of data traffic.

Overhead issue of overhead of periodic TRS:

It was reported in [4] that there is an issue of huge overhead in FR2 due to the having only periodic TRS. So, use of aperiodic TRS can alleviate this problem.

Therefore, we propose to:

Proposal 1: Support aperiodic TRS in Rel-15 NR.

It was proposed in [5] to support UL DCI triggering for aperiodic TRS, which can reuse signaling for aperiodic CSI-RS.

Furthermore, aperiodic CSI-RS is used for aperiodic “measurement”, and fine time/frequency tracking by aperiodic TRS could be the prerequisite of aperiodic measurement. So, aperiodic CSI-RS and aperiodic TRS may have a common point.

Moreover, it is a concern from UE perspective that if aperiodic TRS is supported, BS may overly rely on aperiodic and reduce periodic TRS, which may potentially cause UE performance degradation more or less. It is general view the performance degradation due to BS scheduling is only up to BS implementation, however, it should be considered in standardization that BS should be aware of the degradation as soon as possible. Therefore, it is better to connect aperiodic TRS to CQI reporting.

As a conclusion, we support UL DCI triggering for aperiodic TRS, and propose that connection between triggering aperiodic TRS and CQI reporting should be considered for different use cases.

Proposal 2: UL DCI triggering for aperiodic TRS should be supported, and connection between triggering aperiodic TRS and CQI reporting should be considered in different use cases.

Joint use of TRS and SSB

Joint use of TRS and SSB was described in [5] and discussed in RAN1#92 meeting [6]. Some companies support the joint use, but some other companies have concern on feasibility and standard impact [4].

- 2.15 Joint use of TRS and SSB Nokia(2563) mentions that it is beneficial to include higher layer configured parameter ‘TRS-INFO-SSB’ CSI-RS resource configuration to enable joint use of CSI-RS and synchronization signal block (SSB) resources. Ericsson(2757) instead proposes that the Joint use of TRS and SS-block for fine frequency and time tracking is not supported. The proposals are, - To enable support for the joint use of different CSI-RS resource configurations and SSB resources, new RRC parameter, i.e. TRS-INFO-SSB, is proposed to be included as a part of CSI-RS resource set (Nokia) - Joint use of TRS and SS-block for fine frequency and time tracking is not supported (E///).

“Tight” joint use of TRS and SSB

In [5], joint use of TRS and SSB or fine time/frequency tracking has benefit of efficient use of system resources. General speaking, in case of joint use of TRS and SSB, single-slot TRS may be sufficient, and TRS overhead is reduced.

In [5], joint use of TRS and SSB could be a “new” RS pattern with combination of PBCH DMRS and single-slot TRS, which can be found in the following wording.

From channel estimation perspective above 6GHz at UE-side, the joint use of SSB resources (i.e. two separate DMRS resources associated with PBCH resources in two different symbols) and single-slot CSI-RS resources for time-frequency tracking can be seen as a slightly different RS pattern with respect to the agreement (X=1, N=2, Sf=4, St=4).

It can be regarded as a “tight” joint use, in which the estimation values of SSB and TRS should be “filtered” to form a new value. In our view, the “tight” joint use heavily relies on patterns of SSB and TRS, which may cause potential large standardization effort.

“Loose” joint use of TRS and SSB

In our view, there could be a “loose” joint use, in which SSB is used as resolvable range “keeper” and TRS is used as accuracy “provider”. For instance, if frequency error estimated by TRS is within the resolvable range of frequency error estimated by SSB, frequency error estimated by TRS can be kept, otherwise frequency error estimated by TRS will be thrown. It could be common understanding in the previous discussion.

The resolvable range of frequency error estimation:

The resolvable range depends on symbol spacing. The following table shows the resolvable range under different SCS and symbols spacing [7].

SCS=15KHz; SCS=30KHz; SCS=60KHz; SCS=120KHz. St=2; +-3.5KHz; +-7KHz; +-14KHz; +-28KHz. St=3; +-2.3KHz; +-4.6KHz; +-9.2KHz; +-18.4KHz. St=4; +-1.75KHz; +-3.5KHz; +-7KHz; +-14KHz. St=5; +-1.4KHz; +-2.8KHz; +-5.6KHz; +-11.2KHz. St=7; +-1KHz; +-2KHz; +-4KHz; +-8KHz. St=14; +-500Hz; +-1KHz; +-2KHz; +-4KHz. Table 1, Resolvable range for different symbol spacing (R1-1719581)

It is common understanding that the decreasing ordering of the resolvable range is: SS > PBCH-DMRS > TRS, where St = 2 for PBCH-DMRS, and St = 4 for TRS.

The accuracy of frequency error estimation:

The accuracy is strongly related to the BW/density of RS and symbol spacing [8].

Based on this architecture, the raw estimate accuracy of the frequency offset is strongly related to the number of paired RS for delay correlation, and the two-TRS-symbol spacing St for the delay correlation. Note that the raw estimate accuracy also represents the one-shot accuracy.

It is common understanding that the decreasing ordering of accuracy is: TRS > PBCH-DMRS > SS.

The resolvable range management:

In general, UE may manage the resolvable range for frequency error estimation to avoid frequency drift exceeding the resolvable range of RS.

For instance, while entering DRX-ON with DRX cycle time 2560ms, UE may choose PBCH-DMRS to estimate frequency error, because the max frequency drift is within the resolvable range of PBCH-DMRS.

It could be common understanding, since it is the background of discussion on TRS and SSB transmission before DRX-ON [2].

The frequency drift for different DRX cycle lengths for 5GHz carrier frequency is listed as follows [9]:

In Table 3 we give the frequency and time drift for different DRX cycle lengths for 5GHz carrier frequency and 15kHz subcarrier spacing. Note that this is a tough combination with a small subcarrier spacing in relation to the carrier frequency. Table 3 Frequency and time drift during different DRX cycle lengths assuming a max frequency drift of 0.16ppm/s for 5GHz carrier frequency and 15kHz subcarrier spacing. DRX cycle time [ms]; Max Frequency drift [Hz]; Max Frequency drift [% of SCS]; Max Time drift [s]; Max Time drift [% of CP]. 2560; 2048; 14%; 5.24E-07; 11%. 1000; 800; 5%; 8.00E-08; 2%. 500; 400; 3%; 2.00E-08; 0.4%. 100; 80; 0.5%; 8.00E-10; 0.02%. 10; 8; 0.05%; 8.00E-12; 0.0002%.

Simulation results

Simulations are conducted for purpose of:

- studying performance of aperiodic TRS, or equivalently “one-shot” TRS [8], and

- studying the necessity of the resolvable range management in the “loose” joint use of TRS and SSB.

Simulation assumptions can be found in Appendix.

The simulation results are shown as follows:

- Initial frequency error = 200Hz                                     b) Initial frequency error = 2000Hz

Figure 1: CDF of the estimated frequency error with SNR of 15dB and EPA5 channel.

- Initial frequency error = 200Hz                                     b) Initial frequency error = 2000Hz

Figure 2: CDF of the estimated frequency error with SNR of 15dB and EVA70 channel.

With the initial frequency error is 200Hz, it can be observed that the estimated frequency error based on single-slot TRS (X=1, N=2) is distributed around the initial frequency error with a smaller deviation, while the estimated frequency error based on SSB is distributed around the initial frequency error with a larger deviation.

Observation 1-1: For frequency error estimation and one-shot (aperiodic) TRS, performance of single-slot TRS is much better than that of SSB in EPA5 and EVA70 channels.

It can also be observed that the curve of the single-slot TRS (X=1, N=2) is very close to the curve of the two-slot TRS (X=2, N=2+2), which means that the performance of the one-shot single-slot TRS is similar to that of the one-shot two-slot TRS.

Observation 1-2: For frequency error estimation and one-shot (aperiodic) TRS, performance of single-slot is similar to that of two-slot TRS in EPA5 and EVA70 channels.

Furthermore, it can be observed that deviation is small enough for single-slot TRS (X=1, N=2).

Observation 1-3: For frequency error estimation and one-shot (aperiodic) TRS, performance of single-slot seems good enough in EPA5 and EVA70 channels.

With the initial frequency error is 2000Hz, it can be observed that frequency error estimation based on TRS is not correct any more, because it exceeds the resolvable range of TRS.

Observation 2: The resolvable range management is necessary, otherwise the frequency error estimation is wrong.

Summary

From the discussion and simulation results, the “loose” joint use of TRS and SSB seems sufficient. So, the “tight” joint use of TRS and SSB should be justified for its benefits.

Therefore, we propose that:

Proposal 3: The “tight” joint use of TRS and SSB should be further justified.

Conclusion

In the contribution, we discuss aperiodic TRS and joint use of TRS and SSB, and have the following proposals.

Proposal 1: Support aperiodic TRS in Rel-15 NR.

Proposal 2: UL DCI triggering for aperiodic TRS should be supported, and connection between triggering aperiodic TRS and CQI reporting should be considered in different use cases.

Proposal 3: The “tight” joint use of TRS and SSB should be further justified.

Reference

- R1-1802831, “Remaining issues on TRS”, QC, RAN1#92, Feb. 26th – March 2nd, 2018.

- 3GPP RAN1, “Chairman’s notes of RAN1 90bis”, Oct. 2017.

- R1-1801460, “Remaining issues on TRS design”, HW, RAN1#92, Feb. 26th – March 2nd, 2018.

- R1-1802757, “Corrections related to TRS”, Ericsson, RAN1#92, Feb. 26th – March 2nd, 2018.

- R1-1802563, “Remaining Details of TRS Design”, Nokia, RAN1#92, Feb. 26th – March 2nd, 2018.

- R1-1803304, “Summary for TRS”, MTK, RAN1#92, Feb. 26th – March 2nd, 2018.

- R1-1801670, “Remaining issues on TRS”, MTK, RAN1#92, Feb. 26th – March 2nd, 2018.

- R1-1710832, “On TRS design requirement”, MTK, RAN1 AH 1706, Jun. 27th– 30th, 2017.

- R1-1718451, “Remaining details on TRS”, Ericsson, RAN1#90bis, Oct. 9th – 13th, 2017.

# Appendix-Simulation assumptions

| Channel type | EPA5, EVA70 |
| --- | --- |
| SNR | 2 dB |
| Subcarrier spacing of SSB/TRS | 15kHz |
| TRS pattern | X = 1, N = 2 |
| TRS bandwidth | 50 RBs |
| TRS time spacing | 4 OFDM symbols |
| TRS frequency spacing | 4 REs |
| SSB bandwidth | 20 RBs |
| Simulated samples | 2000 |