3GPP TSG RAN WG1 meeting #92-bis	R1-1804503

Sanya, China, April 16th – 20th, 2018

Agenda item:	7.1.6.1

Source:	NEC

Title:	RRC parameter alignment for EN-DC power control

Document for:	Discussion and decision

Discussion

Higher-layer parameter P-LTE for  for EN-DC described in [1, TS36.213] is not defined in [2, TS38.331][3, TS36.331]. Instead, p-MaxEUTRA is defined in [3] as shown below (highlighted).

To align RAN1 and RAN2 specifications without any impact on RRC, we propose the TP to [1] in the next section.

TS36.331v15.1.0 [3]

RRCConnectionReconfiguration-v15x0-IEs ::= SEQUENCE {

nr-Config-r15					CHOICE {

release							NULL,

setup							SEQUENCE {

endc-ReleaseAndAdd-r15	BOOLEAN,

nr-SecondaryCellGroupConfig-r15	OCTET STRING				OPTIONAL,	-- Need ON

p-MaxEUTRA-r15					P-Max						OPTIONAL	-- Need ON

}

}																	OPTIONAL,	-- Need ON

sk-Counter-r15					INTEGER (0.. 65535)					OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15		OCTET STRING						OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15		OCTET STRING						OPTIONAL,	-- Need ON

tdm-PatternConfig-r15			CHOICE {

release							NULL,

setup							SEQUENCE {

subframeAssignment-r15			SubframeAssignment-r15,

harq-Offset-r15					INTEGER (0.. 9)

}

}																	OPTIONAL,	-- Need ON

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

Text Proposal

7.6.1	EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by higher layer parameter p-MaxEUTRA [x, TS36.331] and a maximum power  for transmissions on the SCG by higher layer parameter p-NR. The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power for the SCG as described Subclauses 7.1 through 7.5 using  as the maximum transmission power for .

Reference

[1] TS38.213v15.1.0

[2] TS38.331v15.0.0

[3] TS36.331v15.1.0