3GPP TSG RAN WG1 Meeting #92bis				R1-1804571

Sanya, China, April 16th – 20th, 2018

Agenda Item:	7.2.3

Source: 	LG Electronics

Title: 	Discussion on PDCCH repetition for URLLC

Document for:	Discussion and decision

- Introduction

Considering ultra-reliability aspects of overall DL transmission and UL transmission, it would be more important to enhance reliability of NR-PDCCH compared to NR-PDSCH or NR-PUSCH. To be specific, if retransmission is assumed to be used for ultra-reliability requirements, PDSCH or PUSCH decoding can be performed after combining coded bits from initial transmission and its retransmission. In this case, BLER requirements of a single PDSCH or PUSCH transmission could be relaxed. Instead, since UE may need to detect both NR-PDCCH scheduling initial transmission and its retransmission, BLER requirement for NR-PDCCH needs to be small enough. Depending on the BLER requirement and feasibility in terms of detection performance, it can be considered to carefully investigate whether compact DCI format is introduced or PDCCH repetition is introduced or both approaches are introduced.

Observation 1: Considering BLER requirement and feasibility of enhancement scheme, it is necessary to carefully investigate whether NR support compact DCI format or PDCCH repetition or both schemes.

In this contribution, we provide our views on ultra-reliability aspects on NR-PDCCH design. Especially, we focus on how to support PDCCH repetition.

- Comparison with Compact DCI

To enhance detection performance of DCI, one approach is to reduce DCI payload size. In this case, UE can enhance DCI detection performance without increasing the number of allocated resources for PDCCH transmission due to the lower code rate of DCI. Alternatively, it can be considered to adopt PDCCH repetition for the same DCI contents or the same TB. The motivation of the above schemes is to make code rate lower. For this, one is to reduce DCI size, the other is to use more resources. PDCCH repetition may require more resource, while compact DCI may need to have scheduling restriction and to have additional BD attempt.

From our perspective, PDDCH repetition may have less specification work load compared to the compact DCI. To be specific, PDCCH repetition may need to define UE behaviour with duplicated DCI while compact DCI may need to investigate how to reduce DCI fields and how to handle BD attempts at UE side. Considering limited working time for this item, we propose to prioritize the discussion on PDCCH repetition for control channel reliability.

Observation 2: In terms of specification effort, PDCCH repetition can be prioritized for control channel reliability

- Higher aggregation level and PDCCH repetition

  - How to support higher aggregation level with resource enlargement

To lower the code rate of DCI for ultra-reliability requirements, it may need to investigate how to increase the number of allocated resources for PDCCH transmission. In this case, following approaches can be considered:

- Option 1: A PDCCH candidate is mapped on a single CORESET supporting higher maximum aggregation level.

- Option 2: A PDCCH candidate can be associated with multiple PDCCH candidates on the same or different CORESET supporting low to moderate maximum aggregation level.

In case of Option 1, CORESET may need to be allocated with wide range of frequency resources and/or a number of time-domain resources to have sufficiently large number of CCEs to support higher aggregation level. This approach would be beneficial in terms of specification work since mapping rule for REG-to-CCE and CCE-to-PDCCH could be the same regardless of a set of aggregation levels to be used for PDCCH. Meanwhile, it is necessary to investigate the impact on blocking among PDCCH candidates.

In Option 2, each component CORESET to make a PDCCH with higher aggregation level does not need to have excessive number of time-and-frequency resources. Instead, it is necessary to design how to map a PDCCH candidate by combining multiple PDCCH candidates mapped on the same or different CORESET. For simplicity, it can be considered that a PDCCH candidate with higher aggregation level is constructed by combining multiple set of PDCCH candidates mapped on different CORESETs. For instance, for a certain aggregation level (e.g. 8), PDCCH candidate #m in a CORESET and PDCCH candidate #m in another CORESET could be combined to make a PDCCH candidate with higher aggregation level (e.g. 16). This approach would be efficient in the perspective of network resource management especially when the network supports multiple UEs with different target aggregation levels. For instance, eMBB UEs need to support up to aggregation level of 8 while URLLC UEs need to support up to aggregation level of 16. Furthermore, depending on the number of supported UEs, the network can have multiple CORESETs. In this case, rather than additionally configuring at least one CORESET with maximum aggregation level of 16, network can support aggregation level of 16 by using two CORESET with maximum aggregation level of 8 which can be used to support eMBB UEs. Furthermore, to achieve diversity gain, a UE can be configured with CORESETs in different carrier or bandwidth part or TRP to support PDCCH with higher aggregation level. For instance, for URLLC, it would be inefficient to perform beam selection procedure due to the latency requirement and relatively small payload size. Instead, it can be considered that PDCCH transmission for URLLC is associated with multiple beams for diversity gain. In this case, PDCCH for URLLC can be transmitted over multiple CORESET with different beam information. Moreover, by constructing higher AL with combined lower AL candidates, nested structure between two ALs are achieved without increasing channel estimation complexity. It is important in URLLC with limited channel complexity budget where higher ALs are monitored generally.

For both cases, it is necessary to investigate how to perform BD handling for PDCCH monitoring. For simplicity, PDCCH candidates to be monitored could be updated depending on the supporting aggregation level set. In this case, the number of PDCCH candidates for lower aggregation level could be reduced.

Proposal 1: To increase the number of allocated resources for PDCCH transmission in frequency domain, a single PDCCH candidate can be constructed by combining multiple PDCCH candidates mapped on the same or different CORESET.

  - How to support multiple transmission of DCI

It can be considered to transmit PDCCH multiple times in time/frequency-domain. If UE has multiple monitoring occasion and/or CORESET configuration and if there are enough unused PDCCH candidates, gNB can transmit same DCI repeatedly on multiple PDCCH candidate mapped to the same or different CORESETs/monitoring occasions. However, In case of PDCCH repetition over multiple monitoring occasions, it is necessary to investigate when the scheduled PDSCH or PUSCH will be transmitted considering latency requirement and UE complexity in terms of UE buffering. To be specific, UE would know how to perform channel estimation and decoding of PDSCH or PUSCH after UE successfully decode the corresponding PDCCH. In that point of view, it can be considered to transmit PDSCH or PUSCH after the end of PDCCH repetition. However, in this case, the latency requirement would not be met. Alternatively, at least PDSCH transmission can be overlapped with the corresponding PDCCH transmission in time-domain. In case of PUSCH transmission, it seems infeasible that PUSCH is transmitted earlier than the end of the corresponding PDCCH transmission. In case of PDSCH, UE may need to buffer the received DL signals for a certain duration of time including all the monitoring occasions where the corresponding PDCCHs are transmitted. In addition to this, we have to consider that possible time-domain RA is configured by RRC. It seems not possible to configure multiple time-domain RAs only for DCIs that repeat over multiple slot. To alleviate this problem, UE may assume multiple DCI transmission only within 1 slot.

Proposal 2: if PDCCH repetition is supported, UE assumes that PDCCH repetition is only in a slot.

In the current specification of 38.214, it has been specified that The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process. It means that UE would ignores PDCCH scheduling same HARQ process as the previously scheduled PDSCH. Thus if gNB send DCI on multiple PDCCH candidates repeatedly, it is expected that UE opportunistically success to decode PDCCH. In other words, PDCCH repetition can be achieved without any specification impact at least for PDSCH if combining over repetition is not assumed. In the case of PUSCH, it is being discussed to re-schedule PUSCH with UL grant received earlier than PUSCH transmission. Considering that, it can be considered that UE distinguishes DCI by the RA information whether it is the DCI repetition or re-scheduling. To be specific, UE may expect to receive UL grant which indicates same RA and same HARQ process for PDCCH repetition. Otherwise, UE can treat the DCI as re-scheduling and cancel previously allocated PUSCH resource.

Proposal 3: NR supports the case where multiple PDCCHs schedule the same PDSCH or PUSCH.

- For PDSCH, A UE ignores PDCCH(s) scheduling PDSCH for previously scheduled HARQ process until the transmission of HARQ-ACK for that HARQ process.

- For PUSCH, A UE ignores PDCCH(s) scheduling PUSCH for previously scheduled HARQ process with same resource allocation as that HARQ process until the PUSCH transmission for that HARQ process

  - FFS: UE behavior when UE receives PDCCH scheduling PUSCH for the previously scheduled HARQ process with different resource allocation from that HARQ process.

- Conclusion

In this contribution, we discuss ultra-reliability aspects on NR-PDCCH design. Our proposals are as follows:

Observation 1: Considering BLER requirement and feasibility of enhancement scheme, it is necessary to carefully investigate whether NR support compact DCI format or PDCCH repetition or both schemes.

Observation 2: In terms of specification effort, PDCCH repetition can be prioritized for control channel reliability

Proposal 1: To increase the number of allocated resources for PDCCH transmission in frequency domain, a single PDCCH candidate can be constructed by combining multiple PDCCH candidates mapped on the same or different CORESET.

Proposal 2: if PDCCH repetition is supported, UE assumes that PDCCH repetition is only in a slot.

Proposal 3: NR supports the case where multiple PDCCHs schedule the same PDSCH or PUSCH.

- For PDSCH, A UE ignores PDCCH(s) scheduling PDSCH for previously scheduled HARQ process until the transmission of HARQ-ACK for that HARQ process.

- For PUSCH, A UE ignores PDCCH(s) scheduling PUSCH for previously scheduled HARQ process with same resource allocation as that HARQ process until the PUSCH transmission for that HARQ process

  - FFS: UE behavior when UE receives PDCCH scheduling PUSCH for the previously scheduled HARQ process with different resource allocation from that HARQ process.