3GPP TSG RAN WG1 Meeting #92bis                      R1-1804576

Sanya, China, April 16th – 20th, 2018

Agenda Item:	7.4.4

Source: 	LG Electronics

Title: 	Initial LLS evaluation for NCMA

Document for:	Discussion and decision

Introduction

In general, for the conventional DL/UL multiple access (MA) scheme, three design criteria have been considered as spectral efficiency, energy efficiency and system complexity. In NR (New RAT) design, three use cases are considered as eMBB, mMTC, and URLLC with several KPIs for IMT 2020 RIT submission [1]. Specially, some KPIs such as massive connectivity, spectral efficiency and latency, might be enormously challenging. As the first step toward NR design, the non-orthogonal multiple access (NOMA) schemes for NR are discussed and UL LLS evaluation results for NOMA schemes are presented.

Followings are the agreements and observations on the multiple access of NR in RAN1 #84bis;

Agreements:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied

Observations:

- Examples non-orthogonal schemes include (but not limited to):

  - For UL, Multi-user shared access (MUSA) (e.g., R1-162226)

  - Resource spread multiple access (RSMA) (e.g., R1-163510)

  - Sparse code multiple access (SCMA) (e.g., R1-162153)

  - Pattern defined multiple access (PDMA) (e.g., R1-163383)

  - Non-orthogonal coded multiple access (NCMA) (e.g., R1-162517)

  - Low code rate spreading (e.g., R1-162385)

  - Frequency domain spreading (e.g., R1-162385)

  - Non-orthogonal multiple access (NOMA) (e.g., R1-163111)

Followings are the agreements of NOMA SI in RAN1 #92;

Agreements:

- Adopt the following table as the metrics for NOMA study from link level point of view.

  - More metrics may be added in the future

| Performance metrics | BLER vs. per UE SNR at a given pair of {per UE SE, # of UEs} Sum throughput v.s. SNR at given BLER level, for a given pair of {per UE SE, # of UEs} MCL |
| --- | --- |
| Implementation related metrics | PAPR/cubic metric Rx complexity and processing latency FFS: Configuration/Scheduling flexibility |

Non-orthogonal Coded Multiple Access (NCMA)

The NOMA schemes have been proposed for the increase of the connectivity or the system throughput. These schemes are based on the non-orthogonal spreading code or the difference of spatial resource and power. In case of specific channel environments with system optimizations (e.g. power allocation or user scheduling); the NOMA schemes can provide the improved connectivity or the increased system throughput compared to the conventional OMA schemes. However, NOMA schemes have some defeats, such as scheduling complexity, encoding/decoding complexity, loss of BLER, and limited environments.

In the NOMA schemes, multiuser interference (MUI) is inherently induced. Considering this, spreading based non-orthogonal coded multiple access (NCMA) scheme was suggested in [2]. This scheme is one of an approach for theoretically minimizing the MUI based on the spreading codes. Here, we define the spreading code as ‘Grassmannian Sequence’ [3][4] and the sequence and quantized version of the sequence can be obtained as follows.

- Grassmannian Sequence

  - Each complex spreading sequence of this sequence set is generated by Grassmannian line packing problem. Let the Grassmannian sequence set defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor. Then, the sequence design problem can be posed in terms of maximizing the minimum chordal distance between sequence pairs: minGmax1≤k<j≤K1-sk*∙sj2, where sk* is the conjugate sequence of sk.

  - Spreading sequence set: G=s(1)…s(K)

- M-QAM quantized Grassmannian Sequence

  - Each complex coefficient of this sequence (which is generated by the Grassmannian sequence) is quantized by M-QAM constellations. Then, the M-QAM quantized Grassmannian sequence set is defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor.

  - Complex coefficient: si(j)=c, i=1≤i≤N, i=1≤j≤K, c∈the set of M-QAM constellations

  - Spreading sequence set: G=s(1)…s(K)

Note that the spreading code design mentioned in [2] is one of the NCMA schemes. From another point of view, coding based NCMA scheme can be also considered as one of other approaches for the NCMA schemes. This scheme is one of an approach to disperse the MUI based on the user-specific scrambling codes in coding domain [5][6].

Link Level Simulation

  - Simulation Parameters

- Simulation parameters are represented as follows:

Table I. UL Link Level Simulation Assumptions

| Waveform | CP-OFDM / DFT-s-OFDM |
| --- | --- |
| Multiple Access | OFDMA w/ Localized Allocation Scheme 1: Spreading based NCMA (w/ Non-orthogonal Code via 64QAM-quantized Grassmannian Sequence [Appendix]) Scheme 2: Coding based NCMA (w/ Codeword level scrambling via LTE PUSCH scrambling) |
| Carrier Frequency | 700 MHz |
| Subcarrier Spacing | 15kHz |
| System Bandwidth / FFT Size | 10MHz / 1024 |
| Transmission Bandwidth | 6PRBs |
| Antenna Configuration | 1T2R |
| TBS per UE | 10, 20, 40 [bytes] |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiple UEs | 1, 3, 6 |
| Channel Coding | LDPC |
| Modulation | QPSK |
| MA Signature Selection | Fixed Allocation |
| Receiver Algorithm | Successive Interference Cancellation (SIC) ; Codeword level IC is assumed, but CRC check is not assumed. |
| Channel Model | TDL-C (300ns), mobility-3km/h |
| Channel Estimation | Ideal / Realistic |

  - BLER vs. per UE SNR

In this section, the initial link level simulation results of NCMA via 64QAM-quantized Grassmannian Sequence (Scheme 1) and LTE PUSCH scrambling (Scheme 2) are compared with the baseline OFDMA scheme in terms of block error rate. In the following simulation results, ‘SF’ represents the spreading factor and ‘UE’ represents the number of UEs.

Figure 1. BLER performance of OFDMA in case of CP-OFDM and ideal/realistic channel estimation

Figure 2. BLER comparisons for Scheme 1, 2 and OFDMA in case of CP-OFDM and TBS = 10 [bytes]

Figure 3. BLER comparisons for Scheme 1, 2 and OFDMA in case of CP-OFDM and TBS = 20 [bytes]

Figure 4. BLER comparisons for Scheme 1, 2 and OFDMA in case of CP-OFDM and TBS = 40 [bytes]

Figure 1 represents the BLER performance of OFDMA. Figures 2, 3 and 4 represent the BLER comparisons for spreading/coding based NCMA and OFDMA with SIC for TBS = 10, 20 and 40 [bytes], respectively. In these simulations, CP-OFDM, TDL-C with 3km/h mobility, QPSK modulation and realistic channel estimation are assumed. Assuming 6 PRBs with 2 DMRS symbols, CRC bits and CRC check are not considered for all schemes and cases. Here, SNR for all the UEs is equal. Above results represent that the Scheme 1 and 2 can provide some connectivity gain or total throughput gain with only marginal loss of BLER. In other words, the Scheme 1 and 2 can accommodate larger number of UEs than OFDMA while performance provided by the Scheme 1 and 2 can be comparable to OFDMA. In the above simulation case, the performance of Scheme 1 is slightly better than that of Scheme 2. Consequently, above results show the potentials of Scheme 1 and 2 which supports multiuser connectivity under given limited resources.

Observation 1: The spreading/coding based NCMA can provide some connectivity gain or total throughput gain with only marginal loss of BLER compared to OFDMA.

Observation 2: The spreading based NCMA can provide slightly better performance compared to the coding based NCMA.

Figure 5. BLER comparisons for Scheme 1, 2 and OFDMA in case of DFT-s-OFDM and TBS = 10 [bytes]

Figure 6. BLER comparisons for Scheme 1, 2 and OFDMA in case of DFT-s -OFDM and TBS = 20 [bytes]

Figures 5 and 6 represent the BLER comparisons for spreading/coding based NCMA and OFDMA with SIC for TBS = 10 and 20 [bytes], respectively. In these simulations, DFT-s-OFDM, TDL-C with 3km/h mobility, QPSK modulation and realistic channel estimation are assumed. Assuming 6 PRBs with 2 DMRS symbols, CRC bits and CRC check are not considered for all schemes and cases. These simulation results also carry the similar aspects from the results in the case of CP-OFDM. Consequently, above results show that the Scheme 1 and 2 can be also considered as candidates of NR NOMA schemes in DFT-s-OFDM system.

Observation 3: The spreading/coding based NCMA can be also considered as candidates of NR NOMA schemes in DFT-s-OFDM system.

System Level Simulation Assumptions

In Rel-14 NR SI, the evaluation assumptions and performance metrics for system level simulation were already discussed. Then, the assumptions and metrics were captured in TR 38.802. Thus, for Rel-15 NR NOMA, we can consider the evaluation assumptions and the performance metrics agreed in Rel-14 NR SI as a starting point.

Proposal: For Rel-15 NR NOMA, the evaluation assumptions and the performance metrics agreed in Rel-14 NR SI for system level simulation need to be considered as a starting point.

Summary

In this document, we presented evaluations for the spreading/coding based NCMA. Following the results, our observation and proposal can be summarized as below.

Observation 1: The spreading/coding based NCMA can provide some connectivity gain or total throughput gain with only marginal loss of BLER compared to OFDMA.

Observation 2: The spreading based NCMA can provide slightly better performance compared to the coding based NCMA.

Observation 3: The spreading/coding based NCMA can be also considered as candidates of NR NOMA schemes in DFT-s-OFDM system.

Proposal: For Rel-15 NR NOMA, the evaluation assumptions and the performance metrics agreed in Rel-14 NR SI for system level simulation need to be considered as a starting point.

Reference

TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

R1-162517, LG Electronics, “Considerations on DL-UL multiple access for NR”

R1-166874, LG Electronics, “Discussion on Non-orthogonal Spreading Sequences for NoMA”

R1-1609230, LG Electronics, “Discussion on Non-orthogonal Spreading Sequence and Codebooks for NoMA”

R1-164557, LG Electronics, “Initial LLS Evaluation Result for NoMA”

R1-166876, LG Electronics, “Performance evaluation of NCMA”

Appendix

- Examples of 64QAM-quantized Grassmannian Sequence based spreading codebook for minimizing the MUI are represented as follows:

Table II. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 2

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 2 | 5+5i5+5i5+5i-5-5i*Pno,2,2 |
| 4 | -3+5i-3+7i3+7i-3+1i-1-3i-7-3i7-3i5+3i*Pno,2,4 |
| 6 | -3+1i-7-3i7-3i-5+3i-1-5i-5+3i-1-7i-5-5i-1-7i-7+1i-1-1i3-3i*Pno,2,6 |
| 8 | -3-7i-7-1i-1-3i1-5i-3-1i-1-5i-1+7i-5-7i-3-1i-7+1i1-7i3+1i-3-7i-1+5i-5+1i-1-7i*Pno,2,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.

Table III. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 4

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 4 | 5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i-5-5i-5-5i-5-5i5+5i*Pno,4,4 |
| 6 | -7+3i-3+1i-5+1i-1-5i3+1i-1-7i1-7i-7+3i-1-7i-1+1i-3+1i3+5i-1-3i-1+7i-3+1i-5+5i-7+5i-3+1i-3-7i5+7i3+3i5-7i-1+3i1+3i*Pno,4,6 |
| 8 | -5-3i-7+1i1-5i1+3i-1-3i-7-3i-7+7i5-1i-1-3i-5-3i7-3i3+1i-5-3i-7+7i-1+5i-5+5i-7-3i1-7i1-3i1-7i-1+3i7+1i-5+7i-3+5i-5+7i-3-5i1+1i-7+7i-5+3i-1-1i7-5i3+1i*Pno,4,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.