3GPP TSG RAN WG1 Meeting #92 bis	R1-1805249

Sanya, P.R. China, April 16 – 20, 2018

Agenda item:		7.1.3.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to 38.214 on UE processing time definitions

Document for:		Discussion and Decision

# 1	Introduction

The UE processing time equations in subclauses 5.3 and 6.4 were left with a few errors.

- The seletion of N1 and N2 is based on the smallest SCS of the {SCSUL, SCSDL}, while the scaling is still always foxed to the DL SCS and UL SCS in the PDSCH and PUSCH processing respectively.

- d1,2  and d2,2 are treated in the equations as if RAN4 defines them in symbols. Furthermore, the intent was to capture the impact of an offset between the PDCCH carrier and the PxSCH carrier, but the equations take this into account naturally and the figure in the summary document R1-1803470 misrepresented the issue;

  - The HARQ-ACK timing is defined from the PDSCH on the carrier of the PDSCH to the HARQ-ACK on the carrier in which the HARQ-ACK is transmitted

  - The PUSCH timing is defined from the PDCCH on the carrier of the PDCCH to the PUSCH on the carrier of the PUSCH. The timings of other carriers do not play a role.

- The last otherwise bullet “otherwise d1,2 = 0.” clause of 5.3 should be part of the previous bullet and refer to what was known d1,3.

- κ is undefined in 38.214 and a reference should be provided.

# 2	Text proposals to TS38.214

## 5.3	UE PDSCH processing procedure time

If the first uplink symbol to carry the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after  after the end of the last symbol of the PDSCH carrying the TB being acknowledged.

-	N1 is based on µ of table 5.3-1 that corresponds to the min (µDL, µUL) where the µDL corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.1 of [4].

-	If HARQ-ACK is transmitted on PUCCH, then d1,1 = 0,

-	If HARQ-ACK is transmitted on PUSCH, then d1,1 = 1.

-	If the PDSCH is mapping type A as given in subclause 7.4.1.1 of [4], and the last symbol of PDSCH is on the ith symbol of the slot where i<7, then , otherwise d1,2 = 0.

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH. The value of  is used both in the case of normal and extended cyclic prefix.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | No additional PDSCH DM-RS configured | Additional PDSCH DM-RS configured |
| 0 | 8 | 13 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Next Modified Subclause

## 6.4	UE PUSCH preparation procedure time

If the first uplink symbol in the PUSCH allocation, including the DM-RS, as defined by the slot offset K2 and the start and length indicator SLIV of the scheduling DCI, is no earlier than at symbol L2 then the UE shall transmit PUSCH where L2 is defined as the next uplink symbol with its CP starting  after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, where N2 is defined by table 6.4-1, where

-	N2 is based on µ of table 6.4-1, that corresponds to the min(µDL, µUL) where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in subclause 4.1 of [4].

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1=0, otherwise d2,1=1.

Otherwise the UE may ignore the scheduling DCI. The value of  is used both in the case of normal and extended cyclic prefix.

Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |