3GPP TSG RAN WG1 Meeting #93		R1-1806979

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.4.1

Source: 	AT&T

Title: 	On Transmitter Side Processing for NOMA

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  The technical specifications were agreed during the December RAN meeting. However, during the initial phase of NR SI, RAN1 studied on non-orthogonal multiple access (NOMA). Due to the necessity of completion of more urgent features, NOMA was delayed in NR. The main objectives of NOMA study are

Non-orthogonal multiple transmission scheme

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

- Receivers for non-orthogonal multiple access: [RAN1, RAN4]

  - MMSE receiver, successive/parallel interference cancellation (SIC/PIC) receiver, joint detection (JD) type receiver, combination of SIC and JD receiver, or other receivers

  - The study should consider performance, receiver complexity, etc.

- Procedures related to the non-orthogonal multiple access  [RAN1]

  - UL transmission detection

  - HARQ, including transmission scheme, feedback scheme, and combining scheme

  - Link adaptation MA signature allocation/selection

  - Synchronous and asynchronous operation

  - Adaptation between orthogonal and non-orthogonal multiple access

- Link and system level performance evaluation or analysis for non-orthogonal multiple access continued from performance metrics identified from Rel-14. The benchmark for comparison is OFDM contention based multiple access. Realistic modelling of Tx/Rx impairment including potential PAPR issue, channel estimation error, power control accuracy, collision, etc. should be considered. [RAN1]

  - Traffic model and Deployment scenarios of eMBB (small packet), URLLC and mMTC

  - Device power consumption

  - Coverage (link budget)

  - Latency and signalling overhead

  - BLER reliability, capacity and system load

  - Physical abstraction (link-to-system mapping model)

Note: targeting common solution for mMTC, URLLC and eMBB small packet.

In this contribution, we provide details of transmitter side processing for applications.

# Transmitter Structures for NOMA

In NOMA systems since the users are sharing the same resources, there will be interference between the users in the same cell.   The interference is more when the number of users are more.  Techniques such as spreading, interleaving or variable power allocation can be applied at the transmitter side. Among various techniques proposed in literature and in previous RAN1 meetings, we prefer a transmitter structure as shown in Figure 1. The spreading operation can be linear or non –linear.

Figure 1 Transmission chain for NOMA

Note that the spreading is done at symbols level.  Since the transmission chain already covers the data scrambling which is UE specific, we don’t prefer additional scrambling as in CDMA/HSPA/EUL systems. Moreover, from our simulation results, we observed additional scrambling does not provide any robustness to inter cell or intra cell user interference.

Proposal 1:  We prefer two types of spreading codes, linear and non-linear spreading for NOMA transmitter

Proposal 2:  Additional scrambling is not needed to reduce the intra/inter user interference

# Multi Code Spreading Transmitter Structure for NOMA

In this section we describe our view on linear codes for NOMA. Note that current studies focus only on assigning single code for a given UE.  That is a pool of codes are defined from the set which satisfy Welch bound and assign a code from the set. However, in our simulations we observed that the cross correlation between the sequences varies. For example let’s take an example of code set generated from [1]

snk≜1Kexpjπk+n2Nwk;       1≤k≤K,  1≤n≤N     					   	   (2)

where w(k) is a perfect sequence of period K, that is

k=1Kwkw*k+l=Kδl.	           						            	  	(3)

Lets say N= 50, and K= 4 and say the user 1 is using spreading code 1 from user 2 is allocated spreading code 20, 10 and 5. It can be observed that the cross correlation depends on the spreading code even though it follows the Welch inequality. That means some sequences from the  pool may  not be useful as the correlation might be higher.

Figure 2 Cross correlation between the sequences which satisfy Welch bound

To alleviate this issue, we propose multi code spreading transmitter structure for NOMA applications. Figure 3 shows the proposed structure of multi code spreading transmitter. This is similar to the structure in Figure 1. However, rather than allocating a single code to the UE, the network assigns multiple codes  say M= 4, SF= 4. These codes are generated from the same pool which satisfy the WB. The advantage of this method is that even though the cross correlation is high among certain UEs,, the other codes might save the packet from interference thanks to the FEC.

TB

Transport

Block

encoder

Layer

mapper

IFFT

Precoder

IFFT

Interleaver , scrambler & modulator

Ant Nt

Ant 1

Spreading

Spreading

Figure 3 Proposed structure of Multi Code Spreading Transmitter for NOMA

Proposal 3:   Multi Code Spreading Transmitter Structure should be studied as part of NOMA studies

# DM-RS for NOMA Transmissions

One interesting scenario for NOMA transmission schemes is the design of DM-RS.  We envision two options for DM-RS transmission.

- Option 1 DM-RS transmission without spreading:  In this option, DM-RS structure among the UEs is orthogonal. Hence the interference is less and the channel estimation is improved. In this case, DM-RS is added after the spreading of data same as that of conventional NR DM-RS.

- Option 2 DM-RS transmission with Spreading:  In this option, rather than using orthogonal DM-RS, we can spread the DM-RS symbols similar to the data. Hence the same resource elements can be reused for the UEs participating in the NOMA transmission.

At present, the benefits of these two options are not clear. Hence we recommend RAN1	should study both the techniques for choosing the optimal DM-RS transmission scheme for NOMA.

Proposal 4:  RAN1 should study DM-RS transmission with and without spreading for optimal DM-RS transmission for NOMA

# Conclusions

In this contribution we outlined our views on the transmitter structures for NOMA applications Based on our observations we recommend

Proposal 1:  We prefer two types of spreading codes, linear and non-linear spreading for NOMA transmitter

Proposal 2:  Additional scrambling is not needed to reduce the intra/inter user interference

Proposal 3:   Multi Code Spreading Transmitter Structure should be studied as part of NOMA studies

Proposal 4:  RAN1 should study DM-RS transmission with and without spreading for optimal DM-RS transmission for NOMA

# References

[1] R1-1804823  ‘Transmitter Side Signal Processing Schemes for NOMA’, Qualcomm Incorporated