3GPP TSG RAN WG1 Meeting #93	R1-1807076

Busan, Korea, May 21st – 25th, 2018

Source:	NTT DOCOMO, INC.

Title:	Discussion on comparison methodology and evaluations assumption for NOMA

Agenda Item:	7.4.4

Document for: 	Discussion and Decision

- Introduction

In RAN1#92 meeting and RAN1#92b meeting, the link-level and system-level evaluation assumptions and performance metric have been agreed [1, 2].

Agreements: - Adopt the parameters in the following table for link-level evaluations of NOMA study. Table 1 Link-level evaluation assumptions Parameters; mMTC; URLLC; eMBB; Further specified values. Carrier Frequency; 700 MHz; 700 MHz or 4 GHz; 4 GHz, 700 MHz as optional. Waveform (data part); CP-OFDM and DFT-s-OFDM; CP-OFDM as starting point; CP-OFDM as starting point. Channel coding; URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC; The choice of channel coding here is only for the performance evaluation purpose for NOMA study. Numerology (data part); SCS = 15 kHz, #OS = 14; Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4; SCS = 15 kHz #OS = 14. Allocated bandwidth; 6 as the starting point; 12 as the starting point; 12 as the starting point; For high payload such as 75 bytes, larger number of RBs can be considered.. TBS per UE; At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional; At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded.; At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded.; #bits per RE calculation does not include DMRS overhead (e.g., REs of one every 7 symbols for DMRS would not be used to carry the data). Target BLER for one transmission; 10%; 0.1%; 10%. Number of UEs multiplexed in the same allocated bandwidth; To be reported by companies.; Companies are encouraged to perform evaulations with various number of UEs Note: refined set of numbers of UEs should be further discussed in the next meeting.. BS antenna configuration; 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional; CDL model in 38.901 should be considered for 8Rx. UE antenna configuration; 1Tx. Propagation channel & UE velocity; TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional. Max number of HARQ transmission; 1 as starting point.; 1 as starting point. More values, 2 for URLLC can be used.; 1 as starting point.. Channel estimation; Ideal channel estimation results should be reported for calibration Realistic channel estimation. MA signature allocation (for data and DMRS); Fixed/Random; Proponents report the details of random MA signature allocation (whether without or with collision). Distribution of avg. SNR; Both equal and unequal; Equal; Both equal and unequal; Uniform discrete values for unequal case,, range [x - a, x + a] (dB) with 1 dB step, where x is the average SNR among UEs, and the deviation [a=3]. Timing offset; 0 as starting point. For grant-free without perfect TA, value is TBD. Frequency error; 0 as starting point. The value(s) is TBD.. Traffic model for link level; Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional.. For link level calibration purpose only; OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also.. Note: for the case when a parameter has a “OR” condition, companies are encouraged to evaluate all the corresponding values Agreements: - Adopt the following table as the metrics for NOMA study from link level point of view. - More metrics may be added in the future Performance metrics; BLER vs. per UE SNR at a given pair of {per UE SE, # of UEs} Sum throughput v.s. SNR at given BLER level, for a given pair of {per UE SE, # of UEs} MCL. Implementation related metrics; PAPR/cubic metric Rx complexity and processing latency FFS: Configuration/Scheduling flexibility. Agreements: - Adopt the parameters in the following table for system-level evaluations of NOMA study Table I: System-level evaluation assumptions Parameters; mMTC; URLLC; eMBB; Further specified values. Layout; Single layer - Macro layer: Hex. Grid. Inter-BS distance; [1732]m; [500m]; 200m. Carrier frequency; 700MHz; [4GHz. 700MHz]; 4GHz. Simulation bandwidth; [6] PRBs; 12 PRBs; 12 PRBs. Number of UEs per cell; Companies report. Channel model; UMa in TR 38.901. UE Tx power; Max 23 dBm. BS antenna configurations; 2 Rx or 4 Rx for 700MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; dH = dV = 0.5λ; BS antenna downtilt: companies to report, FFS a single value 4 Rx or 16 Rx for 4GHz; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; 16 ports: (M, N, P, Mg, Ng) = (10, 8, 2, 1, 1), 16 TXRU; dH = 0.5λ, dV = 0.8λ; BS antenna downtilt: companies to report, FFS a single value. BS antenna height; 25m. BS antenna element gain + connector loss; 8 dBi, including 3dB cable loss. BS receiver noise figure; 5dB. UE antenna configuration; 1Tx as starting point. UE antenna height; Follow the modelling of TR 38.901. UE antenna gain; 0dBi as starting point. UE distribution; For mMTC: [20%] of users are outdoors (3km/h), [80%] of users are indoor (3km/h); Users dropped uniformly in entire cell Companies are encouraged to check whether the percentage of UEs whose CL > 144 dB is significant (e.g., 5%) and the CDF of the CL. Further discuss the percentage of outdoor UEs, to be finalized in May meeting. For URLLC [20%] of users are outdoors (3km/h), [80%] of users are indoor (3km/h); Users dropped uniformly in entire cell For eMBB 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell. UE power control; Open loop PC for mMTC. Companies report the PC mechanisms used for eMBB and URLLC.. HARQ/repetition; Companies report (including HARQ mechanisms).. Channel estimation; Realistic. BS receiver; Advanced receiver, with baseline scheme is MU-MIMO (e.g., has the capability of spatial differentiation) Companies to provide analysis of complexity between baseline vs. advanced receivers. Packet dropping criterion. Note: other values can be considered. Agreements: - The traffic model below is used for NOMA evaluations in mMTC scenario: - Packet arrival per UE: Poisson arrival with arrival rate λ; - Packet size: 20~200 bytes Pareto + higher layer protocol overhead of [29] bytes, as defined in TR 45.820 to be the starting point - Other packet sizes are not precluded. - The traffic model for NOMA evaluations in URLLC scenario is to be decided in May meeting. - The traffic model for NOMA evaluations in eMBB scenario is to be decided in May meeting. Agreements: - Adopt the following performance metrics for NOMA study from system level point of view. For mMTC - Focus on normal coverage. - The performance metrics for mMTC include the following: - Higher layer packet drop rate (PDR) vs. offered load. The definition of PDR is FFS: - Offered load can be at least - Higher layer packet arrival rate (PAR) per cell for massive connectivity - CDF of packet drop rate per UE is optional. - CDF of transmission latency is optional. - CDF of the inter-cell interference-over-thermal (IOT) is optional. - Note: companies are encouraged to provide the curve of resource utilization (RU) vs. offered load. For URLLC - The baseline for performance comparison is UL transmission without dynamic link adaptation (i.e., using configured grant type 1 or type 2) - The performance metrics for URLLC include at least the following: - Percentage of users satisfying reliability and latency requirements vs. packet arrival rate (PAR). - CDF of reliability per UE is optional. - CDF of the inter-cell interference-over-thermal (IOT) is optional. - Note: companies are encouraged to provide the curve of resource utilization (RU) vs. PAR. For eMBB - The performance metrics for eMBB include the following: - Metric 1: Higher layer packet drop rate (PDR) vs. offered load. The definition of PDR is FFS: - Offered load can be at least - Higher layer packet arrival rate (PAR) per cell - CDF of packet drop rate per UE is optional. - CDF of transmission latency is optional. - CDF of the inter-cell interference-over-thermal (IOT) is optional. - Note: companies are encouraged to provide the curve of resource utilization (RU) vs. offered load. - Metric 2: UPT vs. offered load. - CDF of the inter-cell interference-over-thermal (IOT) is optional. - CDF of UE perceived throughput is optional - FFS whether or not to have signalling overhead as one performance metric Agreements: Further clarify the LLS parameters: - For ideal channel estimation, DMRS overhead is 1/7 for #OS 7 and 14, and 1/4 for #OS 4. - For a=[3], companies are encouraged to check RAN4 power control rerquirements aim to conclude in RAN1#93 - FFS timing offset for grant-free without perfect TA, - FFS frequency offset

During the Rel-14 study item of NR, the comparison of various NOMA schemes was carried out independently, where the evaluation assumptions and transceiver design depend on each company. For Rel-15 study item of NOMA, to make more valid comparisons among NOMA schemes, the comparison methodology and assumptions should firstly be discussed.

In this contribution, we firstly discuss the comparison methodology. Then, system-level simulation (SLS) evaluation including assumptions and metrics is discussed.

- Comparison methodology

To make an efficient discussion and performance comparison of NOMA in three usage scenarios, the comparison methodology should be discussed.

  - Unified structure at the transmitter

In the study item of NR in Rel-14, 15 NOMA schemes were proposed, which are listed in Table 1. The features of each NOMA scheme are also highlighted in Table 1. With the help of advanced receivers, signals of multiple users can be detected successfully.

Table 1 Existing NOMA schemes ordered by proposed time

| Scheme | Features | Reference |
| --- | --- | --- |
| SCMA | Multi-dimensional constellation, sparse symbol-to-RE mapping pattern | R1-162155 |
| RSMA | Symbol-level scrambler | R1-162202 |
| MUSA | Random complex spreading sequences | R1-162226 |
| PDMA | Symbol-to-RE mapping pattern | R1-162306 |
| LCRS | Low code rate, bit-level interleaver | R1-162385 |
| SSMA | Short spreading sequences | R1-166552 |
| NCMA | Grassmannian spreading sequences | R1-162517 |
| NOMA | Power domain, orthogonal spreading sequences | R1-163111 R1-167392 |
| IGMA | Bit-level interleaver, symbol-level grid mapping pattern | R1-163992 |
| LDS-SVE | Signature Vector Extension, RB-based sparse mapping pattern | R1-164329 |
| LSSA | Low code rate, user-specific bit-level permutation pattern, group RS pattern | R1-164869 |
| NOCA | LTE defined sequences for uplink RS | R1-165019 |
| IDMA | Low code rate, bit-level interleaver | R1-165021 |
| RDMA | Cyclic-shift Repetition, symbol-level interleaver | R1-167535 |
| GOCA | Orthogonal sequences in one group and non-orthogonal sequences for different groups | R1-167535 |

Even though various signatures are employed by different NOMA schemes, the principle of trying to distinguish users at the receiver by different MA signatures is common. Therefore, the transmitter of NOMA schemes can be unified into one common structure. In Rel-14 NR SI, NOMA schemes were summarized by a high-level block diagram, where bit level and symbol level operations are included. Starting from this, a unified structure at the transmitter consisting of five modules is shown in Figure 1. Note that some modules may be designed jointly.

- Bit-level interleaver/scrambler: For NOMA schemes with bit-level operations, user-specific bit-level interleaver/scrambler combined with low code rate channel coding is used to help multi-user detection (MUD) at the receiver.

- Bit-to-symbol mapping: This module maps the coded bits into modulated symbols, where no user-specific design is considered. For NOMA schemes except SCMA, conventional modulation schemes, e.g. BPSK and QPSK etc., are used. To further exploit the gain of this module, multi-dimensional modulation with shaping gain is used in SCMA.

- Symbol stream generation: Most NOMA schemes with symbol-level operations provide user-specific designs in this module, such as symbol-level interleaver and/or scrambler and user-specific spreading sequences as shown in Table 1.

- Power adjustment: Power is a simple but effective approach to distinguish users. Most NOMA schemes assume equal power, i.e., no power adjustment among users. User/Group-specific power is used by NOMA with user grouping.

- Symbol-to-RE mapping: The symbol stream can be mapped into either all the available physical resources (i.e., full mapping) or part of available physical resources (i.e., sparse mapping). Sparse mapping can reduce the collision on each RE at the cost of less transmission resources and user-specific mapping patterns can be designed to facilitate MUD. Full mapping can fully utilize the resources at the cost of more interference.

Existing NOMA schemes in Table 1 usually utilize one or multiple user-specific modules in Figure 1 as its MA signature to help receivers distinguishing multiple users.

Figure 1 Unified structure of NOMA schemes at the transmitter

In addition, NOMA schemes usually assume single-layer structure, i.e., the information bits are processed through the modules in Figure 1. For further performance enhancement, the information bits can be divided into multiple streams, where each stream is encoded with lower coding rate and lower modulation order as shown in Figure 2. By incorporating the multiple streams with different signatures, e.g., power, sequences or scrambler etc., the signals of multiple layers can be detected at the receiver successfully. Besides, if the coding rates of multiple layers are different, the multi-layer structure could provide multiple reliabilities, which can increase the robustness and overall throughput in grant-free transmission.

Figure 2 NOMA schemes with multi-layer

Observation 1: In addition to single-layer structure, multi-layer structure can be considered for existing NOMA schemes.

  - Module-based comparison methodology

In Rel-14 SI, scheme-based performance comparison was employed and the performance of multiple NOMA schemes were obtained independently under different assumptions. This is possible for preliminary study of uplink NOMA. However, the objective of NOMA SI in Rel-15 is to “further progress on the evaluation of non-orthogonal multiple access schemes focusing on uplink, and provide recommendation on the non-orthogonal multiple access scheme(s) to be specified later.” [3]. To this end, transceiver designs for uplink NOMA should be further studied and evaluated to figure out the optimal combinations of modules at transmitter and receivers in each usage scenarios.

Instead of scheme-based performance comparison in Rel-14 SI, module-based performance comparison can be considered in Rel-15 SI. To investigate the performance gain of NOMA modules, the performance of baseline system without any NOMA schemes should be firstly evaluated. Then, the optimal NOMA scheme can be obtained by exhaustive searching on all the possible combinations of NOMA modules. The performance gain of each NOMA module is evaluated for all use scenarios. The optimal NOMA module and signature can be found by not only the performance gain but also latency and complexity at the transmitter and receiver, etc. With the above module-based comparison methodology, the performance gain of each NOMA module and signature can be well studied.

Proposal 1: Module-based comparison methodology should be considered and further discussed in NOMA SI in Rel-15. FFS whether to treat some modules independently or jointly.

- LLS evaluation assumptions and metrics

During the RAN1#92 and RAN1#92b, some remaining issues for LLS are as follows.

- Unequal SNR distribution

- Timing/frequency offset

- Configuration/Scheduling flexibility

For SNR distribution, although in RAN4, there is some related requirement, it is not easily and directly to be derived the distribution from RAN4 requirement. In this case, the current assumption a = 3 is a suitable value which covers the main part of unequal SNR distribution.

Proposal 2: Confirm the current assumption a = 3 for unequal SNR distribution.

For timing offset, both synchronous and asynchronous operations can be studied during the NOMA SI. For synchronous cases, as the related receiver approach is well studied, the synchronous case can be applied as the starting point. For asynchronous cases, although it is better to use the same assumption for different schemes, as the definition of asynchronous operations is not quite clear, and different companies may have different understanding of the asynchronous operations, it is better to let proponents to report such asynchronous settings if related results are proposed.

For frequency offset, similar with timing offset, 0 can be used as the starting point, and companies can report the values if other values are applied.

Proposal 3: Take synchronous cases (timing offset less than CP) as the starting point. Companies can report related setting of proposed asynchronous cases and corresponding results. Companies can report the value of frequency offset if non-zero values are applied.

For configuration/scheduling flexibility, as discussed in [4], some parameters can be applied to measure the configuration and scheduling flexibility. As in NOMA transmission cases, the number of UEs and the available time-frequency resource may be changed, different overloading factor may be needed for different scenarios, the supportable overloading and spreading factor should be considered as one important metric for configuration and scheduling flexibility.

Proposal 4: The supportable overloading and spreading factor should be considered as one metric for configuration and scheduling flexibility.

On the other hand, different configurations of DMRS and NOMA signature will lead to different performance due to different probability and different types of collision. The collision reduction capability should be also considered as one metric for configuration and scheduling flexibility.

Proposal 5: The collision reduction capability should be considered as one metric for configuration and scheduling flexibility.

- SLS evaluation assumptions and metrics

During the RAN1#92b, most of the SLS evaluation assumptions have been agreed. The main remaining issues for SLS evaluation assumption are as follows.

- Packet dropping criterion for mMTC/URLLC/eMBB

- Traffic model for NOMA evaluations in URLLC/eMBB

- Baseline schemes for mMTC/eMBB

For packet dropping criterion, generally for mMTC/eMBB, the packet dropping can be defined as if this packet failed to be successfully received by destination receiver beyond “Packet dropping timer”. The packet dropping timer for mMTC/eMBB can be set to 1s as the starting point.

Proposal 6: For mMTC/eMBB, the packet dropping is defined by packet dropping timer. The timer is set to 1s as the starting point.

For traffic model for URLLC/eMBB, as in Rel-14 SI, non-full buffer FTP traffic model 3 has been used in many topics, it is suitable to be taken as the starting point.

Proposal 7: For URLLC/eMBB, non-full buffer FTP traffic model 3 can be used as the starting point.

For baseline schemes for eMBB, as Rel-15 NR has already been well studied, both grant-based and grant-free Rel-15 NR schemes can be directly used as the baseline as the starting point. Considering further potential scenarios in further release, grant-free scheme as discussed in Rel-14 NR SI can be further studied.

Proposal 8: For eMBB, both grant-based and Rel-15 NR grant-free schemes can be used as the baselines as the starting point. FFS Rel-14 grant-free schemes.

- Conclusion

In this contribution, we discussed the comparison methodology, link-level, system-level evaluation assumptions and performance metrics for uplink non-orthogonal multiple access in Rel-15. The following observations and proposals are obtained:

Observation 1: In addition to single-layer structure, multi-layer structure can be considered for existing NOMA schemes.

Proposal 1: Module-based comparison methodology should be considered and further discussed in NOMA SI in Rel-15. FFS whether to treat some modules independently or jointly.

Proposal 2: Confirm the current assumption a = 3 for unequal SNR distribution.

Proposal 3: Take synchronous cases (timing offset less than CP) as the starting point. Companies can report related setting of proposed asynchronous cases and corresponding results. Companies can report the value of frequency offset if non-zero values are applied.

Proposal 4: The supportable overloading and spreading factor should be considered as one metric for configuration and scheduling flexibility.

Proposal 5: The collision reduction capability should be considered as one metric for configuration and scheduling flexibility.

Proposal 6: For mMTC/eMBB, the packet dropping is defined by packet dropping timer. The timer is set to 1s as the starting point.

Proposal 7: For URLLC/eMBB, non-full buffer FTP traffic model 3 can be used as the starting point.

Proposal 8: For eMBB, both grant-based and Rel-15 NR grant-free schemes can be used as the baselines as the starting point. FFS Rel-14 grant-free schemes.

References

- Chairman notes, 3GPP RAN1 #92.

- Chairman notes, 3GPP RAN1 #92bis.

- RP-171043, ZTE, “Revision of Study on 5G Non-orthogonal Multiple Access”, June. 2017

- R1-1804823, Qualcomm Incorporated, “Transmitter Side Signal Processing Schemes for NOMA”, April 2018