3GPP TSG RAN WG1 Meeting #93	                                     R1-1807193

Busan, Korea, May 21th – May 25th, 2018

Agenda item:		7.1.2.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Joint L1-RSRP Beam Reporting for SSB and CSI-RS

Document for:		Discussion and Decision

# 1 Introduction

In this contribution, remaining open issues of joint SSB and CSI-RS L1-RSRP beam reporting are discussed.

# 2 L1-RSRP Beam Reporting for SS-block + CSI-RS

The specification TS 38.214, Sec.5.2.1.4 supports UE to be configured for L1-RSRP reporting with both CSI-RS and SSB resources. However, in TS 38.214, technical details are missing how to perform L1-RSRP reporting when both SSB and CSI-RS resources are configured for UE. Therefore, in the following further details for aforementioned configuration to perform L1-RSRP reporting are provided.

Figure 3 shows an example of jointly quasi-colocated (QCL)led SSB-CSI-RS beam report pair computation. Here, wide-beams are associated with SSB resources and narrow beams to CSI-RS resources. Single antenna port SSB resources are spatially QCLed with a single/multiple set(s) of (one or more) single or two antenna port CSI-RS resources. If the value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ UE reports CRIs and L1-RSRP values associated with CRIs based on based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs over SSB and CSI-RS resources being spatially QCL:ed with each others. If the value of higher-layer parameter ReportQuantity, set to ‘SSBRI/Joint-RSRP’, UE reports SSBRIs and L1-RSRP values associated with SSBRIs based on based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs over SSB and CSI-RS resources being spatially QCL:ed with each others. By following above description, individual beam reports can be obtained with already specified beam reporting format defined in TS 38.212.

Figure 3 An example of a joint QCL:ed SSB-CSI-RS beam resource pair.

Proposal 1: In NR Rel-15, support the following joint SSB and CSI-RS L1-RSRP beam reporting configuration:

- If the value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ UE reports CRIs and L1-RSRP values associated with CRIs based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

- If the value of higher-layer parameter ReportQuantity, set to ‘SSBRI/Joint-RSRP’, UE reports SSBRIs and L1-RSRP values associated with SSBRIs based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

Proposal 2: Text proposal to 3GPP TS 38.214:

==========================Text proposal starts===============================

##### 5.2.1.4.2	Report Quantity Configurations

< Unchanged parts are omitted >

If the UE is configured with a CSI-ReportConfig with the higher layer parameter ReportQuantity set to 'CRI/RSRP' or ‘SSBRI/RSRP’ or ‘CRI/Joint-RSRP’ or ‘SSBRI/Joint-RSRP’

< Unchanged parts are omitted >

##### 5.2.1.4.3	L1-RSRP Reporting

< Unchanged parts are omitted >

When value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ or SSBRI/Joint-RSRP’ the UE shall jointly measure L1-RSRP with CSI-RS and SSB resources that are on the same OFDM symbols and spatially QCL:ed.

If the value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ UE reports CRIs and L1-RSRP values associated with CRIs based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

If the value of higher-layer parameter ReportQuantity, set to ‘SSBRI/Joint-RSRP’, UE reports SSBRIs and L1-RSRP values associated with SSBRIs based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

< Unchanged parts are omitted >

=========================== Text Proposal Ends==============================

# 3 Conclusions

In this contribution, the remaining open issues of joint SSB and CSI-RS L1-RSRP beam reporting have been considered. Based on the discussion, the following proposals have been made:

Proposal 1: In NR Rel-15, support the following joint SSB and CSI-RS L1-RSRP beam reporting configuration:

- If the value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ UE reports CRIs and L1-RSRP values associated with CRIs based on based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

Proposal 2: Text proposal to 3GPP TS 38.214:

==========================Text proposal starts===============================

##### 5.2.1.4.2	Report Quantity Configurations

< Unchanged parts are omitted >

If the UE is configured with a CSI-ReportConfig with the higher layer parameter ReportQuantity set to 'CRI/RSRP' or ‘SSBRI/RSRP’ or ‘CRI/Joint-RSRP’ or ‘SSBRI/Joint-RSRP’

< Unchanged parts are omitted >

##### 5.2.1.4.3	L1-RSRP Reporting

< Unchanged parts are omitted >

When value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ or SSBRI/Joint-RSRP’ the UE shall jointly measure L1-RSRP with CSI-RS and SSB resources that are on the same OFDM symbols and spatially QCL:ed.

If the value of higher-layer parameter ReportQuantity, set to CRI/Joint-RSRP’ UE reports CRIs and L1-RSRP values associated with CRIs based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

If the value of higher-layer parameter ReportQuantity, set to ‘SSBRI/Joint-RSRP’, UE reports SSBRIs and L1-RSRP values associated with SSBRIs based on the higher-layer parameter nrofReportedRS largest computed joint L1-RSRPs with SSB and CSI-RS resources being spatially QCL:ed with each others.

< Unchanged parts are omitted >

=========================== Text Proposal Ends==============================