3GPP TSG-RAN WG1 Meeting #93	Tdoc R1-1807682

Busan, South Korea, May 21-25 2018

Agenda Item:	7.1.3.3.5

Source:	Ericsson

Title:	Summary for AI 7.1.3.3.5

Document for:	Discussion/Decision

# 1	Introduction

This document summarizes the contributions submitted to RAN1#93 meeting under AI 7.1.3.3.5.

# 2	Discussion

- [1] proposes that transmit buffer limitation should be according to the dynamically scheduled PDSCH/PUSCH duration for a TB, because otherwise LBRM can effectively be disabled in some cases. Dynamic LBRM was discussed during last year (e.g. transmit buffer limitation according to active BWP size) and RAN1 agreed that TBS_LBRM is based on configured/fixed parameters and not dynamically varying parameters. The proposal in [1] alters basic LBRM functionality at late stage and given the spec is clear and stable and nothing seems to be broken, recommend no spec change.

Proposal: No spec change.

- [2] discusses the technical benefits of uplink LBRM feature. The proposal in [1] (recommending uplink LBRM to be mandatory with signaling) can be handled in UE feature discussion.

Proposal: Discuss in UE feature.

# References

R1-1805889	Remaining issues on soft buffer management	Huawei, HiSilicon

R1-1807259	Remaining issues of Soft buffer management	Ericsson