3GPP TSG RAN WG1 Meeting #94			R1-1809035

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	6.2.2.5

Source: 	Qualcomm Incorporated

Title: 	Support of quality report in msg3 for non-anchor

Document for:	Discussion and Decision

# Background

In RAN#80, a new work item on NB-IoT enhancements was approved (RP-181451) with the following objective:

Improved multi-carrier operation:

- Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2]

In this contribution, we present our views on how to enable msg3 quality reporting for non-anchor access.

# Differences with reporting on anchor carrier

The key differences between quality reporting in anchor and non-anchor are:

- In Rel-14, the UE is not required to recompute msg3 during random access procedure. Rel-15 UEs supporting EDT shall be capable of doing it. Thus, for Rel-16, we should not restrict ourselves to solutions that require constant msg3 payload during random access procedure.

- In anchor carrier NRS is always present. In non-anchor carrier NRS will be present only during the random access response window.

Thus, in the following we propose a methodology and details in line with these differences.

# Methodology

Similar to the CQI definition, we propose to define the DL quality in a similar way the CQI is defined in TS 36.213 for LTE:

-	A single PDSCH transport block with a combination of modulation scheme and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding 0.1.

We make the following proposal on the definition of DL quality reporting for non-anchor carriers.

Proposal 1: The “DL quality reporting in msg3” is defined as “The lowest number of repetitions Rdesired needed for a NPDCCH transmitted in the ‘NPDCCH reference resource’ to be decoded with an error probability not exceeding 0.01”.

# Definition of reference resource and measurement resources

Based on the principle of proposal 1, and in line with what is done for LTE, we need to define at least the following:

- The “reference resource”, i.e., where the placement of the virtual NPDCCH is.

- The “measurement resources”, i.e., where the UE is allowed to measure to determine the DL quality.

For the reference resource, we propose to make the NPDCCH reference resource equal to the first candidate in the entire search space. This has the following advantages with respect to other options:

- The UE always monitors this candidate (it is impossible to receive a random access response before this).

- The NPDCCH reference resource is fixed in time (tying it to the actual received NPDCCH would make the definition more cumbersome).

  - If the NPDCCH reference resource is not fixed in time, the UE may need to continuously compute the DL quality. With this approach, the UE just computes the DL quality once.

- It doesn’t require the UE to make additional wake-ups to measure.

The definition of this NPDCCH reference resource is depicted in Figure 1.

Figure 1 NPDCCH reference resource definition with respect to NPDCCH/NPDSCH/NPUSCH.

Proposal 2: The ‘NPDCCH reference resource’ is defined as ‘A candidate mapped onto the Type-2 common search space with aggregation level of AL=2 and starting in subframe s0”

Proposal 3: The subframe s0 is the first subframe of the first candidate of the first Type-2 search space in the random access response window of the current random access attempt.

On the subframes used for measurement, there is no reason to restrict these, and it should be left to UE implementation. Note that the UE is aware of which subframes contain NRS and which ones do not, the UE will only use NRS when it is present. This being said, it is not desirable to force the UE to wake up additional times just for the sake of having a more accurate quality report, and RAN4 requirements should take this into account. Also, the case where NPDCCH is early terminated and the UE goes to sleep before the entire duration should be taken into account when defining this requirement.

Proposal 4: The computation of Rdesired  is based on “an unrestricted observation interval in time, and an unrestricted observation interval in frequency”.

# Possible set of values for Rdesired

The current specification defines as follows the maximum number of repetitions for random access:

npdcch-NumRepetitions-RA-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

In total, 12 values are allowed. Thus, if we want to have a complete representation of the desired number of repetitions Rdesired we would need to reserve 4 bits in msg3, which seems excessive.

Observation 1: Reporting all the possible values of Rdesired requires 4 bits in msg3.

One observation is that, for a given coverage level, it is extremely unlikely that the UE will report a value of Rdesired that is several orders of magnitude larger/smaller than the configured Rmax for the corresponding coverage level. For example, consider the simple case of Rmax = 1. Then, it is very unlikely that the UE reports 2048 repetitions.

Observation 2: For a given coverage level, the expected set of Rdesired should be much smaller than the complete set of R.

Thus, we propose to link the set of candidates of Rdesired to the configure maximum number of repetitions for NPDCCH for the corresponding coverage level. One example table is shown below for the case of 2 bit reporting.

Table 1 Example set of candidates for Rdesired for the case of 2 bit reporting (‘00’ means not supported)

| Rmax | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘01’ | 1 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 512 |
| ‘10’ | 2 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 1024 |
| ‘11’ | 4 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 2048 |

Proposal 5: The DL quality reporting in msg3 uses N bits (FFS: N=2 or 3). The set of 2N-1 candidates for Rdesired depends on npdcch-NumRepetitions-RA for the corresponding coverage level.

- The value of N is selected depending on feedback from RAN2/RAN4 on overhead (RAN2) and UE accuracy (RAN4).

# Summary of proposals

In this contribution we presented our views on DL quality reporting for NB-IoT. We made the following proposals and observations.

Proposal 1: The “DL quality reporting in msg3” is defined as “The lowest number of repetitions Rdesired needed for a NPDCCH transmitted in the ‘NPDCCH reference resource’ to be decoded with an error probability not exceeding 0.01”.

Proposal 2: The ‘NPDCCH reference resource’ is defined as ‘A candidate mapped onto the Type-2 common search space with aggregation level of AL=2 and starting in subframe s0”

Proposal 3: The subframe s0 is the first subframe of the first candidate of the first Type-2 search space in the random access response window of the current random access attempt.

Proposal 4: The computation of Rdesired  is based on “an unrestricted observation interval in time, and an unrestricted observation interval in frequency”.

Observation 1: Reporting all the possible values of Rdesired requires 4 bits in msg3.

Observation 2: For a given coverage level, the expected set of Rdesired should be much smaller than the complete set of R.

Proposal 5: The DL quality reporting in msg3 uses N bits (FFS: N=2 or 3). The set of 2N-1 candidates for Rdesired depends on npdcch-NumRepetitions-RA for the corresponding coverage level.

- The value of N is selected depending on feedback from RAN2/RAN4 on overhead (RAN2) and UE accuracy (RAN4).