3GPP TSG RAN WG1 Meeting #94															R1- 1809369

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:		6.1.8

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction to AUL-UCI resource allocation in 36.213

Document for:		Discussion and Decision

# 1	Introduction

This document relates to WI “Enhancements to LTE operation in unlicensed spectrum”, and more specifically on transmission of AUL UCI. The following was agreed based on email discussion [91-LTE-11]:

Proposal 3-2b: A new beta offset parameter is defined for AUL-UCI:

However, some of the further details related to beta-offset for AUL-UCI were not agreed.

Following parameter has been captured into 36.331:

betaOffsetAUL Parameter: see TS 36.213 [23,X].

Moreover, 36.212, section 5.2.2.6A defines the following:

…

-	and , where  shall be determined according to [3] depending on the number of transmission codewords for the corresponding AUL PUSCH, and where  is the modulation order of transport block "x".

…

However, corresponding definition of is missing from 36.213. In this contribution we summarize the open issues related to and provide related proposals to complete the work.

# 2	Beta-offset for AUL-UCI

Although it has been agreed to define a beta offset specific to AUL-UCI, it has not been agreed what values it should take. AUL-UCI can be seen as critical control information, since failure in decoding AUL-UCI results in erroneous reception of the associated PUSCH and hence a retransmission of data. To ensure reliable system operation, we see that AUL-UCI should have similar reliability as HARQ-ACK, and consequently, AUL-UCI could use the same beta offset vales as defined for HARQ ACK in Table 8.6.3-1 of 36.213, v 15.2.0.

Proposal 1: The value range for AUL-UCI is the same as that of HARQ ACK, defined in Table 8.6.3-1 of 36.213, v 15.2.0.

We note that for HARQ-ACK, RI, and CSI, there are separate RRC-configured offset values defined for single-and multi-codeword transmission. On the other hand, presently there is no similar agreement in place of AUL-UCI. Given the fairly static nature of AUL transmission, we see that it is sufficient to control AUL UCI transmissions with a single  offset parameters in both single and multi-codeword cases.

Proposal 2: The same parameter is used for AUL-UCI in both single- and multi-codeword cases.

# 3. 	Text Proposal

In the following we provide a text proposal implementing the proposals in Section 2 of this contribution.

TS 36.213, v. 15.2.0:

### 8.6.3	Control information MCS offset determination

Offset values are defined for single codeword PUSCH transmission and multiple codeword PUSCH transmission. Single codeword subframe-PUSCH transmission offsets ,  and  shall be configured to values according to Table 8.6.3-1,2,3 with the higher layer signalled indexes  if the UE transmits no more than 22 HARQ-ACK bits on a PUSCH or if  is not configured, , and , respectively. Single codeword PUSCH transmission offset shall be configured to values according to [Table 8.6.3-1] with the higher layer signalled index  if the UE transmits more than 22 HARQ-ACK bits on a PUSCH and  is configured.

AUL PUSCH transmission offset for AUL-UCI βoffsetAUL-UCI shall be configured to values according to Table 8.6.3-1 with the higher layer signalled index IoffsetAUL-UCI.

If the UE is configured with higher layer parameter ul-TTI-Length,

-	slot-PUSCH transmission offsets, , and  shall be configured via higher layer parameters betaOffsetSlot-ACK-Index, betaOffsetSlot-RI-Index, and betaOffsetSlot-CQI-Index to values according to Table 8.6.3-1, Table 8.6.3-2, and Table 8.6.3-3 with the higher layer signalled indexes  if the UE transmits no more than 22 HARQ-ACK bits on a PUSCH, and , and respectively. Slot-PUSCH transmission offset shall be configured to values according to Table 8.6.3-1 with the higher layer signalled index  if the UE transmits more than 22 HARQ-ACK bits on a slot-PUSCH.

-	subslot-PUSCH transmission offsets, , , , and  shall be configured via higher layer parameters betaOffsetSubslot-ACK-Index, betaOffset2Subslot-ACK-Index, betaOffsetSubslot-RI-Index, betaOffset2Subslot-RI-Index, and betaOffsetSubslot-CQI-Index to values according to Table 8.6.3-1, Table 8.6.3-2, and Table 8.6.3-3 with the higher layer signalled indexes ,  if the UE transmits no more than 22 HARQ-ACK bits on a PUSCH, and ,  and respectively. Subslot-PUSCH transmission offset and shall be configured to values according to Table 8.6.3-1 with the higher layer signalled index , and  if the UE transmits more than 22 HARQ-ACK bits on a subslot-PUSCH. If the Beta offset indicator field in PDCCH/SPDCCH with DCI format 7-0A/7-0B is set to 0, , and , otherwise , and .

Multiple codeword PUSCH transmission offsets ,  and  shall be configured to values according to Table 8.6.3-1,2,3 with the higher layer signalled indexes  if the UE transmits no more than 22 HARQ-ACK bits on a PUSCH or if  is not configured,  and , respectively. Multiple codeword PUSCH transmission offset shall be configured to values according to [Table 8.6.3-1] with the higher layer signalled index  if the UE transmits more than 22 HARQ-ACK bits on a PUSCH and  is configured.

If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell , and if a subframe belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12, then for that subframe, the UE shall use

-	the higher layer indexes , , and  in place of , , , and  respectively in Tables 8.6.3-1,2,3, to determine ,  and  respectively for single codeword PUSCH transmissions, and

-	the higher layer indexes , ,  and  in place of , ,  and  respectively in Tables 8.6.3-1,2,3, to determine ,  and  respectively for multiple codeword PUSCH transmissions.

Table 8.6.3-1: Mapping of HARQ-ACK offset or AUL-UCI offset values and the index signalled by higher layers

| or or IoffsetAUL-UCI | or βoffsetAUL-UCI |
| --- | --- |
| 0 | 2.000 |
| 1 | 2.500 |
| 2 | 3.125 |
| 3 | 4.000 |
| 4 | 5.000 |
| 5 | 6.250 |
| 6 | 8.000 |
| 7 | 10.000 |
| 8 | 12.625 |
| 9 | 15.875 |
| 10 | 20.000 |
| 11 | 31.000 |
| 12 | 50.000 |
| 13 | 80.000 |
| 14 | 126.000 |
| 15 | 1.0 |