3GPP TSG RAN WG1 Meeting #94bis	                                       R1-1811879

Chengdu, China, October 8th– 12th, 2018

Source:	Convida Wireless

Title:	Maintenance for Physical random access channel

Agenda Item:	7.1.1.2

# Document for:     Discussion and Decision

# Background

Revision of R1-1811832.

This document summarizes the submitted contributions for RAN1#94bis for AI 7.1.1.2, " Maintenance for Physical random access channel and procedure". Two issues have been identified in the submitted tdocs for this agenda item.

- Network Synchronization Requirement for Handover

- Msg1 transmission using the combination of 30 kHz Msg1 SCS and 5 MHz BW

- Correction to the CP length for preamble format 1

# Network Synchronization Requirement for Handover

Based on the proposals the following summary is made:

In total eight companies have submitted proposals regarding RACH configuration for handover. Those proposals are added in section 2.1 to 2.8. In RAN1#93 we have reach agreement for paired spectrum and L = 4, the UE assumes an absolute timing difference less than 153600Ts. What remains to agree upon is for unpaired spectrum and L = 4, 8 and 64.

- For L = 8 (FR1) most of the companies propose to have UE assumption about the absolute timing difference

- For L = 64 (FR2) most of the companies propose to have no UE assumption about the absolute timing difference

- For unpaired spectrum and same frequency range there is no need to specify any restriction as network is assumed to be synchronized for unpaired spectrum within same frequency layer.

- For inter frequency handover purpose and unpaired spectrum the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts for L = 4

- For inter frequency handover and target cell in unpaired spectrum uses L = 8 and 64, some more discussion is needed regarding to specify a UE assumption about the absolute timing difference or not.

  - For L = 8 there are two alternatives

    - Use timing assumption of +- 2.5ms (76800Ts)

    - Use timing assumption of +- 5ms (153600Ts), (by using other RS than SSB, e.g if using CSI-RS if configured with 5ms periodicity and supported by the UE)

  - For L = 64 there are two main alternatives

    - Up to UE implementation to derive the SFN and frame boundary. This timing information can later be used for handover purpose for the target cell on that frequency layer

    - The UE timing assumption on XTs

      - X = +-2.5ms (UE need to use another RS than SSB to derive the timing, e.g CSI-RS if configured with 5ms periodicity and supported by the UE)

      - X = is a small value

- Suggestion is that we start to agree on the case with L = 4 followed by L = 8 and finally the case L = 64.

In RAN#81 (RP-182071) a CR on using L = 8 in unpaired spectrum from 2.4GHz to 6GHz was approved under the condition that discussions in RAN1 in inter-frequency handover, solutions based on decoding PBCH in neighbor cell are excluded.

RAN#81: RP-182071 CR 38.213 on Change of carrier frequency lower boundary supporting maximum 8 SS/PBCH blocks CMCC conclusion: This CR is approved under the condition that, in upcoming WGs discussion on inter-frequency RRM/handover, solution(s) based on neighbor cell’s PBCH decoding is excluded The document was approved.

Offline Proposals for section 6.3.3.2 in 38.211

For L = 4

For handover purposes to a target cell in paired or unpaired spectrum where the target cell uses L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in Section 8.1 of [38.213] is not equal to 10ms

For handover purposes within the same frequency range in paired spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For L = 8

Offline proposal:

For inter frequency handover purposes where the source cell is either in paired or unpaired spectrum and the target cell is in unpaired spectrum and uses L=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts

Concerns was raised from two companies.

- For handover from paired to unpaired the +-2.5ms requirement is too high

- Solution to acheive a +-5ms timing requirement

- Adding the following text in section 6.3.3.2 in 38.211, valid for all cases

- For handover purpose, a UE is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter.

- For inter frequency handover purposes in unpaired spectrum where the target cell uses L=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in Section 8.1 of [38.213] is not equal to 10ms

  - Note: require that UE uses other RS than SSB, e.g using CSI-RS for tracking or beam management. The configuration of CSI-RS for tracking is obtained in handover command.

For L = 64

Offline conclusion:

- For intra frequency handover to a target cell in FR2 in same frequency layer, the UE can determine the start of the frame and the SFN based on the timing of the source cell or a cell on the same frequency as the target cell.

  - UE can always assume that the cells are synchronized at +-3us as per RAN4 agreement

- For inter frequency handover to a target cell in FR2, it is up to UE implementation to determine the start of the frame and the SFN on the target cell.

Note: No text is needed in 38.211 for the case of L = 64.

Concerns from one company, proposed alternative solution

- For inter frequency handover purposes in unpaired spectrum where the target cell uses L=64, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than XTs

    - Options for value of X

    - Option 1: X = 76800

    - Option 2: X smaller than 76800

  - Require that UE to use other RS than SSB, e.g using CSI-RS for tracking or beam management. The configuration of those RS is obtained in handover command.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

For handover purposes within the same frequency range in paired spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in Section 8.1 of [38.213] is not equal to 10ms if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For inter frequency handover purposes to target cell in unpaired spectrum where the target cell uses L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in Section 8.1 of [38.213] is not equal to 10ms

## Huawei

Proposal 1: Adopt the text proposal in Appendix A.

# Appendix A: TP of RACH configuration for handover (TS 38.211, f30)

-------------- Start of Text proposal for 38.211 ---------------------------

#### 6.3.3.2	Mapping to physical resources

The preamble sequence shall be mapped to physical resources according to

where  is an amplitude scaling factor in order to conform to the transmit power specified in [5, TS38.213], and  is the antenna port. Baseband signal generation shall be done according to clause 5.3 using the parameters in Table 6.3.3.1-1 or Table 6.3.3.1-2 with  given by Table 6.3.3.2-1.

Random access preambles can only be transmitted in the time resources given by the higher-layer parameter prach-ConfigurationIndex according to Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104].

Random access preambles can only be transmitted in the frequency resources given by the higher-layer parameter msg1-FrequencyStart. The PRACH frequency resources nRA∈0,1,…,M-1, where M equals the higher-layer parameter msg1-FDM, are numbered in increasing order within the initial active uplink bandwidth part during initial access, starting from the lowest frequency. Otherwise, nRA are numbered in increasing order within the active uplink bandwidth part, starting from the lowest frequency.

For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed:

-	15 kHz for FR1

-	60 kHz for FR2.

For handover purposes within the same frequency range in paired spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For inter frequency handover purposes to target cell in unpaired spectrum uses L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For inter frequency handover purposes in unpaired spectrum with target cell uses L=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For inter frequency handover from serving cell in paired spectrum to target cell configured with CSI-RS in unpaired spectrum uses L=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

-------------- End of Text proposal for 38.211 ---------------------------

## ZTE

We can confirm the draft text proposal but several values in the table should be revised just like below:

Table 6.3.3.2-x. Ntime_diff  of the absolute value of the time difference between radio frame 𝑖 in the current cell and radio frame 𝑖 in the target cell

|  |  | Frequency range for target cell |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Paired spectrum with L=4 | Unpaired spectrum with L=4 | Unpaired spectrum with L=8 | Unpaired spectrum with L=64 |
| Frequency range for the current cell | Paired spectrum with L=4 | 153600𝑇s | 153600𝑇s | 153600𝑇s76800𝑇s | 153600𝑇s2SSB symbols or not specified |
|  | Unpaired spectrum with L=4 | 153600𝑇s | 76800𝑇s153600𝑇s for different frequency layer | 76800𝑇s | 76800𝑇s2SSB symbols or not specified |
|  | Unpaired spectrum with L=8 | 153600𝑇s | 76800𝑇s153600𝑇s | 76800𝑇s for different frequency layer | 76800𝑇s2SSB symbols or not specified |
|  | Unpaired spectrum with L=64 | 153600𝑇s | 76800𝑇s153600𝑇s | 76800𝑇s | 76800𝑇s2SSB symbols or not specified for different frequency layer |

Proposal 1: Revised the values in the above table and confirm the draft TP for RACH in handover.

## LGE

Proposal 1:

- If the below assumption is agreed, we can agree the text proposal.

  - UE assume that the tolerance of frame boundary between cells is +/-5ms for two cases (i.e. FDD to TDD, TDD to FDD), and is +/-2.5ms for TDD to TDD case.

  - Text Proposal for 38.211, section 6.3.3.2

===== Start of Text Proposal for 38.211, section 6.3.3.2  ====

For handover purposes, the UE may assume the absolute value of the time difference between radio frame 𝑖 in the current cell and radio frame 𝑖 in the target cell is less than Ttime_diff  in Table 6.3.3.2-x, if any of the following conditions are fulfilled:

- the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where 𝑥 ≠ 1

- the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where 𝑥 = 1 and the association pattern period in Table 8.1-1 of [38.213] is not equal to 1

Table 6.3.3.2-x. Ttime_diff of the absolute value of the time difference between radio frame 𝑖 in the current cell and radio frame 𝑖 in the target cell

|  |  | Frequency range for target cell |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Paired spectrum with L=4 | Unpaired spectrum with L=4 | Unpaired spectrum with L=8 | Unpaired spectrum with L=64 |
| Frequency range for the current cell | Paired spectrum with L=4 | 153600𝑇s | 153600𝑇s | 153600𝑇s | 153600𝑇s |
|  | Unpaired spectrum with L=4 | 153600𝑇s | 76800𝑇s for different frequency layer | 76800𝑇s | 76800𝑇s |
|  | Unpaired spectrum with L=8 | 153600𝑇s | 76800𝑇s | 76800𝑇s for different frequency layer | 76800𝑇s |
|  | Unpaired spectrum with L=64 | 153600𝑇s | 76800𝑇s | 76800𝑇s | 76800𝑇s for different frequency layer |

===== End of Text Proposal ====

## CATT

Proposal 1:

- For intra-frequency handover within the unpaired spectrum, UE assumes the timing between source and target cells are fully synchronized, including the SFN.

- For other intra-frequency and inter-frequency handovers within the unpaired spectrum, or within the  paired spectrum, or between unpaired spectrum and paired spectrum, UE assumes the absolute value, Ntime-diff,  of the time difference between the radio frame i of the current cell and the radio frame i of in the target cell as follows:

  - if L=4 for the target cell, UE assumes Ntime-diff=153600𝑇s

  - if L=8 for the target cell, UE assumes Ntime-diff=76800𝑇s

  - if L=64 for target cell, UE may not assume Ntime-diffis within a certain range.

## Intel

Proposal 3:

- For TDD with L =4, define the same network synchronization requirement for HO as FDD case.

- For TDD with L > 4, define the network synchronization requirement for HO such that cells are synchronized within 2.5msec.

- Adopt the following text proposal.

-------------- Start of Text proposal for 38.211 ---------------------------

6.3.3.2	Mapping to physical resources

-- omitted --

For handover purposes within the same frequency range in paired and unpaird spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For handover purposes within the same frequency range in unpaird spectrum with L>4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts.

-------------- End of Text proposal for 38.211 ---------------------------

## Samsung

Proposal 1: the following text proposal is proposed:

<omitted >

For handover purposes within the same frequency range in paired spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For a target cell in inter-frequency and unpaired spectrum, the UE may for handover purpose assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell is less than 153600 Ts and 76800 Ts for L of target cell equals 4 and 8, respectively, if the any of the following conditions of the PRACH configurations are met:

The association pattern period as determined in [8, TS38.213] is not equal to 10ms;

<omitted>

## Nokia

Proposal 1:

For unpaired spectrum, and L = 4,

- the UE may for handover purpose assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell is less than 153600 Ts, if the any of the following conditions of the PRACH configurations are met:

  - for PRACH configurations in which x is not equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3

  - for PRACH configurations in which x is equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3 and the association period in Table 8.1-1 of [38.213] is not equal to 1

Proposal 4:

For unpaired spectrum, L = 8

- The UE may for handover purpose assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell is less than 76800 Ts.

Proposal 3:

For handover to a target cell in FR2, when useServingCellTimingForSync is set to TRUE, the UE can determine the start of the frame and the SFN based on the timing of the source cell or a cell on the same frequency as the target cell. When useServingCellTimingForSync is not set to TRUE, the UE is expected to decode the BCH channel to determine the start of the frame and the SFN.

## Ericsson

- The already agreed requirements for time difference between cells in paired spectrum with L=4 also applies to unpaired spectrum

- Performance of decoding the PBCH should be sufficient to provide timing of neighbouring cell when L=8

- For FR2, if the UE knows the timing of the target cell from reading the PBCH during neighbour measurements

# Msg1 transmission in the combination of 30 kHz Msg1 SCS and 5 MHz BW

Summary:

RAN1 has agreed to support 30 kHz SCS and length 139 Zadoff Chu sequence for short preamble format based PRACH Msg1 transmission. RAN4, on the other hand, has agreed to use 505 kHz guard band for 5 MHz carrier bandwidth [1]. Hence, as shown below, RAN4 has limited the maximum transmission bandwidth to 11 RBs for the combination of 5 MHz carrier bandwidth and 30 kHz SCS [2].

Table 5.3.2-1 of 38.101: Maximum transmission bandwidth configuration NRB [2]

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 121 | 135 |

However, PRACH Msg1 uses a length 139 sequence and this clearly occupies more BW than 11 RBs (= 132 subcarriers). Besides, the summation of RACH preamble bandwidth and guard band, in this scenario, becomes 5.18 MHz and this is clearly greater than the 5 MHz carrier bandwidth.

Hence, 30 kHz SCS based PRACH Msg1 transmission should not be supported for 5 MHz carrier bandwidth

Proposal: Adopt the following text proposal in section 8.1 of 38.213

CHANGE START

Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission [4, TS 38.211].

-	A preamble index, a preamble subcarrier spacing, , a corresponding RA-RNTI, and a PRACH resource.

A preamble is transmitted using the selected PRACH format with transmission power , as described in Subclause 7.4, on the indicated PRACH resource.

UE is not expected to receive a configuration for PRACH transmission where the PRACH transmission bandwidth is greater than the maximum allowed transmission bandwidth, as defined in table 5.3.2-1 of 38.101-1, for the combination of Msg1 SCS and channel bandwidth.

CHANGE END

# Correction of CP length for preamble format 1

There seems to have been a typo in the GP length for preamble format 1 in the latest version of 38.211, V15.3.0.

In 38.211 v15.2.0, we have the following value of the GP (21024k).

Table 6.3.3.1-1: PRACH preamble formats for  and .

| Format |  |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 1 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 2 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 3 | 839 | 5 kHz |  |  | Type A, Type B |

Proposal:

Add a note in the chairman notes to capture that this typo need to be handle in the next release of 38.211.

Note: The CP length for preamble format 1, table 6.3.3.1-1 in 38.211 shall be 21024, to be updated in the next version of 38.211.

# References

| 1 | R1-1810097 | Remaining issues on physical random access channel and procedure | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-1810182 | Maintenance for Physical random access channel and procedure | ZTE |
| 3 | R1-1810249 | Maintenance for Physical random access channel and procedure | LG Electronics |
| 4 | R1-1810514 | Remaining issues of NR PRACH procedure | CATT |
| 5 | R1-1810747 | Maintenance of PRACH and related procedures | Intel Corporation |
| 6 | R1-1810835 | Remaining Issues on Random Access | Samsung |
| 7 | R1-1811099 | Remaining details on NR Random Access | Nokia, Nokia Shanghai Bell |
| 8 | R1-1811581 | Maintenance for Physical random access channel and procedure | Ericsson |
| 9 | R1-1811228 | Maintenance for Physical random access channel and procedure | Qualcomm Incorporated |