3GPP TSG-RAN WG1 Meeting #96 	R1-1901764

Athens, Greece, 25th February – 1st March,  2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on description of ZP CSI-RS resource configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For each ZP CSI-RS resource, the parameters ‘nrofPorts’, ‘cdm-Type’, ‘density’ and ‘freqBand’ are configured under ‘resourceMapping’ which is the same as NZP CSI-RS resource and described in section 5.2.2.3.1. However, based on the current description on ZP CSI-RS resource, there is no description for ‘density’ and ‘freqBand’. It can be misleading that these parameters are not used for ZP CSI-RS. In addition, ‘nrofPorts’ and ‘cdm-Type’ (instead of CDMType) should be configured in ‘resourceMapping’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cite the description on ‘nrofPorts’, ‘cdm-Type’, ‘density’ and ‘freqBand’ of NZP CSI-RS resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is not complete and not aligned with 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged part omitted>

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

A UE may be configured with any of the following higher layer parameters indicating REs declared as not available for PDSCH:

-	lte-CRS-ToMatchAround in ServingCellConfigCommon configuring common RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell. The configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the LTE carrier centre subcarrier location determined by offset from (reference) point A, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Subclause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) (higher layer parameter zp-CSI-RS-ResourceToAddModList in ZP-CSI-RS-ResourceSet), with each ZP-CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	resourceMapping given by ZP-CSI-RS-Resource defines the number of CSI-RS ports, CDM values and pattern, and the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211]. The parameters NrofPorts, cdm-Type, density and freqBand are provided in resourceMapping as described in Subclause 5.2.2.3.1

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS.

-	The ZP-CSI-RS time domain behavior can be configured to periodic, semi-persistent or aperiodic by highe layer parameters within PDSCH-config. All the resources in a ZP CSI-RS  resource set are configured with the same time domain behavior ('periodic', 'semi-persistent', 'aperiodic').

<Unchanged part omitted>