3GPP TSG-RAN WG1 Meeting #96	 R1-1902880

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.2.1.1

Source:	Ericsson

Title:	Initial access signals and channels for NR-U

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the design of signals and channels for initial access for NR-U, specifically

- DL discovery reference signal (DRS)

- Physical random access channel (PRACH)

# 2	DRS

In the SI phase [1], it was agreed that inclusion of CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) in the NR-U DRS is beneficial. In this section we further discuss ways of achieving efficient multiplexing of SS/PBCH block(s) and RMSI.

In many parts of the world the regulation for unlicensed bands include EIRP limits. Because the beamforming gain is included in the EIRP, using beamforming to increase coverage might not be feasible (assuming the device is able to reach maximum output power with omni-directional transmissions). However, in general beamforming can also be used to reduce interference. NR supports that the UE implicitly indicates the best SS/PBCH block to the gNB during the RACH procedure. This information can then be used in the gNB precoder selection, allowing the gNB to reduce interference for upcoming beamformed PDSCH transmissions. Thus, we think that even for the cases where SS/PBCH block coverage cannot be improved by beamforming, supporting beamformed SS/PBCH blocks has other merits. Thus, we see no reason to restrict this for NR-U.

NR Rel-15 supports beamformed SS/PBCH block transmissions and thus we see no reason to restrict this for NR-U.

One example of a potential compact DRS composition for 30kHz subcarrier spacing is shown in Figure 1 for the case of 4 SS/PBCH blocks within a burst set. This example uses the Case C SS/PBCH time domain block pattern as defined in [5], Section 4.1. As shown in the next section, this time domain pattern is useful when DRS contains RMSI (SIB1), e.g., for standalone deployments. The length is restricted to 1 ms in order to use Cat-2 LBT in-line with agreements in the SI phase [1] Section 7.2.1.3.1:

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms |

Because a PDCCH search space occurs every half slot, i.e., 7 OS (see discussion in next section), LBT can be attempted every half-slot. In the figure, SSBX (Y) denotes an SS/PBCH block with index X and effective SS/PBCH block index Y as discussed in [2]. In this example, the DRS is time shifted by a half slot due to LBT failure at the beginning of the first slot. As discussed in [2], the effective index Y allows the UE to determine which SS/PBCH block from one DRS burst to another that are QCL’d, even though the actual signaled SS/PBCH block indices X may be different from burst to burst because of potential time shifting due to LBT failure.

Figure 1: Example of a potential DRS composition for 30kHz subcarrier spacing based on Case C time domain pattern. LBT fails in the first half-slot, and thus the first SS/PBCH block in the pattern is dropped.

To improve robustness of the SIB1 transmission it is desirable to enable soft combining of different SIB1 redundancy versions. In this example RV0 and RV3 has been chosen as they are self-decodable.

Support soft combining of different SIB1 redundancy versions within the DRS.

During the SI phase, it was discussed if and how to support beam repetition for soft combining of SSBs within the same DRS transmission. Using the mechanism in [2] with N=1, basic repetition can be achieved. In this case, all SS/PBCH blocks in Figure 1 have effective index Y = 0 meaning that all SS/PBCH blocks within the burst are QCL’d. It can be further discussed whether or not soft combining of PBCH within the same DRS transmission is supported given that the SS/PBCH block index in PBCH must change every 8th SS/PBCH block in order for the UE to derive frame timing.

## 2.1	DRS with RMSI (SIB1)

In RAN1 AH 1901, the following agreement was made:

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

which states that for the case of initial access in a standalone deployment, the UE may assume the SS/PBCH block subcarrier spacing is 30 kHz. With this agreement, we consider what changes are needed to NR-Rel-15 in order to achieve efficient multiplexing of SS/PBCH block(s) and RMSI PDCCH/PDSCH in a DRS assuming 30 kHz SCS. In the upcoming sections, we consider two cases separately: DRS with single SS/PBCH block and multiple SS/PBCH blocks. We observe that the changes required to support DRS with a single SS/PBCH block are fewer than for the case of DRS with multiple SS/PBCH blocks. In both cases, the changes ensure that as large as possible number of REs are available for PDSCH carrying SIB1 considering the various multiplexing rules for SS/PBCH blocks, PDCCH, PDSCH, and DMRS that exist in the spec today.

For initial access, the PDSCH carrying SIB1 is scheduled by PDCCH in a Type0-PDCCH common search space associated with CORESET0. The UE determines the bandwidth of CORESET0 from the four most significant bits of pdcch-ConfigSIB1 (included in the MasterInformationBlock) which indicate a single row in one of Tables 13-1 through 13-10 in [5]. For the case SS/PBCH block and CORESET0 using 30 kHz SCS, the appropriate table is 13-4. The two options for CORESET0 bandwidth in this table are 24 and 48 PRBs. However, based on the following agreement in the SI phase (see [1], Section 7.2.1),

Initial active DL/UL BWP is approximately 20MHz for 5GHz band, though the final value will be quantized to number of PRBs. Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band.

we propose the following at least for the case of 20 MHz carriers in unlicensed bands:

The UE expects to be configured with CORESET0 bandwidth of 48 PRBs at least for the case for 20 MHz carriers in unlicensed bands.

The PDSCH carrying SIB1 is scheduled by DCI format 1_0 where the CRC is scrambled by SI-RNTI and where the System Information Indicator field in DCI is set to 0. We observe that for such scheduling, only Type 0 resource allocation in the frequency domain is supported, i.e., PDSCH occupies a set of contiguous resource blocks (RBs). Furthermore, the set of contiguous RBs lie within the same bandwidth of CORESET0.

DCI format 1_0 used to schedule PDSCH carrying SIB1 supports only Type0 PDSCH resource allocation (contiguous RBs) in the frequency domain.

In what follows, we adhere to this constraint so as to avoid introducing a new DCI format for scheduling SIB1.

### 2.1.1	DRS with Single SS/PBCH Block

In this section, we consider a DRS with only a single SS/PBCH block. This configuration is typical in FR1 deployments if neither repeated SSBs nor beamformed SSBs are needed. In order to avoid time gaps in the DRS pattern, it is beneficial to multiplex the SS/PBCH block and the PDSCH carrying SIB1 in the frequency domain. Furthermore, in order to adhere to the contiguous resource allocation for PDSCH signalled by DCI Format 1_0 and at the same time maximize the number of REs available for SIB1, it is beneficial to locate the SS/PBCH block at one edge of the initial BWP. Such a configuration is illustrated in Figure 2. This assumes that the SS/PBCH block is located in the first position of the Case C time domain pattern, i.e., OFDM symbols 2, 3, 4, and 5 assuming indexing starts with zero. While the case of a 2 symbol Type0-PDCCH common search space is shown, the same multiplexing approach may be used for a single symbol search space in which case the PDSCH allocation would start in OFDM symbol 1. The time domain PDSCH allocation is indicated by the 4-bit field of DCI Format 1_0 that indicates a row of the Table 5.1.2.1.1-2 in [6] (Default Table A). For the case of 2-symbol and 1-symbol search space configuration, Row Index 1 and 12 are applicable, respectively. Both correspond to PDSCH Mapping TypeA.

Assuming a PDSCH contiguous bandwidth of 28 PRBs (CORSET0 BW - SS/PBCH block BW) and accounting for the 3 OFDM symbols used for DMRS, the number of available REs for PDSCH carrying SIB1 is 3024 for the case of a 2-symbol search space. Assuming the most robust MCS is used for SIB1 transmission (MCS0), the SIB1 information bit payload is 708 bits.

Figure 2: DRS with FDM multiplexing PDSCH carrying SIB1 and a single SS/PBCH block with 0 RB offset between CORESET0 and the SS/PBCH block.

The only new aspect of this configuration compared to NR Rel-15 is that the SS/PBCH block is located at the edge of the initial BWP which implies an offset of 0 RBs in Table 3-4 of [5]. In Rel-15, only offsets of 12, 14, and 16 PRBs are supported which means that the SS/PBCH block would be located in approximately the middle of the initial BWP. The problem with this is that it would reduce the number of SIB1 payload bits by roughly 50% to ~350 bits, since only contiguous resource allocation for PDSCH is supported in DCI Format 1_0. Clearly, there is a benefit to extending the configurability of the offset to maximize SIB1 payload.

In a DRS containing RMSI, support a 0 RB offset between CORESET0 and the SS/PBCH block(s).

### 2.1.2	DRS with Multiple SS/PBCH Blocks

In this section, we consider a DRS with multiple SS/PBCH blocks, i.e., 2 per slot. For this case, additional changes are needed to the Rel-15 spec. For initial access, the UE determines the monitoring occasions of the Type0-PDCCH common search space from the four least significant bits of pdcch-ConfigSIB1 (included in the MasterInformationBlock) which indicate a single row in one of Tables 13-11 through 13-15 in [5]. For the case of FR1 with SS/PBCH block and CORESET multiplexing pattern 1, the appropriate table is 13-11. To configure the PDCCH search space and the corresponding SS/PBCH block in the same slot, use of a configuration with O = 0 and 2 search space sets per slot can achieve this, e.g., Row Index 1 in Table 13-11. However, in Table 13-11 all configurations have the second PDCCH monitoring occasion starting in the symbol right after the first PDCCH monitoring occasion. For a DRS with 2 SS/PBCH blocks per slot, it is beneficial instead to have the first and 2nd monitoring occasions in the first and 2nd half slots, respectively. For this reason the following agreement was made in RAN1 AH 1901:

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

For the Case C time domain pattern supported in NR Rel-15, the natural place to locate the 2nd monitoring occasion is OFDM symbol 7 (assuming indexing starts at zero). This allows the same number of OFDM symbols for PDSCH carrying SIB1 in each half slot.

Figure 3: SS/PBCH block time domain patterns supported in NR Rel-15

In a DRS containing RMSI with two SS/PBCH blocks per slot, support two Type0-PDCCH CSS monitoring occasions starting in OFDM symbols 0 and 7 (assuming symbol indexing starts at zero).

Using Proposal 3 and Proposal 4, a DRS configuration with two SS/PBCH blocks per slot is shown in Figure 4 assuming two single-symbol Type0-PDCCH common search spaces starting at OFDM symbol 0 and 7, respectively. Assuming a PDSCH contiguous bandwidth of 28 PRBs (CORSET0 BW - SS/PBCH block BW) and accounting for the one OFDM symbol used for DMRS in each half slot, the number of available REs for PDSCH carrying SIB1 is 1680 for the case of a one-symbol search space. Assuming the most robust MCS is used for SIB1 transmission (MCS0), the SIB1 information bit payload is 394 bits. It can be further discussed if this is sufficient for typical deployments.

The time domain SS/PBCH block pattern shown in Figure 4 is the Case C pattern. This allows configuration of only a single-symbol search space in between the two SS/PBCH blocks under the constraint of ensuring an equal number of REs available for SIB1 in each half slot. If a two-symbol search spaces are needed, then there are two possible configuration options. Option 1: Introduce a new time domain pattern for SS/PBCH blocks in which the 2nd block in the slot occupies OFDM symbols 9 -- 12 instead of symbols 8 – 11. This would reduce the number of OFDM symbols available for SIB1 to 4 in each half slot, and hence reduce the SIB1 information bit payload to 315 bits. Option 2: Introduce an additional Type0-PDCCH CSS monitoring occasion starting at OFDM symbol 6. This would even further reduce the number of OFDM symbols available for SIB1 to 3 and reduce the SIB1 information bit payload to 236 bits. Again, it can be further discussed if this is sufficient for typical deployments.

In a DRS containing RMSI with two SS/PBCH blocks per slot, configuration of two 2-symbol Type0-PDCCH CSS monitoring occasions requires introduction of either a new SS/PBCH block time domain pattern (Case C’) or a 2nd Type0-PDCCH monitoring occasion that crosses the half slot boundary. Both options result in a relatively small SIB1 information bit payload, and it should be further discussed whether or not 2-symbol monitoring occasions are supported.

Figure 4: DRS with FDM multiplexing PDSCH carrying SIB1 and two SS/PBCH blocks with 0 RB offset between CORESET0 and the SS/PBCH blocks.

One observation that is common to all options discussed above is that scheduling SIB1 in the first half slot of the DRS is supported already in NR Rel-15 through the PDSCH TypeA mappings in either Row Index 13 or 14 in Table 5.1.2.1.1-2 in [6] (Default Table A). However, scheduling SIB1 in the 2nd half slot of the DRS requires a new TypeB mapping that does not currently exist in Default Table A. There is no TypeB mapping of duration 7 OFDM symbols that starts at OFDM symbol 7. For this reason, we propose the following

In a DRS containing RMSI with two SS/PBCH blocks per slot, support a default PDSCH TypeB mapping starting in OFDM symbol 7 with a length of 7 OFDM symbols (assuming symbol indexing starts at zero).

In summary, to support a DRS design with efficient multiplexing of SS/PBCH block(s) and RMSI PDCCH/PDSCH, the following three changes to NR Rel-15 specifications are identified in order to maximize the SIB1 information bit payload:

- Introduce 0 RB offset between CORESET0 and SS/PBCH block(s) (Proposal 3)

- Introduce a new Type0-PDCCH monitoring occasion at OFDM symbol 7 (Proposal 4)

- Introduce a new default TypeB PDSCH mapping starting at OFDM symbol 7 with duration of 7 symbols (Proposal 5)

For the case of a single SS/PBCH block in a DRS, only #1 is needed. For the case of two SS/PBCH blocks per slot, #1, 2, and 3 are needed. It should be further discussed whether or not a new SS/PBCH block time domain pattern is needed for 30 kHz SCS, or whether the existing Case C pattern is sufficient. Once this is concluded, an LS should be sent to RAN4 with RAN1’s recommendation. The LS should also include the agreement from RAN1 AH 1901 about SCS for SS/PBCH block(s) (shown in Section 2.1 of this paper).

### 2.1.3	SS/PBCH block rate matching assumptions

In NR Rel-15, when receiving the PDSCH scheduled with SI-RNTI in PDCCH-Type0 common search space and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. This assumption restricts the multiplexing possibilities between SS/PBCH blocks and PDSCH carrying RMSI. Specifically, it does not allow the PDSCH to be mapped “around” the SS/PBCH block as shown by the pink shading in Figure 1.

If further optimizations of the number of available resources for SIB1 are needed compared to the designs proposed in Section 2.1.1 and 2.1.2, one would need to revisit a number of assumptions that a Rel-15 UE is allowed to make.

In general, Section 5.1.4 in [6] would need to be modified to allow an SS/PBCH block to be transmitted in REs used by the UE for reception of PDSCH also in the case when the system information indicator in DCI is set to 0.

For the case of a single SS/PBCH block in the DRS, the assumption in Section 7.4.1.1.2 in [3] that “the UE may assume that no DM-RS collides with the SS/PBCH block” would need to be revisited, because the DMRS in OFDM symbol 2 would collide with the first SS/PBCH block if the PDSCH allocation would span all 48 PRBs. The same situation would occur in OFDM symbol 11 for the second SS/PBCH block position in the slot.

For the case of multiple SS/PBCH blocks in the DRS a similar situation would occur in OFDM symbol 8. In addition, no DMRS would be available for the PDSCH REs after the SS/PBCH block in OFDM symbol 12 and 13. In light of this we make the following proposal:

Discuss the trade-off between required changes needed to Rel-15 specifications to allow PDSCH carrying SIB1 to be mapped “around” an SS/PBCH block and the need for additional resources for SIB1.

## 2.2	DRS without RMSI (SIB1)

The previous section focused on design of a DRS with RMSI. While this is needed for standalone deployments, it is also needed in some cases for non-standalone (NSA) deployments, e.g., when ANR is used requiring the UE to read the cell global identifier from SIB1. In some other cases, however, the DRS need not always include RMSI. In such cases, the SS/PBCH block time domain patterns defined in NR Rel-15 are sufficient. However, if a new time domain pattern (e.g., modification of Case C) is supported for DRS with RMSI, it would be beneficial from a simplicity standpoint to use the same time domain pattern for DRS without RMSI. Hence we propose the following

If the UE is configured with SCS for SS/PBCH block(s) on an unlicensed serving cell (i.e., for non-standalone operation), the UE may assume the SS/PBCH block(s) are transmitted with the following time domain pattern:

  - Case A if 15 kHz SCS configured as in NR Rel-15

  - Same pattern as for DRS with RMSI if 30 kHz SCS is configured. FFS: Case C or modified Case C pattern.

# 3	PRACH

In this section, we discuss PRACH design for NR in unlicensed spectrum.

In NR-U, PRACH transmission should be supported for both stand-alone and dual connectivity scenarios (NR-U + LTE and NR-U + NR). In NR, both long (L = 839) and short (L = 139) preamble sequences are supported. The long preambles mainly target large cell deployments. Since NR-U is expected to be deployed in relatively small cells, only short preambles are of interest for NR-U operations.

Only the short PRACH sequence length (LRA = 139) is useful for NR-U.

In NR, for short (L=139) preamble sequences, a number of preamble formats are defined. For each format, the cyclic prefix duration is defined in Table 6.3.3.1-2 in [3], and it is inversely proportional to the subcarrier spacing. Based on the cyclic prefix durations, Table 1 below lists the maximum cell radius for different formats with different subcarrier spacings. Given that small cells are targeted for NR-U deployment, some formats that are designed for rather large cells can be excluded for NR-U. Observing the maximum cell radius for 30 kHz subcarrier spacing, it is clear that formats C0, and C2 target quite large cells.

Only a subset of the short PRACH preamble formats, A1 – A3 and B1 – B4, are useful for NR-U.

Table 1 Maximum cell radius for preamble formats with short sequences

| Preamble format |  | Number of OFDM symbols | Maximum cell radius (in meters) |  |
| --- | --- | --- | --- | --- |
|  |  |  | SCS 15kHz | SCS 30kHz |  |
| A | 1 | 2 | 938 | 469 |  |
|  | 2 | 4 | 2109 | 1054 |  |
|  | 3 | 6 | 3516 | 1758 |  |
| B | 1 | 2 | 586 | 293 |  |
|  | 2 | 4 | 1055 | 527 |  |
|  | 3 | 6 | 1758 | 879 |  |
|  | 4 | 12 | 3867 | 1933 |  |
| C | 0 | 1 | 5300 | 2650 |  |
|  | 2 | 4 | 9200 | 4600 |  |

In NR Rel-15, RMSI (SIB1) contains the IE RACH-ConfigCommon which contains the field msg1-SubcarrierSpacing which has the following description [4]

msg1-SubcarrierSpacing Subcarrier spacing of PRACH. Only the values 15 or 30 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section). If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see 38.211, section XXX).

Clearly, only subcarrier spacings of 15 and 30 kHz are supported for PRACH for FR1. Support for 60kHz SCS for PRACH would require modifications to the signaling specified for Rel-15 NR. Furthermore, from an initial access perspective, since 60 kHz for DRS is out-of-scope for the WI (see WID in [7]), it does not appear to be beneficial to support 60 kHz only for PRACH. Moreover, from a NR-U performance perspective, we have not observed improved performance for 60 kHz SCS compared to 30 kHz for data (see evaluation in [8]). Based on this, we propose the following:

As in NR Rel-15, 60 kHz SCS for PRACH is not supported in FR1.

## 3.1	Design Considerations for PRB-Based Interlaced PRACH

In NR Rel-15, PRACH for a given user consists of the transmission of a length-139 Zadoff-Chu sequence in multiple consecutive OFDM symbols. This is accomplished by mapping values of the FFT of the Z-C sequence to contiguous PRBs in the frequency domain. For the short preamble (length 139), 12 PRBs (144 REs) are needed. Due to the contiguous mapping in the frequency domain, when the IFFT is performed at the UE transmitter, the attractive properties of the Z-C sequence are preserved. Namely, the cross-correlation between the sequence and a cyclically shifted version of the same sequence is non-zero only at the lag corresponding to the cyclic shift (delay). Furthermore, the cross-correlation between two sequences with different roots is constant at all lags with value 1/sqrt(139). Both are desirable from a PRACH detection and timing estimation point of view.

For the case of no interlacing (NR baseline), the output of the PRACH detector is shown in Figure 6(a) when the hypothesized sequence is the same as the transmitted sequence. The PRACH detector performs a correlation between the received sample sequence and the hypothesized PRACH sequence. In this example, the sub-carrier spacing is 30 kHz, an AWGN channel with propagation delay 1 μs is assumed, and the SNR is very large (no noise). Clearly the detector output has a peak at 1 us resulting in an accurate timing estimate. Figure 6(b) shows the PRACH detector output when the hypothesized sequence is different than the transmitted sequence. Evidently, the detector output is small at all lags consistent with the cross-correlation properties of Z-C sequences mentioned above.

(a)	(b)

Figure 6: PRACH detector output for no interlacing, SCS = 30 kHz, AWGN channel, delay = 1 μs when the hypothesized sequence is either (a) same, or (b) different than the transmitted PRACH sequence.

Unlike baseline NR, the Z-C sequence for PRACH is not mapped to contiguous PRBs for the case of a PRB-based interlace design. This affects the autocorrelation properties of the transmitted sequence. Namely, sidelobes (or false peaks) are introduced. If care is not taken in the interlace design, the sidelobes could lead to timing estimation errors. However, if the interlace design ensures that the sidelobe levels are sufficiently low, then this issue is avoided, since the PRACH detection and timing estimation is based on finding the peak with the maximum amplitude. Fortunately, the shape of the sequence autocorrelation and thus the sidelobe levels are a function only of the interlace design, not of individual Z-C sequence roots, thus simplifying the design problem. The key to controlling the sidelobe levels is to introduce some irregularity into the PRB pattern. We have found that it does not take much irregularity to have a well-performing design.

Figure 7 below shows an exemplary design for the case of 20 MHz channel with 30 kHz SCS in which RAN4 has defined 51 PRBs. This example uses the same interlace structure as agreed for PUSCH/PUCCH, i.e., 5 interlaces with either 10 or 11 PRBs per interlace. We note that one RACH occasion (RO) in the frequency domain requires 12 PRBs, whereas a single interlace has only 10 or 11 PRBs. However, this fact can be used to introduce the required irregularity for PRACH. The extra 2 PRBs can be placed in such a way to “break up” the regular interlace pattern. As can be seen in Figure 7, RO1 uses all ten PRBs of Interlace 1 and two PRBs of Interlace 3. Similarly, RO2, uses all ten PRBs of Interlace 2 and two PRBs of Interlace 4. RO3 and RO4 are defined in a similar fashion except minor adjustments to the pattern are needed in case either RO1 or 2 have already used a PRB.

With this design, it is easy to FDM multiplex PUSCH/PUCCH for the same or different user on the unused interlaces. For example, if only RO1 and 2 are configured, Interlaces 3, 4, and 5 are available for PUSCH/PUCCH except in 4 distinct PRB locations (3, 9, 23, and 29) where the extra 2 out of 12 PRBs for PRACH RO1 and RO2 are allocated outside their “primary” interlaces. These exceptions would be known to the UE since it is naturally aware of the PRACH configuration pattern through broadcast signalling (SIB1).

Figure 7: Exemplary PRB-based interlace design for PRACH for 30 kHz using the same interlace structure as for PUSCH/PUCCH. Interlaces 2,3,4,5 have 10 PRBs per interlace; Interlace 1 has 11. Four RACH occasions (RO) in the frequency domain are shown where each RO consists of 12 PRBs (144 sub-carriers).

Figure 8 shows the output of the PRACH detector for the above interlacing design. As in Figure 6, the output is shown for the case that the hypothesized sequence is (a) the same, or (b) different than the transmitted sequence, thus illustrating the auto- and cross-correlation properties. As mentioned previously, the non-contiguous mapping of the PRACH sequence in the frequency domain causes the auto-correlation to have sidelobes as illustrated in Figure 8(a); however, the sidelobe levels have been controlled by introducing some irregularity into the mapping. The resulting autocorrelation has a dominant peak at the true delay (1 μs). Even in the presence of a dispersive channel where the peak can be “smeared,” we have found that with this level of dominance, the performance is still very good, and compares favourably to the case of no interlacing.

We note that further optimization of the patterns is possible, and may be done in such a way to minimize the number of used interlaces for a given number of configured RACH occasions while still maintaining good irregularity resulting in low sidelobe levels. For example, for N ROs, it is possible to construct a pattern where only N + 1 interlaces are touched by PRACH, leaving 5 – (N+1) full interlaces for other signals/channels.

(a)	(b)

Figure 8: PRACH detector output for the PRB-based interlacing design in Figure 7 for SCS = 30 kHz, AWGN channel, delay = 1 μs when the hypothesized sequence is either (a) same, or (b) different than the transmitted PRACH sequence.

## 3.2	Performance of PRB-Interlaced PRACH

In the previous section, it is argued that an interlaced design with controlled sidelobes is desirable. Ultimately, what matters is the PRACH performance in terms of miss-detection probably, false-detection probability, and timing estimation error. In our companion paper [9], we present simulation results of the irregular (non-uniform) PRB-based interlace scheme introduced in the previous section. For comparison purposes, we also evaluate the NR Rel-15 contiguous RE design as well as an alternative contiguous (non-interlaced) design consisting of a frequency domain repetition of the NR Rel-15 design. The motivation for considering the repetition design is that it seems to be the option that requires the least degree of spec changes amongst the 4 alternative schemes identified in the TR for the NR-U SI [1], Section 7.2.1.2. This design enables increased maximum transmit power under a PSD constraint compared to the NR Rel-15 design, albeit at the cost of reduced RACH capacity and loss in multiplexing capability with PUSCH/PUCCH compared to the interlaced scheme proposed here. Setting aside these trade-offs, in [9] we find that both the irregular and repetition schemes perform well for all metrics in the evaluation assumptions agreed in RAN1 AH 1901. Based on this we propose the following:

Down-select amongst the following two enhanced PRACH schemes identified in the TR 38.889 for the NR-U SI: (1) non-uniform (irregular) PRB-based interlace mapping using the same interlace structure as for PUSCH/PUCCH (Alt-2 in the TR), and (2) non-interlaced mapping with repetition of the NR Rel-15 PRACH mapping in the frequency domain (Alt-4 in the TR).

# 4	Conclusion

Based on the discussion in this paper we observed the following:

Observation 1	NR Rel-15 supports beamformed SS/PBCH block transmissions and thus we see no reason to restrict this for NR-U.

Observation 2	DCI format 1_0 used to schedule PDSCH carrying SIB1 supports only Type0 PDSCH resource allocation (contiguous RBs) in the frequency domain.

Observation 3	In a DRS containing RMSI with two SS/PBCH blocks per slot, configuration of two 2-symbol Type0-PDCCH CSS monitoring occasions requires introduction of either a new SS/PBCH block time domain pattern (Case C’) or a 2nd Type0-PDCCH monitoring occasion that crosses the half slot boundary. Both options result in a relatively small SIB1 information bit payload, and it should be further discussed whether or not 2-symbol monitoring occasions are supported.

Observation 4	Only the short PRACH sequence length (LRA = 139) is useful for NR-U.

Observation 5	Only a subset of the short PRACH preamble formats, A1 – A3 and B1 – B4, are useful for NR-U.

Based on the discussion in this paper we make the following proposals for DRS and PRACH design:

Proposal 1	Support soft combining of different SIB1 redundancy versions within the DRS.

Proposal 2	The UE expects to be configured with CORESET0 bandwidth of 48 PRBs at least for the case for 20 MHz carriers in unlicensed bands.

Proposal 3	In a DRS containing RMSI, support a 0 RB offset between CORESET0 and the SS/PBCH block(s).

Proposal 4	In a DRS containing RMSI with two SS/PBCH blocks per slot, support two Type0-PDCCH CSS monitoring occasions starting in OFDM symbols 0 and 7 (assuming symbol indexing starts at zero).

Proposal 5	In a DRS containing RMSI with two SS/PBCH blocks per slot, support a default PDSCH TypeB mapping starting in OFDM symbol 7 with a length of 7 OFDM symbols (assuming symbol indexing starts at zero).

Proposal 6	Discuss the trade-off between required changes needed to Rel-15 specifications to allow PDSCH carrying SIB1 to be mapped “around” an SS/PBCH block and the need for additional resources for SIB1.

Proposal 7	If the UE is configured with SCS for SS/PBCH block(s) on an unlicensed serving cell (i.e., for non-standalone operation), the UE may assume the SS/PBCH block(s) are transmitted with the following time domain pattern:

a.	Case A if 15 kHz SCS configured as in NR Rel-15

b.	Same pattern as for DRS with RMSI if 30 kHz SCS is configured. FFS: Case C or modified Case C pattern.

Proposal 8	As in NR Rel-15, 60 kHz SCS for PRACH is not supported in FR1.

Proposal 9	Down-select amongst the following two enhanced PRACH schemes identified in the TR 38.889 for the NR-U SI: (1) non-uniform (irregular) PRB-based interlace mapping using the same interlace structure as for PUSCH/PUCCH (Alt-2 in the TR), and (2) non-interlaced mapping with repetition of the NR Rel-15 PRACH mapping in the frequency domain (Alt-4 in the TR).

# 5	References

3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum,” v16.0.0, December 2018.

R1-1902884, “Enhancements to initial access procedures,” Ericsson, RAN1#96, February 2019.

3GPP TS 38.211, “Physical channels and modulation” v15.4.0, December 2018

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification” v15.4.0, December 2018.

3GPP TS 38.213, “Physical layer procedures for control” v15.4.0, December 2018.

3GPP TS 38.214, “Physical layer procedures for data” v15.4.0, December 2018.

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018.

R1-1813455, “Frame structure for NR-U,” Ericsson, RAN1#95, November 2018.

R1-1902888, “Evaluation results for enhanced PRACH design,” Ericsson, RAN1#96, February 2019.