3GPP TSG-RAN WG1#96 									R1-1902883

Athens, 25 February – 1 March, 2019

Agenda Item:	7.2.2.2.1

Source:	Ericsson

Title:	Channel access mechanisms for NR-U

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss channel access aspects related to NR-U in both 5 and 6 GHz.

# 2	Discussion

# 2.1	LBT for DL Control messages

During the SI phase, the following note was agreed [1]:

| Channels / signals initiating the COT |  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- | --- |
| DL | DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | when the DRS duty cycle <= 1/20, and the total duration is up to 1 ms: - 25 us Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms, |
|  | DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
|  | PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

  - Note: Applicability of an LBT scheme other than Cat 4 for control messages related to initial/random access, mobility, paging, reference signals -only, and PDCCH-only transmissions, e.g. “RACH message 4”, handover command, GC-PDCCH, or short message paging transmitted either alone or when multiplexed with DRS have been discussed.

In LAA, the choice of LBT priority class for grant-only transmissions, i.e. PDCCH only, is up to eNB.  Similarly, the usage of the lowest priority class value should be allowed for NR-U transmissions that are comprised of control information (broadcast or UE specific).

When a gNB transmission includes only DL control signals/channels/information without any user plane data, the lowest priority class value is used for accessing the channel.

CAT4 LBT with the lowest priority class value can be used for DRS transmissions if CAT2 LBT conditions are not fulfilled.

# 2.2	gNB initiated COT:

During the SI phase, the following was agreed [1]:

Agreement:

- At least for the case where a DL burst follows a UL burst within a gNB-initiated COT and there is no gap larger than 25 us between any two transmissions in the COT, the rules defined below apply for the DL burst following a UL burst:

| Cat 1 Immediate transmission | Cat 2 LBT |
| --- | --- |
| When the gap from the end of the scheduled UL transmission to the beginning of the DL burst is up to 16 sec | When the gap from the end of the scheduled UL transmission to the beginning of the DL burst is larger than 16 sec but not more than 25 us |

Note: a DL burst is defined as a set of transmissions from a given gNB having no gaps or gaps of no more than 16 us. Transmissions from a gNB having a gap of more than 16 us are considered as separate DL bursts.

- FFS: The case where the gap between a DL and UL transmission may be larger than 25 us

We believe that extending the following agreement to cover the case when there are gaps between end of a DL burst and following UL transmission does not violate the regulations.

There is less incentive to support UL to DL burst gap in a gNB initiated COT since the gNB is in control and can prepare/schedule the upcoming transmission in advance without the need for a gap to accommodate for processing delays. A small gap to accommodate for the turnaround time should be enough.

If the UL burst consists of multiple frequency multiplexed UEs, it is understandable to add a maximum limit on the duration of the UL burst if CAT1 is used. However, if only one UE is being scheduled within the UL burst, no limit is needed.

The previous agreement is updated as follows:

- At least for the case where a DL burst follows a UL burst within a gNB-initiated COT and there is no gap larger than 25 us between any two transmissions in the COT, the rules defined below apply for the DL burst following a UL burst:

| Cat 1 Immediate transmission | Cat 2 LBT |
| --- | --- |
| When the gap from the end of the scheduled UL transmission to the beginning of the DL burst is up to 16 sec | When the gap from the end of the scheduled UL transmission to the beginning of the DL burst is larger than 16 sec but not more than 25 us |

Note: a DL burst is defined as a set of transmissions from a given gNB having no gaps or gaps of no more than 16 us. Transmissions from a gNB having a gap of more than 16 us are considered as separate DL bursts.

- FFS: The case where the gap between a DL and UL transmission may be larger than 25 us

- Within a gNB-initiated COT, an UL burst for a UE consisting of one or more of PUSCH, PUCCH, PRACH, and SRS follows the rules defined below:

| Cat 1 Immediate transmission | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| When the gap from the end of the DL transmission to the beginning of the UL burst is not more than 16 sec Note: Maximum limits of the duration of the UL burst other than those already derived from MCOT duration limits should be further discussed when specifications are developed. | For any of the following cases: - When the gap between any two successive scheduled/granted transmissions in the COT is not greater than 25 sec - For the case where a UL transmission in the gNB initiated COT is not followed by a DL transmission in the same COT - Note: the duration from the start of the first transmission within the channel occupancy until the end of the last transmission in the same channel occupancy shall not exceed 20 ms. | N/A |

Note: An UL burst is defined as a set of transmissions from a given UE having no gaps or gaps of no more than 16 us. Transmissions from a UE having a gap of more than 16 us are considered as separate UL bursts.

- Note: the number of LBT attempts within a COT should be discussed further during the WI.

Maximum limits of the duration of the UL burst other than those already derived from MCOT duration limits in case of multiple frequency multiplexed UEs within the burst can be considered.

The gap between UL and the following DL in a gNB initiated COT shall not be larger than 25us.

# 2.3	DL contention window adjustment for standalone

According to BRAN regulations, if at least one transmission that started at the beginning of the Channel Occupancy was successful, the Initiating Device may reset the contention window. Accordingly, it should be straight forward to extend CWS rules to be used also in case of CBG based Ack/Nacks.

For the purpose of contention window adjustment, the HARQ-ACK value corresponding to a reference PDSCH is assumed NACK if all the CBGs of the PDSCH are NACKs in case of CBG based feedback.

In NR-U DL, the reference PDSCH(s) for CWS update are the PDSCH(s) in the starting slot or partial slot of the most recent DL burst for which at least some HARQ feedback is expected to be available.

LAA supports feedback transmission only on licensed carriers. Therefore, the CW adjustment procedure for standalone operation, or licensed assisted NR-U with feedback on unlicensed carriers is not clear. With the flexible PDSCH to ACK timing, the feedback might not necessarily come before the next channel access procedure initiated by the gNB. The same situation can occur if the UE couldn’t send the feedback due to failed LBT. The impact of those situations on the CW adjustment procedure should be specified.  In principle, the situation is very similar to the UL CW adjustment in case of autonomous UL in LAA. It should be straight forward to adopt the same rules of CW adjustment in case of feedback absence and delayed feedback.

Similar rules for CW adjustment in case of absence of feedback and delayed feedback as for UL CW adjustment for autonomous UL in feLAA are adopted for DL CW adjustment in standalone deployment, or any deployment where the feedback is transmitted on unlicensed spectrum.

Before a gNB initiates a COT, the gNB adjusts the CWS according to the following:

  - If the gNB received feedback for at least one the reference PDSCH(s), the contention window size is adjusted as following:

    - The contention window size at the gNB is reset if HARQ-ACK value for at least one of the reference PDSCH(s) is ACK

    - Otherwise, the contention window size at the gNB is increased

  - If there exists at least one previous DL Cat.4 LBT transmission, from the start of the transmission of which, N slots have elapsed, and no feedback for any of the reference PDSCH(s) was received

    - For each such previous Category 4 LBT transmission from the start slot of which, N have elapsed and no feedback was received

      - The contention window size at the gNB is increased to the next higher value

      - Each such previous Category 4 LBT transmission is used to adjust the CWS only once

  - else if the gNB starts a new Cat4. LBT transmission before N slots have elapsed from the previous CAT.4 LBT and no feedback for any of the reference PDSCH(s) is received, the CWS is unchanged.

  - if the gNB receives feedback for one or more previous Category 4 LBT transmission from the start slot of which, N slots have elapsed and no feedback for any of the reference PDSCH(s) was received, it may recompute the CWS as follows:

    - Step 1: it reverts the CWS to the value used to transmit the first burst of such previous Category 4 LBT transmission(s)

    - Step 2: it updates the CWS sequentially in order of the transmission of bursts:

      - if the feedback indicates ACK for the reference PDSCH(s) of that burst, CWS is reset

      - else (if the feedback indicates NACK or there is no feedback), the CWS is doubled

- FFS: value of N and dependency on subcarrier spacing and UE capability

# 2.4	Receiver assisted LBT

It is not clear how RTS/CTS-like handshake would fit into the NR-U frame structure. Here we list some potential issues:

- The RTS/CTS messages will only be understood by an NR-U system and by no other technologies; therefore, the benefits, if any, will be less in a coexistence scenario with a different technology.

- Both gNB and UE should transmit the message using the same format and on the same physical resources. This will require introducing a common physical layer channel for both DL and UL, that is new or extending an existing one into the other transmission direction, e.g. PDCCH-like signal in the UL direction.  Otherwise, any node, gNB or UE, will have to blindly monitor for two different physical channels in every slot.

- Asynchronous detection will always be needed even in a synchronized deployment since the intra-cell UEs are not necessarily time aligned with non-serving gNBs. Similarly, intra- or inter- cell UEs are not time aligned.

We also note that RTS/CTS-like handshake mechanisms result in significant degradation in system throughput in many scenarios.

Supporting receiver assisted LBT requires significant changes to NR physical layer and channel design

# 2.5	Channel access for 6GHz band

The detection threshold used by a node to defer to another node determines the reuse of radio resources used in the system which is a key determinant for system performance. It is obvious that the level of reuse that provides the best system performance will depend on many deployment related factors, such as the pathloss environment, deployment densities, antenna heights etc. NR-U as a technology needs to be able to operate in a diverse range of environments. In some cases (e.g., indoor), it may be beneficial to lower the threshold while in others it may not (e.g., outdoor). In a particular deployment, any technology may always adjust its ED threshold lower than the maximum value allowed by regulation by implementation (but never higher) if it chooses to do so, especially if it benefits that technology.

Our evaluations have consistently shown that the use of a single threshold always is better than the use of two thresholds by a node operating in unlicensed spectrum [2]. This single threshold may or may not be associated with the use of a preamble as a technology specific choice. The situation in the 5 GHz band where a Wi-Fi node is allowed to use a higher ED threshold since it uses 802.11a preamble detection at lower PD threshold is not justified from a system performance perspective. It has always been a non-ideal compromise that has been part of the 5 GHz regime because of the existence of already deployed devices that already use this dual threshold protocol in 5GHz. 6 GHz is greenfield spectrum without prior use of license-exempt technologies.

Therefore, a single common maximum energy detection threshold applicable to all technologies is the best option for 6 GHz. Whether nodes are further required to detect preambles at or below this threshold should be purely a technology choice with each technology having the flexibility to define preambles optimized for its use cases. Each technology can then adapt its detection thresholds freely as long as the single common energy detection threshold is not exceeded. This framework provides the best performance for all users of all technologies and maximum flexibility to optimize reuse in different deployments and environments. There is no technical reason for any technology to warrant preferential exception as is the case in the 5 GHz band.

A single common maximum energy detection threshold across all technologies, with each technology having flexibility to implement technology-specific preambles and to adapt detection thresholds at or below the single common maximum threshold, provides the best performance for all users of unlicensed spectrum.

Given that even with the 5 GHz framework, NR-U can coexist very well with Wi-Fi without the implementation of any Wi-Fi preambles, and even though the regulations for 6 GHz have not been decided yet, there is a clear opportunity for greenfield 6 GHz spectrum to adopt a framework where system performance can be further improved while providing ample deployment and technology specific flexibility without requiring a technology to implement signals and channels from another technology, there is no motivation to consider the adoption of an 802.11a/ax pramble for NR-U in 6 GHz.

Adoption of 802.11a/ax preamble for NR-U in 6GHz should not be considered.

2.5.1	Wideband LBT

The nominal bandwidth in 5GHz is 20MHz. Given that there are already a lot of legacy devices operating on 20MHz channel bandwidth, performing single wideband LBT over bandwidth that is larger than 20MHz is not suitable for the 5GHz band. The situation might be different for operation in 6GHz. The smallest unit for LBT should be equivalent to the nominal channel bandwidth defined by the regulations.

In 6GHz, LBT is performed in units of nominal channel bandwidth defined by the regulations.

# 2.6		On the Reply LS to BRAN

We discuss the LS from ETSI BRAN in detail in [3]. Since this is related to the subject material of this contribution, we reproduce our proposals from [3] here for convenience. .

RAN1 should reply to ETSI BRAN stating that the clarifications provided on EN 301 893 are not clear.

RAN1 should reply to ETSI BRAN stating that the clause on short control signalling in EN 301 893 should not be modified.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Supporting receiver assisted LBT requires significant changes to NR physical layer and channel design

Observation 2	A single common maximum energy detection threshold across all technologies, with each technology having flexibility to implement technology-specific preambles and to adapt detection thresholds at or below the single common maximum threshold, provides the best performance for all users of unlicensed spectrum.

Based on the discussion in the previous sections we propose the following:

Proposal 1	when a gNB transmission includes only DL control signals/channels/information without any user plane data, the lowest priority class value is used for accessing the channel.

Proposal 2	CAT4 LBT with the lowest priority class value can be used for DRS transmissions if CAT2 LBT conditions are not fulfilled.

Proposal 3	The previous agreement is updated as follows:

Proposal 4	Maximum limits of the duration of the UL burst other than those already derived from MCOT duration limits in case of multiple frequency multiplexed UEs within the burst can be considered.

Proposal 5	The gap between UL and the following DL in a gNB initiated COT shall not be larger than 25us.

Proposal 6	For the purpose of contention window adjustment, the HARQ-ACK value corresponding to a reference PDSCH is assumed NACK if all the CBGs of the PDSCH are NACKs in case of CBG based feedback.

Proposal 7	In NR-U DL, the reference PDSCH(s) for CWS update are the PDSCH(s) in the starting slot or partial slot of the most recent DL burst for which at least some HARQ feedback is expected to be available.

Proposal 8	Similar rules for CW adjustment in case of absence of feedback and delayed feedback as for UL CW adjustment for autonomous UL in feLAA are adopted for DL CW adjustment in standalone deployment, or any deployment where the feedback is transmitted on unlicensed spectrum.

Proposal 9	Before a gNB initiates a COT, the gNB adjusts the CWS according to the following:

	If the gNB received feedback for at least one the reference PDSCH(s), the contention window size is adjusted as following:

i.	The contention window size at the gNB is reset if HARQ-ACK value for at least one of the reference PDSCH(s) is ACK

ii.	Otherwise, the contention window size at the gNB is increased

	If there exists at least one previous DL Cat.4 LBT transmission, from the start of the transmission of which, N slots have elapsed, and no feedback for any of the reference PDSCH(s) was received

i.	For each such previous Category 4 LBT transmission from the start slot of which, N have elapsed and no feedback was received

1.	The contention window size at the gNB is increased to the next higher value

2.	Each such previous Category 4 LBT transmission is used to adjust the CWS only once

	else if the gNB starts a new Cat4. LBT transmission before N slots have elapsed from the previous CAT.4 LBT and no feedback for any of the reference PDSCH(s) is received, the CWS is unchanged.

	if the gNB receives feedback for one or more previous Category 4 LBT transmission from the start slot of which, N slots have elapsed and no feedback for any of the reference PDSCH(s) was received, it may recompute the CWS as follows:

i.	Step 1: it reverts the CWS to the value used to transmit the first burst of such previous Category 4 LBT transmission(s)

ii.	Step 2: it updates the CWS sequentially in order of the transmission of bursts:

1.	if the feedback indicates ACK for the reference PDSCH(s) of that burst, CWS is reset

2.	else (if the feedback indicates NACK or there is no feedback), the CWS is doubled

FFS: value of N and dependency on subcarrier spacing and UE capability

Proposal 10	Adoption of 802.11a/ax preamble for NR-U in 6GHz should not be considered.

Proposal 11	In 6GHz, LBT is performed in units of nominal channel bandwidth defined by the regulations.

Proposal 12	RAN1 should reply to ETSI BRAN stating that the clarifications provided on EN 301 893 are not clear.

Proposal 13	RAN1 should reply to ETSI BRAN stating that the clause on short control signalling in EN 301 893 should not be modified.

# References

[1]	 Chairman’s notes RAN1#95, Spokane, US

[2]	R1-1814021 On the use of a preamble for NR-U, Ericsson

[3]	R1-1902890	On the LS from ETSI BRAN, Ericsson