3GPP TSG RAN WG1 #97		R1-1905940

Reno, USA, May 13th – 17th, 2019

3GPP TSG-RAN WG2#105bis Meeting	R2-1905473

# Xi’an, China, 8th April – 12th April, 2019

# Title:	LS on SPS/CG for IIoT

Response to:	-

Release:	Rel-16

Work Item:	NR_IIOT-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:	-

Contact Person:

Name:	Tianyang Min

E-mail Address:	tianyang.min.ex@nttdocomo.com

Attachments:	      None.

# 1. Overall Description:

RAN2 has discussed some aspects of supporting multiple active SPS/CG configuration and shorter SPS periodicities and has reached the following agreements:

R2 assumes that the maximum number of active SPS configurations for a given BWP of a serving cell in the specification is 8 or 16 (FFS). R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption. Will support “short” SPS periodicities, at least down to 0.5ms Ask R1 on feasibility, and additionally the feasibility to go down to even lower values, e.g. 2 symb. R2 assumes that activation/deactivation is done by DCI. RAN1 should address activation/deactivation DCIs related with configured grant Type 2 and SPS in the case of multiple configurations When multiple UL CG or DL SPS configurations is configured, an offset for each configuration is needed for the calculation of the HARQ process ID

Based on the above, RAN2 would like to request RAN1 answer the following questions:

- RAN2 agreed that the maximum number of active SPS configurations for a given BWP of a serving cell should be either 8 or 16, but could not reach a final conclusion. From RAN2 perspective 8 was proposed as it seems to be sufficient for current TSC requirements. On the other hand, the current maximum number of simultaneous HARQ processes in the UE is 16, so this is why this number was proposed.

Q1: RAN2 would like to kindly ask RAN1 whether from PHY layer perspective there is a difference or preference, e.g. in terms of complexity to, to support 8 or 16 configurations.

- From RAN2 point of view, supporting short periodicity at least down to 0.5 ms is required. Support of even shorter periodicities (e.g. down to 2 symbols) could be useful for support of TSC traffic patterns with periodicities non-aligned with NR frame structure, i.e. periodicities which are not multiple of NR slot or symbol period, which are used to configure CG/SPS periodicity. On the other hand, some companies indicated that the same issue can be addressed by providing the UE with multiple active SPS configurations and there was no consensus on how to address issue.

Q2: RAN2 would like to kindly ask RAN1 about:

- the feasibility to support SPS periodicities of at least 0.5 ms

- the feasibility to support SPS periodicities shorter than 0.5 ms, e.g. down to 2 symbols

- any additional limitations for the above e.g. in terms of supported SCS, HARQ-ACK feedback

- RAN2 assumes that activation/deactivation of multiple SPS/CG configurations is done by DCI. There is no consensus in RAN2 on activation/deactivation of multiple SPS/CG configurations to be done by one DCI for multiple configurations or by one DCI per configuration (as in LTE rel-15).

Q3: RAN2 would like to kindly ask RAN1 the feasibility of activation/deactivation of multiple SPS/CG configurations via DCI signalling.

In addition, RAN2 would like to stress that the motivation to have an HARQ process ID offset for each configuration is to have a separate and non-overlapping HARQ process pool for each configuration when multiple UL CG or DL SPS configurations are configured.

# 2. Actions:

# To RAN WG1 group.

# ACTION:

# RAN2 would like to kindly ask RAN1 to

- provide answers to the above questions.

- take the above agreements into account for their work in Rel-16.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #106	13th May – 17th May 2019 	Reno, Nevada, USA

TSG-RAN WG2 Meeting #107	26th Aug. – 30th Aug. 2019 	Czech Republic