3GPP TSG-RAN WG1 Meeting #97	R1-1905965

Reno, USA, May 13th – 17th 2019

Agenda Item:	6.2.2.1

Source:	Ericsson

Title:	UE-group wake-up signal in NB-IoT

Document for:	Discussion, Decision

# Introduction

One of the objectives with the Rel-16 WID for additional MTC enhancements for LTE is to specify [1]:

Improved DL transmission efficiency and/or UE power consumption: - … - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

Related to the above objective, the following agreements were made for group WUS in RAN1 #96bis [2]:

Up to 2 time-multiplexed WUS resources, for both legacy WUS and group WUS, may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not. Group WUS location in relation to legacy WUS may be configured such that: - If one group WUS resource is configured, that group WUS resource may be configured to coincide with the legacy WUS resource or to occur immediately before the legacy WUS resource, and, - If two group WUS resources are configured, the first group WUS resource coincides with the legacy WUS resource and the second group WUS resource occurs immediately before the first group WUS resource. Group WUS sequence design is down selected between the following in RAN1#97 - Legacy WUS with shifted scrambling codes - Legacy WUS with time-frequency short OCCs - Phase shifted legacy WUS using a suitable subset of the phase increments. - Legacy WUS with Gold codes Companies are encouraged to provide evaluation results Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation. Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

In this contribution, we further elaborate on our views on the design of the group WUS. Accompanying papers present our higher layer views in [3][4].

The structure of the paper is that in the first section, fundamental features are treated whereas optional features that are not necessary for WUS functionality follows in a subsequent section.

# Fundamental group WUS functionality

## Group WUS resource allocation

It has been agreed that either one or two WUS resources may be time multiplexed, one of which is shared with legacy WUS if legacy WUS is also configured. Hence, the locations of the resources may be determined implicitly if two resources are configured. This can be used to minimize configuration signaling, since that only for the case where one group WUS resource is configured does its location need to be signaled.

Signal the configured number of group WUS resources, and if only one resource is configured, also signal its time location.

Related to the resource allocation is the matter of group WUS co-existence with legacy WUS. In order to avoid to complex configurations, depending on whether legacy WUS is configured or not, an independent configuration of group WUS is preferred.

Group WUS resources are configured independently of legacy WUS, hence resource sharing with legacy WUS is configured implicitly.

## WUS power

Group WUS power configuration also remains to be agreed. There are essentially two options how this can be done:

- Independently from legacy WUS, or

- Same as legacy WUS.

The former option would allow more flexibility but also more overhead, whereas the latter option would limit flexibility but also overhead. From the network perspective, a cell is configured to provide a certain coverage, regardless of supported releases. Hence, the value of the extra configurability is likely marginal and can be omitted.

A cell is configured to provide a certain coverage level, regardless of WUS release. Hence, WUS coverage requirements are the same for legacy WUS and group WUS.

A further complicating issue regarding power configuration is the agreement that legacy WUS is the common WUS for the case where legacy WUS and group WUS share resources. For this case, it is all but impossible to allow different power levels.

Legacy WUS is included in group WUS design by, e.g., common WUS configuration. Hence, power levels or duration cannot differ between legacy WUS and group WUS.

As presented above, there are few benefits but substantial problems in using different power configurations for legacy WUS and group WUS. However, it may also be the case that legacy WUS is not defined. In that case group WUS power should still be configured, preferably with the same configuration as legacy WUS. That leads us to the following proposal:

If legacy WUS is configured, group WUS use the power configuration of legacy WUS. If legacy WUS is not configured, group WUS power is configured with an optional configuration parameter using the same parameter values as legacy WUS.

## Number of UE groups

For most paging rates, it has been shown to be preferable to utilize as many groups as possible [4], and where the maximum number of groups will be given by the sequence design. This is also true if multilevel grouping is agreed, in which case the number of groups will be fewer than the number of sequences, see also Sec. 3.2. Furthermore, it was agreed in RAN1 #95 that the number of groups should be configurable, hence, the matter of configurability remains. Considering RAN2 is responsible for the grouping basis, which may affect the number of groups, it is beneficial if RAN2 also takes responsibility for the configuration of the number of UE groups.

RAN2 is responsible for grouping basis, which may also affect the configuration of the number of groups.

Send LS to RAN2 asking RAN2 to determine the configuration of number of UE groups.

## Sequence design

Based on previous agreements and input from RAN1 #96b, we have evaluated the following group WUS designs:

Incremental phase shift (DELTA132X)

These designs are based on the legacy WUS, extended with an incremental phase shift with a 2pi/132 quanta per RE. The group-specific sequence in subframe x is thus defined as

wgroupm'=wm'ej2πgm132

Where w(m') is the legacy WUS sequence in subframe x, and the sequence index is

m=0,1,…,131

m'=m+132x

Due to deterministic, strong, cross-correlation between sequences with some pairwise combinations of g, the values of g is restricted. We propose two subsets that we find have good properties:

DELTA13213 where g is defined as

g=0IDgroup=02+13×IDgroup-11≤IDgroup≤NgrpsUE

NgrpsUE≤10

Cross-correlation analysis of this set reveals that all possible sequence pairs are free of correlation peaks within the maximum possible offset within a symbol duration.

DELTA1327 for which g is defined as

g=7×IDgroup0 ≤IDgroup≤NgrpsUE

NgrpsUE≤18

Cross-correlation analysis of this set reveals that all possible sequence pairs are free of strong correlation peaks within an offset less than ±5/12 of a symbol duration.

In both cases, IDgroup=0 yield the legacy WUS.

In addition to the incremental phase shift, three additional designs have been proposed:

Time-frequency short orthogonal cover code (TF-sOCC)

TF-sOCC is a length-12 frequency-domain cover code combined with the length-11 Barker code in the time domain [5]. We have evaluated the complete set of 11 group-WUS sequences and the legacy sequence.

Gold cover code (GOLDx)

GOLDx is a length-127 Gold cover code [6]. We have evaluated this scheme with 12, 22 and 43 groups in addition to the legacy signal, labeled GOLD12, GOLD22 and GOLD43.

Shifted scrambling sequence (SCRS1031)

This design is based on the proposal in [7] with the per-group shift increment set to 1031 instead of 262. The motivation for the change in shift increments is to avoid aliases between subframes. This scheme has been evaluated with varying numbers of groups from 12 and upwards.

### Simulation results

We have assumed that group-WUS may be configured to share resources with legacy WUS. Since a legacy UE is expected to employ a WUS detection threshold based on white noise, we have chosen to assume the same for group-WUS UEs. This means detection performance will not differ between sequence designs. It also means the cross-sequence false-alarm probability (xFAR) is a key performance indicator.

Table 1 summarizes the used simulation parameters.

Table 1: Simulation parameters for group WUS sequence evaluation.

| Parameter | Value |
| --- | --- |
| Fs | 1.92e6 |
| Carrier bandwidth | 6 RB |
| FFT | 128 |
| WUS bandwidth | 2RB |
| WUS repetitions | 1, 4 |
| Channel model | EPA, 1Hz |
| Rx frequency error | 40 Hz |
| Rx coherence | 1 SF |
| SNR | 2.5 dB, -12.5 dB |
| Detection threshold | 1% white noise FAR |
| Reciever time window | ±5 µs, ±10 µs, ±25 µs |

Figure 1 presents results in terms of the CDF of the detected peak correlation value for single subframe (R=1) WUS with the proposed sequences for timing drift values corresponding to UE network synchronization periods of up to 28.5 s, which is by far more than what is necessary. It reveals that the DELTAx performs slightly better than TF-sOCC and clearly better than GOLD codes.

Figure 1: Cumulative distribution of peak correlation values for 1% white-noise false-alarm threshold for single-subframe WUS at 2.5 dB SNR and detection window lengths of, left, ±10 µs and right, ±25 µs.

Figure 2 presents results for WUS with 4 subframes (R=4). The plots indicate that performance with TF-sOCC and DELTA132_13 is very similar. Comparing R=1 and R=4 also indicate that the difference between the designs is less for longer repetitions. In these simulations we also see that scrambling sequence shift has clearly inferior performance, although it could be pointed out that doubling the coherence-interval from one subframe to two would likely reduce this disadvantage.

Figure 2: Cumulative distribution of peak correlation value for 1% white-noise false-alarm threshold for 4-subframe WUS at -12.5 dB SNR and detection window lengths of, left, ±5 µs and right, ±25 µs.

The simulation results are summarized in Table 2.

Table 2: Performance of the different evaluated group WUS sequences.

| Design | Maximum number of groups (excluding legacy) | Cross-sequence FAR at 1% white-noise FAR threshold (R=1, SNR=2.5 dB) |  | Cross-sequence FAR at 1% white-noise FAR threshold (R=4, SNR=-12.5 dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ±10 µs | ±25 µs | ±5 µs | ±25 µs |
| DELTA1327 | 18 | 3% | 6% | 1% | 2% |
| DELTA13213 | 10 | 0.5% | 3% | 1% | 2% |
| TF-sOCC | 11 | 2% | 7% | 1% | 2% |
| GOLD12 | 12 | 16% | 18% | 4% | 8% |
| GOLD22 | 22 | 16% | 18% | 4% | 8% |
| GOLD43 | 43 | 16% | 18% | 4% | 8% |
| SCRS1031 | >21 | 22% | 25% | 7% | 10% |

Incrementally phase shifted DELTA13213 performs slightly better than time-frequency short orthogonal cover codes, with average cross-sequence false alarm rates very close to white noise.

Time-frequency short orthogonal cover codes allows one group more than incrementally phase shifted DELTA13213

Gold cover codes have inferior performance compared to incrementally phase shifted DELTA13213 and time-frequency short orthogonal cover codes.

Shifted scrambling codes have inferior performance compared to Gold cover codes.

Incrementally phase shifted DELTA13213 is adopted as group WUS sequence.

# Optional group WUS functionality

In this section we present our opinions on features that are not fundamental and may thus be omitted. As a first design criterion here, it makes sense to keep the design as simple as possible and only introduce features that improve performance for a wide range of use case.

Optional features should only be introduced if gains can be shown.

## UE group weighting

It has been proposed that UE group weighting is introduced without any clear description on how proper weights should be determined. Weighting is further studied below for the case where the common WUS is the legacy WUS, since this is the only case that is likely to benefit from weighting. There are several good reasons why weighting should be avoided. From the network perspective, it is unlikely that weighting is implemented in the MME-eNB interface, since it requires substantial interoperability testing among network operators, resulting in huge costs as a consequence. Furthermore, it will make the implementation of any other grouping strategy than UE ID grouping increasingly difficult. Weighting will discriminate UEs based on their UE ID or some other parameters that is used for grouping basis in a way that is undesirable for both network vendors and operators. Finally, the benefits of weighting are small at best, assuming a heavily skewed distribution of legacy and group WUS UEs, in which case there will be little performance difference with separating group WUS resources from legacy WUS UEs altogether.

Weighting will complicate grouping-bases not only relying on UE_ID.

Nevertheless, false wake-up performance for UE group weighting is considered in Figure 3, for relations of the number of group WUS UEs to legacy WUS UEs of 2 and 8, respectively, together with the case where the common WUS is not the legacy WUS. It is evident that for more even relations (2x), weighting will not improve performance for hardly any combination of weight level and UE paging rate. Furthermore, for a given weight, performance for UE groups belonging to the shared resource or not, depend on the UE paging rate. Hence, it is not possible to select a weight that fits all UEs. Finally, performance for weighting does not for any weight or UE paging rate equal that of the case where the common WUS is not the legacy WUS, i.e., if colliding, WUSs are sent on separate POs.

Performance of group weighting depends on UE paging rates, implying it is impossible to select optimal weights.

For the more uneven relation between group WUS UEs and legacy WUS UEs (8x), weighting may improve performance, provided the network is able to accurately estimate the correct weights. If that is not the case, as is shown in the figure, performance among different UE groups will significantly diverge. Also for the more uneven relation, not using the legacy WUS as the common WUS will in average provide better and more coherent performance.

Performance gains from group weighting is marginal at best and is worse than the case where the common WUS is not the legacy WUS.

Figure 3: False wake-up rate as relation to UE paging rates for different UE group weighting values together with the case where the common WUS is not the legacy WUS. In the leftmost figure, the rlation between group WUS UEs and legacy WUS UEs is 2 and the rightmost figure the corresponding relation is 8. A weight of 0.25 implies that 25% of the UEs in the resource shared with legacy WUS have been moved to the non-shared resource. Two group WUS resources are assumed in total with 8 UE groups per resource.

UE group weighting is not supported.

## Multilevel grouping

The sequence design will result in a limited number of sequences. These sequences can be used for either many unique UE groups and no intermediary level between these and the common WUS, i.e., dual level grouping, or fewer unique groups with an additional intermediary level based on combinations of groups between the unique UE groups and the common WUS, i.e., multilevel grouping. Considering the sequence design candidates, it is likely that a design scheme with approximately 11 sequences will be defined. Hence, the dual level grouping and multilevel grouping alternatives are assessed with this number of available sequences. Table 3 presents the two alternatives regarding how the sequences are disposed according to the two grouping alternatives.

Table 3: Dual level and multilevel grouping configurations.

| Sequence index | Dual level grouping | Multilevel grouping |
| --- | --- | --- |
| 0 | WUS group 0 | WUS group 0 |
| 1 | WUS group 1 | WUS group 1 |
| 2 | WUS group 2 | WUS group 2 |
| 3 | WUS group 3 | WUS group 3 |
| 4 | WUS group 4 | WUS groups 0+1 |
| 5 | WUS group 5 | WUS groups 0+2 |
| 6 | WUS group 6 | WUS groups 0+3 |
| 7 | WUS group 7 | WUS groups 1+2 |
| 8 | WUS group 8 | WUS groups 1+3 |
| 9 | WUS group 9 | WUS groups 2+3 |
| 10 | Common WUS | Common WUS |

Figure 4 presents the false wake-up performance for the above two cases. It is clear in the figure that for most paging rates, dual level grouping with more groups is beneficial to multilevel grouping with fewer groups and combinations of groups.

Figure 4: False wake-up rates as a function of paging probabilities for both dual level grouping with 10+1 groups (red line), and multilevel grouping with 4+6+1 groups (blue line) for the case of 40 UEs per PO.

Taking further into account both the increased detection complexity required to implement multilayer grouping and the increased false paging rate arising from the need to detect more sequences, multilevel grouping is not justifiable.

Multilevel grouping has a higher false wake-up rate compared to dual level grouping utilizing the same total number of WUS sequences.

Multilevel grouping is not supported.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A cell is configured to provide a certain coverage level, regardless of WUS release. Hence, WUS coverage requirements are the same for legacy WUS and group WUS.

Observation 2	Legacy WUS is included in group WUS design by, e.g., common WUS configuration. Hence, power levels or duration cannot differ between legacy WUS and group WUS.

Observation 3	RAN2 is responsible for grouping basis, which may also affect the configuration of the number of groups.

Observation 4	Incrementally phase shifted DELTA13213 performs slightly better than time-frequency short orthogonal cover codes, with average cross-sequence false alarm rates very close to white noise.

Observation 5	Time-frequency short orthogonal cover codes allows one group more than incrementally phase shifted DELTA13213

Observation 6	Gold cover codes have inferior performance compared to incrementally phase shifted DELTA13213 and time-frequency short orthogonal cover codes.

Observation 7	Shifted scrambling codes have inferior performance compared to Gold cover codes.

Observation 8	Weighting will complicate grouping-bases not only relying on UE_ID.

Observation 9	Performance of group weighting depends on UE paging rates, implying it is impossible to select optimal weights.

Observation 10	Performance gains from group weighting is marginal at best and is worse than the case where the common WUS is not the legacy WUS.

Observation 11	Multilevel grouping has a higher false wake-up rate compared to dual level grouping utilizing the same total number of WUS sequences.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Signal the configured number of group WUS resources, and if only one resource is configured, also signal its time location.

Proposal 2	Group WUS resources are configured independently of legacy WUS, hence resource sharing with legacy WUS is configured implicitly.

Proposal 3	If legacy WUS is configured, group WUS use the power configuration of legacy WUS. If legacy WUS is not configured, group WUS power is configured with an optional configuration parameter using the same parameter values as legacy WUS.

Proposal 4	Send LS to RAN2 asking RAN2 to determine the configuration of number of UE groups.

Proposal 5	Incrementally phase shifted DELTA13213 is adopted as group WUS sequence.

Proposal 6	Optional features should only be introduced if gains can be shown.

Proposal 7	UE group weighting is not supported.

Proposal 8	Multilevel grouping is not supported.

# References

RP-190757, “Additional enhancements for NB-IoT,” Huawei, RAN #82, Shenzhen, P.R. China, March 2019.

R1-1905771, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT,” Huawei, RAN1 #96bis, Xi’an, P.R. China, April 2019.

R2-1907908, “UE_ID based WUS grouping,” Ericsson, RAN2 #106, Reno, USA, May 2019.

R2-1907912, “Mobility WUS group and paging multiplexing,” Ericsson, RAN2 #106, Reno, USA, May 2019.

R1-1905529, UE-group wake-up signal for NB-IoT, Intel, RAN1 #96bis, Xi’an, P.R. China, April 2019.

R1-1904526, UE-group wake-up signal, Qualcomm, RAN1 #96bis, Xi’an, P.R. China, April 2019.

R1-1904270, UE-group wake-up signal for NB-IoT, Intel, RAN1 #96bis, Xi’an, P.R. China, April 2019.