3GPP TSG RAN WG1 Meeting #97	R1-1905977

Reno, USA, 13-17 May, 2019

Agenda Item:	6.2.1.7

Source:	Huawei, HiSilicon

Title:	Coverage enhancement for Non-BL UE

Document for:	Discussion and decision

# Introduction

In RAN1 #96bis [1] and RAN2 #105bis [2] meetings, the following agreements were achieved on coverage enhancement for non-BL UEs.

Agreements in RAN1#96bis meeting:

Agreement

CSI-RS based CSI feedback is only supported in TM9

The supported number of CSI-RS ports is only 8

Conclusion

No further discussion on the modification on the design or configuration for support of CSI-RS for non-BL CE UEs in CE mode A in Rel-16. The baseline is Rel-15 CSI-RS.

Agreement

For CSI feedback of non-BL CE UE, RI is fixed to 1 if it is included as part of reporting on PUCCH or PUSCH

Agreement

- MPDCCH and PDSCH are punctured around the REs used for the CSI-RS transmission.

Agreements in RAN2#105bis meeting:

- Working assumption: In connected mode non-BL UEs in CE monitor MPDCCH to receive ETWS indication and/or CMAS indication using CSS in the same narrowband where unicast transmission can be received.

In this contribution, further considerations on coverage enhancement for non-BL UEs are provided.

# Discussion

## UE demodulation performance requirements for 2 RX antennas

For BL UEs, in order to achieve low cost, a single receive RF chain is assumed. The coverage mechanism is common for both BL UEs in CE mode and non-BL UEs in CE mode. Therefore, UE demodulation performance requirement in coverage enhancement is based on a single receive RF chain.

However, compared to two receive RF chains supported by non-BL UE, about 4dB coverage loss can be envisaged for a single receive RF chain, which implies more repetitions especially in extreme channel condition. Since non-BL UEs definitely support 2 RX antennas, it is beneficial to support UE demodulation performance requirements for 2 RX antennas. With 2 RX antennas, the number of repetitions for DL reception can be significantly reduced, and resource utilization can be improved.

Proposal 1: UE demodulation performance requirements for 2 RX antennas are supported for non-BL UEs in coverage enhancement.

## ETWS/CMAS in connected mode

RAN2 has agreed in #105bis meeting that connected mode non-BL UEs in CE monitor MPDCCH to receive ETWS indication and/or CMAS indication using CSS in the same narrowband where unicast transmission can be received. In addition, the following questions were arisen in the LS [3]:

RAN2 would like to know:

- Whether it is feasible for a non-BL UE in CE in connected mode to monitor MPDCCH to receive ETWS indication and/or CMAS indication using CSS in the same narrowband where unicast transmission can be received?

- Whether it is possible for a non-BL UE in CE in connected mode to monitor USS and CSS simultaneously in the same narrowband?

In the current specifications, type 0 CSS is just in the same narrowband where unicast transmission can be received and BL/CE UEs can monitor USS and type 0 CSS simultaneously.

Observation 1: It is feasible for a non-BL UE in CE in connected mode to simultaneously monitor USS and type 0 CSS, which is in the same narrowband where unicast transmission can be received.

Proposal 2: Send LS to RAN2 to answer that:

1. It is feasible for a non-BL UE in CE in connected mode to monitor MPDCCH to receive ETWS indication and/or CMAS indication using type0 CSS in the same narrowband where unicast transmission can be received.

2. It is possible for a non-BL UE in CE in connected mode to monitor USS and type0 CSS simultaneously in the same narrowband.

From RAN1 perspective, three options can be considered to notify the ETWS/CMAS considering RAN2 working assumption:

Option 1: By implementation, eNB releases the UE to idle mode.

Option 2: Support non-BL UEs to monitor the CSS for paging in connected mode.

Option 3: Using the DCI in USS or UE-group DCI format 3/3A in type 0 CSS.

For option 1, considering the latency requirement of ETWS/CMAS and the actual latency of hundreds of RRCConnectionRelease repetitions and MPDCCH repetitions for ETWS/CMAS notification, it is not efficient by implementation. Option 2 is a straightforward way, but it will require the UE to monitor both CSS for paging and USS which may be transmitted on two different narrowbands. So option 2 will waste a lot of energy and have big impacts on current specifications, which is not preferred. Option 3 offers an easier way to notify the ETWS/CMAS and has little impact on current specifications. Compared with UE-specific DCI, UE-group DCI format 3/3A in type 0 CSS is more efficient, which also coincides with the agreement in RAN2.

For example, as shown in Figure 1, one of the multiple TPC commands in DCI format 3/3A is used for notifying the ETWS/CMAS reception and will not be configured to any UEs as TPC command.

Figure 1. Using DCI format 3/3A for notifying ETWS/CMAS reception

Proposal 3: DCI format 3/3A in type0 CSS is used to notify the ETWS/CMAS reception for connected mode non-BL UEs in coverage enhancement.

# Conclusions

In this contribution, further considerations on coverage enhancement for non-BL UEs are given, and following observation and proposals are provided:

Observation 1: It is feasible for a non-BL UE in CE in connected mode to simultaneously monitor USS and type 0 CSS, which is in the same narrowband where unicast transmission can be received.

Proposal 1: UE demodulation performance requirements for 2 RX antennas are supported for non-BL UEs in coverage enhancement.

Proposal 2: Send LS to RAN2 to answer that:

1. It is feasible for a non-BL UE in CE in connected mode to monitor MPDCCH to receive ETWS indication and/or CMAS indication using type0 CSS in the same narrowband where unicast transmission can be received.

2. It is possible for a non-BL UE in CE in connected mode to monitor USS and type0 CSS simultaneously in the same narrowband.

Proposal 3: DCI format 3/3A in type0 CSS is used to notify the ETWS/CMAS reception for connected mode non-BL UEs in coverage enhancement.

# References

- 3GPP, “Chairman's Notes RAN1 #96bis final”, Xi’an, China, 8th-12th April 2019

- 3GPP, “Chairman's Notes RAN2 #105bis final”, Xi’an, China, 8th-12th April 2019

- 3GPP, R2-1905278 “LS on ETWS/CMAS in connected mode narrowband”.