3GPP TSG RAN WG1 Meeting #97			                                R1-1905991

Reno, USA, 13th - 17th May, 2019

Source:               ZTE, Sanechips

Title:                    Simulation results for 2-step Rach

Agenda item:      7.2.1.3

Document for:    Discussion and Decision

# Introduction

Agreement in R1-1903834:

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

The motivation of evaluation and the corresponding LLS assumptions are agreed in [7] during the email discussion.

In this contribution, two parts of preliminary evaluation results for 2-step RACH are provided, the first part shows the performance with different payload size in various scenarios. Both long preamble sequence Format 0 and short preamble sequence Format A1 are considered in payload size evaluation. The second part studies different mapping schemes between PRACH preamble and PUSCH-DMRS, to have better understanding on the performance impacted by the potential collision. Besides, the effects of receiving antenna numbers and preamble formats are also considered.

# Payload size evaluations

The methodology of the payload size evaluation is as follows:

Step 1: For each scenario with a specific assumption for timing offset (based on cell size), for various payload sizes, derive the required SNRs at the target BLER (e.g. 10%) by LLS, for the given resource size (bandwidth).

Step 2: Calculate the supported MCL based on the required SNR (corresponding to SINR in SLS) for the given occupied bandwidth, Tx power and noise figure.

Step 3:  Draw the CDF of coupling loss for each scenario and find the target percentage of CL, e.g. 95%.

Step 4: Draw the MCL vs TBsize curve for each scenario with different PRB numbers and receiving antennas. Find the maximum supported TBsize at the target percentage of CL.

## Link level simulation results

3 scenarios are considered:  200m, UMi, 4 GHz; 500m, UMa, 4 GHz and 1732m, RMa, 700 MHz.

The following 2 scenarios 200m, UMi, 4 GHz and 500m, UMa, 4 GHz are evaluated with Preamble format A1, the subcarrier spacing at 4GHz is 30kHz. The PUSCH resource size is [1 3 6] PRBs + 1slot, with 1/7 DMRS overhead.

The scenario 1732m, RMa, 700 MHz is relatively large cell and simulated with Preamble Format 0, the subcarrier spacing at 700MHz is 15kHz. The PUSCH resource size is [1 6 12] PRBs + 1slot, with 1/7 DMRS overhead.

Both 2 and 4 receiving antennas are considered. More detailed simulation assumptions following the agreements can be found in Table A1 in the Appendix.

- 200m, UMi, 4 GHz

Assume the total PUSCH frequency resource is 1, 3, 6 PRB, single UE, TDL-A channel. The BLER performance for the considered scenarios can be found in Figure 1. The timing offset is estimated by preamble and the channel estimation is done by DMRS with compensated TO. The frequency offfset is 0.15ppm, i.e. 600 Hz, without estimation/compensation. It can be seen that in general larger allocated bandwidth can provide better link-level performance, and the performance of 4 receiving antennas is much better than 2 receiving antennas.

(a) 1 PRB

(b) 3 PRBs

(c) 6 PRBs

Figure 1 BLER vs. SNR for 200m, UMi, 4 GHz

- 500m, UMa, 4 GHz

Assume the total PUSCH frequency resource is 1, 3, 6 PRB, single UE, TDL-A channel. Similar observations as those of UMi scenario can be found.

(a) 1 PRB

(b) 3 PRBs

(c) 6 PRBs

Figure 2 BLER vs. SNR for 500m, UMa, 4 GHz

- 1732m, RMa, 700 MHz

Assume the total PUSCH frequency resource is 1, 6, 12 PRB, single UE, TDL-A channel. Although the cell radius is larger than UMi/UMa, the performance is even better mainly due to less imposed frequency offset. The trends of BLER vs. bandwidth/antenna number are similar to that of other scenarios.

(a) 1 PRB

(b) 6 PRBs

(c) 12 PRBs

Figure 3 BLER vs. SNR for 1732m, RMa, 700 MHz

## Link budget calculation

Since the link budget calculation process of different scenarios is similar, here we consider the 4GHz + 200m results in Figure 1(c), 4Rx as an example. The elements (1)(2)(3)(4)(9) are the same for all cases, and element(5) is the channel bandwidth. The element (7) is the required SNR at 0.1 working point BLER derived from the figures in Section 2.1.

Table 1 Link budget calculation with 4G+200m Format A1

|  | 10bytes | 40bytes | 80bytes | 120bytes | 160bytes |
| --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 | 23 |
| Receiver |  |  |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 2160000 | 2160000 | 2160000 | 2160000 | 2160000 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -105.7 | -105.7 | -105.7 | -105.7 | -105.7 |
| (7) Required SINR (dB) (Target BLER=0.1) | -7.2 | -4.3 | -0.8 | 4.1 | 10.6 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -112.9 | -110 | -106.5 | -101.6 | -95.1 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 135.9 | 133 | 129.5 | 124.7 | 118.1 |

## Coupling loss distribution from SLS

System level simulation can be adopted to assist the payload size evaluation by drawing the coupling loss distribution for various scenarios. The simulation assumptions can be found in appendix, where most of the parameters are taken from the calibration campaign in the previous study of NOMA [8].

Figure 4 Coupling loss distribution in different scenarios

## Maximum supported TBsize

The MCL vs TBsize figures are drawn from the BLER vs SNR curves and link budget calculation. The 95% percentage of MCL is obtained from the coupling loss distribution from SLS. Then we plot a line of dashes on the MCL vs TBsize figure, and the maximum supported Tbsize can be observed clearly in the Figure 5, 6, 7.

It can be found that in general 10 bytes payload size can be satisfied with more than 95% of the coverage for the evaluated scenarios, i.e. ISD with 200/500/1732m with appropriate settings on the PRB number and received antenna.  Allocating more bandwidth or adding more receiving antennas can enhance the performance for large payload size, e.g. for 200m ISD, up to 160bits can be supported with 4Rx and 6PRB allocated bandwidth.

The percentage is decreasing when the cell size is enlarged due to higher path loss, and when the payload size is increased due to the loss of decoding performance.

- 4GHz + 200m

Figure 5 MCL vs TBsize of  scenario 4GHz + 200m

It can be found that more than 160 bytes can be supported in this scenario.

- 4GHz + 500m

Figure 6 MCL vs TBsize of  scenario 4GHz + 500m

It can be found that more than 11 bytes can be supported in this scenario.

- 700MHz + 1732m

Figure 7 MCL vs TBsize of  scenario 700MHz+1732m

It can be found that more than 29 bytes can be supported in this scenario.

Observation 1: With appropriate settings of resource size and antenna number, the minimum payload size same as msg3 (i.e. 56/72 bits) can be supported in different scenarios (cell size with 200, 500, 1732m). Larger payload sizes or cells sizes can be supported by allocating more resources or adding receiving antennas, e.g. for 200m ISD, up to 160bits can be supported with 4Rx and 6PRB allocated bandwidth.

# Preamble and PUSCH occasions mapping methods

For contention-based RA, when more than one UEs transmit preambles in the same RO, they may or may not transmit payload in the same PUSCH occasion, depending on the mapping schemes between PRACH preamble and PUSCH occasion. One way to minimize the PUSCH collision (which means sharing the same PUSCH occasion) and DMRS collision (which means different UEs select the same PUSCH occasion and the same DMRS port) is to allocate multiple PUSCH occasions (orthogonal) to be associated with the preambles in one PRACH occasion. While on the other hand, the code rate is enlarged under if we do not allocate more total reserved resources associated with a RO. Therefore, it is necessary to have some evaluations on the link performance regarding the number of PUSCH occasions associated to one RO.

## Evaluation results with 1 DMRS sequence

With the same evaluation assumptions listed in Table A1, and under the same total resource overhead (6 or 12 PRBs), 6 mapping types between Preamble and PUSCH occasions are considered for the evaluation. Two UEs with no preamble collision but possible collision of PUSCH transmission are assumed. This is mainly because the probability of more than two UEs shared on the same PUSCH resource is extremely small if the preamble collision probability is 1%, according to the analysis in our previous contribution [6]. In this case, only one DMRS sequence with 12 DMRS ports is considered.

The timing offset is estimated by preamble and the channel estimation is done by DMRS with compensated TO. The frequency offset is considered with 0.15ppm, i.e. 600Hz, without compensation. Both 2 and 4 receiving antennas are considered, the scenario is 4GHz + 200m with Format 0 and A1.

The transmitting power of the following schemes is normalized as the same:

For total 6 PRBs, PUSCH resource can be equally divided into 2, 3 and 6 groups, each UE can use 3, 2 and 1 PRBs respectively, denoted as scheme 1, 2, 3;

For total 12 PRBs, PUSCH resource can be equally divided into 4, 6 and 12 groups, each UE can use 3, 2 and 1 PRBs respectively, denoted as scheme 4, 5, 6.

For the 6 mapping types above, scheme 1 and 2 with total 6 PRBs has a DMRS collision probability of 1/24 and 1/36 respectively. Scheme 4 with total 12 PRBs has a DMRS collision probability of 1/48. The other 3 mapping types have no DMRS collision probability.

Note: X label is normalized SNR in order to ensure that each scheme has the same transmit power.

(a). 1T2R

(b). 1T4R

Figure 8 BLER vs. SNR for 2UEs with Preamble Format 0

.

(a). 1T2R

(b). 1T4R

Figure 9 BLER vs. SNR for 2UEs with Preamble Format A1

Based on the evaluation results shown in figure 9, it can be found that with normalized SNR, different mapping schemes show similar performance at the 0.1 BLER working point. Mapping types scheme 1 and 2 have error floor in high SNR especially for 2 receiving antennas.

Performance of different formats does not show significant difference, which means the accuracy of TO estimation by short preamble format A1 should be good enough for 4G+200m scenario.

Schemes with total 12 PRBs have less than 1dB performance better than total that of total 6 PRBs.

Although scheme 1 has the highest DMRS collision probability, the overall impact of performance loss can be covered by the benefit from low code rate.

Observation 2: With the aligned transmit power and total reserved resource overhead, the performance of different mapping schemes is similar.

## Comparison between different DMRS sequences

There is an opinion to introduce more DMRS sequences to avoid DMRS collision without increasing the total resource allocation. The following simulation results show the comparison between the performance of 1 DMRS sequence with collision and multiple DMRS sequences without collision. The scheme 1, 2 and 4 are considered in this case.

(a)1T2R

(b) 1T4R

Figure 10 Comparison between different DMRS sequences with Preamble Format 0

(a). 1T2R

- . 1T4R

Figure 11 Comparison between different DMRS sequences with Preamble Format A1

Observation 3: Performance of multiple DMRS sequences without collision is similar with that of 1 DMRS sequence with potential collision.

# Conclusion

Observation 1: With appropriate settings of resource size and antenna number, the minimum payload size same as msg3 (i.e. 56/72 bits) can be supported in different scenarios (cell size with 200, 500, 1732m). Larger payload sizes or cells sizes can be supported by allocating more resources or adding receiving antennas, e.g. for 200m ISD, up to 160bits can be supported with 4Rx and 6PRB allocated bandwidth.

Observation 2: With the aligned transmit power and total reserved resource overhead, the performance of different mapping schemes is similar.

Observation 3: Performance of multiple DMRS sequences without collision is similar with that of 1 DMRS sequence with potential collision.

# References

- RP-182894, “New work item: 2-step RACH for NR”, ZTE Corporation, Sanechips, 3GPP RAN#89, Sorrento, Italy, December 2018.

- “Chairman’s notes”, 3GPP RAN1#96, Athens, Greece, February 2019.

- R1-1901626, “Considerations on the channel structure of msgA”, ZTE Corporation, Sanechips, 3GPP RAN1#96, Athens, Greece, February 2019.

- R1-1903435, “Summary of 7.2.1.1 Channel Structure for Two-stepRACH”, ZTE Corporation, 3GPP RAN1#96, Athens, Greece, February 2019.

- R1-1903878, “On the channel structure of msgA”, ZTE Corporation, Sanechips, 3GPP RAN1#96bis, Xi’an, China, April 2019.

- R1-1903880, “Analysis on the Collision Probability of msgA”, ZTE Corporation, Sanechips, 3GPP RAN1#96bis, Xi’an, China, April 2019.

- R1-1903834, “Email discussion on potential link-level simulation assumption for 2-step RACH”, ZTE Corporation, 3GPP RAN1#96, Athens, Greece, Feb. 2019.

- 3GPP TR38.812, “Study on Non-Orthogonal Multiple Access (NOMA) for NR”, Release 15, v1.0.0

# Appendix

## A.1 Link Level simulation assumptions

Table A1: Link-level evaluation assumptions

| Parameters | Values |
| --- | --- |
| Scenario | 1) For evaluation of schemes: 200m, UMi, 4 GHz. 2) For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz. |
| Preamble format | 839 long Format 0 139 short Format A1 |
| Waveform (data part) | CP-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30kHz at 4 GHz |
| TBS | As described in section 2. |
| MCS and Resource size | QPSK or 16QAM, [1 3 6 12] PRBs |
| Number of UEs | 1 or 2, Equal avg. SNR is assumed for the two UEs |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx or 4Rx |
| Propagation channel & UE velocity | TDL-A 30ns, 3km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 105 Hz for 700MHz, 600 Hz for 4 GHz |
| Max number of HARQ transmission | 1 |
| Receiver | MMSE-IRC |
| Channel estimation | Realistic for both channel estimation and TO estimation. |
| Target BLER | 1% |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

## A.2 System Level simulation assumptions

Table A2 System level simulation assumptions

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |
| Inter-BS distance | 1732m | 500m | 200m |
| Minimum distance between UE and BS | 35m | 10m | 10m |
| Carrier frequency | 700MHz | 4GHz | 4GHz |
| Bandwidth | 10MHz |  |  |
| Channel model | UMa in TR 38.901 |  |  |
| Building penetration loss | The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz |  |  |
| BS Tx power | 46 dBm | 46 dBm | 41 dBm |
| BS antenna configurations | 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), +-45 Polarization; dH = dV = 0.8λ; |  |  |
| TXRU mapping to antenna elements on BS side | One TXRU per vertical dimension per polarization |  |  |
| TXRU mapping weights on BS side | TXRU virtualization only in the vertical dimension, i.e., sub-array partition model with 1D virtualization, refer to TR36.897 |  |  |
| Polarized antenna modeling | Model-2 in TR36.873 |  |  |
| BS antenna downtilt | 92 | 98 | 102 |
| BS antenna height | 25m |  |  |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |  |  |
| BS receiver noise figure | 5dB |  |  |
| UE Tx power | Max 23 dBm |  |  |
| UE antenna configuration | 1 (vertical polarization) |  |  |
| UT array orientation | uniformly distributed on [0,360] degree |  |  |
| UE antenna gain | 0dBi |  |  |
| UE receiver noise figure | 9dB |  |  |
| UE distribution and antenna height | 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell. |  |  |
| UE power control | Open loop PC, P0 = [-90] dBm, alpha = 1. |  |  |
| HARQ/repetition | 1 |  |  |
| Handover margin | 0dB |  |  |
| Wrapping method | Geographical distance based wrapping |  |  |
| UE attachment | Based on RSRP |  |  |