3GPP TSG RAN WG1 Meeting #97	R1-1905992

Reno, USA, May 13 – 17, 2019

Agenda Item:	7.2.5.1

Source:	Huawei, HiSilicon

Title:	Discussion on performance evaluation for NTN

Document for:	Discussion and Decision

Introduction

A new Study Item on “solutions evaluation for NR to support Non Terrestrial Network” was approved in RAN plenary #80, with the following objectives related to physical layer [1]:

Physical layer

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1].

To verify the effectiveness of various proposed solutions, a simulation framework with high fidelity and acceptable complexity is required. In this contribution, we discuss both link level and system level simulation assumptions for NTN performance evaluation.

System level simulation assumptions

The following three reference NTN scenarios are identified in [1]:

- Scenario A: GEO transparent satellite based NTN with fixed beams

- Scenario C2: LEO transparent satellite based NTN with moving beams

- Scenario D2: LEO regenerative satellite based NTN with moving beams

In practice, a constellation of LEO satellites are deployed in order to provide global coverage, because the typical foot print size provided by a single LEO satellite is much smaller than the coverage provided by a GEO satellite [2]. However, single satellite simulation may serve as a baseline to evaluate most of the physical layer solutions. An extended wrap-around mechanism can be used to model the intra-satellite interference for beams near satellite nadir point. As shown in Figure 1(a), in the wrap-around method used for terrestrial network, one or several layers comprising multiple hexagonal cells are created outside the reference cells. Differently, the extended wrap-around mechanism for NTN adds one or several layers comprising different beam bore-sight points. The bore-sight points can be placed at the center of hexagonal cells as in Figure 1(b), but the beam layout is not necessarily a hexagon.

From inter-satellite interference evaluation perspective, multiple satellites can be beneficial to model the interference for beams far from satellite nadir point. For those beams, beams from neighboring satellites will generate non-negligible interferences. To reduce the simulation complexity, instead of defining a complete satellite constellation for scenarios C2 and D2, it is better to define positions for up to 7 satellites (including 2 neighboring intra-orbit satellites, 4 neighboring inter-orbit satellites and the reference satellite itself). While for scenario A, it is sufficient to give positions for up to 3 satellites (including 2 neighboring intra-orbit satellites and the reference satellite itself).

Figure 1. Different wrap-around methods: (a) terrestrial; (b) NTN

Proposal 1: An extended wrap-around mechanism by adding one or several layers comprising different beam bore-sight points can be adopted for intra-satellite interference modeling.

Proposal 2: Multiple satellites should be considered to study the inter-satellite interference for beams at low elevation angle.

Satellite characteristics

In typical NR system level simulations, a full frequency reuse is adopted, i.e., all cells share the same frequency band as depicted in Figure 2(a). However, since the inter-cell interference assuming full frequency reuse can be an issue in NTN, an N-color scheme (e.g., N = 3 or 4) can be considered to handle non-negligible inter-beam/co-channel interferences [3]. As defined in [4], a color is characterized by both the frequency reuse factor (FRF) and polarization. Each coloring scheme defines a base coloring pattern by assigning each beam to a color. For example, a 4-color scheme shown in Figure 2(b) can mean 4 sub-bands and one polarization or 2 sub-bands and 2 polarizations. Comparing the full frequency reuse and N-color frequency reuse, one may need to balance between the available bandwidth in each beam and inter-beam co-channel interference. To strike a good balance and achieve better average beam throughput, a 3-color or 4-color scheme is widely used in satellite communication [5]. Considering that UEs for S band and Ka band generally adopt different polarization (i.e., linear and circular dual polarization, respectively), the FRF can be different as well.

Observation 1: Full frequency reuse is adopted in NR while N-color frequency reuse is usually employed for current satellite communication.

Proposal 3: A 3-color or 4-color scheme can be considered for NTN performance evaluation.

Figure 2. Different frequency reuse patterns: (a) full frequency reuse; (b) 4-coloring scheme

In practice, a satellite may equip with tens or even hundreds of beams to provide large coverage and high throughput. For typical scenario settings (e.g., LEO 1200km with 100 km beam radius and 10 degree elevation angle), a satellite can contain more than 500 beams in total. To reduce complexity of system-level evaluation, a 19-beam layout with satellite nadir point located at the centermost beam may be more realistic for interference modeling considering different coloring schemes. Besides, if lower elevation angles for the beam center are required, other beam layout configurations (e.g., satellite position or bore-sight beam directions) can be further considered.

Proposal 4: A 19-beam layout with possibly different configurations should be considered for system level simulation in NTN.

Performance metrics and detailed assumptions

Two metrics can be used for calibration purpose: Coupling loss and Geometry. Similar to typical NR evaluation, user perceived throughput (UPT) and packet delay can be employed as the performance metrics for NTN system level simulation. Other metrics can also be considered if needed. The detailed simulation assumptions for system level simulation are listed in Table A1, Table A2 and Table A3 in the appendix.

Proposal 5: Take the simulation parameters in Table A1, Table A2 and Table A3 as the starting point for NTN system level evaluation.

Link-level simulation assumptions

Generally, the link level simulation assumptions for NR [6] should be taken as guideline. Some parameters should be revised to adapt the characteristics of the NTN channels and payloads of satellites.

Downlink synchronization

In downlink synchronization, the Doppler shift is a major NTN-specific characteristic. The Doppler value due to the satellite movement can be up to several times of the SCS value, which would be a great challenge for PSS detection [2]. The value is the function of satellite orbits and elevation angles. Pre-compensation can be applied at the satellite side to mitigate the major part of the Doppler offset, however, the residual frequency offset is still large in cells around the nadir point. Furthermore, the maximum UE speed in NTN scenarios can be up to 1000km/h. Those impacts should be considered in the downlink synchronization.

Proposal 6: A maximum frequency offset value that reflects the typical NTN deployment scenarios should be determined for both regenerative and transparent cases.

The simulation assumptions for downlink synchronization are listed as follows in Table 1.

Table 1:  Simulation assumptions for NTN downlink synchronization

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | TDL/CDL model in TR38.811, with scaling factor 30ns for sub6GHz, 300ns for above 6GHz |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| SNR range | > -6 dB | > -18dB |
| Search window | NR as baseline |  |
| Antenna Configuration at the TRP | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | UE: uniform distribution [+/- 0.1 ppm] BS: uniform distribution +/- 0.05 ppm Doppler in channel: max. +/-25.7 ppm for D2 and C2 - Max. UE speed:1000km/h - Min. Elevation angle for both sat-gateway and user equipment: 10 degree |  |
| Number of interfering TRPs | 0 TRP as start point FFS interference from other TRPs, based on the frequency reuse pattern or constellation |  |
| Phase noise | Table 6 in R1-1905216 | Table 7 in R1-1905216 |

The following performance metrics should be reported:

- PSS detection rate for one shot

- FAR detection rate

- Residual frequency offset

PRACH

The main challenge of PRACH in NTN is the differential delay which is much larger than terrestrial network. So new PRACH formats may be needed. When a PRACH signal is transmitted by the UE, the frequency offset at the UE side is smaller than that in DL synchronization since the major part has been compensated based on the downlink frequency tracking. In Table A.1.5-2 of TR38.802, 0.1 ppm is assumed to model the UE side frequency offset. However, for LEO-based access, the PSS detection algorithm has to be revised to address the huge Doppler shift at the receiver side, and the value of frequency offset at UE side is FFS. Therefore, the value assumed in TR38.802 needs to be revisited based on the evaluation results of downlink synchronization.

The simulation assumptions for PRACH are listed as follows in Table 2.

Table 2:  Simulation assumptions for NTN for PRACH

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | TDL/CDL model in TR38.811, with scaling factor 30ns for sub6GHz, 300ns for above 6GHz |  |
| Antenna Configuration at the TRP | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | UE: value FFS, based on the residual frequency offset of DL synchronization BS : uniform distribution +/- 0.05 ppm |  |
| PRACH design | FFS |  |
| Phase noise | Table 6 in R1-1905216 | Table 7 in R1-1905216 |

The following performance metrics should be reported:

- PRACH detection rate

- FAR detection rate

Data transmission

The main challenges for NTN data transmission are the limited link budget and large propagation delay. For the first challenge, we suggest to start the evaluation based on the MCS table introduced for NR URLLC (Table 5.1.3.1-3 of TS38.214 for DL and Table 6.1.4.1-2 for UL). The large propagation delay will impact the efficiency of CSI feedback. Both the open-loop and the closed-loop AMC should be evaluated.

The simulation assumptions for data transmission are listed as follows in Table 3.

Table 3:  Simulation assumptions for NTN for data transmission

| Parameters | Value |  |
| --- | --- | --- |
| Carrier frequency | 2GHz | DL 20GHz |
| Duplex | FDD |  |
| Subcarrier spacing | 15kHz, 30kHz | 60kHz, 120kHz |
| CSI feedback | No feedback / periodic feedback |  |
| Modulation order, Coding rate | NR MCS Table (introduced for URLLC) |  |
| Channel coding scheme | NR channel coding |  |
| Link adaptation | Open-loop and closed-loop |  |
| HARQ | Disabled |  |
| Channel estimation | Realistic estimation |  |
| Frequency offset model | Frequency offset in case of non-initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 0.1 ppm |  |
| UE speed | 3 km/h, 120 km/h, 1000km/h |  |
| Channel model | TDL/CDL model in TR38.811, with scaling factor 30ns for sub6GHz, 300ns for above 6GHz |  |
| TRP antenna configuration | 1Tx | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element | (4, 8, 2), with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise | Table 6 in R1-1905216 | Table 7 in R1-1905216 |

The following performance metrics should be reported:

- BLER or throughput

Conclusions

In this contribution, both system level and link level simulation assumptions are elaborated. Based on the discussion in the previous sections, we made the following observations and proposals:

Observation 1: Full frequency reuse is adopted in NR while N-color frequency reuse is usually employed for current satellite communication.

Proposal 1: An extended wrap-around mechanism by adding one or several layers comprising different beam bore-sight points can be adopted for intra-satellite interference modeling.

Proposal 2: Multiple satellites should be considered to study the inter-satellite interference for beams at low elevation angle.

Proposal 3: A 3-color or 4-color scheme can be considered for NTN performance evaluation.

Proposal 4: A 19-beam layout with possibly different configurations should be considered for system level simulation in NTN.

Proposal 5: Take the simulation parameters in Table A1, Table A2 and Table A3 as the starting point for NTN system level evaluation.

Proposal 6: A maximum frequency offset value that reflects the typical NTN deployment scenarios should be determined for both regenerative and transparent cases.

- References

RP-182880, “SID NR-NTN solutions_v60”.

TR 38.811,” Study on New Radio (NR) to support non terrestrial networks”

M. Á. Vázquez, A. Pérez-Neira, D. Christopoulos, S. Chatzinotas, B. Ottersten, P.-D. Arapoglou, A. Ginesi, and G. Tarocco, “Precoding in multibeam satellite communications: Present and future challenges”.  IEEE Wireless Commun., vol. 23, no. 6, pp. 88-95, Dec. 2016.

Yoann Couble, Catherine Rosenberg, Jean-Baptiste Dupé, Cédric Baudoin, and André-Luc Beylot, “Two-Color Scheme for a Multi-Beam Satellite Return Link: Impact of Interference Coordination,” IEEE Journal on Selected Areas in Comm., vol. 36, no. 5, pp. 993-1003, May 2018.

H. Fenech, S. Amos, A. Tomatis, and V. Soumpholphakdy, “High throughput satellite systems: An analytical approach,” IEEE Trans. Aerosp. Electron. Syst., vol. 51, no. 1, pp. 192-0202, Jan. 2015.

TR 38.802, “Study on New Radio (NR) Access Technology; Physical Layer Aspects”.

- Appendix

Table A1.  System level simulation parameters for configurations A, C2 and D2

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | S-band / Ka- Band |
| Maximum Bandwidth per beam (DL + UL) | S-band : DL 30 MHz and UL 30 MHz Ka band : DL 400 MHz and UL 400 MHz |
| Reference satellite position | GEO: (42157,0,0) LEO-600: (6971,0,0) LEO-1200: (7571,0,0) |
| Gateway position | (6371,0,0) |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table A2 |
| Satellite antenna pattern | TR 38.811v15.0.0 section 6.4.1 Bessel function |
| Satellite polarization configuration | Circular |
| Beam layout | Bore-sight points placed at the center of hexagonal cells as in Figure 1(a) |
| Number of beams | 19 |
| Frequency re-use factor | S band: 3 or 4 for linear polarization Ka band: 4 for linear polarization and 2 for dual circular polarization |
| Deployment scenarios | Base-line : Rural Additional deployment scenario results can be provided |
| Fast fading model | Frequency selective channel model from TR 38.811v15.0.0 |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | X=10 UEs per beam with uniform distribution in all the beams Other values of X: FFS |
| UE configuration | S-band : - Handheld - Others (optional) Ka-band : - VSAT - Others |
| UE orientation | VSAT: Ideal Tracking serving beam; Handheld: Random Others: FFS |
| UE antenna and polarization configurations | S-band : - Handheld : Linear - Others: FFS Ka-band : - VSAT : Circular - Others : FFS |
| UE antenna noise figure, antenna temperature, G/T | S-band : - Handheld : Table 4.4-1 in TR38.811 V15 - Others: FFS Ka-band : - VSAT : Table 4.4-1 in TR38.811 V15 - Others : FFS |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base line : Coupling loss, Geometry Optional : FFS |
| Receiver | MMSE-IRC |
| Reference signals configuration | Table A3 |
| Scheduler | PF |
| Traffic model | FTP 3 for handheld, packet size 0.5Mbytes, arrival ratio TBD; TBD for VSAT; |
| Channel estimation | Realistic |
| MIMO mode | IT-1R |
| CSI feedback | Periodic |
| Metrics for performance evaluation | Baseline: UE throughput (5%, 50%, 95%) Optional : Packet delay |

Table A2.  System level simulation parameters for downlink transmission and uplink reception

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Equivalent satellite antenna aperture (Note1) | S-band | 22m | 2m | 2m |
| Satellite EIRP density （dBW/MHz） |  | 25.3 | 31.3 | 55.3 |
| Tx gain (dB) |  | 50 | 29 | 29 |
| Rx gain (dB) |  | 50 | 29 | 29 |
| Satellite Noise Temp. （dB (K)） |  | 26 | 26 | 26 |
| G/T（dB K-1） |  | 24 | 3 | 3 |
| Equivalent satellite antenna aperture (Note1) | Ka-band | 2.2 m | 0.2m | 0.2m |
| Satellite EIRP density （dBW/MHz） |  | 15.2 | 21.2 | 45.2 |
| Tx gain (dB) |  | 50 | 29 | 29 |
| Rx gain (dB) |  | 53.4 | 32.5 | 32.5 |
| Satellite Noise Temp. （dB (K)） |  | 27 | 27 | 27 |
| G/T（dB K-1） |  | 26.4 | 5.5 | 5.5 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR38.811v15.0.0. |  |  |  |  |

Table A3.  Reference signals configuration

| DL | PDCCH | 2 symbols* |
| --- | --- | --- |
|  | DMRS | Type II, 2 symbols |
|  | CSI-RS | 1 port for each beam |
|  | SSB | 1 SSB for each beam, per 20 ms* |
|  | TRS | 2 consecutive slots per 20 ms, 1 port* |
|  | PTRS | 1 port PT-RS, (L,K) = (1,4)* |
| UL | PUCCH | 2 RBs, 14 OFDM symbols* |
|  | DMRS | Type II, 2 symbols |
|  | SRS | 2 symbols per 5 slots,8 RBs per symbol* |
|  | PTRS | 1 port PT-RS, (L,K) = (1,4)* |
| Note: Parameter value with mark * is reused from ITU self-evaluation. Reference in R1-1808073. |  |  |