3GPP TSG RAN WG1 Meeting #97	R1-1905997

Reno, USA, May 13 – 17, 2019

Agenda Item:	7.2.5.5

Source:	Huawei, HiSilicon

Title:	Discussion on link budget for NTN

Document for:	Discussion and Decision

Introduction

A new SI was approved by RAN#80 on studying a set of necessary features/adaptations enabling the operation of NR in non-terrestrial networks (NTN) [1]. Specifically, the link budget for NTN is different from terrestrial network [2]:

| Non Terrestrial Network Design Constraints | Differences between NTN and cellular systems |
| --- | --- |
| Power limited link budget | The main design drivers of satellite and HAPS based communication systems are; - To maximise the throughput for a given transmit power from the UE on the UL and from the satellite/HAPS on the DL. - To maximise the availability of the service under deep fading situations (typically between 20 and 30 dB in Ka band for 99.95% availability) |

The most significant difference of NTN compared with terrestrial network is that both UL and DL in NTN are power limited. In this contribution we present our views on link budget and system throughput, and provide some simulation results as a benchmark for further study.

Discussion

The link budget (C/(N+I)) in free space between a transmitter and a receiver consists of two parts, carrier power over noise density (C/N) and carrier power over interference noise density (C/I). The general formulas are expressed as below [3]:

CN+I dB=10×LOG(1/(10-0.1×CN(dB)+10-0.1×CI(dB)))                          (1)

where

CNdB=EIRPdBW+GTdBK-kdBWKHz-PLOSSdB-ALOSSdB

-PMARGINSdB-PPOLdB-PCabledB-10LOG(BW)                                      (2)

•C/N (dB): carrier power over noise density ratio

•EIRP (dBW): effective isotropic radiated power

•G/T (dB/K): figure of merit of the receiver, where G is the receiver antenna gain, and T the equivalent system temperature.

•PLoss (dB): free space loss

•ALOSS (dB): atmospheric path loss due to gases and rain fades

•PMARGINS (dB): shadowing margin

•PPOL (dB): polarization mismatch loss

•PCable (dB): cable loss

•k (dBW/K/Hz): Boltzmann Constant: -228.6

S Band of LEO and GEO satellites

The following Table 2.1 presents the link budget analysis for S band as well as the assumptions. The terminal is assumed to be identical for different orbit satellites and the parameters are set in accordance with reference [2]. The satellite EIRP and satellite G/T are different for GEO and LEO and we refer the values from [4] and [5].

Table 2.1 Link budget analysis for S band

|  | LEO 600 km |  | LEO 1200km |  | GEO 35786 km |  |
| --- | --- | --- | --- | --- | --- | --- |
| Link Parameters |  |  |  |  |  |  |
| DL/UL | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| Carrier frequency (GHz) | 2 | 2 | 2 | 2 | 2 | 2 |
| Total bandwidth (MHz) | 30 | 30 | 30 | 30 | 30 | 30 |
| Frequency reusing factor | 3 | 3 | 3 | 3 | 3 | 3 |
| Channel bandwidth (MHz) | 10 | 10 | 10 | 10 | 10 | 10 |
| Satellite altitude (km) | 600 | 600 | 1200 | 1200 | 35786 | 35786 |
| SCS (kHz) | 15 | 15 | 15 | 15 | 15 | 15 |
| Number of PRBs | 52 | 6 | 52 | 6 | 52 | 1 |
| PRBs bandwidth (MHz) | 9.36 | 1.08 | 9.36 | 1.08 | 9.36 | 0.18 |
| Elevation angle () | 30 | 30 | 30 | 30 | 30 | 30 |
| Distance between satellite and UE (km) | 1075.1 | 1075.1 | 1998.9 | 1998.9 | 38609 | 38609 |
| Free space path loss (dB) | 159.1 | 159.1 | 164.5 | 164.5 | 190.2 | 190.2 |
| Atmospheric loss (Clear sky) (dB) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Polarization loss (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| Cable loss (dB) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Shadowing margin (dB) | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Average C/I within a satellite beam (dB) | 16 | 16 | 16 | 16 | 16 | 16 |
| Channel model | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Satellite Parameters |  |  |  |  |  |  |
| EIRP per channel (dBW) | 35 | - | 41 |  | 65 | - |
| EIRP density per channel (dBW/MHz) | 25.3 |  | 31.3 |  | 55.3 |  |
| Equivalent satellite antenna aperture [Note1] | 2 | 2 | 2 | 2 | 22 | 22 |
| Satellite Tx antenna Gain (dBi) | 29 |  | 29 |  | 50 |  |
| Satellite Rx antenna Gain (dBi) | - | 29 |  | 29 | - | 50 |
| Satellite antenna polarization | Circular | Circular | Circular | Circular | Circular | Circular |
| Rx antenna noise figure (dB) |  | 2.7 |  | 2.7 |  | 2.7 |
| Rx antenna ambient temperature (K) | 290 | - | 290 |  | 290 | - |
| Rx antenna temperature (K) | 150 | - | 150 |  | 150 | - |
| Satellite G/T (dB/K) | - | 3 |  | 3 | - | 24 |
| Terminal Parameters |  |  |  |  |  |  |
| Terminal type | 3GPP Class 3 |  |  |  |  |  |
| Terminal G/T (dB/K) | -33.62 | - | -33.62 |  | -33.62 | - |
| Terminal antenna type | Omnidirectional antenna (linear) |  |  |  |  |  |
| Terminal antenna polarization | Linear | Linear | Linear | Linear | Linear | Linear |
| Terminal receive antenna gain (dBi) | 0 | - | 0 |  | 0 | - |
| Terminal noise figure (dB) | 9 | - | 9 |  | 9 | - |
| Terminal ambient temperature (K) | 290 | - | 290 |  | 290 | - |
| Terminal antenna temperature (K) | 290 | - | 290 |  | 290 | - |
| Terminal transmit power (dBW) | - | -7 |  | -7 | - | -7 |
| Terminal transmit antenna gain (dBi) | - | 0 |  | 0 | - | 0 |
| Link Budget Results |  |  |  |  |  |  |
| Terminal C/(N+I) (dB) | -4.42 | - | -3.83 | - | -5.52 | - |
| Satellite C/(N+I) (dB) | - | -0.43 | - | -5.75 | - | -2.71 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of [2]. |  |  |  |  |  |  |

Observation 1: The path loss of GEO scenario is around 30 dB higher than 600 km LEO scenario. Although the EIRP and satellite G/T of GEO scenario is about 30 dB and 21 dB higher than 600km LEO scenario respectively, the C/(N+I) of GEO is still lower than that of LEO.

Observation 2: Due to the transmitter power limitation, it is necessary to enhance the terminal receiver capability when higher C/(N+I) is required.

Observation 3: The EIRP per channel is determined by the satellite payload power capacity and EIRP density is highly related to the frequency reuse factor.

Ka Band of LEO and GEO satellites

The following Table 2.2 presents the link budget analysis for Ka band and the assumptions are also listed in the table. The terminal is assumed to be identical for different orbit satellites and the parameters are set in accordance with reference [2]. The satellite EIRP and satellite G/T are different and referred from [4] and [5].

Table 2.2 Link budget analysis for Ka band

|  | LEO 600 km |  | LEO 1200km |  | GEO 35786 km |  |
| --- | --- | --- | --- | --- | --- | --- |
| Link Parameters |  |  |  |  |  |  |
| DL/UL | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| Carrier frequency (GHz) | 20 | 30 | 20 | 30 | 20 | 30 |
| Total bandwidth (MHz) | 400 | 400 | 400 | 400 | 400 | 400 |
| Frequency reusing factor | 4 | 4 | 4 | 4 | 4 | 4 |
| Channel bandwidth (MHz) | 100 | 100 | 100 | 100 | 100 | 100 |
| Satellite altitude (km) | 600 | 600 | 1200 | 1200 | 35786 | 35786 |
| SCS (kHz) | 120 | 120 | 120 | 120 | 120 | 120 |
| Number of PRBs | 66 | 32 | 66 | 32 | 66 | 16 |
| PRBs bandwidth (MHz) | 95.04 | 46.08 | 95.04 | 46.08 | 95.04 | 23.04 |
| Elevation angle () | 30 | 30 | 30 | 30 | 30 | 30 |
| Distance between satellite and UE (km) | 1075.1 | 1075.1 | 1998.9 | 1998.9 | 38609 | 38609 |
| Free space path loss (dB) | 179.1 | 182.62 | 184.5 | 188.01 | 210.2 | 213.73 |
| Atmospheric loss (Clear sky) (dB) | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Polarization loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| Cable loss (dB) | 2 | 2 | 2 | 2 | 2 | 2 |
| Shadowing margin (dB) | 5 | 5 | 5 | 5 | 5 | 5 |
| Average C/I within a satellite beam (dB) | 16 | 16 | 16 | 16 | 16 | 16 |
| Channel model | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Satellite Parameters |  |  |  |  |  |  |
| EIRP per channel (dBW) | 35 | - | 41 | - | 65 | - |
| EIRP density per channel (dBW/MHz) | 15.2 |  | 21.2 |  | 45.2 |  |
| Equivalent satellite antenna aperture [Note1] | 0.2 | 0.2 | 0.2 | 0.2 | 2.2 | 2.2 |
| Satellite Tx antenna Gain (dBi) | 29 | - | 29 | - | 50 | - |
| Satellite Rx antenna Gain (dBi) | - | 32.5 | - | 32.5 | - | 53.4 |
| Satellite antenna polarization | Circular | Circular | Circular | Circular | Circular | Circular |
| Rx antenna noise figure (dB) |  | 3.4 |  | 3.4 |  | 3.4 |
| Rx antenna ambient temperature (K) | 290 | - | 290 | - | 290 | - |
| Rx antenna temperature (K) | 150 | - | 150 | - | 150 | - |
| Satellite G/T (dB/K) | - | 5.5 | - | 5.5 | - | 26.5 |
| Terminal Parameters |  |  |  |  |  |  |
| Terminal type | VSAT |  |  |  |  |  |
| Terminal G/T (dB/K) | 15.86 | - | 15.86 |  | 15.86 | - |
| Terminal antenna type | 60 cm equivalent aperture diameter (circular) |  |  |  |  |  |
| Terminal antenna polarization | Circular | Circular | Circular | Circular | Circular | Circular |
| Terminal receive antenna gain (dBi) | 39.7 | - | 39.7 | - | 39.7 | - |
| Terminal noise figure (dB) | 1.2 | - | 1.2 | - | 1.2 | - |
| Terminal ambient temperature (K) | 290 | - | 290 | - | 290 | - |
| Terminal antenna temperature (K) | 150 | - | 150 | - | 150 | - |
| Terminal transmit power (dBW) | - | 3 | - | 3 | - | 3 |
| Terminal transmit antenna gain (dBi) | - | 43.2 | - | 43.2 | - | 43.2 |
| Link Budget Results |  |  |  |  |  |  |
| Terminal C/(N+I) (dB) | 10.57 | - | 10.99 | - | 9.75 | - |
| Satellite C/(N+I) (dB) | - | 10.96 | - | 6.67 | - | 5.07 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of [2]. |  |  |  |  |  |  |

Observation 4: Due to the high gain of terminal antenna, Ka band obtains higher C/(N+I) than S band , even though the propagation loss is higher and the channel bandwidth is larger.

Observation 5: The path loss of Ka band is 20dB higher than S band for all considered scenarios.

Analysis

Based on all the parameters (C/I, EIRP, G/T, k, Ploss, Aloss, Pmargin, PAD), the link budget can be calculated for different scenarios based on formula (1). However, the system throughput is not only related to link budget, but also the system bandwidth and spectrum efficiency. Different systems require different SNR for the same MCS, and the MCS table for downlink and uplink are also different [6]. The detailed simulation assumptions for S band and Ka band are listed in Table A.1 and B.1, respectively. The MCS table introduced for URLLC in NR Rel-15 are employed. Table A.2-4 and Table B.2-4 provide the mapping between MCS and SNR for PDSCH and PUSCH. Each MCS entry is mapped to a specific SNR and throughput, as well as spectrum efficiency. With these tables, the corresponding throughput of each scenario can be obtained and the summary of link budget analysis is shown in Table 3.1 and Table 3.2.

Table 3.1 Downlink Link budget summary of different scenarios

| Downlink | S Band |  |  | Ka Band |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | LEO 600 km | LEO 1200 km | GEO 35786 km | LEO 600 km | LEO 1200 km | GEO 35786 km |
| Terminal | 3GPP Class 3 |  |  | VSAT |  |  |
| C/(N+I) (dB) | -4.42 | -3.83 | -5.52 | 10.57 | 10.99 | 9.75 |
| PRB bandwidth (MHz) | 9.36 | 9.36 | 9.36 | 95.04 | 95.04 | 95.04 |
| Throughput (kbps) | 2224.8 | 2793.6 | 1735.2 | 195437 | 217555 | 184378 |
| Spectrum efficiency (bit/s/Hz) | 0.24 | 0.30 | 0.18 | 2.10 | 2.33 | 1.98 |

Table 3.2 Uplink Link budget summary of different scenarios

| Uplink | S Band |  |  | Ka Band |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | LEO 600 km | LEO 1200 km | GEO 35786 km | LEO 600 km | LEO 1200 km | GEO 35786 km |
| Terminal | 3GPP Class 3 |  |  | VSAT |  |  |
| C/(N+I) (dB) | -0.43 | -5.72 | -2.65 | 10.96 | 6.67 | 5.07 |
| PRB bandwidth (MHz) | 1.08 | 1.08 | 0.18 | 46.08 | 46.08 | 23.04 |
| Throughput (kbps) | 518.4 | 158.4 | 36 | 106963.2 | 68256 | 29491.2 |
| Spectrum efficiency (bit/s/Hz) | 0.48 | 0.15 | 0.20 | 2.32 | 1.48 | 1.28 |

Observation 6: The Ka band has higher spectrum efficiency and larger channel bandwidth than S band, which implies that Ka band may be more suitable for eMBB applications.

Proposal 1: The upper limit of satellite EIRP per beam for different scenarios should be discussed and determined.

Proposal 2: The frequency reuse factor should be considered when evaluating the link budget.

Proposal 3: Capture the link budget analysis results in this contribution into TR.

Conclusion

In this document, we discuss the link budget for NR NTN. The following observations were presented:

Observation 1: The path loss of GEO scenario is around 30 dB higher than 600 km LEO scenario. Although the EIRP and satellite G/T of GEO scenario is about 30 dB and 21 dB higher than 600km LEO scenario respectively, the C/(N+I) of GEO is still lower than that of LEO.

Observation 2: Due to the transmitter power limitation, it is necessary to enhance the terminal receiver capability when higher C/(N+I) is required.

Observation 3: The EIRP per channel is determined by the satellite payload power capacity and EIRP density is highly related to the frequency reuse factor.

Observation 4: Due to the high gain of terminal antenna, Ka band obtains higher C/(N+I) than S band , even though the propagation loss is higher and the channel bandwidth is larger.

Observation 5: The path loss of Ka band is 20dB higher than S band for all considered scenarios.

Observation 6: The Ka band has higher spectrum efficiency and larger channel bandwidth than S band, which implies that Ka band may be more suitable for eMBB applications.

Proposal 1: The upper limit of satellite EIRP per beam for different scenarios should be discussed and determined.

Proposal 2: The frequency reuse factor should be considered when evaluating the link budget.

Proposal 3: Capture the link budget analysis results in this contribution into TR.

- References

RP-181370, “Study on solutions evaluation for NR to support Non-Terrestrial Network”, Thales, RAN#80, La Jolla, USA, June 11-14, 2018

3GPP TR 38.811 V 15.0.0, “Study on New Radio (NR) to support non terrestrial networks (Release 15)”, August 10, 2018

RP-181366, “NR-NTN: update on link budget analysis”, Thales, Ligado, HNS, ESA, 3GPP RAN#80, La Jolla, USA, 11th June – 14th June 2018.

Inigo del Portilloa, Bruce G. Cameron, Edward F. Crawley, “A Technical Comparison of Three Low Earth Orbit Satellite Constellation Systems to Provide Global Broadband”, 69th International Astronautical Congress (IAC), Bremen, Germany, 1-5 October 2018.

R1-1901723, “Link Budget Analysis in NTN”, Nokia, Nokia Shanghai Bell, RAN#96, Athens, Greece, Feb. 25-Mar.1, 2019

3GPP TS 38.214, “NR; Physical layer procedures for data (Release 15)”.

- Appendix A: Simulation assumptions and results for S band scenarios

Table A.1: Simulation assumptions for S band scenarios

| Scenario | LEO 600km |  | LEO 1200km |  | GEO |  |
| --- | --- | --- | --- | --- | --- | --- |
| DL/UL | DL | UL | DL | UL | DL | UL |
| Simulation | Simulation 1 | Simulation 2 | Simulation 1 | Simulation 2 | Simulation 1 | Simulation 3 |
| MCS table | 38.214 Table 5.1.3.1-3 | 38.214 Table 6.1.4.1-2 | 38.214 Table 5.1.3.1-3 | 38.214 Table 6.1.4.1-2 | 38.214 Table 5.1.3.1-3 | 38.214 Table 6.1.4.1-2 |
| MCS range | 0~28 | 0~27 | 0~28 | 0~27 | 0~28 | 0~27 |
| SCS | 15KHz | 15KHz | 15KHz | 15KHz | 15KHz | 15KHz |
| # RB | 52 | 6 | 52 | 6 | 52 | 1 |
| # total RE | 8736 | 1008 | 8736 | 1008 | 8736 | 168 |
| # data RE | 8112 | 936 | 8112 | 936 | 8112 | 156 |
| LDPC decoder | LOMS 20 | LOMS 20 | LOMS 20 | LOMS 20 | LOMS 20 | LOMS 20 |
| Channel type | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |

Table A.2: Required SNR, THP and spectral efficient of Simulation 1

| MCS | Tbsize (bit) | SNR@BLER=0.1 (dB) | THP(Kbps) | Spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 0 | 480 | -11.8 | 432 | 0.05 |
| 1 | 640 | -10.6 | 576 | 0.06 |
| 2 | 808 | -9.6 | 727.2 | 0.08 |
| 3 | 1032 | -8.6 | 928.8 | 0.10 |
| 4 | 1256 | -7.7 | 1130.4 | 0.12 |
| 5 | 1608 | -6.7 | 1447.2 | 0.15 |
| 6 | 1928 | -6 | 1735.2 | 0.18 |
| 7 | 2472 | -4.8 | 2224.8 | 0.24 |
| 8 | 3104 | -3.9 | 2793.6 | 0.30 |
| 9 | 3976 | -2.6 | 3578.4 | 0.38 |
| 10 | 4864 | -1.8 | 4377.6 | 0.47 |
| 11 | 6016 | -0.7 | 5414.4 | 0.58 |
| 12 | 7040 | 0.2 | 6336 | 0.69 |
| 13 | 8192 | 1.2 | 7372.8 | 0.80 |
| 14 | 9480 | 2.2 | 8532 | 0.91 |
| 15 | 10760 | 3.5 | 9684 | 1.02 |
| 16 | 12040 | 4.2 | 10836 | 1.15 |
| 17 | 13832 | 5.2 | 12448.8 | 1.31 |
| 18 | 15624 | 6.1 | 14061.6 | 1.51 |
| 19 | 17424 | 6.9 | 15681.6 | 1.70 |
| 20 | 19464 | 7.9 | 17517.6 | 1.89 |
| 21 | 21000 | 9.4 | 18900 | 2.02 |
| 22 | 22032 | 9.9 | 19828.8 | 2.11 |
| 23 | 24576 | 10.8 | 22118.4 | 2.37 |
| 24 | 27144 | 11.8 | 24429.6 | 2.56 |
| 25 | 29192 | 12.6 | 26272.8 | 2.82 |
| 26 | 31752 | 13.6 | 28576.8 | 3.07 |
| 27 | 33816 | 14.3 | 30434.4 | 3.27 |
| 28 | 36896 | 15.4 | 33206.4 | 3.52 |

Table A.3: Required SNR, THP, and spectral efficient of Simulation 2

| MCS | Tbsize (bit) | SNR@BLER=0.1 (dB) | THP(Kbps) | Spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 0 | 48 | -10.4 | 43.2 | 0.04 |
| 1 | 72 | -9.2 | 64.8 | 0.06 |
| 2 | 88 | -8.5 | 79.2 | 0.07 |
| 3 | 112 | -7.7 | 100.8 | 0.09 |
| 4 | 136 | -7 | 122.4 | 0.11 |
| 5 | 176 | -6.1 | 158.4 | 0.15 |
| 6 | 224 | -5.2 | 201.6 | 0.19 |
| 7 | 288 | -4.3 | 259.2 | 0.24 |
| 8 | 352 | -3.5 | 316.8 | 0.29 |
| 9 | 456 | -2.4 | 410.4 | 0.38 |
| 10 | 576 | -1.3 | 518.4 | 0.48 |
| 11 | 704 | -0.3 | 633.6 | 0.59 |
| 12 | 848 | 0.7 | 763.2 | 0.71 |
| 13 | 984 | 1.6 | 885.6 | 0.82 |
| 14 | 1128 | 2.5 | 1015.2 | 0.94 |
| 15 | 1256 | 3.3 | 1130.4 | 1.05 |
| 16 | 1416 | 4.5 | 1274.4 | 1.18 |
| 17 | 1608 | 5.3 | 1447.2 | 1.34 |
| 18 | 1800 | 6 | 1620 | 1.50 |
| 19 | 2024 | 7 | 1821.6 | 1.69 |
| 20 | 2280 | 8.1 | 2052 | 1.90 |
| 21 | 2408 | 8.6 | 2167.2 | 2.01 |
| 22 | 2536 | 9.1 | 2282.4 | 2.11 |
| 23 | 2865 | 10.4 | 2578.5 | 2.39 |
| 24 | 3104 | 11.6 | 2793.6 | 2.59 |
| 25 | 3368 | 12.5 | 3031.2 | 2.81 |
| 26 | 3752 | 13.8 | 3376.8 | 3.13 |
| 27 | 4224 | 15.4 | 3801.6 | 3.52 |

Table A.4: Required SNR, THP and spectral efficient of Simulation 3

| MCS | Tbsize (bit) | SNR@BLER=0.1 (dB) | THP(Kbps) | Spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 0 | 24 | -4.2 | 21.6 | 0.12 |
| 1 | 24 | -4.2 | 21.6 | 0.12 |
| 2 | 24 | -4.2 | 21.6 | 0.12 |
| 3 | 24 | -4.2 | 21.6 | 0.12 |
| 4 | 24 | -4.2 | 21.6 | 0.12 |
| 5 | 24 | -4.2 | 21.6 | 0.12 |
| 6 | 32 | -3.7 | 28.8 | 0.16 |
| 7 | 40 | -3.1 | 36 | 0.20 |
| 8 | 56 | -2.1 | 50.4 | 0.28 |
| 9 | 72 | -1.2 | 64.8 | 0.36 |
| 10 | 88 | -0.4 | 79.2 | 0.44 |
| 11 | 112 | 0.7 | 100.8 | 0.56 |
| 12 | 136 | 1.6 | 122.4 | 0.68 |
| 13 | 160 | 2.5 | 144 | 0.80 |
| 14 | 176 | 3.1 | 158.4 | 0.88 |
| 15 | 208 | 4.2 | 187.2 | 1.04 |
| 16 | 224 | 4.9 | 201.6 | 1.12 |
| 17 | 272 | 6.1 | 244.8 | 1.36 |
| 18 | 304 | 6.9 | 273.6 | 1.52 |
| 19 | 336 | 7.8 | 302.4 | 1.68 |
| 20 | 368 | 8.5 | 331.2 | 1.84 |
| 21 | 408 | 9.5 | 367.2 | 2.04 |
| 22 | 432 | 10.1 | 388.8 | 2.16 |
| 23 | 480 | 11.3 | 432 | 2.40 |
| 24 | 528 | 12.5 | 475.2 | 2.64 |
| 25 | 576 | 13.4 | 518.4 | 2.88 |
| 26 | 608 | 14.1 | 547.2 | 3.04 |
| 27 | 704 | 16 | 633.6 | 3.52 |

- Appendix B: Simulation assumptions and results for Ka band scenarios

Table B.1: Simulation assumptions for Ka band scenarios

| Scenario | LEO 600km |  | LEO 1200km |  | GEO |  |
| --- | --- | --- | --- | --- | --- | --- |
| DL/UL | DL | UL | DL | UL | DL | UL |
| Simulation | Simulation 4 | Simulation 5 | Simulation 4 | Simulation 5 | Simulation 4 | Simulation 6 |
| MCS table | 38.214 Table 5.1.3.1-3 | 38.214 Table 6.1.4.1-2 | 38.214 Table 5.1.3.1-3 | 38.214 Table 6.1.4.1-2 | 38.214 Table 5.1.3.1-3 | 38.214 Table 6.1.4.1-2 |
| MCS range | 0~28 | 0~27 | 0~28 | 0~27 | 0~28 | 0~27 |
| SCS | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| # RB | 66 | 32 | 66 | 32 | 66 | 16 |
| # total RE | 11088 | 5376 | 11088 | 5376 | 11088 | 2688 |
| # data RE | 10092 | 4896 | 10092 | 4896 | 10092 | 2448 |
| LDPC decoder | LOMS 20 | LOMS 20 | LOMS 20 | LOMS 20 | LOMS 20 | LOMS 20 |
| Channel type | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |

Table B.2: Required SNR, THP and spectral efficient of simulation 4

| MCS | Tbsize (bit) | SNR@BLER=0.1 (dB) | THP(Kbps) | Spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 0 | 608 | -12 | 4377.6 | 0.05 |
| 1 | 808 | -10.8 | 5817.6 | 0.06 |
| 2 | 984 | -9.9 | 7084.8 | 0.07 |
| 3 | 1256 | -8.8 | 9043.2 | 0.09 |
| 4 | 1544 | -8 | 11116.8 | 0.12 |
| 5 | 2024 | -6.8 | 14572.8 | 0.15 |
| 6 | 2408 | -6 | 17337.6 | 0.18 |
| 7 | 3104 | -4.8 | 22348.8 | 0.23 |
| 8 | 3824 | -3.9 | 27532.8 | 0.29 |
| 9 | 4872 | -2.7 | 35078.4 | 0.38 |
| 10 | 6016 | -1.8 | 43315.2 | 0.46 |
| 11 | 7424 | -0.7 | 53452.8 | 0.56 |
| 12 | 8712 | 0.4 | 62726.4 | 0.68 |
| 13 | 10248 | 1.3 | 73785.6 | 0.80 |
| 14 | 11784 | 2.2 | 84844.8 | 0.89 |
| 15 | 13320 | 3.6 | 95904 | 1.01 |
| 16 | 14856 | 4.3 | 106963 | 1.13 |
| 17 | 16896 | 5.2 | 121651 | 1.28 |
| 18 | 18960 | 6 | 136512 | 1.48 |
| 19 | 21504 | 7 | 154829 | 1.67 |
| 20 | 24072 | 7.8 | 173318 | 1.82 |
| 21 | 25608 | 9.4 | 184378 | 1.98 |
| 22 | 27144 | 9.8 | 195437 | 2.10 |
| 23 | 30216 | 10.8 | 217555 | 2.33 |
| 24 | 33816 | 11.8 | 243475 | 2.56 |
| 25 | 35856 | 12.6 | 258163 | 2.80 |
| 26 | 38936 | 13.6 | 280339 | 2.95 |
| 27 | 42016 | 14.5 | 302515 | 3.26 |
| 28 | 45096 | 15.4 | 324691 | 3.49 |

Table B.3: Required SNR, THP and spectral efficient of simulation 5

| MCS | Tbsize (bit) | SNR@BLER=0.1 (dB) | THP(Kbps) | Spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 0 | 288 | -11.4 | 2073.6 | 0.05 |
| 1 | 384 | -10.3 | 2764.8 | 0.06 |
| 2 | 480 | -9.3 | 3456 | 0.08 |
| 3 | 608 | -8.5 | 4377.6 | 0.10 |
| 4 | 768 | -7.5 | 5529.6 | 0.12 |
| 5 | 984 | -6.6 | 7084.8 | 0.15 |
| 6 | 1160 | -5.8 | 8352 | 0.18 |
| 7 | 1544 | -4.7 | 11116.8 | 0.24 |
| 8 | 1864 | -4 | 13420.8 | 0.29 |
| 9 | 2408 | -2.7 | 17337.6 | 0.38 |
| 10 | 2976 | -1.7 | 21427.2 | 0.47 |
| 11 | 3624 | -0.6 | 26092.8 | 0.57 |
| 12 | 4224 | 0.3 | 30412.8 | 0.66 |
| 13 | 4992 | 1.2 | 35942.4 | 0.78 |
| 14 | 5760 | 2.1 | 41472 | 0.90 |
| 15 | 6528 | 3 | 47001.6 | 1.02 |
| 16 | 7168 | 4.1 | 51609.6 | 1.12 |
| 17 | 8192 | 5.2 | 58982.4 | 1.28 |
| 18 | 9480 | 6 | 68256 | 1.48 |
| 19 | 10504 | 7 | 75628.8 | 1.64 |
| 20 | 11784 | 7.9 | 84844.8 | 1.84 |
| 21 | 12552 | 8.6 | 90374.4 | 1.96 |
| 22 | 13320 | 9.2 | 95904 | 2.08 |
| 23 | 14856 | 10.2 | 106963.2 | 2.32 |
| 24 | 16136 | 11.8 | 116179.2 | 2.52 |
| 25 | 17424 | 12.4 | 125452.8 | 2.72 |
| 26 | 18960 | 13.6 | 136512 | 2.96 |
| 27 | 22032 | 15.5 | 158630.4 | 3.44 |

Table B.4: Required SNR, THP and spectral efficient of simulation 6

| MCS | Tbsize (bit) | SNR@BLER=0.1 (dB) | THP(Kbps) | Spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 0 | 136 | -11.1 | 979.2 | 0.04 |
| 1 | 184 | -10 | 1324.8 | 0.06 |
| 2 | 240 | -9.1 | 1728 | 0.08 |
| 3 | 304 | -8.1 | 2188.8 | 0.10 |
| 4 | 368 | -7.5 | 2649.6 | 0.12 |
| 5 | 480 | -6.5 | 3456 | 0.15 |
| 6 | 576 | -5.6 | 4147.2 | 0.18 |
| 7 | 768 | -4.5 | 5529.6 | 0.24 |
| 8 | 928 | -3.8 | 6681.6 | 0.29 |
| 9 | 1224 | -2.5 | 8812.8 | 0.38 |
| 10 | 1480 | -1.6 | 10656 | 0.46 |
| 11 | 1864 | -0.5 | 13420.8 | 0.58 |
| 12 | 2152 | 0.3 | 15494.4 | 0.67 |
| 13 | 2536 | 1.4 | 18259.2 | 0.79 |
| 14 | 2856 | 2.3 | 20563.2 | 0.89 |
| 15 | 3240 | 3.2 | 23328 | 1.01 |
| 16 | 3624 | 4.4 | 26092.8 | 1.13 |
| 17 | 4096 | 5 | 29491.2 | 1.28 |
| 18 | 4608 | 5.9 | 33177.6 | 1.44 |
| 19 | 5248 | 7 | 37785.6 | 1.64 |
| 20 | 5888 | 7.8 | 42393.6 | 1.84 |
| 21 | 6272 | 8.5 | 45158.4 | 1.96 |
| 22 | 6656 | 9 | 47923.2 | 2.08 |
| 23 | 7296 | 10.1 | 52531.2 | 2.28 |
| 24 | 8064 | 11.6 | 58060.8 | 2.52 |
| 25 | 8712 | 12.4 | 62726.4 | 2.72 |
| 26 | 9480 | 13.2 | 68256 | 2.96 |
| 27 | 11016 | 15.6 | 79315.2 | 3.44 |