3GPP TSG RAN WG1 Meeting #97	R1-1906011

Reno, USA, May 13-17, 2019

Agenda Item:	7.2.4.2.2

Source:	Huawei, HiSilicon

Title:	Sidelink resource allocation mode 2 for NR V2X

Document for:	Discussion and decision

# Introduction

At RAN#83, a WI on NR V2X was agreed [1]. The WI has the following objective related to sidelink resource allocation:

  - Mode 2

    - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome

In this paper, we discuss mode 2 resource allocation for NR V2X sidelink and provide our views on resource allocation methods/features for mode 2, including sensing and resource/pattern selection for both out-of-coverage and in-coverage scenarios.

# Mode 2 resource allocation

In mode 2, UE may autonomously select sidelink resources for sidelink transmission within the (pre)-configured resource pools (RPs) or within (pre-)configured resources within the RPs.

## Resource allocation granularity & (pre-)configuration

SL grant-free transmission can provide very low latency and high reliability that can satisfy URLLC requirements as motivated in NR uplink. It can be used for both in-coverage and out-of-coverage UEs. More details about mode 2 GF transmission (mode 2c) were provided in our earlier contributions [4].

NR’s higher reliability target than LTE-V requires a higher maximum number of retransmissions larger than 2, and can be further enhanced by avoiding potential collisions between the SL retransmissions of different UEs. This can be achieved in a grant-free transmission mode, by (pre)-configuring either a single time/frequency repetition pattern (TFRP) or a set of TFRPs within a RP. The TFRPs indicate the time and frequency location of each repetition of a TB. The (pre-)configuration takes into account the UE needs and the radio conditions. The TFRP selection is performed at least once within the periodicity of the (pre-)configured grant resources. An example of a TFRP pool is depicted in Figure 1.

Figure 1. Example of a TFRP pool.

In order to alleviate the half-duplex constraint, the pool of (pre-)configured TFRPs should be such that any two distinct TFRPs do not collide in at least one time unit. This achieves the following benefits:

- Due to near-far effects and in-band emissions, simultaneous PSSCH transmissions from multiple UEs may interfere with each other even though those transmissions take place on different resources in the frequency domain. Such detrimental impacts are avoided as long as those devices select different TFRPs.

- A UE is not able to transmit and receive PSSCH simultaneously. Grant-free transmissions using the TFRP set allows UEs to transmit PSSCH and receive PSSCH from multiple UEs as long as those transmissions are carried out using distinct TFRPs.

- Repetition not triggered by feedback is required for mode 2 operation in order to meet the latency and reliability requirements of NR V2X.

Therefore, we have the following proposal:

Proposal 1: Resource granularity for sensing & resource (re)-selection is based on UE-specific or common (pre-)configured time-frequency repetition patterns (TFRPs).

## Support for blind retransmission of a TB in Mode 2

At RAN1#96 meeting, it was agreed that blind retransmission of a TB for NR SL V2X is supported and details of the support are for WI phase. In this Section, we discuss the support of blind retransmission in Mode 2.

Blind retransmission refers to retransmission of a TB that is not triggered by HARQ feedback or scheduling grant.  As proposed in Section 2.1 and supported by system level simulation results captured in TR 38.885 [3], (pre-)configured TFRPs can support repetition, alleviate the half duplex impact and significantly reduce the collision probability of a TB. Therefore, mode 2 blind retransmission is inherently supported by (pre)configuring a set of TFRPs and allowing UEs to select a TFRP to carry out both the initial transmission and blind retransmissions of a TB.

Observation 1: (Pre-)configured TFRPs inherently support blind retransmissions of a TB.

## Reservation of retransmission of a TB

In RAN#96, the following agreement was achieved regarding reservations of retransmissions of a TB:

NR V2X Mode-2 supports reservation of sidelink resources at least for blind retransmission of a TB

  - Whether reservation is supported for initial transmission of a TB is to be discussed in the WI phase

  - Whether reservation is supported for potential retransmissions based on HARQ feedback is for the WI phase

- Advance indication/reservation of initial transmission of a TB induces latency which may prevent meeting the stringent latency requirements of advanced NR V2X use case. Such advanced indication/reservation requires a dedicated reservation signal to be sent prior to the initial transmission of a TB in order to reserve resources for the initial transmission and potential retransmissions of the TB. In order to send such reservation signals in advance, either a dedicated channel for the reservation signal, or a SCI sent in advance has to be defined. UEs may have to agree with certain timing window for the reservation signal in order to make any sensing procedure effective. There are also potential collisions of advanced SCI or reservation signals which affects the reliability of sensing Standalone PSSCH for the purpose of advanced reservation of initial transmission or retransmission of a TB also adds to the half-duplex issue in the sense that a UE transmitting standalone PSSCH will not be able to simultaneously receive data transmissions. As such standalone PSSCH for the purpose of resource reservation should not be supported. On the other hand, supporting reservation of retransmission of a TB can be implemented simply by blindly detecting the DMRS of the initial transmission. If the collision of retransmission is avoided with such reservation, the additional benefits of reservation for initial transmission is much smaller. Given the latency implications and additional complexity and overhead induced by the reservation signal for initial transmission and limited benefits of such reservation, it should not be supported.

- Similarly, implicit reservation of potential retransmissions based on HARQ feedback can also be indicated by PSSCH DMRS of initial transmission.

- Proposal 2: Standalone PSSCH carrying reservation for initial transmission or retransmission of a TB is not supported in NR V2X mode 2

- Proposal 3: The reservation of blind retransmissions of a TB is transmitted along with the initial transmission of the corresponding TB

- Proposal 4: Support implicit resource reservation signaling for the repetition(s) of a TB (i.e. blind retransmissions) or potential HARQ-based retransmissions indicated via PSSCH DMRS

## Sensing & resource selection

Sidelink sensing is the procedure where the UE identifies occupied sidelink resources. The current functionalities under consideration include decoding of sidelink control channel transmissions, sidelink measurements, detection of sidelink transmissions. Other options are not precluded, including combination of the above options.

Sidelink resource selection is the mechanism where the UE selects resources for PSCCH and PSSCH transmission. Which information is used by UE for resource selection may depend on the outcome of the sensing procedure and any information available at the UE such as from decoding of sidelink control channel and/or channel measurements at the time it needs to perform the resource selection operation.

Below, we share our views on potential sensing and resource selection procedures, namely sensing based on SL measurements, sensing based on SCI decoding, LBT based on short-term sensing, and LTE-V mode 4 based on long-term sensing as described in current LTE-V specifications.

## Sensing based on SL measurements

In RAN#96, it was agreed that mode-2 sensing procedure utilizes L1 SL-RSRP based on sidelink DMRS when the corresponding SCI is decoded and FFS whether/which measurement is used if the corresponding SCI is not decoded e.g. SL-RSRP after blind DMRS detection, SL-RSSI etc.

Two examples were provided as candidates for SL measurements in the case where the corresponding SCI was not decoded: SL-RSRP after blind DMRS detection and SL-RSSI. SL-RSSI is used for LTE V2X, however, the SL-RSSI is measured based on the assumed periodicity of semi-persistent transmission by the Tx UE. The measurement may be useful for LTE V2X where most of the applications target periodic traffic. For aperiodic traffic in NR V2X, periodicity does not correspond to actual transmissions and SL-RSSI may not be an accurate measurement.  Including SL-RSSI may result in unnecessary exclusion of resources which may actually decrease the overall sensing performance. Similar observation has been made in [5]. A solution to address this problem was also proposed therein which relies on removing the contributions from aperiodic packets from the long term SL-RSSI measurement, based on standalone advanced reservation SCI. As explained in Section 2.3, reservation SCI for initial transmissions induces latency which cannot be afforded for advanced use case with stringent latency and reliability requirements, not to mention the compounding of the HD problems. In addition,  a (standalone) SCI would not necessarily indicate whether a certain traffic is periodic or aperiodic, thus complicating the measurement of SL-RSSI and reducing its accuracy. Therefore, in our view SL-RSSI measurement should not be used in NR V2X sensing procedure.

SL-RSRP based on DMRS provides reliable sensing performance as further discussed below. The case where SCI is not decoded may refer to two different scenarios: The first scenario is that there is an SCI associated with the PSSCH transmission but UE does not decode the SCI (successfully). The second scenario is that no SCI is transmitted along with the PSSCH. For GF transmission using (pre-)configured TFRPs, no SCI needs to be associated with the SL data transmission. Sensing and detection can be based on PSSCH DMRS blind detection as further discussed below. Therefore, GF transmission corresponds to the 2nd scenario of “the corresponding SCI is not decoded” and SL-RSRP of PSSCH DMRS blind detection can be used for sensing instead of SCI decoding. Sensing based on PSSCH DMRS blind detection provides significant benefits compared to SCI decoding in terms of overhead savings, reliability enhancement, complexity reduction and support of contention based transmission, which are further discussed below and demonstrated by our simulation results.

Proposal 5:

- Mode 2 supports sensing based on PSSCH DMRS RSRP

- Blind DMRS detection is supported

## TFRP selection based on PSSCH DMRS blind detection

For mode 2 UEs (pre-)configured with a UE-specific TFRP, no sensing or resource selection by the UE is needed.

For mode 2 UEs (pre-)configured with TFRP pools, TFRP selection is needed. For the TFRP selection, the UE may use some knowledge it obtains from detecting PSSCH DMRS of other UEs to improve the reliability of mode 2 operation. A mapping between a detected DMRS and an associated TFRP implies that no TFRP indication message needs to be sent as part of the SCI, thus saving on signaling overhead. As illustrated in Figure 2, sensing in the form of DMRS detection can lead to a reduction of TFRP collisions. By keeping track of the currently used patterns, the UE can select one pattern that does not collide with the in-use patterns. Importantly, sensing for TFRP selection in mode 2 does not impact latency as may be the case for other sensing schemes, such as those based on LBT.

Figure 2. Example usage of sensing for TFRP selection to improve the reliability of GF transmission. (a) without sensing a TFRP collision may occur; (b) with sensing the TFRP collision can be avoided.

It should be noted that TFRPs are (pre-)configured in mode 2, therefore time-frequency resources need not be dynamically indicated in SCI and Rx UE can decode PSSCH based on DMRS blind detection. As the UE blindly decodes PSSCH, it will also be able to sense which TFRPs are being transmitted by other UEs based on the DMRS blind detection. In fact, with (pre-)configured TFRPs, not only there is no requirement that a PSSCH be associated with a PSCCH meaning SCI overhead is reduced, but also and more importantly, the reliability is increased compared to a transmission scheme which relies on SCI decoding.

## TFRP selection procedure based on sensing and/or reservation

In RAN1 # 96bis, the following agreement has been achieved for Mode 2 resource allocation

Agreements:

- NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure

- NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure

  - This functionality can be enabled/disabled by (pre-)configuration

In this section, we discuss sensing and selection procedure in line with the above agreement.  The basic TFRP selection procedure is described as follows:

Configuration: A UE is configured with a TFRP pool and possibly an initial/default TFRP. The TFRP pool may include the periodicity and the size of the pattern window. UE may initialize the transmission using the initial TFRP if configured, otherwise, UE randomly selects an initial TFRP within the TFRP pool.

Sensing: As discussed in Section 2.4.1.1, before TFRP selection, UE shall perform sensing based on DMRS blind detection. The DMRS detection provides the information on the TFRPs used by other UEs during the sensing window. Note that the sensed TFRP based on DMRS blind detection may provide two types of information that will be used for TFRP selection. Firstly, the retransmission resource of the same TB, which is basically a reservation of retransmission resource via implicit DMRS indication; and secondly, the currently in use TFRP by the UE. The latter information is useful in case UE keeps the same TFRP for future transmissions or retransmissions.

TFRP selection: Based on sensing results, Tx UE should try to avoid TFRPs conflicting with retransmission resources implicitly indicated by other UEs.

If Tx UE does not detect any conflict with other UEs’ resources, Tx UE may reuse the same TFRP, in which case DMRS can implicitly reserve an initial transmission of the next TB on the same TFRP.

If UE switches TFRP for the current transmission, the sensing and resource selection procedure may not be able to take into account the initial transmission resource of that TFRP, which is in line with the agreement that NR supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure. If Tx UE reuses the same TFRP, sensing UEs can avoid this TFRP based on sensing results from the previous TB transmissions, which is in line with the agreement that NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure.

Proposal 6: NR supports implicit reservation of an initial transmission of a TB using PSSCH DMRS of a different TB.

## PSSCH DMRS blind detection vs SCI decoding

There are several advantages for not transmitting SCI and exploiting DMRS PSSCH as an indication of TFRP for sensing purposes:

- URLLC requirements satisfaction & SCI overhead savings:

PSSCH DMRS mapping to TFRP pool in mode 2 enables the Rx UE to decode PSSCH without relying on SCI to indicate time/frequency resources.

In LTE V2X, the SCI uses 2 PRBs for each sub-channel. However, NR V2X reliability requirements are much higher than LTE V2X. Note that if PSCCH were to indicate time/frequency resources, then it should be designed to be more reliable than PSSCH. Therefore, in order to meet such higher reliability targets, more SCI resources would be needed (e.g. multiple symbols could be used for time-domain repetition [5]). Similar study on DCI reliability has been carried out in the context of URLLC PDCCH enhancement in Rel-16, where it was shown that in order to meet high DCI reliability requirements, at least 8 CCEs or 48 PRBs in one symbol are needed [6]. The overhead cost of transmitting SCI could be doubled or tripled in NR V2X compared to LTE V2X in order to meet the NR V2X latency and reliability requirements.

As a concrete example, consider the case of vehicle platooning applications and more precisely the scenario of cooperative driving for vehicle platooning information exchange between a group of UEs supporting V2X application. The scenario of reporting needed for platooning between UEs supporting V2X application and between a UE supporting V2X application and RSU, also has similar traffic requirements. For these two scenarios, the traffic payload size can be as small as 50 bytes [7]. LTE SCI format 1 size is 32 bits according to TS 36.212, which when accounting for QPSK modulation, rate 1/3 FEC and CRC attachment amounts to more than 100 encoded bits. This represents an overhead of more than 25% and 50% compared to the traffic payload size of 50bytes modulated with 64QAM and encoded with a FEC code rate of 1/3 and 2/3, respectively.  Such an overhead is quite significant even when not accounting for the expected increase in SCI resources (compared to LTE) in order to meet the higher latency and reliability requirements of NR V2X.

In addition, for some advanced driving applications, such as emergency trajectory alignment between UEs supporting V2X application, it is required to deliver large packet sizes (2000 bytes) within 3 ms latency and with very high reliability of 99.999%. For some other applications such as sensor information sharing between UEs supporting V2X applications, the same stringent latency and reliability requirements are expected to be met to deliver SL data rates as high as 50Mbps [7]. The primary target of (pre-)configured grant schemes is to achieve high reliability within a small latency budget. Thus by saving the SCI overhead more repetitions or retransmissions can be allowed for configured grant transmissions compared with LTE V2X SPS solution (which uses SCI for each TB), when UE processing time latencies are taken into account. More repetitions or retransmissions within the small latency budget automatically translate into higher reliability. Also, for packet sizes of 2000 bytes and 50 Mbps data rates too many radio resources would be wasted on SCI that would otherwise be used for data transmission in order to achieve the stringent latency and reliability requirements for the above referenced applications.

- Improved reliability:

DMRS sequences that enable to decode PSSCH can be designed to be very reliably detected. PSCCH reliability can also be improved, but the resources used for PSCCH transmission have to be scaled up significantly in order to meet the most stringent reliability requirements of NR V2X [5].

Note that PUSCH decoding performance for UL configured grant, which relies on DMRS blind detection, has been extensively studied in the context of URLLC in Rel-16. Those studies showed that the reliability of DMRS blind detection can satisfy even the most stringent reliability requirements of URLLC applications.  It has indeed been concluded in RAN1 Adhoc#1901, that [2]

PUSCH miss detection performance highly depends on the PUSCH configurations such as DMRS configuration, resource allocation, and false-alarm target setting.

  - If a configured grant PUSCH resource is not shared by multiple UEs,

    - 7 companies observed that if the reliability requirement is to be met by a single transmission, all the results show that PUSCH miss detection probability is lower than the PUSCH target BLER under the respective evaluation assumptions (e.g., MCS levels, etc.).

It is to be noted that the above observation is valid for a single transmission of PUSCH. If more than one transmission is considered as is expected to be the case in NR V2X, then DMRS-blind detection performance would be even better.

- Support of contention based transmission:

Although different sensing techniques can be used to reduce PSSCH collision probability in mode 2, collisions cannot be completely avoided. In case that two UEs use the same PSSCH time frequency resources, their corresponding SCIs may also collide. If the receiving UE cannot decode the SCI due to collision, it may not be able to decode PSSCH if the SCI is used to indicate the time frequency resources. However, PSCCH DMRS sequences can still be resolved even when they map to the same PSSCH resources. If two UEs end up using the same PSSCH resources, they still have a high probability of reliably detecting DMRS. Therefore their PSSCH is likely to be decodable. This is especially true when using soft combining of repeated SL transmissions if as expected more than one transmission per TB is supported in NR V2X.

- Reduced decoding complexity:

PSSCH sensing based on DMRS blind detection relies on a (pre-)configured mapping between PSSCH DMRS sequences and TFRPs. For SCI based sensing on the other hand, the UE needs to detect DMRS of PSCCH (potentially blindly detect it depending on the design), then perform channel estimation and attempt to decode SCI.

Therefore, both sensing methods may potentially involve blind detection of DMRS (i.e. PSSCH DMRS vs PSSCH DMRS), while SCI based sensing requires extra procedures of channel estimation and PSCCH decoding. Note also that detection of DMRS can be based on a simple correlator, which as is well known has very low complexity.

For sensing based on SCI decoding, UE needs to attempt to decode SCI for every sub-channel, and if there are multiple possible locations of SCI or multiple SCI formats as is expected in NR V2X, more blind detection of SCI would be required. For sensing based on measurements and in particular DMRS blind detection, only the DMRS corresponding to the overlapping (pre-)configured TFRPs need to be blindly detected, which can be a very small subset of the overall TFRPs. As such, the DMRS pool size can be made very small as discussed in the next paragraph.

## Link-level performance comparison between PSSCH DMRS blind detection and SCI decoding

In Figure 3, we demonstrate the reliability of DMRS blind detection for the purpose of TFRP indication in comparison with SCI decoding reliability for sensing and detection purposes through link level simulations. If DMRS blind detection is used for sensing, we measure the missed detection probability. If SCI decoding is used for sensing, we measure the BLER of SCI decoding for reliability. Three steps may be needed in order to correctly decode SCI: 1) (blind) detection of DMRS for PSCCH; 2) Channel estimation based on the detected DMRS for PSCCH 3) Decode PSCCH correctly and check CRC. In this simulation, only the last two steps are simulated and any missed detection of PSCCH DMRS may further reduce the reliability of PSCCH decoding. The main simulation assumptions are captured in Table 1.

Different DMRS pool sizes are created for potential mapping of TFRPs. 2 OFDM symbols are used for PSSCH DMRS and we follow the NR Uu DMRS configuration Type 1 to generate the DMRS pool. Urban NLOS channel is used. For PSSCH, the resource size of 12 RBs are used, which is the same assumption as the system level simulations in Section 3. For PSCCH, it is assumed to be TDMed with PSSCH and 4RB is used based on the assumption of minimum PSSCH size being 4RB. The pay load size of SCI is chosen to be the same as LTE SCI format 1. The code rate of PSCCH is derived based on pay load and available resources. Different DMRS pool sizes of 4, 16 and 64 are considered with a false alarm probability set to 1%. It can be seen that, the missed detection rate of PSSCH DMRS is much less than BLER of SCI decoding for all pool sizes. For example, for a DMRS pool size of 4, there is a 14 dB gap between missed detection probability of PSSCH DMRS and PSCCH decoding BLER at 10-4 error rate. Therefore, we can observe that DMRS detection is much more reliable than SCI decoding.

In addition to indicating TFRPs, DMRS could also be used to indicate MCS for the purpose of PSSCH decoding. One option for (pre-)configured grant could be to use a MCS pool that includes a limited number of potential MCS values.

Table 1: LL simulation assumption for PSSCH DMRS blind detection and PSCCH decoding

|  | PSSCH DMRS detection | PSCCH decoding |
| --- | --- | --- |
| Frequency Resources | 12 RB | 4RB |
| PSCCH time resource | --- | 2 OS |
| DMRS symbols | 2 adjacent OS | 2 adjacent OS |
| DMRS density | NR Uu Type 1 | 1 in 4 REs |
| MIMO Mode | 2T4R, Precoder Cycling | 2T4R, Precoder Cycling |
| PRB bundling size | 4 RB | 4 RB |
| Subcarrier spacing | 60khz | 60khz |
| Channel Model | Urban NLOS | Urban NLOS |
| PSCCH channel coding | --- | Polar Code |
| PSCCH Modulation | --- | QPSK |

Figure 4: DMRS blind detection vs SCI decoding reliability comparison.

Note that for sensing the TFRPs in the (pre-)configured TFRP pool, the DMRS pool size needed to indicate the TFRPs does not need to be large. In fact, no indication is needed for sensing among non-overlapping TFRPs as there is no ambiguity in the DMRS associated with a given TFRP, and even for partially overlapping TFRPs, given the location of TFRPs is (pre-)configured, the potential choice of different TFRPs can be very small. For example, in the partial overlapping TFRP pool described in Fig. 8 of [4], only 3 choices would need to be indicated by DMRS for sensing purposes as the location of data transmission associated with the DMRS is already known.

Observation 2: For mode 2 UEs with (pre-)configured TFRPs, PSSCH resources need not be indicated in SCI.

Proposal 7: For mode 2 UEs PSSCH reception, SCI indication is optional. For mode 2 UEs (pre-)configured with a TFRP or TFRP pool, no SCI is used

Proposal 8: For mode 2 UEs (pre-)configured with a TFRP pool, TFRP selection is needed. Sensing for TFRP selection can be performed implicitly based on DMRS blind detection.

Proposal 9: (Pre)-configured grant should be supported for mode 2 SL resource allocation in order to meet the latency and reliability requirements of NR V2X advanced use cases.

## Short-term sensing

A potential collision avoidance technique is listen-before-talk (LBT) where a UE performs a clear channel assessment and potentially random back-off before accessing the channel.  It is not clear at this stage if the LBT procedure defined for the unlicensed spectrum with unlicensed band regulations in mind can be readily reused for V2X communications over licensed spectrum. Specifying a new short-term sensing procedure specific to V2X operation may entail a lot of specification work far beyond the necessary scope of the current study and ensuing work items.

One potential issue with LBT is the random backoff. This can severely affect latency. In particular, when the system is highly loaded, latency may easily exceed the latency budget. Besides, with sensing, the hidden and exposed node problems may further affect the performance, especially in terms of latency. It has been suggested that a modified LBT with random backoff whereby UE is allowed to break the random backoff procedure in order to transmit a packet if its latency budget is about to expire, could mitigate the latency issue. However, under loaded conditions, such a modified LBT scheme is equivalent to random resource allocation which means that the benefit of LBT short-term sensing would be completely lost. Besides it is well documented that random resource allocation performs worse than sensing based resource allocation.

Furthermore, reaching the extremely high reliability targets for some services (e.g., 99.999%) might be very challenging within the latency budget.

Observation 3: LBT based on short-term sensing increases latency due to random backoff. It needs to be determined whether LBT can reach the V2X latency target (3ms for some use cases) under typical traffic loads.

# System level simulation of different resource allocation schemes

In this section, we provide some preliminary system level simulation results of different mode-2 schemes. In particular, we compare NR Mode 2 with TFRPs to NR Mode-2 without TFRPs based on long term sensing, and NR Mode-2 without TFRPs based on short term sensing.  Packet reception rate (PRR) and packet inter-reception (PIR) performance as in TR 37.885 are used for the performance evaluation. The simulation profiles that are agreed in RAN1#94bis are used for the simulations. The detail simulations assumptions and results of PRR and PIR in different scenarios have been provided in [8]; in this Section, we provided additional latency results for aperiodic traffic.

Figures 6 and 7 compare the CDFs of packet latencies for mode 2 with TFRPs and the two versions of mode 2 without TFRPs based on long-term and short-term sensing, respectively, for aperiodic traffic with 60khz SCS. The corresponding PRR and PIR performance is also shown in Fig 8 and 9. As expected, it can be seen that mode 2 with TFRPs has shorter latency than both versions of Mode 2 without TFRPs while providing better PRR and PIR performance than both long term and short term sensing based Mode 2 without TFRPs. This is because Mode 2 with TFRPs mainly relies on TFRP design to resolve potential collisions and half duplex issues, while long term sensing may introduce latency due to its selection window for aperiodic traffic, and short term sensing may introduce latency due to the back off schemes.

Figure 5: Latency performance of Mode 2 with TFRPs vs Mode 2 without TFRPs in highway aperiodic mode 1-traffic, Broadcast 60 kHz

Figure 6: Latency performance of Mode 2 with TFRPs vs Mode 2 without TFRPs in Urban aperiodic mode 1-traffic, Broadcast 60 kHz

Figure 7. PRR and PIR performance of mode 2 with TFRPs vs mode 2 without TFRPs in Highway Aperiodic model-1 traffic, broadcast channel, 60 kHz

Figure 8. PRR and PIR performance of mode 2 with TFRPs versus mode 2 without TFRPs in Urban Aperiodic model-1 traffic, broadcast channel, 60 kHz

# Conclusions

In this contribution, we discussed the resource allocation for UE autonomous transmission for NR V2X sidelink transmission.   We have the following observations and proposals:

Proposal 1: Resource granularity for sensing & resource (re)-selection is based on UE-specific or common (pre-)configured time-frequency repetition patterns (TFRPs).

Proposal 2: Standalone PSSCH carrying reservation for initial transmission or retransmission of a TB is not supported in NR V2X mode 2

Proposal 3: The reservation of blind retransmissions of a TB is transmitted along with the initial transmission of the corresponding TB

- Proposal 4: Support implicit resource reservation signaling for the repetition(s) of a TB (i.e. blind retransmissions) or potential HARQ-based retransmissions indicated via PSSCH DMRS

Proposal 5:

- Mode 2 supports sensing based on PSSCH DMRS RSRP

- Blind DMRS detection is supported

Proposal 6: NR supports implicit reservation of an initial transmission of a TB using PSSCH DMRS of a different TB.

Proposal 7: For mode 2 UEs PSSCH reception, SCI indication is optional. For mode 2 UEs (pre-)configured with a TFRP or TFRP pool, no SCI is used.

Proposal 8: For mode 2 UEs (pre-)configured with a TFRP pool, TFRP selection is needed. Sensing for TFRP selection can be performed implicitly based on DMRS blind detection.

Proposal 9: (Pre)-configured grant should be supported for mode 2 SL resource allocation in order to meet the latency and reliability requirements of NR V2X advanced use cases.

Observation 1: (Pre-)configured TFRPs inherently support blind retransmissions of a TB.

Observation 2: For mode 2 UEs with (pre-)configured TFRPs, PSSCH resources need not be indicated in SCI.

Observation 3: LBT based on short-term sensing increases latency due to random backoff. It needs to be determined whether LBT can reach the V2X latency target (3ms for some use cases) under typical traffic loads.

# References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LGE, Huawei, HiSilicon, RAN#83, Shenzhen, China March 2019.

- Chairman’s Notes, 3GPP RAN1 #1901

- TR 38.885 v16.0.0, “Study on Vehicle-to-Everything (Release 16)”.

- R1-1900026, “Sidelink resource allocation mode 2”, Huawei, HiSilicon, RAN1 Adhoc#1901, Taipei, Taiwan, January 2019.

- R1-1905334, “Discussion of Resource Allocation for Sidelink - Mode 2”, Nokia, Nokia Shanghai Bell, RAN1#94, Xi’An, China, April 2019.

- R1-1808937, “Sidelink control channel design of NR V2X”, Huawei, HiSilicon, RAN1#94, Gothenburg, Sweden, August 2018.

- R1-1900043, “PDCCH Enhancements for URLLC”, Huawei, HiSilicon, RAN1#1901, Taipei, Taiwan, January 2019.

- TS 22.186, “Enhancement of 3GPP support for V2X scenarios; Stage 1”.

- R1-1901540, “Sidelink resource allocation mode 2”, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25-March 1, 2019.