3GPP TSG RAN WG1 Meeting #97	R1-1906013

Reno, USA, May 13-17, 2019

Agenda Item:	7.2.4.8

Source:	Huawei, HiSilicon

Title:	System-level evaluations on sidelink for NR V2X

Document for:	Discussion and decision

# Introduction

At RAN#83, a WI on NR V2X with the following objective was agreed [1]:

- Sidelink physical layer procedures as per the study outcome

  - HARQ procedures [RAN1, RAN2]

  - CSI acquisition for unicast [RAN1]

      - CQI/RI reporting is supported and they are always reported together. No PMI reporting is supported in this work. Multi-rank PSSCH transmission is supported up to two antenna ports.

      - In sidelink, CSI is delivered using PSSCH (including PSSCH containing CSI only) using the resource allocation procedure for data transmission.

  - Power control [RAN1, RAN2]

In addition, the following agreement were made in RAN1#96bis [2]:

Agreements:

- Support at least Sidelink CSI-RS for CQI/RI measurement

  - Sidelink CSI-RS is confined within the PSSCH transmission

Agreement:

- For further evaluation of NR V2X,

  - 2Tx/2Rx is taken as an additional simulation assumption for FR1.

In this contribution, we provide system-level performance evaluations based on CSI reporting. We also show results for the scenarios with the additional antenna configuration.

# System-level evaluation assumptions

In this paper, the system simulation evaluations for CSI reporting and the additional antennal parameter are provided. The basic profiles are shown in Table 1. The other simulation assumptions follow TR 37.885. SFBC is used for transmit diversity with a fixed MCS, and NR type-1 codebook is used for rank adaption MIMO precoder matrix selection. For resource allocation, both schemes use NR SL Mode 1 with dynamic grants. Transmission for one-shot transmission without HARQ is evaluated in this contribution.

Table 1: Basic simulation assumptions for V2V links

| Parameter | Assumption |
| --- | --- |
| Frequency | 6 GHz |
| Simulation bandwidth | 20 MHz |
| Sub-carrier spacing | 60 kHz |
| Scheduling | NR-V2X resource allocation mode 1 |
| SA assumption | FDM multiplexing with data. SA occupies 2 RBs |
| In-band emission | According to TR 36.885 evaluation assumptions, with {W, X, Y, Z} = {3, 6, 3, 3} |
| Synchronization | ideal time frequency synchronization |
| Link type | Direct vehicle-to-vehicle link |
| VUE antenna model | TR 37.885 Option 1 |
| MIMO scheme | SFBC/NR type-1 codebook |
| Traffic model | Periodic-2: Medium intensity, 50 ms inter-packet arrival, 50% vehicles generate packets. Updated Periodic-2: Medium intensity, 30 ms inter-packet arrival, 100% vehicles generate packets. Aperiodic-1: Medium intensity, 50ms plus an exponential random variable with the mean of 50 ms, 100% vehicles generate packets. Updated Aperiodic-1: Medium intensity, 15ms plus an exponential random variable with the mean of 15 ms, 100%vehicles generate packets. |
| Deployment and UE drop | Highway: Option A |
| Number of Tx/Rx antennas | 2Tx/4Rx and 2Tx/2Rx |

For RB allocation, when one packet is too big to fit into one slot, the packet is segmented into several smaller packets. The calculation and statistics of PRR and PIR follow TR 37.885. Packets that are available at the UE, but not scheduled, thus not transmitted, are included in the PRR computation.

# System evaluation results for 6 GHz

## CSI reporting evaluation

In this section, the performance of CSI reporting is evaluated.

### Highway-A UE drop

The unicast performance for highway-A deployment is given in this sub-section. The receiver is randomly selected among the UE located from 320 meters from the transmitter for the highway-A drop.

#### Periodic-2 traffic

The simulation results of closed-loop mode with CSI reporting under the Periodic-2 traffic profile in RAN1#94bis are shown in Table 2. For CQI reporting, the 4 bits quantized CQI feedback as used in NR Uu and the ideal CQI value without quantization are compared. Due to the low traffic density with the Periodic-2 model, the performance of the two schemes are close to optimal. The differences between these two modes are negligible, and the quantization of CQI has no performance impact.

Table 2: Unicast performance evaluation for Highway-A-Periodic 2

| CSI mode | Performance metric |  |  |
| --- | --- | --- | --- |
|  | PRR (%) | Throughput (Mbps) | PIR (sec) |
| AMC & rank adaption (CQI ideal) | 99.52 | 0.166 | 0.050 |
| AMC & rank adaption (CQI NR Uu) | 99.36 | 0.165 | 0.050 |

#### Aperiodic-1 traffic

The simulation results of CSI reporting with the Aperiodic-1 traffic profile are shown in Table 4. Due to the low traffic density, the performance of the two schemes are also close to optimal.

Table 3: Unicast performance evaluation for Highway-A-Aperiodic 1

| CSI mode | Performance metric |  |  |
| --- | --- | --- | --- |
|  | PRR (%) | Throughput (Mbps) | PIR (sec) |
| AMC & rank adaption (CQI ideal) | 97.45 | 0.143 | 0.094 |
| AMC & rank adaption (CQI NR Uu) | 96.54 | 0.141 | 0.094 |

The results together show that, for both periodic and aperiodic traffic mode, the quantization error from the 4-bit design reused in NR Uu does not have any meaningful impact on system performance.

Observation 1: The quantization error from the 4-bit design reused from NR Uu to the ideal CQI has no noticeable impact on system performance metrics.

Proposal 1: SL CQI reporting re-uses the NR Uu CQI table, and therefore uses 4 bits per TB.

# Conclusions

In this contribution, system level evaluation results of NR V2X are presented. Based on the discussion, we make the following observations:

Observation 1: The quantization error from the 4-bit design reused from NR Uu to the ideal CQI has no noticeable impact on system performance metrics.

Proposal 1: SL CQI reporting re-uses the NR Uu CQI table, and therefore uses 4 bits per TB.

# References

RP-190786, New WID on 5G V2X with NR sidelink, RAN#83, Shenzhen, China, March 2019.

Chairman’s notes, RAN1#96bis, Xi’an, China, April, 2019.

TR 37.855, “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR”

TR 36.885, “Study on LTE-based V2X Services”