3GPP TSG RAN WG1 Meeting #97	 R1-1906016

Reno, USA, May 13-17, 2019

Agenda Item:	7.2.4.6

Source:	Huawei, HiSilicon

Title:	QoS management for congestion control in NR V2X

Document for:	Discussion and decision

# Introduction

One objective of the V2X WI is to specify congestion control for NR to support advanced V2X services [1] . Agreements, etc. to date include [2], [4], [5]:

Agreements:

- Introduce at least one congestion metric for NR sidelink

  - FFS details – to be done in WI phase (if included)

- Congestion control is supported at least for sidelink mode 2

  - Note: details of congestion control can be covered in the work item phase, not in this SI.

Conclusion:

- It is deemed beneficial to report Sidelink Congestion Metrics(s) to a gNB

- Consequently, it is recommended to specify the corresponding details in the WI phase

Agreements:

- Support at least NR CBR as congestion metric for NR sidelink congestion control.

  - LTE CBR is the baseline for defining NR CBR.

The agreement of last meeting introduces CBR. In this contribution, we discuss some issues about NR sidelink congestion control related to CBR.

# Discussion

## NR sidelink congestion control

### NR sidelink CBR

LTE sidelink congestion metrics include Channel busy ratio (CBR) and Channel occupancy ratio (CR). The definition of CBR and CR are described in 36.214 [6]. For PSSCH, CBR measured in subframe n is defined as the portion of sub-channels whose S-RSSI measurements exceed a (pre)configured threshold observed during subframes [n-100, n-1]. In LTE sidelink, PSCCH and PSSCH are FDM multiplexed and have separate resource pools

Based on LTE sidelink, CBR measurement for PSSCH is necessary in NR sidelink. Whether CBR measurement for PSCCH can be done separately from PSSCH should be further studied. As agreed in RAN1#94bis [2], at least option 3 multiplexing for PSCCH and associated PSSCH is supported. CBR measurement for PSSCH can be done for the whole slot, and a separate CBR measurement for PSCCH is not needed since no dedicated PSCCH resources are configured. Different from LTE sidelink, NR sidelink supports physical-layer groupcast and unicast in addition to broadcast, and HARQ is supported for both groupcast and unicast. For PSFCH, at least TDM multiplexing for PSSCH and PSFCH is supported. In a resource pool, PSFCH resources can be (pre)configured periodically with a period of N slots, thus CBR measurement for PSSCH should be performed by excluding the PSFCH resources.

Proposal 1: A single PSCCH/PSSCH CBR measurement is defined on the resources occupied by PSCCH and PSSCH, and excluding PSFCH, for multiplexing option 3.

- FFS how to define CBR if other PSCCH/PSSCH multiplexing options are supported.

### NR sidelink CR

For LTE sidelink, channel occupancy ratio (CR) evaluated at subframe n is defined as the total number of sub-channels used for its transmission in subframes [n-a, n-1] and granted in subframes [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b] (a>0, b>=0). The UE obtains the corresponding channel occupancy ratio limit value (CR_limit) based on CBR measurements and adapts its TX parameters to meet CR_limit. For the same purpose, CR in NR sidelink congestion metric should be supported. For the periodic traffic, the definition of CR in LTE sidelink can be reused since LTE sidelink only has periodic traffic. For the aperiodic traffic, it is difficult to use past samples to predict the probability of occurrence for dynamic scheduling traffic. In this case, reusing the definition of CR in LTE sidelink may cause imprecise measurement when b is not equal to zero. Thus, for aperiodic traffic and mixed periodic and aperiodic traffics, how to calculate the CR should to be further studied.

Proposal 2: CR based congestion control is supported.

- FFS how to calculate the CR for aperiodic traffic, and mixed periodic and aperiodic traffic.

### Congestion level report

In LTE V2X, both eNB-assisted and UE-autonomous congestion control are supported. UE does CBR measurement and reports to the eNB to assist the eNB scheduling. As agreed in RAN1#96 meeting [4], it is beneficial to report sidelink congestion metric(s) to a gNB. Then PSSCH/PSCCH CBR should also be reported to the gNB. With the congestion level report, gNB can adjust resource pool related parameters such as set of slots and symbols that belong to PSSCH resources.

Proposal 3 Support reporting of CBR to gNB in Mode 1.

# Conclusions

In this contribution, we discuss NR sidelink congestion control. Based on above discussions, following proposals are given.

Proposal 1: A single PSCCH/PSSCH CBR measurement is defined on the resources occupied by PSCCH and PSSCH, and excluding PSFCH, for multiplexing option 3.

- FFS how to define CBR if other PSCCH/PSSCH multiplexing options are supported.

Proposal 2: CR based congestion control is supported.

- FFS how to calculate the CR for aperiodic traffic, and mixed periodic and aperiodic traffic.

Proposal 3: Support reporting of CBR to gNB in Mode 1.

# References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, RAN#83, Shenzhen, China, March 18-21, 2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #94bis Chengdu, China, October 8th – 12th, 2018

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting Ad-Hoc Meeting 1901 Taipei, Taiwan, 21st – 25th  January, 2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96 Athens, Greece, February 25th – March 1st,  2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96bis Xi’an, China, 8th – 12th April, 2019

- 3GPP TS 36.214 V15.3.0(2018-09); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); physical layer; Measurements (Release 15)

- S2-1904823, “LS response to SA2 on unicast, groupcast and broadcast in NR sidelink”, SA WG2 Meeting #S2-132, 08-12 April, 2019, Xi’an, China