3GPP TSG RAN WG1 Meeting #97	R1-1906023

Reno, USA, May 13 – 17, 2019

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Consideration on self evaluation of IMT-2020 for area traffic capacity in NR

Document for:	Discussion and Decision

Introduction

According to 3GPP work plan on IMT-2020 submission, the final submission will be made in June 2019. More test environments or configurations for NR can be evaluated for the final submission. In this contribution, the consideration and evaluation results for area traffic capacity in Indoor Hotspot - eMBB test environment with configuration C are provided.

Definition and requirements of area traffic capacity

In Report ITU-R M. 2410 (see [1]), the following definition and requirements of area traffic capacity are defined in subsection 4.6.

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6). Carea = ρ × W × SEavg (6) In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands. This requirement is defined for the purpose of evaluation in the related eMBB test environment. The target value for Area traffic capacity in downlink is 10 Mbit/s/m2 in the Indoor Hotspot – eMBB test environment.

Therefore, area traffic capacity can be derived analytically through equation (6), where SEavg is the average spectral efficiency as defined in subsection 4.5 in Report ITU-R M.2410.

Observation 1: Area traffic capacity can be derived analytically through equation (6), where SEavg is the average spectral efficiency as defined in subsection 4.5 in Report ITU-R M.2410.

Evaluation methodology of area traffic capacity

From Report ITU-R M.2412 (see [2]), we can see the following evaluation methodology.

The evaluation is conducted in Indoor Hotspot-eMBB test environment where a single band is considered. Area traffic capacity is derived based on the achievable average spectral efficiency, TRxP density and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg as follows: Carea = ρ × W × SEavg

It should be noted that the TRxP density ρ needs to be calculated according to the network layout defined in subsection 8.3.1 in Report ITU-R M.2412. According to Figure 8-F1 illustrated in Report ITU-R M.2412 (see below), the simulation area of Indoor Hotspot – eMBB test environment is S=120×(15+20+15)=6000 m2. And there are 12 BSs/sites which are placed in 20 meter spacing in this simulation area. It is stated in Report ITU-R M.2412 that “The type of site deployed (e.g. one TRxP per site or 3 TRxPs per site) is not defined and should be reported by the proponent”. When one TRxP per site is assumed, there will be 12 TRxPs in the simulation area, and in this case, ρ=12 TRxP/6000 m2=0.002TRxP/m2. When 3 TRxPs per site is assumed, there will be 36 TRxPs in the simulation area, and in this case, ρ=36 TRxP/6000 m2=0.006TRxP/m2. Therefore, one observe that

Observation 2: According to Indoor Hotspot network layout as defined in Report ITU-R M.2412, the TRxP density is calculated to be:

-   For one TRxP per site (12 TRxP in simulation area), ρ=0.002TRxP/m2

-   For 3 TRxP per site (36 TRxP in simulation area), ρ=0.006TRxP/m2

Figure 8-F1

Indoor Hotspot sites layout

Evaluation results of area traffic capacity

Based on the definition and evaluation methodology, we derive the evaluation results of area traffic capacity in Indoor Hotspot-eMBB at 70 GHz based on the average spectral efficiency evaluation provided in [3]. In [3], the effective bandwidth that accounts for the time-frequency resource used for DL/UL transmission (including GP symbols) is considered for the calculation of spectral efficiency in TDD. Therefore, the channel bandwidth, W, in equation (6) should be substituted by the effective bandwidth in TDD. In addition, the reduced guard band ratio and PDCCH overhead for larger bandwidth is modeled to calculate the spectral efficiency for larger bandwidth in [3]. It is noted that more TRxPs can help to reach the target with smaller bandwidth. Detailed evaluation assumptions are found in [3].

Table 2 – Downlink area traffic capacity (Mbit/s/m2) in Indoor Hotspot-eMBB at 70 GHz 
(12 TRxP, Channel model B)

| System bandwidth (MHz) | Average spectral efficiency SEavg [bps/Hz/TRxP] | 12TRxP (TRxP density ρ=0.002TRxP/m2) | Requirements [Mbit/s/m2] |
| --- | --- | --- | --- |
|  | TDD 200 MHz bandwidth per CC with 60 kHz SCS | TDD (DDDSU) |  |
| W = 200 | 9.611 | 2.911 | 10 |
| W = 400 |  | 5.821 |  |
| W = 600 |  | 8.731 |  |
| W = 800 |  | 11.642 |  |

Observation 3: IMT-2020 requirement on area traffic capacity can be met by NR at 70 GHz.

Conclusions

In this document, the evaluation results of downlink area traffic capacity in Indoor-Hotspot eMBB test environment is given. Based on the discussions, the following observations are obtained.

Observation: IMT-2020 requirement on area traffic capacity can be met by NR at 70 GHz.

- References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1906022, “Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency in NR”, Huawei, HiSilicon, May 2019.