3GPP TSG RAN WG1 Meeting #97	R1-1906032

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.8.5

Source:	Huawei, HiSilicon

Title:	Discussion on UE complexity of DFT-based compression codebook

Document for:	Discussion and Decision

- Introduction

It has been agreed in previous meetings [1] that:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

In this contribution, a problem of phase randomization and correction for DFT-based compression codebook is discussed which is for UE implementation and affects the performance if this point is ignored. In addition, the impact of oversampling factor O3 on the system performance is also discussed.

- Space-frequency compression codebook

The design of type II codebook with DFT-based compression is introduced in [2-3]. In this design, the space-frequency matrix is compressed in both spatial domain and frequency domain, by a set of spatial basis vectors and a set of frequency basis vectors.

The space-frequency matrix W can be represented and approximated by the following formulation

W=W1W2WfH

where W1 and Wf are composed of selected basis vectors from the spatial codebook and frequency codebook, respectively. The dimension of the coefficients matrix W2 is 2L×M, with L and M as the number of selected spatial and frequency basis vectors, respectively. The UE only needs to feedback the indices of selected spatial and frequency basis vectors, i.e. W1 and Wf, as well as the combination coefficients W2.

Since the basic codebook structure has been agreed with some remaining loose ends, the precoder of j-th subband for each layer can be represented as

wj=1Pjl=0L-1p0(ref)blm=0M-1pl,mφl,mum*(j)l=0L-1p1(ref)blm=0M-1pl+L,mφl+L,mum*(j)

where bl is the l-th beam from selected spatial basis, um*(j) is the conjugate of the j-th entry of the m-th selected frequency basis, p0(ref) and p1(ref) are the reference amplitude for two polarizations, pl,m and φl,m are the quantized differential amplitude and phase corresponding the spatial-frequency pair (l,m), pl,m=0 if it is not included in the size-K0 subset, and Pj is a normalization factor.

- Phase correction for eigenvectors

In the design of space-frequency compression codebook, the space-frequency matrix W is obtained by concatenating the precoding vectors of different subbands. Then the space-frequency matrix is compressed in both spatial domain and frequency domain. The effect of frequency domain compression highly depends on the smoothness of the coefficients along subbands.

In one implementation, the leading eigenvector is utilized as the precoding vector for each subband. Note that if a vector v is the eigenvector of a matrix A, ejθ*v is also its eigenvector. We should be cautious that eigenvectors, as the output of a SVD solver, may have a “random” phase rotation. Whether such phase rotation exists may depend on the numerical algorithm of the SVD solver. To achieve more efficient frequency domain compression, random phase rotation should be corrected.

To implement phase correction, the eigenvector for each subband should be multiplied by a proper phase. Then the space-frequency matrix is obtained with rotated eigenvectors. It should be noted that phase correction between adjacent subbands will provide more efficient frequency domain compression. A better phase correction will lead to the result that a larger proportion of power concentrates within the selected frequency components and then a smaller M is enough in the frequency domain compression. A potential phase correction implementation is to compensate the phase of eigenvector corresponding to the (i+1)-th subband (ei+1) with the phase of the inner product of the i-th and (i+1)-th eigenvectors (phase of ei+1H*ei), i.e., ei+1=phaseei+1H*ei*ei+1.

This procedure has no impact over codebook structure and associated PMI quantization. Whilst PMI quantization is purely a UE implementation, a proper phase correction will ensure PMI coefficient compression in the frequency domain more efficiently.

For higher rank codebook design, a space-frequency matrix can be obtained for each layer and phase correction is performed for each layer. For better PMI compression, phase correction for each layer may be different.

  - CDF curves for power ratio

We define power ratio as the ratio of preserved power after spatial-frequency compression and the total power before the compression. The power ratio reflects the level that the power of coefficients concentrates on the selected spatial and frequency basis vectors.

Figure 1. CDF curves for power ratio with and without phase correction.

Fig.1 illustrates the CDF curves of power ratio with and without phase correction. With the help of a proper phase correction, the power concentrates on the selected frequency basis more, which leads to a more efficient frequency domain compression.

Observation 1: Phase correction by a proper UE implementation should be considered to avoid random phase rotation caused by a SVD solver in order to achieve efficient frequency domain compression.

  - Insight on the phase correction

The reconstructed space-frequency matrix at gNB side can be expressed as:

W=W1W2WfH=W1W1HHR Wf,oWfH=W1W1HHR Wf,oWf,oHRH

The phase correction essentially is multiplying a phase correction matrix to the ideal space-frequency matrix H:

H'=H∆

where

∆=diagφ1φ2…φN3

and φr is the phase correction multiplied to the r-th subband. With phase correction, the reconstructed space-frequency matrix at gNB side can be expressed as:

W=W1W2WfH=W1W1HH'R Wf,oWf,oHRH=W1W1HH∆R Wf,oWf,oHRH

It can be found in the equation above, phase correction can seriously impact on the reconstructed space-frequency matrix due to the matrix ∆. Essentially, the aim of phase correction is to strive to reduce reported coefficients of space-frequency matrix so that PMI quantization can be concentrated or constrained to the selected W1 and  Wf.

In previous meetings, the necessity of oversampling in frequency beam is discussed. In fact, the effect of oversampling is certain implementation of phase correction. Although the rotation factor is not reported, the oversampling is suggested to be implemented at UE side for improving performance.

  - Simulation results

System-level simulation results for the proposed space-frequency compression codebook with or without phase correction are included in this section.

Figure 2. The trade-off between performance and overhead with/without phase rotation for (L, M) = (6, 4).

The spatial basis reuses the oversampled 2D-DFT beam, whilst the frequency basis uses the oversampled DFT beam with oversampling factor of 4. Each coefficient within matrix W2 is quantized with A bits for amplitude and P bits for phase. Detailed parameters are shown in Appendix I. The trade-off between performance and overhead with and without phase rotation is shown in Fig. 2. If phase rotation is not conducted, the performance of frequency domain compression is even worse than Type II codebook. The phase rotation procedure will significantly improve the performance and compress the coefficients in the frequency domain more efficiently.

Observation 2: Without proper phase correction, frequency domain compression shows worse performance than type II codebook assuming the same feedback overhead.

- Computation complexity analysis

An example of the basic procedure of UE implementation for PMI feedback is depicted in figure 3, and detailed procedure is discussed in Table I.

Figure 3. Main computation for UE implementation.

Table I. The procedure of UE implementation for PMI feedback

Step 1: Calculating Covariance matrix for each PMI subband Calculate the sum covariance matrix of RB channel per PMI subband, Cj=i=0NRBSB-1HiHHi, j=0, ⋯, NSB-1, where NRBSB is the number of PRBs in each PMI subband; - NRB(total number of RBs) complex matrix multiplications NTX×NRX and NRX×NTX; Step 2: Calculating Un-quantized precoder for each PMI subband For each layer, take one eigenvector of the sum covariance matrix for each PMI subband, and concatenate them into spatial-frequency matrix W=w0,⋯, wNSB-1; - NSB SVDs of NTX×NTX matrix is needed for each layer in this step; Step 3: Calculating Sum Covariance matrix and Searching WB SD beams Calculate the wideband covariance matrix of channel, C=j=0NSB-1Cj, where NSB is the number of subbands; Obtain the leading eigenvector vWB of the wideband covariance matrix C, as the wideband precoder; Calculate the inner products of all spatial beams with vWB, and then obtain L spatial beams b0,⋯, bL-1 using vWB; - A SVD of NTX×NTX matrix is needed in this step; Step 4: Calculating coefficients per PMI subband after SD Compressing Calculate the combination coefficients of all subbands, by the inner products of b0,⋯, bL-1 and wj(j=0, ⋯, NSB-1); - Two complex matrix multiplications with size L×(NTX2) and (NTX2)×NSB; Step 5: Calculating WB coefficients after FD compressing and Searching WB FD beams Compress using FD basis and find the best FD beams as well as corresponding coefficients; - Two complex matrix multiplications with size L×NSB and NSB×NSB;

As discussed above, the additional implementation complexity brought by type II compression codebook compared with type II codebook is the complexity in Step 5, in which only additional two complex matrix multiplication is required. For different configuration of L, the implementation complexity differs in step 4 and 5 with larger dimension of matrix multiplication, which counts a small ratio in the total implementation complexity. For different rank, the implementation complexity differs also in step 4 and 5 with production step 4 and 5 for rank times.

- Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following observation and proposal are made.

Observation 1: Phase correction by a proper UE implementation should be considered to avoid random phase rotation caused by a SVD solver in order to achieve efficient frequency domain compression.

Observation 2: Without proper phase correction, frequency domain compression shows worse performance than type II codebook assuming the same feedback overhead.

- References

- “RAN1 Chairman’s Notes”, Spokane, USA, November 12-16, 2018.

- R1-1812242, Huawei, HiSilicon, “Discussion on CSI enhancement”, Spokane, USA, November 12-16, 2018.

- R1-1813002, Samsung, “Summary of CSI enhancement for MU-MIMO support”, Spokane, USA, November 12-16, 2018.

- Appendix I

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz |
| System bandwidth | 10MHz (13 subbands, 4 PRB for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1) for overhead reduction; (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2) for higher rank of Type II; the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for overhead reduction; SU-MIMO for higher rank of Type II |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |