3GPP TSG RAN WG1 Meeting #97	R1-1906049

Reno, USA, May 13 – 17, 2019

Agenda Item:	7.2.2.3

Source:	Huawei, HiSilicon

Title:	Performance evaluation of initial signal

Document for:	Discussion and decision

# Introduction

The following agreements has been achieved in AdHoc meeting [1]:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

  - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

    - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

  - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

  - FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

It was captured in TR38.889 [2]

- The detection of a gNB's transmission burst by the UE has been studied, and concerns on the UE power consumption required for Tx burst detection e.g. if the UE needs to frequently detect/monitor the PDCCH have been raised. The proposals that have been made by contributions regarding these topics include existing NR signal(s) with potential enhancement(s), a channel such as PDCCH with potential enhancement(s), and the 802.11a/802.11ax preamble with potential enhancement(s); consensus was not achieved on any of these proposals. The detection/decoding reliability of each of the proposals has not been sufficiently evaluated for a complete evaluation of the proposals against each other.

In this contribution, we mainly discuss the details of all candidates that could be used as initial signal and corresponding link-level simulation results are also provided.

# Overview of NR initial signal candidates

NR-U is expected to have multiple mini-slot/symbol-level starting points to allow prompt access into channels. One issue arising along with the flexible starting point of DL/UL transmission is how to identify the start of DL/UL burst. In DL, unnecessary PDCCH blind-decoding could be avoided by introducing an ‘initial signal’ since blind-decoding imposes high requirement of UE capability and significantly decreases battery life of the device.

Group-common PDCCH DMRS could be a potential candidate as initial signal. An alternative approach is through detection of group-common PDCCH. 802.11a preamble is also identified as a candidate for initial signal which also facilitates co-existence between NR-U and WiFi devices.

In the following sub-sections, details of each candidate for evaluation will be provided. As initial signal is only detected by the UE that already connected the serving cell, synchronization is assumed before the initial signal detection.

## NR PDCCH DMRS

As shown in Figure 1, RE density of PDCCH DMRS is 3 RE/RB and it spreads across configured CORESET CCE(s). The configured symbols of PDCCH DMRS within each slot could be 1-3 symbols.

The current sequence generation of DMRS in Rel-15 NR depends on the index of the slot and the symbol where the RS is located. The gNB needs to prepare multiple versions of the identification signal or regenerate it in a short time repeatedly after it is contending for channel access, as it doesn’t know when channel access succeeds. In our evaluation, we assume sequence generation of PDCCH DMRS is time independent.

## 802.11a preamble

802.11a preamble can be considered as a legacy preamble that is appended to all PHY frames for the purpose of backward compatibility. In our investigation, both Non-HT Short Training Field (L-STF) and Non-HT Long Training Field (L-LTF) are employed for DL burst identification, which are shown as follows. Since both sequences will be repeated twice in time domain, then L-STF is mapped to the 1st and 2nd symbol while L-LFT is mapped to the 3rd and 4th symbol within each slot. All sequences are mapped to the center frequency of BWP, as illustrated in Figure 2, which occupy 52 sub-carriers with 312.5 kHz SCS.

Figure 1 Illustration of resource mapping of NR PDCCH DMRS

Figure 2 Illustration of 802.11 L-STF/L-LTF

# Requirement of detection performance

Reliable identification of serving cell DL burst is very important for NR-U UE to determine whether to start PDCCH monitoring and receiving PDSCH in the following COT. In Figure 3, we provide system level evaluation in indoor scenario to show the impact on the DL mean UPT assuming different detection performance of initial signal. We observed that 1% miss detection rate can achieve almost all the UPT. Further decreasing miss detection rate of initial signal has marginal gain. Moreover, if the detection sensitivity of the initial signal increases above -76dBm, UPT will drop dramatically because UE cannot receive initial signal with high probability and will stop decoding the PDCCH and PDSCH.

Observation 1: 1% miss detection rate of initial signal can achieve almost all DL UPT.

Figure 3 impact of initial signal detection on the UPT indoor scenario

# Performance evaluation of candidates for initial signal

As illustrated in Figure 4, although 802.11a preamble occupies 4 symbols, detection performance of 802.11a preamble is the worst. This is because L-LTF and L-STF should work together with L-SIG field that modulated with BPSK ½ BCC, which is the bottle neck of the preamble detection [3]. Moreover, L-STF and L-LTF requires different numerology (with 312.5 kHz SCS), which may impose additional requirement on hardware implementation.

Observation 2: Detection performance of 802.11a preamble as initial signal is bottle-necked by L-SIG. It also imposes additional requirement on hardware implementation.

Detection performance of PDCCH DMRS is better than that of 802.11a preamble. According to agreements of UE power saving AI in [1], the false alarm rate of 1% is agreed as baseline for evaluation of power saving signal/channel for wake-up purpose. In our investigation, detection performance of PDCCH DMRS with both 0.1% false alarm rate is also provided. It is observed that miss detection performance with higher false alarm rate outperforms that with lower false alarm rate. However, higher false alarm rate leads to more power wastage at UE side. Due to limited sequence length, there is a considerable gap between the performance of PDCCH DMRS and that of group-common DCI with lower payload size (e.g. 40 bits) for both 30 kHz SCS and 60 kHz SCS case. Furthermore, additional SFI could be carried by group-common DCI, which could also facilitate UE’s further DL reception.

Observation 3: Detection performance of group-common DCI is better compared with that of PDCCH DMRS. Besides, additional SFI could be carried by group-common DCI, which could also facilitate UE’s further DL reception.

Detection performance of group common DCI mainly depends on DCI payload size and SCS. With the same aggregation level, smaller DCI payload size corresponds to lower coding rate and hence results in better miss detection performance. While with the same DCI configuration, detection performance of group common DCI with 60 kHz SCS is better than that with 30 kHz SCS due to higher frequency diversity gain.

Observation 4: Detection performance of group-common DCI depends on its numerology, aggregation level and DCI payload size.

Figure 4. Detection performance of group common DCI, PDCCH DMRS and 802.11a preamble

# Conclusions

In this contribution, we mainly discuss the details of all candidates that could be used as initial signal and the corresponding link-level simulation evaluations are also provided. Based on the discussion and evaluations, we have made the following observations and proposals:

Observation 1: 1% miss detection rate of initial signal can achieve almost all DL UPT.

Observation 2: Detection performance of 802.11a preamble as initial signal is bottle-necked by L-SIG. It also imposes additional requirement on hardware implementation.

Observation 3: Detection performance of group-common DCI is better compared with that of PDCCH DMRS. Besides, additional SFI could be carried by group-common DCI, which could also facilitate UE’s further DL reception.

Observation 4: Detection performance of group-common DCI depends on its numerology, aggregation level and DCI payload size.

# References

Chairman’s Notes RAN1 AdHoc#1901

TR 38.889 V16.0.0

IEEE802.11-15/0579, “802.11ax Preamble Design and Auto-detection-r4”, Marvell

# Appendix I: system level simulation assumptions

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 30KHz and 60KHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms for NRU |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #2 |
| Link adaptation | CQI feedback + OLLA |
| Scheduling | Proportional Fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | DL only |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

# Appendix II: link level simulation assumptions

| System bandwidth | 106 RBs (15 kHz SCS) 51 RBs (30 kHz SCS) 24 RBs (60 kHz SCS) |
| --- | --- |
| Numerology | 15/30/60 kHz SCS |
| Channel | CDL-C |
| Channel delay | 100 ns |
| Number of Antennas | gNB Tx=1 UE Rx =1 |
| NID(1),NID(2) | 0 |
| Probability of false alarm | 0.1% , 1% |
| UE moving speed | 3 km/h |