3GPP TSG RAN WG1 Meeting #97	R1- 1906068

Reno, USA, May 13 – 17, 2019

Agenda Item:	6.2.3.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on power control for additional SRS

Document for:	Discussion and Decision

Introduction

In RAN1 #96 meeting, it was agreed that:

Agreement

Same power control configuration applies for all additional SRS symbols configured to a single UE

In addition, there is a FFS that

Further study on power control enhancements of additional SRS symbols (including whether these features are needed):

- Independent power control of additional SRS symbols from legacy SRS symbols

- Reuse of SRS power control for carriers without PUCCH/PUSCH

- Consider SRS configurations to reduce the range of power variations in a subframe

For SRS triggering, an agreement was achieved in RAN1 #96bis meeting that

Agreement

A codepoint in the same DCI triggers SRS transmission for one of the following:

- Only aperiodic legacy SRS symbols

- Only aperiodic additional SRS symbols

- Both aperiodic legacy and aperiodic additional SRS symbols within the same subframe

The association of the codepoint and one of the above is configured by RRC signalling. A separate codepoint will be supported for the case of no SRS triggering.

In this contribution, we discuss the power control of SRS for additional SRS symbols.

Discussion

In the last meeting, the following possible power control enhancements for additional SRS were discussed:

- Independent power control of additional SRS symbols from legacy SRS symbols

- Reuse of SRS power control for carriers without PUCCH/PUSCH

- Consider SRS configurations to reduce the range of power variations in a subframe

In this section, aspects of the power control for additional SRS are discussed.

Principle of the power control for additional SRS

Currently the power control of legacy SRS is coupled with PUSCH for carrier configured with PUSCH transmission, i.e., sharing the same SNR target, path loss offset value, and TPC command.

On one hand, considering that the purpose of SRS enhancement is to provide more SRS transmission occasions and enhance downlink transmission efficiency via channel reciprocity for a TDD cell with dominated downlink services, the uplink PUSCH transmission for a UE may not happen often. Then, there would be no power control parameters for additional SRS when PUSCH is absent.

On the other hand, the additional SRS symbols and legacy SRS symbols may have different bandwidth and transmission durations, and they may work in different scenarios. For example, the legacy SRS symbols can be used periodically to get a coarse channel estimation, and then additional SRS symbols can be used to get a further refined channel estimation. Therefore, they would have different SINR targets, and thus they need to have independent power control.

Thus, based on the above discussion, it is necessary to introduce a separate set of power control parameters for additional SRS from legacy SRS and the formula of additional SRS power control should be decoupled with PUSCH.

Proposal 1:  Introduce a separate set of power control parameters for additional SRS from legacy SRS and the formula of the additional SRS power control is decoupled with PUSCH.

Design of the power control for additional SRS

In LTE Rel-14, the power control of SRS for carriers without PUCCH/PUSCH was specified to achieve independent power control from legacy SRS and the decoupling from PUSCH as in section 5.1.3.1 of 36.213:

The setting of the UE Transmit power for the SRS transmitted on subframe i for serving cell  is defined by:

for serving cell  with frame structure type 2, and not configured for PUSCH/PUCCH transmission

[dBm]

In our understanding, reuse such design for Rel-16 additional SRS is a simple way to achieve independent power control. To this end, the higher layer parameters PO_SRS,c, aSRS,c and PO_PUSCH,c, ac can be independently configured for SRS and PUSCH. Accordingly, the power of SRS transmission can be determined by

[dBm]     (Eq. 1)

For the TPC, since the additional SRS could be triggered by Rel-15 DCI format together with or without legacy SRS, we discuss the TPC issue case by case. In the current specification, the TPC carried in the DCI and its usage are summarized as in Table 1.

Table 1. TPC signaling and usage

| TPC in the DCI | TPC Usage |
| --- | --- |
| In UL grants | PUSCH |
| In DL grants | PUCCH |
| In DCI format 3/3A | PUCCH and PUSCH |
| In DCI format 3B | SRS |

- For the DCI format 3B, the dynamic TPC adjustment can be indicated by TPC command on the carrier without PUSCH/PUCCH configuration. Thus, the TPC is only used for SRS. Such design and the power control formula (Eq. 1) can be easily reused for the additional SRS and the specification impact is very minimal.

- For the UL grants or DCI format 3/3A, the TPC value is used for the power control but also can be used for the additional SRS if triggered. There are two cases:

- Case 1: Trigger only aperiodic additional SRS

This case is simple because only additional SRS is triggered and thus the TPC value in the corresponding DCI applies to both the additional SRS and PUSCH. However, the power control of the additional SRS can reuse (Eq. 1).

- Case 2: Trigger both legacy and additional SRS symbols within the same subframe

In this case, the simple way is that the one TPC value applies to both additional SRS and PUSCH. The additional SRS could use (Eq. 1) while the legacy SRS follows the power control coupled with PUSCH.

- For the DL grant, the TPC value just can be used for PUCCH and the additional SRS. Also, formula (Eq. 1) is used for additional SRS.

In summary, when the additional SRS is triggered, no matter by what DCI formats, the TPC value in the DCI applies to the additional SRS. And the power control formulas of SRS for DCI format 3B without PUSCH/PUCCH can be reused.

Proposal 2: The TPC value in the Rel-15 DCI format applies to the additional SRS if the addition SRS are triggered.

- Re-use the power control mechanism defined for carriers without PUSCH/PUCCH, where higher layer parametersPO_SRS,c, aSRS,c andPO_PUSCH,c, ac can be independently configured, even PUSCH/PUCCH is configured.

Conclusions

This contribution mainly discusses power control issue when additional SRS is configured. We have the following proposals

Proposal 1:  Introduce a separate set of power control parameters for additional SRS from legacy SRS and the formula of the additional SRS power control is decoupled with PUSCH.

Proposal 2: The TPC value in the Rel-15 DCI format applies to the additional SRS if the addition SRS are triggered.

- Re-use the power control mechanism defined for carriers without PUSCH/PUCCH, where higher layer parametersPO_SRS,c, aSRS,c andPO_PUSCH,c, ac can be independently configured, even PUSCH/PUCCH is configured.