3GPP TSG-RAN WG1 Meeting #97	R1-1906077

Reno, USA, 12th – 17th May, 2019

Agenda item:     	7.2.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NR V2X Sidelink Synchronization mechanism

Document for:		Discussion and Decision

# 1 	Introduction

In the RAN1#96 meeting discussion on NR V2X sidelink synchronization continued and the further agreements were made.

In this contribution we discuss details of NR V2X sidelink synchronization procedure and S-SSB design.

# 2 	Discussion on synchronisation procedures

Selection of synchronization source for NR SL transmission was discussed in the RAN1#96 meeting and the following agreement and was made:

Agreements:

- Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured.

  - The following table is a working assumption

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| - P0: GNSS - P1: the following UE has the same priority: - UE directly synchronized to GNSS - P2: the following UE has the same priority: - UE indirectly synchronized to GNSS - P3: the remaining UEs have the lowest priority. | - P0: gNB/eNB - P1’: UE directly synchronized to gNB/eNB - P2’: UE indirectly synchronized to gNB/eNB - P3’: GNSS - P4’: UE directly synchronized to GNSS - P5’: UE indirectly synchronized to GNSS - P6’: the remaining UEs have the lowest priority. |

Compared to the synchronization priority table defined for LTE the column describing priority order of synchronization sources for GNSS based operation is modified so that SLSS signals from the UEs synchronized to gNB/eNB are removed. We think this kind of simplification is reasonable. Regarding the column describing priorities for synchronization sources for gNB/eNB based operation, it is reasonable to keep the option to use GNSS as low priority synchronization source because there can be large areas without network coverage and it makes sense to try to find common synchronization source for all the NR SL UEs.

It should be noted that SL UE may be able to receive signals from multiple eNBs and gNBs that are not necessarily synchronized to each other. When network-based synchronization is configured UE should not use any eNB or gNB that it is able to receive as the synchronization source, but in the case that shared Uu and SL carrier is used for SL, the gNB or eNB operating in the carrier should be used as the highest priority synchronization source. If UE cannot receive any eNB or gNB in the shared carrier it should use SLSS from the UEs that are synchronized to base station in the shared carrier according to table above. If network-based synchronization is used for SL transmission in the carrier where there are not gNBs/eNBs then UE should synchronize to base stations in the (pre)configured carrier.

The synchronization priority table above does not distinguish LTE UEs and NR UEs. According to the WID ‘not co-channel’ coexistence of LTE and NR sidelinks is assumed. Because of this we think that when network-based synchronization is prioritized, NR SL UE can only use SLSS from NR UEs as the synchronization source. On the other hand, when GNSS based synchronization is used, also SLSS from LTE SL UE in some other carrier could be used.

In the network-based synchronization, the timing obtained from certain eNB/gNB is not necessarily exactly the same than the timing obtained from some other gNB/eNB in the same carrier due to synchronization accuracy and propagation delay. There is even possibility that eNBs/gNBs in the shared carrier are not synchronized although this is probably not a typical deployment. Because of this, it makes sense to have the possibility to define mapping between eNB/gNB cell-ID and SLSS-ID similarly as in LTE.

Proposal 1: The working assumption on the synchronization priority table is confirmed with the clarification that when network-based synchronization is used the carrier for eNB or gNB signals for synchronization is (pre)configured

Proposal 2: When network-based synchronization is (pre)configured only SLSS from NR SL UE is used for synchronization of NR SL UE. When GNSS based synchronization is (pre)configured also SLSS from LTE SL UE can be used for synchronization

Proposal 3: Mapping between SLSS-ID and cell-ID is supported when network-based synchronization is used.

In the RAN1#96 meeting SL reference signal based or data-aided synchronization was discussed, and the following agreement was made:

Agreements:

- NR V2X supports using a sidelink RS for synchronization purpose

  - Applicable only on unlicensed (ITS) carrier with no network deployment on this carrier

  - This RS is not a standalone RS and not part of SLSS.

  - This RS will not appear in the synchronization procedure for the selection of sync sources.

  - RS used for the synchronization purpose would not impact any sidelink RS design

  - FFS:  Whether this RS is DM RS or other RS

  - FFS: Whether this could be achieved by UE implementation

  - FFS: Specification impact

Our understanding is that this kind of synchronization is used by the UEs that do not support SLSS based synchronization. One example could be two cars close to each other and having GNSS based synchronization. When they drive to tunnel and GNSS is lost, the UEs can still be synchronized to each other based on DM RS (and data) sent periodically. When the UEs come out from the tunnel, synchronization of the UEs may be drifted compared to GNSS timing.

Another example could be UE in underground parking lot without GNSS coverage. If the UE has been powered off and is booting up, it could synchronize to other UEs in the parking lot and start sending V2V messages. However, we think that UE that supports reference signal-based synchronization but does not support SLSS based synchronization should not be independent synchronization source i.e. that kind of UE can only start to transmit SL messages if it has synchronized to some external synchronization source.

Proposal 4: DMRS should be selected as the working assumption, when reference signal-based synchronization method is studied.

Proposal 5: UE that uses reference signal-based synchronization for its transmission cannot be independent synchronization source, but it needs to synchronize to some external source

In case of unicast or groupcast communication, synchronization source changes can be problematic when the UEs move in and out of the coverage area of GNSS or NR/LTE network and the highest priority synchronization changes and UEs need to resynchronize to the new source. Loss of synchronization could be avoided if the SLSS of a specific UE (e.g. platoon head) could be prioritized. Obviously, this kind of higher priority UE should use the highest priority synchronization source available for its own SLSS generation, so that all the transmissions in the same carrier use the same synchronization as much as possible. We assume that Rx UE can follow the SLSS transmission from other UEs also when it uses GNSS or gNB for synchronizing its own transmissions.

Proposal 6: In unicast and groupcast, the SLSS of a specific UE can be prioritized over other SLSS transmissions

# 3	Discussion on SL synchronisation signal design

In RAN1#98bis following agreements were made in context of SL synchronisation channel design:

Agreements: - RAN1 will consider the design of NR SLSS where - S-PSS of LTE and NR candidates have similar PAPR. - S-SSS of LTE and NR candidates have similar PAPR. - SLSS design should take into consideration PAPR difference between S-PSS and S-SSS. Agreements: - In NR V2X, S-SSB bandwidth is 11RBs. - PSBCH spans 11RBs. - The S-SSB is designed following combination 1. - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS - Two symbols are used for each of S-PSS and S-SSS, respectively

In this section we discuss on matters related to the SL SS design and S-SSB structure.

## 3.1 NR SL SS design

During, in RAN1#96, an agreement was made under physical layer structure that Rel-16 NR sidelink would only support CP-OFDM. Following from this, and accounting the agreements made in RAN1#95, that S-SSB should support same numerology as PSCCH/PSSCH, it would seem natural SL-SS would also be based on CP-OFDM only.

Observation 1: SL-SS uses CP-OFDM only.

As per agreement made in RAN1#95, NR-Uu sequence design should be used as a starting point for S-PSS and S-SSS design. Correspondingly it was agreed in RAN1#AH1901 that m-sequence is used for S-PSS and Gold-sequence for S-SSS. In RAN1#96bis it was agreed that the sequence length is 127. Using same or similar sequence design as for NR-Uu, has the merit that the UE detector can be (at least partially) re-used, removing the need to implement new detector algorithms adding complexity.

One important factor to considering especially the design of S-PSS, is the orthogonalization against NR-Uu PSS and SSS. In LTE-V2X sidelink the roots of the PSS sequence were changed to prevent possible confusion by LTE Uu devices. When considering NR design, similar approach, by changing sequence properties, could also be considered. It could be considered adjusting either the cyclic shifts (selected based on NID(2)), adjusting the polynomial used, and/or the initialization. In addition to the sequence domain changes, also frequency domain orthogonalization could be considered for NR-V2X sidelink synchronisation signal. In NR based synchronisation design, the SSB is not required to be placed to the middle of the carrier, but can is placed freely (within) the carrier band. The valid locations of the SSB are defined by the synchronisation raster. However, as this would require in some extent agreeing and ensuring that the S-SSB locations would not collide with the NR-Uu locations, it would be preferable to achieve the orthogonalization by sequence design.

Thus to ensure low complexity synchronization signal search for NR V2X SL, it could be desirable that the S-PSS would have low cross-correlation against NR-PSS, to prevent possible ‘false alarms’ for the legacy NR-Uu devices. On the other hand, it could be desirable to also to ensure also that the S-PSS does not correlate too well with NR-Uu SSS to prevent possible errors in NR-Uu cell ID detection (e.g. in case of collision). Of course, when NR V2X device does synchronisation search, it does not necessarily matter if, with the same search, also NR-Uu can be detected. It could, in certain sense also seen as a benefit.

As a first, from design perspective the simplest approach would be to use exactly same baseline m-sequence design as for NR-Uu, but use different cyclic shift values than in NR-Uu, ([0 43 86]) to achiever orthogonalization. For NR-PSS perspective this would ensure good cross-correlation properties and, as NR-PSS polynomial (145) is also used to generate the NR-SSS, thus the cross-correlation performance be good (2^((n+1)/2)+1=17).

Observation 2: It is possible to select different cyclic shift values from NR-PSS polynomial (145) and achieve good cross-correlation performance against NR-SSS.

If it is seen desirable to use completely different polynomial to avoid any possible legacy device design artefacts, it would be possible to consider using different generator polynomial with same order/length. To achieve this, it could be possible to use the second generator polynomial used to generate NR-SSS sequence, 131 (i.e. x^7+x+1). This would also ensure good correlation properties against NR-PSS and NR-SSS. To ensure best cross-correlation properties against NR-PSS, the cyclic shifts would need to be selected. Cyclic shift values that have minimum cross-correlation value against all of the NR-PSS cyclic shifts are listed in Table 1.

Observation 3: Polynomial 131 (2nd generator polynomial of NR-SSS) can also achieve good gross correlation properties against NR-PSS and NR-SSS.

Table 1: Cyclic shifts of polynomial 131 giving minimum cross-correlation against NR-PSS

| Generator polynomial | Cyclic shifts |  |
| --- | --- | --- |
| 131 | 0 | 56 |
|  | 4 | 67 |
|  | 13 | 74 |
|  | 24 | 99 |
|  | 31 | 100 |
|  | 33 | 117 |
|  | 38 | 118 |
|  | 43 | 122 |
|  | 51 |  |

All but one of the remaining possible generator polynomials (of max 18) of order 7 can also provide minimum cross-correlation against NR-PSS at least with one selected cyclic shifts (polynomial 137 does not give any). However their cross-correlation properties against NR-SSS, with the selected cyclic shifts (giving good cross correlation against NR-PSS) is not as good as with polynomials 145 and 131.

Observation 4: Other polynomials (of order 7) do not offer similar cross-correlation properties against both, NR-PSS and NR-SSS, as polynomials 145 and 131.

From detection performance perspective the side slopes time and frequency domain correlation magnitudes (i.e. ambiguity function) was evaluated for polynomial 131 and compared against NR-PSS properties. In Table 2 the max correlation peaks and mean correlation for the side slopes (excluding the main peak) are presented for both. Evaluation was done assuming time domain range of [-64…63] samples (no oversampling) and frequency domain range of [-11 … 11] tones. It can be seen that with selected cyclic shifts for polynomial 131 similar of better (time/frequency) ambiguity properties can be achieved as for NR-PSS.

Observation 5: M-sequence generated with polynomial 131 can achieve similar or better time and frequency domain correlation properties (ambiguity function) as NR-PSS.

Table 2: Correlation magnitudes for polynomials 131 and 145

| Polynomial 131 |  |  | Polynomial 145 (NR-PSS) |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic shift | max. side lopes | mean side lopes | Cyclic shift | max. side lopes | mean side lopes |
| 0 | 24.5564 | 6.6915 | 0 | 21.3519 | 6.6954 |
| 4 | 22.9556 | 6.7171 | 43 | 20.8243 | 6.6496 |
| 13 | 23.0613 | 6.6980 | 86 | 21.1668 | 6.6888 |
| 24 | 19.9906 | 6.7365 |  |  |  |
| 31 | 20.6924 | 6.6903 |  |  |  |
| 33 | 19.4044 | 6.6821 |  |  |  |
| 38 | 19.3419 | 6.7302 |  |  |  |
| 43 | 19.3502 | 6.7680 |  |  |  |
| 51 | 18.6379 | 6.7228 |  |  |  |
| 56 | 18.8715 | 6.7436 |  |  |  |
| 67 | 20.0996 | 6.7231 |  |  |  |
| 74 | 19.9881 | 6.7097 |  |  |  |
| 99 | 18.9270 | 6.7124 |  |  |  |
| 100 | 20.4296 | 6.6853 |  |  |  |
| 117 | 23.4732 | 6.6847 |  |  |  |
| 118 | 21.3719 | 6.6626 |  |  |  |
| 122 | 23.3129 | 6.6439 |  |  |  |

Following the above approach, it would seem possible to orthogonalize with NR-Uu design, by (re-)using the polynomial 131 for S-PSS. Note that while in above analysis it was shown that there can be found multiple suitable values for cyclic shifts, the number supported for S-PSS should be considered based on total number of identities required and S-SSS design (number of supported IDs).

Proposal 7: Either different cyclic shift of NR-PSS (polynomial 145) or polynomial 131 is used as S-PSS.

When considering the size of ID space to be supported, there could be benefit to consider different use-cases compared to LTE SL. In LTE the id space is divided between in-coverage and out-of-coverage and also the SL id is determined by gNB id (in-coverage). In NR there are different use-cases (unicast/groupcast) and additional synchronisation sources (GNSS, gNB, NR UE, and eNB), which should be given different priorities, as taken as a working assumption in RAN1#96. If the UE can separate different SL synchronisation sources based on the S-SSB, in addition to in-coverage and out-coverage, it could simplify the synchronisation procedure.

Proposal 8: Identity space supported by S-SSB accounts possibility to separate different synchronisation sources and possibly also use cases.

## 3.2 S-SSB design

In RAN1#96bis it was agreed that the S-SSB BW is 11PRB. The number of available symbols per SSB would be upper bound by the number of symbols in slot. Following from the agreements made in RAN1#95, that S-SSB should support same numerology as PSCCH/PSSCH, i.e. both normal and extended CP at least for 60kHz, the S-SSB design should account both 14 symbol and 12 symbol slot structure. For simplicity would be preferable if the S-SSB design would be common for NCP and ECP, but as there is no need for the UEs to consider different hypothesis for CP detection (due to (pre-)configured SL-BWP), it may not be strictly required. Note that if PSS is repeated over two symbols, the concatenated PSS symbols could be used to detect the applied cyclic prefix.

Observation 6: S-SSB design needs to support both, 14 symbol and 12 symbol slots structure.

With narrower S-SSB BW selected, the number of symbols may need to be increased compared to NR to maintain the same coding rate or the payload decreased. In NR-Uu SSB design, PBCH decoding performance is limiting factor in terms of the one-shot performance. Thus, if the coverage of S-SSB is to be increased (with similar payload), the number of symbols would need to be increased. The LTE SL PSBCH occupies 7 symbols (+2 for DMRS) over 6PRBs. Now, assuming that S-PBCH payload is similar as for NR and that we would like to keep the coding rate and DMRS density (every 4th RE) similar, approximately 8 to 9 symbols would be needed for PSBCH.

Additional aspect of the time domain design is whether there is need to preserve, like in NR-Uu, preserve symbols from the start and end of the slot for DL/UL control or whether the slot is preserved fully for S-SSB. In NR-Uu, it was assumed that cell could schedule data on RBs not occupied by SSB, but in case of SL this would not seem necessity from power budget perspective. Thus assuming that no symbols are reserved for control at the beginning of the slot, and that common basic structure targeted for NCP and ECP, PSBCH could accommodate 8 symbols in maximum. This would allow doubling the energy compared to NR-Uu design. This would also enable leaving, for 14 symbol slot format, 2 symbols free, e.g. at the end the slot. Also to facilitate the CP hypothesis determination, S-PSS and S-SSS symbols could be mapped in non-consecutive symbols.

Figure 3. Illustration of possible NR-V2X S-SSB design

S-SSB/PSBCH content

The PBCH payload size in NR is 56 bits (including 24 bit CRC) carried by 48 PRBs (including DMRS at ¼-density). In LTE the SL-MIB carries timing related information (SFN, sub-frame), slot pattern and transmission bandwidth configuration together with the coverage case indication. In NR, part of the information needed by the is carried e.g. by DRMS. Like discussed in Section 3.1, S-SSB would need to carry information to separate different synchronisation conditions, as well possible sources and use cases.

In terms of timing related information, in NR this is provided partly by MIB (6 MSB bits of SFN), PBCH payload (four LSBs of SFN) and also informed via SSB index (in PBCH payload till Lmax=8, and also in MIB for Lmax=64) together with predefined SSB time location pattern. This gives the timing for the UE down to slot level. In LTE-V2X, corresponding information is carried by MIB. In NR the number of slots in a radio frame will depend on the used numerology and thus the number of bits needed to provide the information may increase. One approach, used in NR is to use predefined candidate time pattern (e.g. within 5ms), together with corresponding indexing to a carry additional timing information down to slot level. This would reduce the amount of information that would need to be carrier by S-SSB. However, if, the difference compared to NR is that the slots where S-SSB can be sent would be restricted e.g. by UL-DL slot configuration, thus it might not be possible to determine single pattern to enable support of multiple S-SSBs within a period. Other alternative would be to provide the slot index directly. However, when considering higher subcarrier spacings, e.g. 60kHz or 120kHz, the number of slots in one radio frame gets rather high (i.e. 40 of 80), thus up to 7 bits would be needed. Reducing the size of slot timing indication by indicating only few slots from the start of the frame, might not be desirable as these might not align well with UL-DL slot pattern, unless the UL-DL slot pattern is accounted in some manner.

Observation 7: To support different deployments with different UL-DL slot configurations, it might not be possible to support fixed S-SSB candidate time location pattern.

Observation 8: Indicating slot index directly might not be feasible due to larger number of bits needed.

Proposal 9: SFN information is carried as a part of PSBCH payload.

For unpaired spectrum operation, there would need to be provide UL-DL slot configuration (TDD-UL-DL-ConfigCommon) for the UE. In LTE V2X the possible UL-DL slot configuration options cover the configurations given in Table 4.2-2 in 36.211. In NR V2X, this information can be rather large, depending on the used sub-carrier spacing. However it also provides additional flexibility and forward compatibility to the deployments, and therefore it would seem preferable not to restrict this information. Thus it would be good to consider whether this information can be carried by S-SSB.

Proposal 10: UL-DL slot configuration (TDD-UL-DL-ConfigCommon) is carried as part of PSBCH payload.

In LTE V2X the transmission bandwidth configuration is provided as a part of the MIB. In NR-Uu the initial DL BWP is set to be same as the CORESET#0 BW (BW and location). It has was taken as a working assumption in RAN1AH1901 (later confirmed in RAN1#95) that only one SL-BWP is configured per carrier and agreed that SL-BWP configuration is separate from the NR-Uu BWP configuration. It also has been agreed that one SL BWP is (pre-)configured for RRC IDLE and out-of-coverage NR V2X UEs in a carrier (RAN1#95). Thus based on these agreements, it would appear that the SL-BWP is known by the UE, and there is no need to provide it again in MIB. As PSCCH/PSSCH will have same numerology as S-SSB, there would not be any need to include any numerology indication to SL-MIB.

Observation 9: SL-BWP (also for IDLE) can be assumed to be (pre-)configured for the UE (including the numerology), thus there would not appear to be any no need to provide the information again in MIB.

In NR, the SSB provides UE the information on the frequency relation of the SSB and the initial BWP. In addition, in NR, an offset was introduced to enable reducing the synchronization raster points by enabling SSBs to be placed off the PRB raster of the data (while to same subcarrier raster). For NR V2X, it has been agreed (in RAN1#AH1901) that S-SSB frequency location are (pre-)configured in advance. Following from this and as the SL-BWP configuration can also be assumed to be known (as noted above), there seems to be no need to provide information on the frequency relation of BWP and S-SSB in MIB.  If the S-SSB candidate frequency locations are preconfigured, there would neither be need to have this information included to the PSBCH, if it can be assumed that S-SSB is always on same PRB grid as data.

Observation 10: As SS raster location is (pre-)configured, it would appear to be possible to assume that S-SSB is always aligned with common PRB raster of the BWP and there is no need to provide this information in PSBCH.

# 4	Conclusions

In this contribution, we have discussed synchronization aspects of NR V2X. In Section 2, we have the following observations and proposals in relation to synchronisation procedures:

Proposal 1: The working assumption on the synchronization priority table is confirmed with the clarification that when network-based synchronization is used the carrier for eNB or gNB signals for synchronization is (pre)configured

Proposal 2: When network-based synchronization is (pre)configured only SLSS from NR SL UE is used for synchronization of NR SL UE. When GNSS based synchronization is (pre)configured also SLSS from LTE SL UE can be used for synchronization

Proposal 3: Mapping between SLSS-ID and cell-ID is supported when network-based synchronization is used.

Proposal 4: DMRS should be selected as the working assumption, when reference signal-based synchronization method is studied.

Proposal 5: UE that uses reference signal-based synchronization for its transmission cannot be independent synchronization source, but it needs to synchronize to some external source

Proposal 6: In unicast and groupcast, the SLSS of a specific UE can be prioritized over other SLSS transmissions

In Section 3.1 we discussed the SL synchronisation signal design, namely PSS and SSS and made following proposals and observations:

Observation 1: SL-SS uses CP-OFDM only.

Observation 2: It is possible to select different cyclic shift values from NR-PSS polynomial and achieve good cross-correlation performance against NR-SSS.

Observation 3: Polynomial 131 (2nd generator polynomial of NR-SSS) can also achieve good gross correlation properties against NR-PSS and NR-SSS.

Observation 4: Other polynomials (of order 7) do not offer similar cross-correlation properties against both, NR-PSS and NR-SSS as polynomials 145 and 131.

Observation 5: M-sequence generated with polynomial 131 can achieve similar or better time and frequency domain correlation properties (ambiguity function) as NR-PSS.

Proposal 7: Either different cyclic shift of NR-PSS (polynomial 145) or polynomial 131 is used as S-PSS.

Proposal 8: Identity space supported by S-SSB accounts possibility to separate different synchronisation sources and possibly also use cases.

The S-SSB design has been discussed in Section 3.2 and following observations and proposals have been made:

Observation 6: S-SSB design needs to support both, 14 symbol and 12 symbol slots structure.

Observation 7: To support different deployments with different UL-DL slot configurations, it might not be possible to support fixed S-SSB candidate time location pattern.

Observation 8: Indicating slot index directly might not be feasible due to larger number of bits needed.

Proposal 9: SFN information is carried as a part of PSBCH payload.

Proposal 10: UL-DL slot configuration (TDD-UL-DL-ConfigCommon) is carried as part of PSBCH payload.

Observation 9: SL-BWP (also for IDLE) can be assumed to be (pre-)configured for the UE (including the numerology), thus there would not appear to be any no need to provide the information again in MIB.

Observation 10: As SS raster location is (pre-)configured, it would appear to be possible to assume that S-SSB is always aligned with common PRB raster of the BWP and there is no need to provide this information in PSBCH.

# References

- RP-190766,	New WID on 5G V2X with NR sidelink, LG Electronics, Huawei

- 3GPP TS 38.133 V15.3.0 (2018-09), Requirements for support of radio resource management

- R1-1813521, Discussion on NR V2X Sidelink Synchronization, Nokia, Nokia Shanghai Bell

- R1-1609567, On UL Waveforms below 40 GHz, Nokia, Nokia Shanghai Bell

# Annex A: Earlier agreements

RAN1#94:

RAN1#95:

Agreements:

- BWP is defined for NR sidelink.

  - In a licensed carrier, SL BWP is defined separately from BWP for Uu from the specification perspective.

    - FFS the relation with Uu BWP.

  - The same SL BWP is used for both Tx and Rx.

  - Each resource pool is (pre)configured within a SL BWP.

  - Only one SL BWP is (pre)configured for RRC idle or out of coverage NR V2X UEs in a carrier.

  - For RRC connected UEs, only one SL BWP is active in a carrier. No signalling is exchanged in sidelink for activation and deactivation of SL BWP.

    - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

      - Revisit in the next meeting if significant issues are found

  - Numerology is a part of SL BWP configuration.

Note: This does not intend to make restriction in designing the sidelink aspects related to SL BWP.

Note: This does not preclude the possibility where a NR V2X UE uses a Tx RF bandwidth the same as or different than the SL BWP.

Agreements:

- S-SSB has the same numerology, which includes SCS and CP length, as that of control and data channels for a given carrier

Agreements:

- The transmission bandwidth for S-SSB is within the BW of the (pre)-configured SL-BWP.

  - FFS:  The actual transmission BW for S-SSB and sync raster

RAN1#AH1801

Agreements:

- The frequency location for S-SSB is (pre-) configured

  - Note: it implies that there is no intended hypothses detection in frequency location of S-SSB performed by the UE for a carrier in a given band

  - Note: the potential frequency locations for the (pre-)configured frequency location may be restricted, up to RAN4

Agreements:

- Confirm the working assumption

  - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE