3GPP TSG RAN WG1 #97	R1-1906083
Reno, USA, May 13th – 17th, 2019

Source:				Panasonic

Title:	UL signals and channels for NR-U

Agenda item:	7.2.2.1.3

Document for:			Discussion and Decision

Introduction

This document discusses the following NR-U UL aspects:

Block interlaced PUSCH

Block interlaced PUCCH

SRS enhancement

Discussion on block interlaced PUSCH

This chapter is the update from [1] to describe the merit of cluster-level PUSCH allocation on FDM with PRACH.

Flexible starting point subject to LBT

NR-U TR [2] mentions the following discussion related to the PUSCH starting point:

The following options have been identified as possible candidate at least for the first PUSCH(s) transmitted in the UL transmission burst. - Option 1: PUSCH(s) as in Rel-15 NR - Option 2: Multiple starting positions in one or multiple slot(s) are allowed for PUSCH(s) scheduled by a single UL grant (i.e., not a configured grant) and one of the multiple PUSCH starting positions can be decided depending on LBT outcome. It is noted that for above options, the ending position of the PUSCH is fixed as indicated by the UL grant.

We propose Option 1 only. We understand Option 2 is resource-efficient as the PUSCH partial slot can start immediately after LBT pass when LBT frequently fails. However, our position is that the grant-based PUSCH is sent in gNB shared COT with LBT cat 1 or 2 to simplify the gNB reception process and receiver side (at gNB) LBT assurance. On this assumption, the possibility of LBT failure would be low, and then the gain from option 2 seems marginal.

Even if the gain from Option 2 is important, further discussion on the following aspects are needed to support this option:

How gNB detect the PUSCH starting point without the knowledge of the UE LBT’s situation

Whether puncture or perform rate-matching related to the part of PUSCH before LBT pass.

Taking account of the standardization effort, to support option 1 is reasonable.

Proposal 1:	For grant-based PUSCH, NR-U supports the same starting points only as in Rel-15 NR (i.e. support Option 1 only).

PUSCH resource allocation scheme in frequency domain

In LAA, PRB-based block-interlace design is applied for PUSCH transmission and, uplink resource allocation type 3 is used as the PUSCH resource allocation. The uplink resource allocation type 3 can allocate the interlace resource of 100 PRBs at maximum. The resource allocation field in UL grant consists of a resource indication value (RIV). The RIV uniquely corresponds to the starting PRB in a cluster and continuous allocation length in a cluster. The RIV is indicated within a cluster, and it is applied to all clusters.

Similar scheme can be applied in NR-U. However, NR-U supports operating bandwidth of 20 MHz or more (e.g. 80MHz). Further, indicating the resource of 20MHz or more by one DCI has been discussed in RAN1. If the interlace allocation indicated by one RIV is always applied to all the clusters in the operating band, the number of PRBs per UE increase. It is inefficient for UE with only small data. Also, in the case of cell edge UE, transmission power may be limited.

Besides, the cluster-level PUSCH allocation is useful to realize the FDM with NR-U PRACH. As PRACH with contiguous mapping (i.e. non-interlace manner) is only supported, PUSCH can be mapped on the part of an interlace by the cluster-level PUSCH allocation and this avoids the collision with the RACH occasion.

Therefore, in addition to the interlace allocation by RIV within cluster like LAA, which cluster or which 20MHz sub-band units are applied should be indicated as Figure 1. Such indication can be another RIV or additional bitmap.

Proposal 2:	For NR-U PUSCH resource allocation scheme in frequency domain, in addition to the interlace allocation by RIV within cluster like LAA, which cluster or which 20MHz sub-band units are applied should be indicated.

Figure 1 Resource allocation scheme

Waveform

In our understanding, whether to support DFT-s-OFDM is still open for NR-U PUSCH.

In [3], the PAPR evaluations for B-IFDMA (i.e. interlace with DFT-s-OFDM) and interlaced mapping with OFDMA are compared. According to this paper, DFT-s-OFDM can have less PAPR by 1-2 dB than OFDMA. Therefore, it is beneficial to support DFT-s-OFDM at least for the UL cell-edge case.

Proposal 3:	Support interlace with DFT-s-OFDM for NR-U PUSCH.

Discussion on block interlaced PUCCH

Enhancement to support small UCI payload (1-2 bits)

This section is the resubmission of [1].

According to the agreement [4], to support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3 have been agreed. Therefore, transmission of UCI payloads larger than 2 bits can be realized with relatively small efforts. On the other hand, how to support the small payload with 1-2 bits is still FFS. The alternatives are as follows:

Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3

Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1

We prefer to support Alt-1. We think the enhanced PF2 and PF3 with e.g. zero-padding can work as a method to carry small UCI payload. Although it can degrade the resource efficiency, it saves the standardization effort by reducing the number of the enhanced PUCCH formats. On the other hand, there is not a strong motivation to support the enhanced PF0 or PF1 (Alt-2) as the user multiplexing is supported by the enhanced PF2 and PF3.

Proposal 4:	Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3 for NR-U (Alt-1).

Waveform

According to the agreement [4], it is FFS whether to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3 or not.

According to the NR-U TR [2], the legacy Rel-15 PUCCH format 3 is identified beneficial for NR-U. Therefore, NR-U UE would implement DFT precoder and then it can be utilized for enhanced PUCCH format 3 also. Besides, to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3 causes some spec impacts. Eventually, we don’t see the strong motivation to replace waveform from DFT-s-OFDM as Rel-15.

Proposal 5:	Support DFT-s-OFDM for enhanced PUCCH format 3 for NR-U as well as NR Rel-15 (Do NOT replace).

Transmission timing

This section is the resubmission of [1].

For NR-U, there are multiple types of COT. One is the gNB-shared COT and the other one is UE-initiated COT. It has been identified beneficial that HARQ-A/N for the corresponding data is sent in the same gNB-shared COT. We propose that HARQ-A/N over PUCCH for the separated COT is also sent in the gNB-shared COT only but not in UE-initiated COT.

Increased opportunity

In the gNB-shared COT, if the switching gap between DL and UL is less than 16 us, LBT before UL transmission can be omitted. Therefore, the opportunity to transmit the UCI indicated by DCI can be relatively increased. This can help to fast feedback HARQ-A/N. On the other hand, for UE-initiated COT, LBT is necessary before UL transmission. Then the feedback can be delayed.

Increased multi-user CDM/FDM feasibility

As stated above, if LBT before UL can be omitted in the gNB-shared COT, the time domain position of each user’s UL transmission can be aligned. Therefore, multi-user CDM/FDM of PUCCH can be realized. This is also beneficial for the fast UCI feedback. On the other hand, for UE-initiated COT, the LBT with randomized contention time is required for each UE. Therefore, the timing position of each UE’s UL may not be aligned.

Similarly, to send CSI reporting on PUCCH within gNB-shared COT is useful to allow flexible resource allocation of PUCCH. This can be enabled by group-PDCCH based trigger of CSI report on PUCCH. We propose such PUCCH report on gNB-shared COT and we call it as aperiodic PUCCH. In addition to such aperiodic PUCCH, to have CSI report over PUCCH in UE-initiated COT may or may not be beneficial. These aspects should be studied further.

For the reasons above, we would propose the following:

Proposal 6:	For NR-U, HARQ-A/N over PUCCH should be transmitted in gNB-shared COT only.

Proposal 7:	For NR-U, CSI report over PUCCH triggered by group based PDCCH in gNB-shared COT should be supported.

Discussion on SRS

This chapter is the resubmission of [6].

Periodicity

In LTE eLAA, only aperiodic SRS is supported. For NR-U also, aperiodic SRS should be supported as it is efficient that aperiodic SRS in COT is requested by gNB on demand. Periodic / semi-persistent SRS may not be resource-efficient as available time resource by COT is limited compared with license-based operation.

Proposal 8:	Only aperiodic SRS is supported for NR-U.

SRS with/without PUSCH

In LTE eLAA, both SRS with and without PUSCH are supported. For NR-U also, both SRS should be supported to increase the opportunity of sounding. Further, SRS without PUSCH can be useful to fill the gap between the transmissions if the continuous TDM of UL transmissions from multiple UEs is allowed by the regulation. For example, when UL data is absent, SRS may be sent during the preparation time for the HARQ feedback.

Proposal 9:	Both SRS with and without PUSCH are supported for NR-U.

Frequency domain resource

In NR Rel-15, SRS with and without frequency hopping are supported. SRS without frequency would be mapped to cover whole BWP to sound it once. SRS with frequency hopping was designed for the PSD boosting.

For single 20 MHz sub-band on NR-U, SRS without hopping would be sufficient and beneficial to sound the whole sub-band within the limited sounding opportunity and to meet the wide OCB requirement. On the other hand, SRS with hopping may not be beneficial as it may not bring the benefit of power boosting due to PSD regulation on unlicensed band.

SRS frequency resource across multiple 20 MHz sub-bands can be discussed after how to allocate SRS to multiple sub-bands is concluded in agenda 7.2.2.2.5.

Proposal 10:	SRS without frequency-hopping is supported on single 20 MHz sub-band for NR-U.

Association between SRS and beam/antenna

In NR Rel-15, SRS can be associated with analog beam and antenna in order to support the beam management. The association between SRS and beam/antenna would be:

Each SRS resource can be associated with beam/antenna by UE.

SRS request field can indicate the SRS resource for beam measurement.

Based on the SRS measurement, SRI can indicate the SRS resource associated with the good beam(s) for PUSCH transmission.

For NR-U, this association can be the baseline. The required enhancement on this should be further discussed.

Proposal 11:	NR Rel-15 is the baseline for the association between SRS and beam/antenna for NR-U.

Conclusion

For NR-U block interlaced PUSCH, we would propose the following:

Proposal 1:	For grant-based PUSCH, NR-U supports the same starting points only as in Rel-15 NR (i.e. support Option 1 only).

Proposal 2:	For NR-U PUSCH resource allocation scheme in frequency domain, in addition to the interlace allocation by RIV within cluster like LAA, which cluster or which 20MHz sub-band units are applied should be indicated.

Proposal 3:	Support interlace with DFT-s-OFDM for NR-U PUSCH.

For NR-U block interlaced PUCCH, we would propose the following:

Proposal 4:	Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3 for NR-U (Alt-1).

Proposal 5:	Support DFT-s-OFDM for enhanced PUCCH format 3 for NR-U as well as NR Rel-15 (Do NOT replace).

Proposal 6:	For NR-U, HARQ-A/N over PUCCH should be transmitted in gNB-shared COT only.

Proposal 7:	For NR-U, CSI report over PUCCH triggered by group based PDCCH in gNB-shared COT should be supported.

For NR-U SRS, we would propose the following:

Proposal 8:	Only aperiodic SRS is supported for NR-U.

Proposal 9:	Both SRS with and without PUSCH are supported for NR-U.

Proposal 10:	SRS without frequency-hopping is supported on single 20 MHz sub-band for NR-U.

Proposal 11:	NR Rel-15 is the baseline for the association between SRS and beam/antenna for NR-U.

Reference

R1-1904594, Panasonic, RAN1 #96bis, Xi’an, April 2019

3GPP TR 38.889 V16.0.0

T. Svensson, et al., “Block Interleaved Frequency Division Multiple Access for Power Efficiency, Robustness, Flexibility, and Scalability,” EURASIP Journal on Wireless Communication and Networking, Volume 2009, Mar. 2009

RAN1 Chairman’s Notes, RAN1 #96, Athens, February 2019

RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm Inc., RAN#82

R1-1900240, Panasonic, RAN1 AH-1901, Taipei, January 2019

Appendix - Agreements from past meetings

The following UL aspects for NR-based Access to Unlicensed Spectrum (NR-U) were captured in NR-U TR [2]:

For scenarios in which a block-interlaced waveform is used for UL transmission, a PRB-based block-interlace design has been identified as beneficial at least for 15 and 30 kHz SCS, and potentially for 60 kHz SCS. One identified benefit is better link budget with given PSD constraint. However, it has been observed that power boosting gains decrease with increasing SCS. Another identified benefit is as one option to efficiently meet the occupied channel bandwidth requirement. Compared with sub-PRB interlace design, the PRB-based block-interlace design has comparatively less specification impact. For sub-PRB block interlace designs, in some scenarios, sub-PRB block interlacing can be beneficial in terms of power boosting. However, the sub-PRB block interlace design has at least the following specification impacts: Reference signal design (e.g., DMRS); Channel estimation aspects; Resource allocation. Both PRB and sub-PRB interlacing for 60 kHz have been studied. For sub-PRB interlacing the following aspects have been considered: - Power boosting potential depending on resource allocation size - PUSCH DMRS configuration aspects - Channel estimation performance - Number of REs per interlace unit It has been identified as beneficial to support a block-interlaced structure in which the number of interlaces (M) decreases with increasing SCS, and the nominal number of PRBs per interlace (N) is similar for each SCS (in a given bandwidth) at least for 15 and 30 kHz SCS, and potentially 60 kHz depending on supported interlace design. From a RAN1 perspective it has been identified that supporting a non-uniform interlace structure in which the number of PRBs per interlace is allowed to be different for different interlaces is beneficial from a spectrum utilization point of view. It is up to RAN4 to investigate whether or not the non-uniform interlace structure has an impact on MPR/A-MPR requirements for PUSCH. Within a 20 MHz bandwidth, the following candidate PRB-based interlace designs have been identified where M is the number of interlaces and N is the number of PRBs per interlace in a 20 MHz bandwidth. Where two values are listed for N, it means that some interlaces have one more PRB than others (non-uniform interlace design) SCS; M; N. 15 kHz; 12; 8 or 9. 10; 10 or 11. 8; 13 or 14. 30 kHz; 6; 8 or 9. 5; 10 or 11. 4; 12 or 13. 60 kHz; 4; 6. 3; 8. 2; 12. 60 kHz (if 26 PRBs is supported in a 20 MHz bandwidth); 4; 6 or 7. 2; 13. 3; 8 or 9. For carriers with bandwidth larger than 20 MHz, two candidate interlace designs have been identified: - Alt-1: Same interlace spacing for all interlaces regardless of carrier BW. This alternative uses Point A as a reference for the interlace definition - Alt-2: Interlacing defined on a sub-band (20 MHz) basis. (Note: Possible interlace spacing discontinuity at edges of sub-band). Additional candidates have been identified, but consensus has not been achieved, e.g., (1) for carriers with bandwidth larger than 20 MHz, retain the same number of PRBs per interlace (N) for all interlaces regardless of carrier BW; (2) Partial interlace allocation. Detailed design can be further discussed when specifications are developed taking RF aspects into account.

The following UL aspects for NR-U are captured in NR-U WID [1]:

- UL control including extension of PUCCH format(s) to support PRB-based frequency block-interlaced transmission and use of Rel-15 NR PUCCH formats 2 and 3 for NR-U operation. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1. - UL data channel including extension of PUSCH to support PRB-based frequency block-interlaced transmission; support of multiple PUSCH(s) starting positions in one or multiple slot(s) depending on the LBT outcome with the understanding that the ending position is indicated by the UL grant; design not requiring the UE to change a granted TBS for a PUSCH transmission depending on the LBT outcome. The necessary PUSCH enhancements based on CP-OFDM. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1. - SRS including the introduction of additional flexibility in configuring/triggering SRS in line with agreements during the study phase.

The agreements from RAN1 AH-1901:

Agreement: For interlace transmission of at least PUSCH and PUCCH, the following PRB-based interlace design is supported for the case of 20 MHz carrier bandwidth: a. 15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace b. 30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace Note: PRACH design to be considered separately, including multiplexing aspects with PUSCH and PUCCH Working assumption: - For a given SCS, the following interlace design is supported at least for PUSCH: - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth - Point A is the reference for the interlace definition - For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths - FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz - FFS: Whether and how partial interlace allocation is supported

The agreements from RAN1 #96 [4]:

Agreement: - Support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3. The enhancements include at least the following aspects: - For a 20 MHz carrier bandwidth, support mapping to physical resources of at least one full interlace - Mechanism to support user multiplexing for both data and reference symbols of PUCCH - The following aspects are FFS: - Support for small payloads (1 and 2 bits) - Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3 - Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1 - Whether or not to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3 Agreement: Support configuration of an SRS resource in additional OFDM symbol locations other than the last 6 symbols of a slot with PUSCH and SRS time division multiplexed as in Rel-15. - FFS: which symbols locations. Agreement: Sub-PRB interlace design for PUSCH and PUCCH is not supported.

The agreements from RAN1 #96bis:

Agreement: For a 20 MHz carrier bandwidth, if enhancements to PF0 and PF1 are supported, a mapping to physical resources of at least one full interlace is supported - FFS: Whether or not to support enhancements to PF0/1. - Companies are encouraged to provide user multiplexing capacity and UCI payload analysis for enabling the decision for relevant use cases Agreement: Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13. Agreement: Decisions on which additional PUCCH formats (enhanced or combination of legacy and enhanced) are supported should be at least based on the following. - Which PUCCH format(s) are to be used at least for the following use cases: - HARQ ACK prior to dedicated PUCCH resource configuration - HARQ ACK, SR, CSI and combinations thereof after dedicated PUCCH resource configuration - Specification impact, e.g., UE procedures in 38.213 and 38.212, for all proposed PUCCH formats to be supported - User multiplexing capacity and UCI payload analysis for all proposed PUCCH formats to be supported - In-band and out-of-band emissions Agreement: The following common set of evaluation assumptions and reporting metrics are used for enhanced PUCCH design Evaluation assumptions Property; Value. Carrier frequency; 5 GHz. Channel bandwidth; 20 MHz. Channel model; TDL-C. Delay scaling; At least 10ns, 100ns with 300 ns optional. Antenna configuration at BS*; (M,N,P) = (1,1,2) with omni-directional antenna element. Antenna configuration at UE; Single omni-directional antenna element. Antenna port virtualization; No beamforming and no beam selection. Frequency offset; 0 ppm. UE speed; 3 km/h. Subcarrier spacing; 15/30 kHz (with other SCS optional). Number of code-division multiplexed users if applicable; 1 user, with other cases optional. Interference assumption; No inter-cell interference. * See Table 7-1 of R1-1704144. Reporting metrics Parameter; Value; Notes. Enhanced PUCCH Format; e.g., E-PF0, E-PF1, E-PF2, E-PF3. Number of OFDM symbols used for PUCCH resource; e.g., 1, 2, 4, 14. Number of RBs used for PUCCH resource (N_RB); At least 1 full interlace assumed (Assume interlace design for 20 MHz carrier bandwidth as agreed in RAN1AH#1901 for 15kH and 30 kHz is used). Frequency domain OCC configuration details (if applicable); Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols, OCC cycling (if applicable). Time domain OCC configuration details (if applicable); Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols. Number of multiplexed users, e.g., by code division if applicable; 1 user is assumed as baseline Companies are to report other cases if evaluated. Waveform; e.g., CP-OFDM or DFT-s-OFDM. PUCCH encoder type; e.g., Reed Muller or Polar. SCS; 15KHz, 30KHz. Noise level, Np (dBm); Np = -174 + 10*log10(SCS*12*N_RB) + NF NF = 5dB. Required SNR (dB); Required SNR needed to fulfil detection criterion(1)(2), read from simulation curve. Cubic Metric; Note: Single value reported in dB. P_max (dBm); Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used for the PUCCH resource. Backoff (dB); Backoff is computed as the cubic metric. Note: If cubic metric is not used, information on the backoff metric used should be provided.. P_TX (dBm); P_TX = min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff. MCL (dB); MCL = P_TX – SNR – Np. PUCCH payload size(s) (bits); If multiple payload sizes evaluated, then MCL to be plotted vs. PUCCH payload size. PUCCH encoding rate(s); If multiple payload sizes evaluated, then multiple encoding rates to be reported (if applicable). - For PUCCH payloads of 1 or 2 bits, detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and is defined as P(ACK to Error) ≤ 1% and P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only. - For PUCCH payloads greater than 2 bits follow the requirements in TS38.104 Section 8.3 Note: - Companies should provide details of RE mapping in frequency and time of enhanced PUCCH formats that are evaluated - Companies should provide details about channel estimation assumptions, e.g. practical, ideal - Definition of SNR to be reported with evaluation metrics.