3GPP TSG RAN WG1 Meeting #97	R1-1906085

Reno, USA, May 13th – 17th, 2019

| Source: | Nokia, Nokia Shanghai Bell |
| --- | --- |
| Title: | Considerations on Performance Evaluation in NTN |
| Document for: | Discussion and Decision |
| Agenda Item: | 7.2.5.1 |

# 1 Introduction

A new Study Item (SI) on “Study on Solutions for NR to Support Non-Terrestrial Networks” was approved in RAN#80 meeting [1] and further updated in RAN#82 meeting [2] and RAN#83 meeting [3] with the considered scenarios of transparent GEO satellite and transparent/regenerative LEO satellite (moving beam on earth) for pedestrian UEs and on board vehicle UEs in NTN. The objectives of this SI for physical layer are as follows.

- Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]:

  - Physical layer control procedures (e.g. CSI feedback, power control)

  - Uplink Timing advance/RACH procedure including PRACH sequence/format/message

  - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

- Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

RAN1#96bis discussed the simulation assumptions of performance evaluation in NTN with the agreement summarized in [4].

In this contribution, we shared our views on link-level and system-level performance evaluation in NTN for this SI.

# 2 Calibration

The calibration of simulation platform is beneficial for following performance evaluation including throughput evaluation in RAN1 and mobility evaluation in RAN2 at least. In mobility evaluation, multiple satellites should be involved to analyze the handover performance from one satellite to another satellite due to the motion of satellites and UEs. Additionally, throughput evaluation also needs to consider inter-satellite interference besides intra-satellite interference, especially for the satellite-edge UEs. Therefore, satellite constellation should be considered in the calibration of system-level simulator in NTN.

Observation 1: Satellite constellation should be considered in the calibration for simulation platform in NTN.

Inclination satellite constellation is generally used as LEO satellite constellation and also can be used for GEO satellite constellation. Therefore, it is proposed to use inclination satellite constellation in this SI. Additionally, even using same beam layout for each satellite, active beam layout for each LEO satellite might be changed due to satellite motion to void strong interference especially in high latitude area. However, it is an implementation issue how to active beams based on satellite position. Therefore, it is suggested to focus on partial geographical area for simulation platform calibration and following system-level evaluation.

Proposal 1: Use inclination satellite constellation and focus on partial geographical area for simulation platform calibration and following system-level evaluation.

Table 2-1 provides one example of inclination satellite constellation configuration for each scenario as well as corresponding UE distribution. For LEO satellite, UEs are uniformly distributed in geographical beam coverage of main satellite for evaluation purpose. The neighbor satellites, which are visible by these UEs, will be involved for performance statistics, including serving beam selection and interference statistics. For GEO satellite, UEs are uniformly distributed in geographical coverage of given beam pattern of main satellite with the consideration of big coverage of each GEO satellite and simulation complexity. The neighbor beams, which might leak interference to these UEs, will be involved for performance statistics, including serving beam selection and interference statistics.

Table 2-1 Parameters for inclination satellite constellation and UE deployment.

| Satellite constellation | Type 1 | Type 2 | Type 3 |
| --- | --- | --- | --- |
| Satellite type | GEO | LEO | LEO |
| Satellite altitude [km] | 35786 | 1200 | 600 |
| Inclination angle () | 0 | 80 | 85 |
| Number of orbits | 1 | 9 | 10 |
| Number of satellites per orbit | 4 | 18 | 20 |
| Relative phase difference satellites in adjacent planes () | - | 10 | 9 |
| Beam layout | [TBD] | [TBD] | [TBD] |
| Main satellite position | (42157,0,0) km | (7571,0,0) km | (6971,0,0) km |
| UE deployment | Uniform distribution in geographical coverage of given beam pattern of main satellite | Uniform distribution in geographical beam coverage of main satellite | Uniform distribution in geographical beam coverage of main satellite |

Note: d is beam diameter.

Proposal 2: Use Table 2-1 for satellite and UE deployment in calibration.

Proposal 3: For LEO satellite, the neighbor satellites, which are visible by one UE, will be included for this UE’s performance statistics, including serving beam selection and interference statistics.

Proposal 4: For GEO satellite, the neighbor beams, which might leak interference to one UE, will be included for this UE’s performance statistics, including serving beam selection and interference statistics.

The large-scale channel model part includes LOS probability, free space path loss, atmospheric absorption loss, rain and cloud attenuation loss, ionospheric/tropospheric scintillation loss, O2I penetration loss, and shadow fading but not all appear simultaneously. Atmospheric absorption loss may be neglected at frequencies below 10 GHz normally but is recommended for calculation in the case of the elevation angles below 10 degrees. TR 38.811 provides the baseline method for atmospheric absorption loss in system level simulation. Therefore, it is recommended to calculate the atmospheric absorption loss for large-scale channel model calibration in Ka band but neglect in S band with 10 as the minimal elevation angle configuration.

Proposal 5: Calculate atmospheric absorption loss for calibration in Ka band but neglect it in S band with 10 as the minimal elevation angle configuration.

The path loss due to rain and cloud attenuation is location specific and can be neglected for the frequencies below 6 GHz [3]. As mentioned in TR 38.811, the baseline for system-level simulation is to consider the clear sky conditions only. Therefore, it is recommended to neglect the path loss due to rain and cloud attenuation in calibration.

Proposal 6: It is recommended to consider clear sky condition as baseline in calibration and neglect the path loss due to rain and cloud attenuation.

The pathloss of ionospheric scintillation can be only applied for the carrier frequency below 6 GHz and neglected for other carrier frequencies. For the carrier frequency below 6 GHz, the ionospheric scintillation pathloss can be obtained by Equation (6.6-13) in TR 38.811 for the latitudes within ±20, is zero for the latitudes between ±20 and ±60, and equals 12.02 dB for other latitudes. The peak-to-peak amplitude fluctuations in Equation (6.6-13) for path loss calculation can be obtained by the amplitude index S4. Therefore, for simplification, it is proposed to provide the detailed value for S4 in the simulation assumptions if the latitude is possible within ±20 and moreover the carrier frequency is below 6GHz.

Proposal 7: The value of amplitude index S4 should be given in the simulation assumptions if the latitude is possible within ±20 and moreover the carrier frequency is below 6GHz.

Non-zero handover margin configuration is mandatory in the system-level simulation evaluation for throughput performance. Therefore, it is proposed to use non-zero handover margin (for example 2dB) for the calibration of handover margin function since this function is related to serving beam selection.

Proposal 8: Use non-zero handover margin (for example 2dB) as simulation assumption to calibration handover margin function for serving beam selection.

# 3 Link Budget Analysis

The scenarios with elevation angle of 90 and 10 are the best case and the worst case for link budget analysis respectively. Therefore, beside evaluation angle of 30, it is proposed to consider both 90 and 10 as elevation angle for link budget analysis.

Proposal 9: The values of elevation angles for link budget analysis can be 90, 30 and 10.

Lower BLER target is preferred in NTN, especially for GEO satellite, due to longer propagation delay. Therefore, it is proposed to use 10%, 0.1% and 0.001% as BLER target in link budget analysis of NTN.

Proposal 10: The values of BLER targets for link budget analysis can be 10%, 0.1% and 0.001%.

The link-level channel model in TR 38.811 includes NTN-CDL-A, NTN-CDL-B, NTN-CDL-C, NTN-CDL-D, NTN-TDL-A, NTN-TDL-B, NTN-TDL-C, NTN-TDL-D, where NTN-CDL-A and NTN-CDL-B are constructed for NLOS, NTN-CDL-C and NTN-CDL-D are constructed for LOS, TDL models are filtered from CDL models for simplified evaluation, e.g., for non-MIMO elevation. It is proposed to use one link-level channel model in TR 38.811 for link budget evaluation to minimize the effort. For comparison, AWGN model can be used as baseline.

Proposal 11: Use AWGN model as baseline and select additional one link-level channel model in TR 38.811 for link budget analysis.

The simulation assumptions in Table 3-1, Table 3-2 and Table 3-3 can be considered for link budget analysis in NTN with achievable throughputs as output. The detailed values for satellite parameter configuration 2 can be based on the outcome of agreed simulation assumptions for calibration.

Proposal 12: The simulation assumptions in Table 3-1, Table 3-2 and Table 3-3 can be considered for link budget analysis in NTN with achievable throughputs as output.

Table 3-1 Simulation assumptions for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| Bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Satellite altitude | 600km, 1200km, 35786km |
| SCS | 30kHz (S band), 120kHz (Ka band) |
| Elevation angle | 10, 30, 90 |
| Atmospheric loss | 0.1 dB (S band), 1 dB (Ka band) |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Average C/I within a satellite beam | 16 dB |
| Frequency reuse factor (Note 1) | 4 |
| Terminal type | - Ka band: VSAT, (M, N, P) = (8, 4, 2) - S band: (M, N, P) = (1,1,1), (M, N, P) = (1,1,2), and (M, N, P) = (1, 2, 2) |
| Terminal speed | 0 km/h |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Fast fading channel model | AWGN, [NTN-CDL-D] in TR 38.811 |
| Target BLER | 10%, 0.1%, 0.001% |
| Note 1: Frequency reuse factor of 1 is not precluded from simulation studies. |  |

Table 3-2 Terminal parameters

| Terminal Type | VSAT | (M, N, P) = (1,1,1) | (M, N, P) = (1,1,2) | (M, N, P) = (1, 2, 2) | (M, N, P) = (8, 4, 2) |
| --- | --- | --- | --- | --- | --- |
| Terminal receive antenna element gain (dBi) | 39.7 | 0 | 0 | 0 | 5 |
| Terminal noise figure (dB) | 1.2 | 7 | 7 | 7 | 10 |
| Terminal ambient temperature (K) | 290 | 290 | 290 | 290 | 290 |
| Terminal antenna temperature (K) | 150 | 290 | 290 | 290 | 290 |
| Terminal maximal EIRP (dBW) | 45.75 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal EIRP (dBW) | 44 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal output back off (dB) | 1.75 | 0 | 0 | 0 | 0 |
| Terminal transmit power (dBW) | 3 | -7 | -7 | -7 | -7 |
| Terminal transmit antenna element gain (dBi) | 43.2 | 0 | 0 | 0 | 5 |
| Terminal cable loss (dB) | 0.45 | 0 | 0 | 0 | 0 |

Table 3-3 Satellite parameters

|  |  | Configuration 1 |  |  | Configuration 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 |
| S band | Satellite EIRP density (dBW/MHz) | 59 | 40 | 34 |  |  |  |
|  | Satellite G/T (dB/K) | 19 | 11.2 | 11.2 |  |  |  |
| Ka band | Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |  |  |  |
|  | Satellite G/T (dB/K) | 22 | 13 | 13 |  |  |  |

# 4 Random Access

## 4.1 Requirements

PRACH detection requirement evaluation shall follow the methodology in 8.4 and 11.4 of [15]. The time error tolerance for different SCS shall be the same as provided in Tables 8.4.2.1-1 and 11.4.2.1-1 of [15]. Table 4-1 provides the proposed timing estimation errors for AWGN and NTN TDL-B300-100 and NTN TDL-B30-300. The table does not contain potentially new SCS values and preamble formats for FR1 and FR2.

Table 4-1 Timing estimation error tolerance.

| Preamble Format | SCS [kHz] | Time error tolerance |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | FR1 |  | FR2 |  |
|  |  | AWGN | NTN TDL-B300-100 | AWGN | NTN TDL-B30-300 |
| 0 | 1.25 | 1.04 s | 2.55 s | n.a. | n.a. |
| C2 | 15 | 0.52 s | 2.03 s | n.a. | n.a. |
| C2 | 30 | 0.26 s | 1.77 s | n.a. | n.a. |
| C2 | 60 | n.a. | n.a. | 0.13 s | 0.28 s |
| C2 | 120 | n.a. | n.a. | 0.07 s | 0.22 s |

Table 4-2 summarizes the channel profiles that shall be used in performance requirement tests.

Table 4-2 Channel model parameters for FR1 and FR2.

| Combination Name | Channel Model | Spectrum | Delay Spread | max. excess tap delay (span) | max. Doppler Frequency | Delay resolution |
| --- | --- | --- | --- | --- | --- | --- |
| NTN TDL-B300-100 | NTN TDL-B | FR1 | 300 ns | 2595 ns | +/-100 Hz | 5 ns |
| NTN TDL-B30-300 | NTN TDL-B | FR2 | 30 ns | 290 ns | +/-300 Hz | 5 ns |

The test preambles follow Table A.6-1 and A.6-2 of [15]. Detailed preamble configurations are given in Table 4-3.

Table 4-3 Test preamble formats.

| Preamble Format | SCS [kHz] | N_CS | Logical Sequence Index |
| --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 |
| C2 | 15 | 23 | 0 |
| C2 | 30 | 46 | 0 |
| C2 | 60 | 69 | 0 |
| C2 | 120 | 69 | 0 |

Minimum required SNR values for at least 99% detection probability of are provided in Table 4-4 (FR1) and Table 4-5 (FR2):

Table 4-4 Minimum required SNR for FR1.

|  |  |  |  |  | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number Tx Antenna | Rx Antenna | Propagation Conditions | Frequency Offset [Hz] | Format | SCS = 1.25 kHz | SCS = 15 kHz | SCS = 30 kHz |
| 1 | VSAT (Radius = 0.32m) | AWGN | 0 | 0 | TBD | n.a. | n.a. |
|  |  | NTN TDL-B300-100 | +/- 400 |  | TBD | n.a. | n.a. |
|  |  | AWGN | 0 | C2 | n.a. | TBD | TBD |
|  |  | NTN TDL-B300-100 | +/- 400 |  | n.a. | TBD | TBD |

Table 4-5 Minimum required SNR for FR2.

|  |  |  |  |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number Tx Antenna | Rx Antenna | Propagation Conditions | Frequency Offset [Hz] | Format | SCS = 60 kHz | SCS = 120 kHz |
| 1 | VSAT (Radius = 0.032m) | AWGN | 0 | C2 | TBD | TBD |
|  |  | NTN TDL-B30-300 | +/- 4000 |  | TBD | TBD |

Proposal 13: Discussion on performance requirements for PRACH based on proposed assumptions and methodology.

## 4.2 Simulation Assumptions

Table 4-6 provides the link-level simulation assumptions for PRACH and can be used for PRACH performance evaluation in NTN.

Proposal 14: Use link-level simulation assumptions in Table 4-6 for PRACH performance evaluation in NTN.

Table 4-6 Link-level simulation assumptions for PRACH.

| Parameter | Configuration |  |
| --- | --- | --- |
| Satellite | LEO | GEO |
| Satellite height above earth (informative) | 600 km | 35786 km |
| Scenario | Single cell, single mobile |  |
| Carrier Frequency | 2, 20 GHz |  |
| Channel Model | AWGN, NTN-CDL-A, NTN-CDL-C [10] |  |
| Delay Scaling | 2 GHz: 250 ns 20 GHz: 30 ns (see 7.3.5.2.2 of [3]) |  |
| Channel Angle Spread Scaling | ASDesired = ASModel |  |
| Channel Mean Angle Scaling | Satellite: AOD and ZOD provided by model Mobile: AOA uniformly distributed in range [-180°,+180°] and ZOA provided by model |  |
| Elevation Angle Scaling | Uniformly distributed [80°,90°] [10] (distribution is consistent with mobiles located within area with approx. 200 km beam diameter and satellite 600 km above ground) | Uniformly distributed [89°,90°] [10] (distribution is consistent with mobiles located within area with approx. 1000 km beam diameter and satellite 35786 km above ground) |
| Satellite Antenna Pattern | [4] |  |
| Satellite Antenna Radius | 2 GHz: 0.32 m 20 GHz: 0.032 m | 2 GHz: 4.8 m 20 GHz: 0.48 m |
| Satellite Antenna Orientation | ΩSat,α=0°, ΩSat,β=180°, ΩSat,γ=0° |  |
| Satellite Center Frequency Offset | 0.05 ppm |  |
| Satellite Speed | 7561 m/s | 0 m/s |
| Satellite Movement Direction | Along x-axis |  |
| Satellite preamble detection | Each company should provide details on used algorithm and used accumulation over repetition symbols (coherent or non-coherent combining) |  |
| Mobile Antenna Pattern | 2 GHz + 20 GHz: (1,1,1) omni directional 20 GHz: VSAT |  |
| Mobile Antenna Orientation | omni directional: ΩUE,α uniformly distributed [0,360°], ΩUE,β=0° , ΩUE,γ=0° VSAT: Antenna maximum gain direction points towards strongest cluster |  |
| Mobile Movement Direction | Uniformly distributed [0,360°] |  |
| Mobile Center Frequency Offset | 0.1 ppm |  |
| Mobile Speed | 3, 120, 500, 1000 km/h |  |
| Mobile Power Control | Ideal power control in frequency domain |  |
| Preamble format | Each company provides details on configuration (i.e. format, SCS, N_CS etc). New formats are not precluded. Preferred Format is C2 with =0 and N_CS=0. |  |
| Initial timing offset | Assume precompensation in delay. Initial timing offset is uniformly distributed [0, TOmax], where TOmax is the maximum supported timing offset of the selected preamble format and N_CS parameter. |  |
| False alarm probability with noise only | 0.1 % |  |

## 4.3 Performance Metric

For PRACH detection performance metrics:

Error cases:

- Missed detections: The missed detection probability is the ratio between the total number of undetected preambles and the total number of transmitted preambles. A preamble is considered as not detected if a) the preamble has not been detected at all due to e.g. low SNR or b) the preamble is detected correctly but with absolute timing estimate that is larger than half of the PUSCH CP (e.g. if PUSCH SCS is 15 kHz and CP is 5.2 s, then absolute estimation error should be less than 2.6 s)

- False alarms: Detection of a preamble that has not been transmitted.

Fig. 4-1 Illustration of miss-detection probability

Fig. 4-2 Illustration of false-alarm probability

Fig. 4-3 Illustration of timing estimation error

Mandatory:

Provide minimum required SNR to achieve 1% missed detection.

Plot observed missed detection probability as function of SNR. It is suggested to have an SNR range to cover missed detection from at least 10-4 to around 1 (Fig. 4-1)

Optional:

Plot False alarm probability as function of SNR (Fig. 4-2)

Plot CDF of timing estimation error at minimum required SNR for 0.1 % missed detection rate (Fig. 4-3)

Proposal 15: Performance metrics for PRACH evaluation should include 1) the minimum required SNR to achieve 1% missed detection, and 2) observed missed detection probability as function of SNR with the SNR range to cover missed detection from at least 10-4 to around 1 at least.

Proposal 16: False alarm probability as function of SNR and CDF of timing estimation error at minimum required SNR for 0.1 % missed detection rate can be used as optional performance metrics for PRACH evaluation.

## 4.4 Angle Scaling Procedure Including Subpaths

Step 1: To cluster rays provided in Table 6.9.1-1 to 6.9.1-4 in [10] add cluster subpaths using equation 7.7-0a [11] and considering that cluster-wise rms cZSD=cASD=0:

θn,m,ZOA,model=θn,ZOA,model+cZSA∙αm

θn,m,ZOD,model=θn,ZOD.model+0∙αm

ϕn,m,AOA,model=ϕn,AOA.model+cASA∙αm

ϕn,m,AOD,model=ϕn,AOD,model+0∙αm

Step 2: Determine circular mean (μmodel) and spread (ASmodel) including subpaths using equation (A-1 and A-2 of [11])

Step 3: Scale angles to desired values employing equation 7.7-5 [11], 6.9-1 and 6.9-2 [10]

θn,m,ZOA,scaled=ASZSA,desiredASZSA,modelθn,m,ZOA,model-μZOA,model+μZOA,desired-(αZOA,desired-50°)

θn,m,ZOD,scaled=θn,m,ZOD,model+(αZOD,desired-50°)

ϕn,m,AOA,scaled=ASASA,desiredASASA,modelϕn,m,AOA,model-μAOA,model+μAOA,desired

ϕn,m,AOD,scaled=ASASD,desiredASASD,modelϕn,m,AOD,model-μAOD,model+μAOD,desired

Step 4: Wrap azimuth angles ϕn,m,AOA/AOD,scaled within [0,360°]. Zenith angles  180°<θn,m,ZOA/ZOD,scaled≤360° are set to (360°-θn,m,ZOA/ZOD,scaled)

Proposal 17: Use angle scaling procedure including subpaths in Section 4.4 for performance evaluation of PRACH.

# 5 Throughput

UE throughput (5%, 50%, 95%) and beam throughput (5%, 50%, 95%) can be used as performance metric for throughput evaluation of throughput in NTN. Due to the potential stronger inter-beam interference, the solutions for interference mitigation might need to be used for performance elevation of UE throughout and beam throughput in NTN, for example frequency reuse pattern, dynamic point selection and blanking, and so on.

Proposal 18: UE throughput (5%, 50%, 95%) and beam throughput (5%, 50%, 95%) can be used as performance metric for throughput evaluation in NTN.

Proposal 19: Solutions for interference mitigation can be used for throughput evaluation in NTN.

# 6 Mobility

The radio mobility issues have been discussed in RAN WG#2 and the agreements are captured in [13]. The need for both IDLE mode and ACTIVE mode mobility enhancements have been identified. The main mobility performance requirements are the enhancements for: reduced hand-over failure rates, reduced radio link failure rates, and cell (re-)selection. Service continuity between terrestrial networks (TN) and NTN is also to be addressed in RAN WG#2, and it requires definition of dedicated evaluation scenario(s), such as proposed in [14]. The mobility enhancements are to be studied in RAN WG#2, ideally, using the reference scenarios listed Table 2-1.

Proposal 20: For RAN WG#2 radio mobility performance studies, use the NTN reference scenarios defined in Table 2-1.

# 7 UE Location

The simulation assumptions for calibration can be reuses for performance evaluation of UE positioning with the UE positioning accuracy as performance metric, i.e. UE positioning accuracy (5%, 50%, 80%, 95%) (based on CDF of horizontal positioning accuracy and vertical positioning accuracy in global inertial coordinate system).

Proposal 21: Use horizontal UE positioning accuracy (5%, 50%, 80%, 95%) and vertical UE positioning accuracy (5%, 50%, 80%, 95%) as metric for performance evaluation of UE location.

# 8 Conclusions

In this contribution, we share our views on link-level and system-level performance evaluation for NTN with following proposals.

Observation 1: Satellite constellation should be considered in the calibration for simulation platform in NTN.

Proposal 1: Use inclination satellite constellation and focus on partial geographical area for simulation platform calibration and following system-level evaluation.

Proposal 2: Use Table 2-1 for satellite and UE deployment in calibration.

Proposal 3: For LEO satellite, the neighbor satellites, which are visible by one UE, will be included for this UE’s performance statistics, including serving beam selection and interference statistics.

Proposal 4: For GEO satellite, the neighbor beams, which might leak interference to one UE, will be included for this UE’s performance statistics, including serving beam selection and interference statistics.

Proposal 5: Calculate atmospheric absorption loss for calibration in Ka band but neglect it in S band with 10 as the minimal elevation angle configuration.

Proposal 6: It is recommended to consider clear sky condition as baseline in calibration and neglect the path loss due to rain and cloud attenuation.

Proposal 7: The value of amplitude index S4 should be given in the simulation assumptions if the latitude is possible within ±20 and moreover the carrier frequency is below 6GHz.

Proposal 8: Use non-zero handover margin (for example 2dB) as simulation assumption to calibration handover margin function for serving beam selection.

Proposal 9: The values of elevation angles for link budget analysis can be 90, 30 and 10.

Proposal 10: The values of BLER targets for link budget analysis can be 10%, 0.1% and 0.001%.

Proposal 11: Use AWGN model as baseline and select additional one link-level channel model in TR 38.811 for link budget analysis.

Proposal 12: The simulation assumptions in Table 3-1, Table 3-2 and Table 3-3 can be considered for link budget analysis in NTN with achievable throughputs as output.

Proposal 13: Discussion on performance requirements for PRACH based on proposed assumptions and methodology.

Proposal 14: Use link-level simulation assumptions in Table 4-6 for PRACH performance evaluation in NTN.

Proposal 15: Performance metrics for PRACH evaluation should include 1) the minimum required SNR to achieve 1% missed detection, and 2) observed missed detection probability as function of SNR with the SNR range to cover missed detection from at least 10-4 to around 1 at least.

Proposal 16: False alarm probability as function of SNR and CDF of timing estimation error at minimum required SNR for 0.1 % missed detection rate can be used as optional performance metrics for PRACH evaluation.

Proposal 17: Use angle scaling procedure including subpaths in Section 4.4 for performance evaluation of PRACH.

Proposal 18: UE throughput (5%, 50%, 95%) and beam throughput (5%, 50%, 95%) can be used as performance metric for throughput evaluation in NTN.

Proposal 19: Solutions for interference mitigation can be used for throughput evaluation in NTN.

Proposal 20: For RAN WG#2 radio mobility performance studies, use the NTN reference scenarios defined in Table 2-1.

Proposal 21: Use horizontal UE positioning accuracy (5%, 50%, 80%, 95%) and vertical UE positioning accuracy (5%, 50%, 80%, 95%) as metric for performance evaluation of UE location.

# References

- RP-181370, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#80, La Jolla, USA, June 11 - 14, 2018.

- RP-182444, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#82, Sorrento, Italy, December 10 - 13, 2018.

- RP-190710, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#83, Shenzhen, China, March 18 - 21, 2019.

- R1-1905921, “Report of RAN1#96bis meeting”, 3GPP RAN1#97, Reno, USA, May 13th – 17th, 2019.

- TR 38.811, “Study on New Radio (NR) to support non terrestrial networks”, 3GPP.

- TR 38.821, “Solutions on NR to support non-terrestrial networks”, 3GPP.

- R1-1903995, “Corrections for TR38.811 Chapter 6 Non-Terrestrial Networks channel models”, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Thales, Fraunhofer HHI, Fraunhofer IIS, Keysight, ZTE, ESA, Intelsat, 3GPP RAN1#96bis, Xi’an, China, April 8 – 12, 2019.

- TS 38.213, “NR; Physical layer procedures for control”, “Uplink power control” in Subclause 7, 3GPP.

- TS 36.213, “E-UTRA; physical layer procedures”, “Uplink power control” in Subclause 5, 3GPP.

- R1-1905120, “Link budget analysis in NTN”, Nokia, Nokia Shanghai Bell, RAN1#96bis, Xi’an, China, 8th – 12th April 2019.

- R1-1902621, “NTN link level channel models”, Nokia, Nokia Shanghai Bell, Fraunhofer HHI, Fraunhofer IIS, RAN#89, Athens, Greece, Feb 11 – Mar 14, 2019.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, 3GPP, V15.0.0.

- R2-1902225, “Report of email discussion [104#53] [NR - NTN] - Performance requirements for NTN”, RAN2#104, Athens, Greece, Feb 11 – Mar 14, 2019.

- R2-1900564, “Report of email discussion [104#52] [NR - NTN] - TP on RAN2 agreements”, RAN2#104, Athens, Greece, Feb 11 – Mar 14, 2019.

- R2-1901468, “Mobility aspects between Non-Terrestrial and Terrestrial Networks”, RAN2#104, Athens, Greece, Feb 11 – Mar 14, 2019.

- TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP.

# Annex A: Text Proposal for Requirements and Simulation Assumptions of PRACH in TR 38.821

Start of changes

X.Y Requirements for PRACH

PRACH detection requirement evaluation shall follow the methodology in 8.4 and 11.4 of TS 38.104. The time error tolerance for different SCS shall be the same as provided in Tables 8.4.2.1-1 and 11.4.2.1-1 of TS 38.104. Table x.y.1 provides the proposed timing estimation errors for AWGN and NTN TDL-B300-100 and NTN TDL-B30-300. The table does not contain potentially new SCS values and preamble formats for FR1 and FR2.

Table x.y.1 Timing estimation error tolerance.

| Preamble Format | SCS (kHz) | Time error tolerance |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | FR1 |  | FR2 |  |
|  |  | AWGN | NTN TDL-B300-100 | AWGN | NTN TDL-B30-300 |
| 0 | 1.25 | 1.04 s | 2.55 s | n.a. | n.a. |
| C2 | 15 | 0.52 s | 2.03 s | n.a. | n.a. |
| C2 | 30 | 0.26 s | 1.77 s | n.a. | n.a. |
| C2 | 60 | n.a. | n.a. | 0.13 s | 0.28 s |
| C2 | 120 | n.a. | n.a. | 0.07 s | 0.22 s |

Table x.y.2 summarizes the channel profiles that shall be used in performance requirement tests.

Table x.y.2 Channel model parameters for FR1 and FR2.

| Combination Name | Channel Model | Spectrum | Delay Spread | max. excess tap delay (span) | max. Doppler Frequency | Delay resolution |
| --- | --- | --- | --- | --- | --- | --- |
| NTN TDL-B300-100 | NTN TDL-B | FR1 | 300 ns | 2595 ns | ± 100 Hz | 5 ns |
| NTN TDL-B30-300 | NTN TDL-B | FR2 | 30 ns | 290 ns | ± 300 Hz | 5 ns |

The test preambles follow Table A.6-1 and A.6-2 of TS 38.104. Detailed preamble configurations are given in Table x.y.3.

Table x.y.3 Test preamble formats.

| Preamble Format | SCS [kHz] | N_CS | Logical Sequence Index |
| --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 |
| C2 | 15 | 23 | 0 |
| C2 | 30 | 46 | 0 |
| C2 | 60 | 69 | 0 |
| C2 | 120 | 69 | 0 |

Minimum required SNR values for at least 99% detection probability of are provided in Table x.y.4 (FR1) and Table x.y.5 (FR2):

Table x.y.4 Minimum required SNR for FR1.

|  |  |  |  |  | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number Tx Antenna | Rx Antenna | Propagation Conditions | Frequency Offset (Hz) | Format | SCS = 1.25 kHz | SCS = 15 kHz | SCS = 30 kHz |
| 1 | VSAT (Radius = 0.32m) | AWGN | 0 | 0 | TBD | n.a. | n.a. |
|  |  | NTN TDL-B300-100 | +/- 400 |  | TBD | n.a. | n.a. |
|  |  | AWGN | 0 | C2 | n.a. | TBD | TBD |
|  |  | NTN TDL-B300-100 | +/- 400 |  | n.a. | TBD | TBD |

Table x.y.5 Minimum required SNR for FR2.

|  |  |  |  |  | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number Tx Antenna | Rx Antenna | Propagation Conditions | Frequency Offset (Hz) | Format | SCS = 60 kHz | SCS = 120 kHz |
| 1 | VSAT (Radius = 0.032m) | AWGN | 0 | C2 | TBD | TBD |
|  |  | NTN TDL-B30-300 | +/- 4000 |  | TBD | TBD |

Discussion on performance requirements for PRACH based on proposed assumptions and methodology.

End of changes

Start of changes

X.Y Simulation Assumptions

X.Y.Z PRACH

## The link-level simulation assumptions for PRACH performance evaluation is provided in Table X.Y.Z.

Table X.Y.Z Simulation assumption for PRACH

| Parameter | Configuration |  |
| --- | --- | --- |
| Satellite | LEO | GEO |
| Satellite height above earth (informative) | 600 km | 35786 km |
| Scenario | Single cell, single mobile |  |
| Carrier Frequency | 2, 20 GHz |  |
| Channel Model | AWGN, NTN-CDL-A, NTN-CDL-C [R1-1902621] |  |
| Delay Scaling | 2 GHz: 250 ns 20 GHz: 30 ns (see 7.3.5.2.2 of [TR 38.811]) |  |
| Channel Angle Spread Scaling | ASDesired = ASModel |  |
| Channel Mean Angle Scaling | Satellite: AOD and ZOD provided by model Mobile: AOA uniformly distributed in range [-180°,+180°] and ZOA provided by model |  |
| Elevation Angle Scaling | Uniformly distributed [80°,90°] [R1-1902621] (distribution is consistent with mobiles located within area with approx. 200 km beam diameter and satellite 600 km above ground) | Uniformly distributed [89°,90°] [R1-1902621] (distribution is consistent with mobiles located within area with approx. 1000 km beam diameter and satellite 35786 km above ground) |
| Satellite Antenna Pattern | TR 38.811 |  |
| Satellite Antenna Radius | 2 GHz: 0.32 m 20 GHz: 0.032 m | 2 GHz: 4.8 m 20 GHz: 0.48 m |
| Satellite Antenna Orientation | ΩSat,α=0°, ΩSat,β=180°, ΩSat,γ=0° |  |
| Satellite Center Frequency Offset | 0.05 ppm |  |
| Satellite Speed | 7561 m/s | 0 m/s |
| Satellite Movement Direction | Along x-axis |  |
| Satellite preamble detection | Each company should provide details on used algorithm and used accumulation over repetition symbols (coherent or non-coherent combining) |  |
| Mobile Antenna Pattern | 2 GHz + 20 GHz: (1,1,1) omni directional 20 GHz: VSAT |  |
| Mobile Antenna Orientation | omni directional: ΩUE,α uniformly distributed [0,360°], ΩUE,β=0° , ΩUE,γ=0° VSAT: Antenna maximum gain direction points towards strongest cluster |  |
| Mobile Movement Direction | Uniformly distributed [0,360°] |  |
| Mobile Center Frequency Offset | 0.1 ppm |  |
| Mobile Speed | 3, 120, 500, 1000 km/h |  |
| Mobile Power Control | Ideal power control in frequency domain |  |
| Preamble format | Each company provides details on configuration (i.e. format, SCS, N_CS etc). New formats are not precluded. Preferred Format is C2 with =0 and N_CS=0. |  |
| Intitial timing offset | Assume precompensation in delay. Initial timing offset is uniformly distributed [0, TOmax], where TOmax is the maximum supported timing offset of the selected preamble format and N_CS parameter. |  |
| False alarm probability with noise only | 0.1 % |  |
| Performance metric | - Minimum required SNR to achieve 1% missed detection - Observed missed detection probability as function of SNR (Note 2) - False alarm probability as function of SNR (optional) (Note 3) - CDF of timing estimation error at minimum required SNR for 0.1% missed detection rate (optional) |  |

Note 1: The missed detection probability is the ratio between the total number of undetected preambles and the total number of transmitted preambles. A preamble is considered as not detected if a) the preamble has not been detected at all due to e.g. low SNR or b) the preamble is detected correctly but with absolute timing estimate that is larger than half of the PUSCH CP (e.g. if PUSCH SCS is 15 kHz and CP is 5.2 s, then absolute estimation error should be less than 2.6 s).

Note 2: SNR range should cover missed detection from at least 10-4 to around 1.

Note 3: False alarm is the detection of a preamble that has not been transmitted.

End of changes