3GPP TSG RAN WG1 Meeting #97	R1-1906093

Reno, USA, May 13-17, 2019

Agenda Item:	7.2.6.3

Source:	Ericsson

Title:	PUSCH Enhancements for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN#83 the new WI on physical layer enhancements for NR URLLC was approved [1]. One objective of the WI is specification of PUSCH enhancements for both grant-based PUSCH and configured grant based PUSCH, in particular the following kind of PUSCH scheduling:

o	For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots

This is based on the following agreement and conclusion from RAN1#96 [2].

Agreements: - Capture the descriptions of option 1 to 6 (see R1-1903797 and previous agreements) in the TR. Conclusion: - Finalize the details regarding how to use “option 1” vs. “option 2” during the WI phase using option 4, 5, and 6 (as in R1-1903797) as a starting point.

Option 1 and option 2 above as also known as mini-slot repetition and multi-segment transmission respectively and have been discussed extensively during the study item phase. The descriptions of option 4 and 6 from [3] are given below. These solutions have in common that they try to support both option 1 and option 2.

Option 4: One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. - The number of the repetitions signaled by gNB represents the “nominal” number of repetitions. The actual number of repetitions can be larger than the nominal number. - FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition. - The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols. - FFS the detailed interaction with the procedure of UL/DL direction determination - If a “nominal” repetition goes across the slot boundary or DL/UL switching point, this “nominal” repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. - Handling of the repetitions under some conditions, e.g., when the duration is too small due to splitting, is to be further investigated in the WI phase. - No DMRS sharing across multiple PUSCH repetitions - The maximum TBS size is not increased compared to Rel-15. - FFS: L > 14 - S+L can be larger than 14 - FFS: The bitwidth for TDRA is up to 4 bits. - Note: different repetitions may have the same or different RV. 4 symbols, 4 repetitions Slot boundary 4 symbols, 2 repetitions Slot boundary 14 symbols, 1 repetition Slot boundary Note: this case requires S+L>14. Option 6: One or more PUSCH repetitions in one slot, or two or more PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates an entry in the higher layer configured table - The nnumber of repetitions, starting symbols of each repetition, length of each repetition, and mapping of the repetitions to slots can be obtained from each entry in the table. - More than one repetition can be mapped to one slot - The resource assignment for each repetition is contained within one slot. Each transmitted repetition is contained within one UL period in a slot. - FFS: increasing the number of bits for TDRA field in DCI - FFS other details - The maximum TBS size is not increased compared to Rel-15. Notes on option 6 family: - Proponents of option 6 raised the issue of protecting short PUCCH and SRS, e.g. how to allow the PUSCH to skip the last one or two symbols in a slot to avoid conflict with PUCCH/SRS. - One advantage of option 6 is that for dynamic PUSCH, UE does not need to rely on at least UL/DL configuration (including dynamic indication) and PUCCH/SRS presence to decide on start/length of a repetition. - The issue of interaction with UL/DL configuration may still need to be handled for configured grant PUSCH, which is common for option 4-6. - Concerns raised regarding option 6: - Does it require a lot of TDRA entries depending on the number of repetitions, the starting symbol, TDD slot structure? - For configured grant type 2, does it require different TDRA tables or more entries in the TDRA table for different starting symbols in the TDRA field in activation DCI?

In RAN1#96bis [4] , the following agreements relating to PUSCH were made:

Agreements: - Option 5 is not considered further as part of PUSCH enhancements. Agreements: For option 4, dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH is supported for PUSCH enhancements. The dynamic indication can be enabled or disabled by the gNB. - FFS the exact signaling method - FFS the exact DCI format(s) - FFS the exact mechanism to enable or disable - FFS the DCI activating type 2 configured grant PUSCH Agreements: For option 6, - For dynamic PUSCH - For semi-static DL symbol(s), to down-select - Option 1: it is not expected that the resource allocation has conflict with semi-static DL symbol(s). - Option 2: if the resource allocation has conflict with semi-static DL symbol(s), the repetition is not transmitted. - For dynamically indicated DL symbol(s) (via format 2_0), it is not expected at the UE that the resource allocation has conflict with dynamically indicated DL symbol(s). - Note: this is the same as Rel-15 behavior. - For configured grant PUSCH, - For type 1 configured grant PUSCH, and PUSCH other than the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, - If a repetition conflicts with semi-static DL symbol(s), the repetition is not transmitted. - FFS: If a repetition conflicts with dynamically indicated DL symbol(s) (via format 2_0), the repetition is not transmitted. - FFS For the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, follow the same handling as dynamic PUSCH. Agreements: - For option 6, at least for dynamic grants, it is not expected that one repetition (i.e., one SLIV) spans across slot boundary. Proposals: For option 4, when one nominal repetition is split into multiple repetitions due to segmentation at the slot/UL period boundary, - For front-loaded-only DMRS, DMRS is transmitted at the beginning of each repetition. - FFS the case when additional DMRS is configured for the transmission - FFS whether it is handled differently when there is only one symbol in the repetition Discuss till next meeting (also consider type A vs. type B DM-RS aspects) Agreements: For both option 4 and 6, frequency hopping is supported - FFS details

In this contribution we discuss the above details for option 4 and option 6.

# 2	Discussion

In this section we discuss option 4 and 6  above and see whether they can support both mini-slot repetition and multi-segment transmission. In the summary below we focus on slot boundary issues, but UL/DL switching points can be treated in a similar way.

## 2.1	Option 4

Note that with the restriction of L <= 14, option 4 does not support multi-segment transmission with a total length of all repetitions larger than L = 14 symbols. Consider for example an allocation starting in symbol 4 in a certain slot with a total length of the whole allocation equal to 28 symbols. With multi-segment transmission this would be split into three segments, each in a different slot as illustrated in Figure 1.

Figure 1: Multi-segment transmission with S = 4, total length = 28 symbols spanning three slots.

The only way to reach a total length of 28 symbols is to set the nominal number of repetitions larger than one, but this would result in both nominal repetitions crossing the slot boundary resulting in four total repetitions as illustrated in Figure 2.

Figure 2: Result of option 4 with S = 4, L = 14, and two nominal repetitions.

Based on our understanding of the proponents of mini-slot repetitions, the result in Figure 2 is actually not a desired allocation of repetitions, and this combination of S, L, and number of nominal repetitions would not be scheduled. Instead, if a desired total length of 28, with starting point near the middle of the slot is desired, then either S = 7, L = 14 with two nominal repetitions, or S = 2, L = 2 with 14 nominal repetitions would be chosen.

Option 4 supports mini-slot repetition.

Option 4 does not support multi-segment transmission with the restriction L <= 14. Removing this restriction allows option 4 to support multi-segment transmission.

## 2.2	Option 6

Option 6 allows for explicit signalling of each repetition’s starting point, length, and slot. Hence it can support both multi-segment transmission and mini-slot repetition.

Option 6 supports both multi-segment transmission and mini-slot repetition.

As we show in section 4, option 6 allows for not configuring redundant allocations or allocations that are unlikely to be scheduled. Hence it is unlikely to introduce excessive signalling overhead due to the need of explicitly configuring entries for irregular availability of symbols available for PUSCH transmission, due to for example TDD pattern, SRS, PUCCH.

The signalling overhead of option 6 is not a concern in practice.

# 3	Modified option 4

Based on the above discussion, option 4 seems like a promising candidate for supporting both multi-segment transmission and mini-slot repetition. In this section we propose two different modifications to option 4 to make it better support multi-segment transmission,

## 3.1	Option 4a

The easiest modification to option 4 is to allow L larger than 14.

Modify option 4 to allow L to take values larger than 14.

## 3.2	Option 4b

In this modification of option 4 we keep the restriction that L <= 14, but use the total number of slots spanned by the overall resources for all the nominal repetitions to determine whether to use mini-slot repetition or multi-segment transmission. As above, let K be the nominal number of repetitions signalled by the gNB.

- Let M denote the number of slots spanned by the overall resources. We note that M can easily be calculated as the smallest integer M such that S + L*K <= M*14.

- If M >= K, use multi-segment transmission, otherwise use mini-slot based repetition.

In the figures below we give some examples showing the use of option 4b:

Figure 3: Example of mini-slot repetition with (S=4, L=4, K=2). Here M = 1, and since M < K, mini-slot repetition is used with 2 repetitions.

Figure 4: Example of multi-segment transmission with (S=4, L=14, K=1). Here M = 2, and since M >= K, multi-segment transmission is used with 2 repetitions, one in each slot.

Figure 5: Example of mini-slot repetition with (S=12, L=2, K=4). Here M = 2, and since M < K, mini-slot repetition is used with 4 repetitions.

Figure 6: Example of multi-segment transmission with (S=12, L=8, K=1). Here M = 2, and since M >= K, multi-segment transmission is used with 2 repetitions, one in each slot.

Figure 7: Example of mini-slot repetition with (S=0, L=7, K=2). Here M = 1, and since M < K, mini-slot repetition is used with 2 repetitions.

Recall the examples given above in Figure 1 where it was not possible to schedule using option 4. This is now possible with option 4b as illustrated in Figure 5.

Figure 8: Example of multi-segment transmission with (S=4, L=14, K=2). Here M = 3, and since M >= K, multi-segment transmission is used.

In general, option 4b allows for scheduling of both mini-slot repetitions and multi-segment transmission when the length of the first nominal repetition as signalled by L is smaller than or equal to 7:

Option 4b efficiently enable both mini-slot repetition and multi-segment transmission when the length of the first nominal repetition is smaller than or equal to 7.

Based on these observations we have the following proposal:

Adopt either option 4a or option 4b as described above to support both mini-slot repetition and multi-segment transmission.

# 4 Signalling nominal number of repetitions for option 4

In this section we discuss how to dynamically indicate the nominal number of repetitions in the DCI. We compare the required DCI size when signalling the number of repetitions in an entry in the TDRA table compared to signalling in separate DCI fields. We note that joint signalling of the number of repetitions together with S and L in the TDRA table potentially can save DCI bits compared to separate signalling. We first consider an example with the following TDRA table:

Table 1: Example TDRA table

| Entry | S | L |
| --- | --- | --- |
| 0 | 0 | 7 |
| 1 | 7 | 7 |
| 2 | 0 | 14 |
| 3 | 0 | 4 |

and the following repetition table:

Table 2: Example repetition table

| Entry | Nominal number of repetitions K |
| --- | --- |
| 0 | 1 |
| 1 | 2 |

We see that to separately signal these entries, 2 bits are needed for the TDRA table, and one bit is needed for the repetition table. We now consider signalling these fields in an extended TDRA table as follows:

Table 3: Signalling of S, L, and K in TDRA table. Similar entries are highlighted in the same color.

| Entry | S | L | Nominal number of repetitions K |
| --- | --- | --- | --- |
| 0 | 0 | 7 | 1 |
| 1 | 7 | 7 | 1 |
| 2 | 0 | 14 | 1 |
| 3 | 0 | 4 | 1 |
| 4 | 0 | 7 | 2 |
| 5 | 7 | 7 | 2 |
| 6 | 0 | 14 | 2 |
| 7 | 0 | 4 | 2 |

We see that joint signalling of these parameters requires the same number of bits as separate signalling.

Joint signalling of S, L, and the number of repetitions in the TDRA table does not increase the DCI size.

Another thing that can be noticed is that many of the entries have similar total allocation length therefore are likely to have similar BLER performance. For example, both entry 2 and 4 have a total length of 14 and start in symbol 0. Similarly, entry 7 and 0 both start in symbol 0 and have lengths 7 and 8 respectively. Thus it is reasonable to only configure one of each of these similar entries, thus making the TDRA table smaller. The smaller TDRA table can then potentially decrease signalling overhead compared to separate signalling, if enough redundant entries can be found. Even if enough redundant entries to save bits in DCI signalling cannot be found, these entries can be replaced by other entries signalling more useful combinations that would have needed increased DCI size to signal separately. One example is given below where the replaced entries are highlighted. Note that in order to signal the same combinations with separate signalling, 4 bits are needed, but with joint signalling only 3 bits are needed.

Table 5: Joint signalling of S, L, K with some entries changed.

| Entry | S | L | Nominal number of repetitions K |
| --- | --- | --- | --- |
| 0 | 0 | 7 | 1 |
| 1 | 7 | 7 | 1 |
| 2 | 0 | 14 | 1 |
| 3 | 0 | 4 | 1 |
| 4 | 0 | 14 | 4 |
| 5 | 7 | 7 | 2 |
| 6 | 0 | 14 | 2 |
| 7 | 0 | 14 | 8 |

Even though this example is somewhat small, we can see gains from joint signalling of S, L, and K in the TDRA table. With a larger TDRA table and a larger set of K to signal we expect more entries to be redundant or unlikely to be scheduled in practice, leading to larger gains from joint signalling.

Joint signalling of S, L, and the number of repetitions in the TDRA table can decrease the DCI size compared to separate signalling through removal of redundant entries.

We note that this mechanism is easy to enable or disable through RRC reconfiguration of the TDRA table, and it applies to DCI activating type 2 configured grant without further changes needed. The method is general and applies to any DCI format for which the TDRA table can be reconfigured, including possible new DCI formats.

Joint signalling of the nominal number of repetitions in the TDRA table is easy to enable or disable, supports all DCI formats with reconfigurable TDRA table, and supports type configured grant without any needed changes.

Signal the nominal number of repetitions in the TDRA table to potentially reduce DCI size for option 4 and option 5.

Note that similar arguments also apply to option 6 with explicit signalling of starting point, length, and slot of each repetition. For example, we can describe Table 3 in the following way:

Table 5: Version of Table 3 with explicit indication of start, length and slot of each repetition.

| Entry | (S0, L0, slot) | (S1,L1, slot) |
| --- | --- | --- |
| 0 | (0, 7, n) | - |
| 1 | (7,7, n) | - |
| 2 | (0, 14, n) | - |
| 3 | (0, 4, n) | - |
| 4 | (0,7, n) | (7,7, n) |
| 5 | (7,7, n) | (0,7, n + 1) |
| 6 | (0, 14, n) | (0, 14, n + 1) |
| 7 | (0, 4, n) | (4, 4, n) |

# 5 Handling of symbols unavailable for PUSCH transmission using option 4

One of the advantages of option 6 as compared to option 4 that has been noted is that option 6 allows for dynamic avoidance of symbols used for e.g. SRS or PUCCH by other UEs. For PDSCH the similar problem of how to avoid LTE CRS resources, or CORESETs belonging to other UEs was solved by introducing reserved resources. Sets of reserved resources can be configured, and the gNB can dynamically indicate if the PDSCH is rate-matched around these resources. A similar solution can be used for PUSCH, where sets of OFDM symbols can be configured and a dynamic indication can be used to indicate whether the UE should transmit PUSCH on these symbols or not.

Reserved resources in the UL can be used to avoid PUSCH and SRS collision for option 4.

# 6 Frequency hopping

Both option 4 and 6 support scheduling of shorter PUSCHs repeated multiple times, so called mini-slot repetition, and longer repetitions which only end at slot or period boundaries, so called multi-segment PUSCH. For mini-slot repetition, it is beneficial to support inter-repetition frequency hopping. In this case intra-repetition frequency hopping causes additional DMRS overhead with small diversity gain. An example is shown in Figure 9.

Figure 9: Inter-repetition hopping for mini-slot repetition.

For multi-segment PUSCH, inter-repetition hopping can also be beneficial, e.g. in the case of two shorter repetitions close to the slot or UL period boundary. An example is shown in Figure 10

Figure 10: Inter-repetition hopping for multi-segment PUSCH.

In cases where segments differ a lot in size it might be preferable to use intra-repetition hopping instead of inter-repetition hopping. An example is shown in

Figure 11: Intra-repetition hopping for multi-segment PUSCH.

Support both inter-repetition and intra-repetition hopping for both option 4 and option 6.

We note that Rel. 15 supports both inter-slot frequency hopping, and intra-slot frequency hopping. Thus the rules for inter-repetition frequency hopping can be based on the Rel. 15 rules for inter-slot frequency hopping, while the rules for intra-repetition hopping can be based on the rules for intra-slot frequency hopping.

Base the rules for inter-repetition frequency hopping on the Rel. 15 rules for inter-slot frequency hopping. Base the rules for intra-repetition frequency hopping on the Rel. 15 rules for intra-slot frequency hopping.

# 7 Transport block size determination when repeating mini-slots

When scheduling a PUSCH transmission, the target code rate and modulation order is determined from the MCS index signaled in DCI typically. The transport block size is then calculated from the target code rate, modulation order, number of layers, and the allocated resources as described in Section 6.1.4.2 of TS 38.214.

In the following, the issues of the Rel-15 procedure are analyzed, when mini-slot repetition is applied.

### 7.1 Reduced scheduling flexibility

In the case of slot aggregation of Rel-15, the transport block size is determined using parameters for the first slot, and the same transport block size is then used in each of the aggregated slots. The same approach can be used for mini-slot repetition, where the transport block size is determined by the amount of allocated resources in the first mini-slot, together with a signaled target code rate and modulation order. One drawback of this is that it can affect scheduling flexibility. With repetitions, if a TB needs to be transmitted with low MCS indices, very large bandwidth is required. In some cases, it is not even possible to schedule the given TB using some MCS indices since the required bandwidth would be too large. We illustrate this problem by considering three different ways of transmitting an 8 OS long PUSCH, either

- by 4 repetitions of a 2 OS long PUSCH, or

- by 2 repetitions of a 4 OS long PUSCH, or

- a single 8 symbol long PUSCH.

(a) and (b) represents the case where the TBS is determined based on parameters for the first transmission, then the TB is repeated multiple times. (c) represents the case where the TBS is determined based on the total amount of resource used for the TB.

We consider three different target packet sizes from the evaluation assumptions, 100 bytes, 250 bytes, or 1370 bytes. We assume a BW of 40 MHz and SCS = 30 kHz, giving a maximum number of PRBs equal to 106 for CP-OFDM. We examine all MCS indices in MCS table 5.1.3.1-3 from TS 38.214 (low SE 64QAM table) and find the number of PRBs needed to support the target TBS. Sometimes the same number of PRBs gives close to equal TBS when using adjacent MCS indices. In this case we only select the MCS index with the smallest spectral efficiency, corresponding to the highest reliability.

| Target TBS = 100 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 OS, 4 repetitions |  | 4 OS, 2 repetitions |  | 8 OS, 1 repetition |  |
|  | MCS | NPRB | MCS | NPRB | MCS | NPRB |
| Smallest MCS | 11 | 88 | 6 | 92 | 2 | 95 |
| Largest MCS | 28 | 15 | 28 | 5 | 24 | 3 |

| Target TBS = 250 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 OS, 4 repetitions |  | 4 OS, 2 repetitions |  | 8 OS, 1 repetition |  |
|  | MCS | NPRB | MCS | NPRB | MCS | NPRB |
| Smallest MCS | 17 | 96 | 10 | 90 | 6 | 99 |
| Largest MCS | 28 | 36 | 28 | 12 | 26 | 6 |

| Target TBS = 1370 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 OS, 4 repetitions |  | 4 OS, 2 repetitions |  | 8 OS, 1 repetition |  |
|  | MCS | NPRB | MCS | NPRB | MCS | NPRB |
| Smallest MCS | Not possible |  | 23 | 100 | 15 | 98 |
| Largest MCS | Not possible |  | 28 | 67 | 28 | 29 |

In the examined cases, basing the TBS determination on the number of resources available in the first repetition leads to worse flexibility in the available {MCS, NPRB} combinations for achieving the target TBS, as compared to basing TBS determination on the total number of resources. For example, for all three TB sizes above, option (a) provides less {MCS, NPRB} flexibility than option (c). By flexibility here we mean the range of MCS and NPRB that can be used in the transmission. For instance, in the table on the top, with 8 OS and 1 repetition, it is possible to MCS indices from 2 to 24 and NPRB from 3 to 95, which means that depending on the channel quality, a small or large allocation, as well as a low or high MCS can be used. However, in the first column of the table on the top with 2 OS and 4 repetitions, the range of MCSs is smaller (the lowest MCS index is 11) and also the smallest allocation is 15 PRBs. With large TBS sizes the situation is even worse, and as the last table shows it is not even possible to transmit the TBS in only 2 OS and 4 repetitions case. Thus, basing the TBS on the total amount of resources available (up to the resources of one slot) provides the best flexibility in terms of {MCS, NPRB} combinations.

Another advantage of basing the TBS on the total number of resources available is that it is possible to change the TBS both by changing the number of PRBs allocated as well as the number of OFDM symbols. It is not as easy to change the TBS when using mini-slot repetition and Rel-15 TBS determination procedure, where the number of OS in the first transmission may need to stay fixed to keep the alignment delay low. Changing the number of repetitions only changes the total transmission length, but does not change the number of OS in the first transmission, which is used to determine the TBS in Rel-15.

Note that for very large allocations, larger than 14 symbols in total, the above procedure could lead to a TBS that is larger than the maximum TBS in Rel. 15, potentially affecting existing hardware implementation. This is not the intention here, and the maximum amount of resource elements used in TBS determination is limited by what’s available in one slot for the given number of PRBs. A simple way of achieving this is to account for all allocated resources when determining N’RE, the total number of REs allocated for PUSCH with a PRB. Since the total number of REs allocated for PUSCH in Rel. 15 is given by NRE = min(156, N’RE)×nPRB the largest TBS will not exceed the maximum supported in Rel. 15, which corresponds to NRE=156×nPRB.

Basing the TBS determination on the allocated resources in the first transmission can lead to inflexible scheduling, and poor usage of the MCS table.

In the examined cases, it is not possible to reach the lowest spectral efficiency in the Rel-15 MCS table with 1 repetition even when using the full bandwidth. Thus using more repetitions and basing TBS determination on the allocated resources in the first transmission does not give noticeable gains in spectral efficiency compared to the Rel-15 MCS table.

For multi-segment PUSCH and for mini-slot repetitions, TBS determination is based on the total amount of allocated resources when determining N’RE.

### 6.2 Mismatch in modulation order and base graph

When mini-slot aggregation with K repetition is used, the transmission uses {RMCS,K, QMCS,K, K}, where RMCS,K is the code rate signaled by the MCS index, QMCS,K is the modulation order signaled by the MCS index, K is the number of repetitions.

Alternatively, if assuming the MCS is selected according to the total amount of occupied resources, the TBS would be sent with {RMCS,1, QMCS,1, 1}.

When applying the Rel-15 approach, RMCS,1 could deviate so much from RMCS,K, such that QMCS,1 QMCS,K. When this happens, the link performance will suffer significantly. The reason is that in this case the base graph in this case does not match the new modulation order, since the base graphs are optimized for different code rates. To illustrate this, we compare two cases:

- 4 repetitions of a 2OS PUSCH, and

- 1 repetition of an 8 OS PUSCH.

For (A), the first OFDM symbol is occupied by DMRS, followed by 7 OFDM symbols occupied by PUSCH payload. For (B), DMRS is artificially made to occupy only the first OFDM symbol of the first repetition, so that the same DMRS overhead (=1 os) is used in both cases. A target TBS of 32 or 100 bytes is used. The BLER performance of (A) vs (B) is shown in Figure 6 and 7 below, using the assumed transmission parameters.

Figure 12: Performance degradation when improper modulation order is used in mini-slot repetitions.

Figure 13: Performance degradation when improper modulation order is used in mini-slot repetitions.

We see that (A) with 4 repetitions performs about 1.5 – 1.8 dB worse than (B) with 1 repetition. The reason is, (A) uses {RMCS,4, QMCS,4=64QAM, K=4}, (B) uses {RMCS,1 RMCS,4/16, QMCS,1=QPSK, 1} which is the right choice when taking into account all occupied resources.

In the case with target TBS = 800 bits, a similar problem occurs to the base graph selection as well, since base graph selection is partly determined by the target code rate in the MCS. Since the target code rate is above 0.67, the base graph switches from BG2 to BG1. Since the mother code rate of BG1 is 1/3, and the mother code rate of BG2 is 1/5, this also impacts performance negatively due to circular buffer repetition being used below rate 1/3 for BG1 instead of fresh parity bits.

When (mini-)slot aggregation is used, basing the TBS determination on the allocated resources in the first transmission may lead to excessively high target code rate, resulting in modulation order and base graph mismatch.

In Rel-16 eURLLC, different relevant use cases can be considered with potentially different reliability requirements. In some use cases, a very strict reliability of 1-10-6 is required as for the example mentioned in [1]. It is worth noting that techniques for enhancing reliability can be done at different layers in the protocol stack. Requiring overall transmission reliability of 1-10-6 does not necessarily mean that all the solutions must come from the physical layer. For example, NR supports higher layer reliability enhancement in the form of PDCP duplication. With PDCP duplication, the reliability requirement on the physical layer can be relaxed.

In NR Rel-15, a new CQI table for CQI report corresponding to 10-5 BLER target was introduced. This aims to support URLLC DL transmission with high reliability requirement. Moreover, a new MCS table supporting new MCS entries with low spectral efficiency values was introduced to support very robust PDSCH and PUSCH transmissions. These PHY reliability enhancements made in NR Rel. 15 can be considered sufficient for eURLLC.

# Conclusion

In this contribution, we discussed our view how to enhance PUSCH transmission to meet the URLLC requirements and made the following observations:

Observation 1	Option 4 supports mini-slot repetition.

Observation 2	Option 4 does not support multi-segment transmission with the restriction L <= 14. Removing this restriction allows option 4 to support multi-segment transmission.

Observation 3	Option 6 supports both multi-segment transmission and mini-slot repetition.

Observation 4	The signalling overhead of option 6 is not a concern in practice.

Observation 5	Option 4b efficiently enable both mini-slot repetition and multi-segment transmission when the length of the first nominal repetition is smaller than or equal to 7.

Observation 6	Joint signalling of S, L, and the number of repetitions in the TDRA table does not increase the DCI size.

Observation 7	Joint signalling of S, L, and the number of repetitions in the TDRA table can decrease the DCI size compared to separate signalling through removal of redundant entries.

Observation 8	Joint signalling of the nominal number of repetitions in the TDRA table is easy to enable or disable, supports all DCI formats with reconfigurable TDRA table, and supports type configured grant without any needed changes.

Observation 9	Reserved resources in the UL can be used to avoid PUSCH and SRS collision for option 4.

Observation 10	Basing the TBS determination on the allocated resources in the first transmission can lead to inflexible scheduling, and poor usage of the MCS table.

Observation 11	In the examined cases, it is not possible to reach the lowest spectral efficiency in the Rel-15 MCS table with 1 repetition even when using the full bandwidth. Thus using more repetitions and basing TBS determination on the allocated resources in the first transmission does not give noticeable gains in spectral efficiency compared to the Rel-15 MCS table.

Observation 12	When (mini-)slot aggregation is used, basing the TBS determination on the allocated resources in the first transmission may lead to excessively high target code rate, resulting in modulation order and base graph mismatch.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Modify option 4 to allow L to take values larger than 14.

Proposal 2	Adopt either option 4a or option 4b as described above to support both mini-slot repetition and multi-segment transmission.

Proposal 3	Signal the nominal number of repetitions in the TDRA table to potentially reduce DCI size for option 4 and option 5.

Proposal 4	Support both inter-repetition and intra-repetition hopping for both option 4 and option 6.

Proposal 5	Base the rules for inter-repetition frequency hopping on the Rel. 15 rules for inter-slot frequency hopping. Base the rules for intra-repetition frequency hopping on the Rel. 15 rules for intra-slot frequency hopping.

Proposal 6	For multi-segment PUSCH and for mini-slot repetitions, TBS determination is based on the total amount of allocated resources when determining N’RE.

# References

R1-190726, New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)

3GPP RAN1#96, Chairman’s notes.

R1-1903797	Summary of Friday offline discussion on potential enhancements for PUSCH for NR URLLC (AI 7.2.6.1.3)	Nokia, Nokia Shanghai Bell

3GPP RAN1#96bis, Chairman’s notes.