3GPP TSG RAN WG1 Meeting #97                                  R1-1906115

Reno, USA, May 13th – 17th, 2019

Agenda Item:    7.2.7.2

Source:	ZTE

Title:	NR FR2 spectral efficiency in the Indoor Hotspot eMBB test environment for IMT-2020 self-evaluation

Document for:	Discussion

Introduction

There was an approved Study Item on Self Evaluation towards IMT-2020 submission in the RAN#75 meeting, which mainly focuses on evaluated RAN technologies based on Rel-15 and beyond to satisfy all ITU-R IMT-2020 requirements. Then 3GPP RAN#81 has finished working on the finalization of the updated submission of 3GPP “5G” technology to ITU-R WP5D.

At RAN#77 meeting, the general work plan of self evaluation is approved [1]. Moreover, “[ITU-R AH 01] Calibration for self-evaluation” [2] has been set up for discussion on calibration and collection of calibration results. It is observed that the calibration results are well aligned according to the results collected so far for InH_x, UMa_x and RMa_x in all IMT-2020 defined test environments.

Moreover, in this contribution, some considerations on evaluation details of eMBB usage scenarios will be shown for Frequency Range 2 (FR2).

Indoor Hotspot-eMBB in Frequency Range 2

In this section, the results for Configuration C (70GHz) are presented for Indoor Hotspot eMBB (downlink and uplink). Meanwhile, the ITU-R report said “70 GHz represents frequency ranges of 66 GHz – 86 GHz;” in the section 8.4, so TDD duplex is used in a typical deployment for NR. Furthermore, both candidate bands on ITU-R WRC-1 AI 1.13 also are covered in Configuration C. The detailed overhead assumptions and system level simulation assumptions are presented in Appendix.

It is notable that Configuration B (FR2, Frequency carrier 30GHz)’s evaluated results has been shown contribution (R1-1811064) in RAN1# 94bis.

- Observation 1: Some candidate identified IMT bands in WRC-19 AI 1.13 are covered in Configuration C with Indoor hotspot eMBB test environment.

Table2-1 DL spectral efficiency evaluation in the Indoor Hotspot – eMBB Config C

| Indoor Hotspot – eMBB Configuration B(30GHz) |  | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDD （DDDSU） | ITU Requirement | TDD （DDDSU） | ITU Requirement |
| 256Tx cross-polarized antenna (M,N,P,Mg,Ng) (16,8,2,1,1) 32TXRU (Mp,Np,P,Mg,Ng) (4,4,2,1,1) | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) (2,4,2,1,2) 8RXRU (Mp,Np,P,Mg,Ng) (1,2,2,1,2) | 10.7 | 9 | 0.32 | 0.3 |

Table2-2 UL spectral efficiency evaluation in the Indoor Hotspot – eMBB Config C

| Indoor Hotspot – eMBB Configuration C(70GHz) |  | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDD （DDDSU） | ITU Requirement | TDD （DDDSU） | ITU Requirement |
| 256Rx cross-polarized antenna (M,N,P,Mg,Ng) (16,8,2,1,1) 32TXRU (Mp,Np,P,Mg,Ng) (4,4,2,1,1) | 32Tx cross-polarized antenna (M,N,P,Mg,Ng) (2,4,2,1,2) 8TXRU (Mp,Np,P,Mg,Ng) (1,2,2,1,2) Omni | 7.14 | 6.75 | 0.24 | 0.21 |

It can be seen from the above tables that the IMT-2020 requirement of 0.3 bits/s/Hz for 5th percentile downlink spectral efficiency and 9 bits/s/Hz for downlink average spectral efficiency is fulfilled by NR Indoor Hotspot-eMBB test environment for in FR2. Meanwhile, the IMT-2020 requirement of 0.21 bits/s/Hz for 5th percentile uplink spectral efficiency and 6.75 bits/s/Hz for average uplink spectral efficiency is easily fulfilled.

- Observation 2: IMT-2020 requirements on uplink spectral efficiency can be fulfilled for configuration C of Indoor Hotspot-eMBB test environment in the NR.

Conclusion

In this document, we provide our considerations on evaluated detail towards IMT-2020 submission.

- Observation 1: Some candidate identified IMT bands in WRC-19 AI 1.13 are covered in Configuration C with Indoor hotspot eMBB test environment.

Observation 2: IMT-2020 requirements on uplink spectral efficiency can be fulfilled for configuration C of Indoor Hotspot-eMBB test environment in the NR.

References

[1] RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al.

[2] Report ITU-R M.2410 - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

[3] Report ITU-R M.2412 - Guidelines for evaluation of radio interface technologies for IMT-2020.Report ITU-R M.2412 - Guidelines for evaluation of radio interface technologies for IMT-2020.

- Appendix

Table-A1 system level configuration parameters for DL Indoor Hotspot-eMBB 30GHz

| Parameter | Value |
| --- | --- |
| Test environment | Indoor Hotspot – eMBB |
| Evaluation configuration | Configuration C |
| Channel model | InH_B |
| Subcarrier spacing | TDD：60 kHz |
| TDD frame structure | DDDSU, S(10D,2G,2U) |
| Symbols number per slot | 14 |
| ISD | 20 m |
| Carrier Frequency | 70 GHz |
| System bandwidth | 80MHz |
| Transmit power per TRxP | 21 dBm |
| Mechanic tilt | 180deg in GCS (pointing to the ground) |
| Electronic tilt | 90deg in LCS |
| UT attachment | Based on RSRP (formula) from CRS port 0 |
| Scheduling | MU-PF |
| MIMO mode | MU-MIMO with rank 4 adaptation per user Maximum MU layer = 12 |
| BS receiver type | MMSE-IRC |
| Number of antenna elements per TRxP | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) (8,8,2,1,1) |
| Number of TXRU per TRxP | 32TXRU (Mp,Np,P,Mg,Ng) (4,4,2,1,1) |
| Number of antenna elements per UE | 16Rx cross-polarized antenna (M,N,P,Mg,Ng) (2,4,2,1,2) |
| Number of RXRU per UE | 8RXRU (Mp,Np,P,Mg,Ng) (1,2,2,1,1) |
| UE speeds of interest | Indoor users: 3 km/h |
| BS noise figure | 7 dB |
| UE noise figure | 10 dB |
| RB number | 105 |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC |
| Overhead | 32.14% |
| Precoder derivation | Non-precoded SRS based |
| Beam set at TRxP | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -1*pi/8, 1*pi/8, 3*pi/8] Zenith angle θj = [pi/4 3*pi/4] |
| Beam set at UE | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; |

Table-A2 system level configuration parameters for UL Indoor Hotspot-eMBB 70GHz

| Parameter | Indoor Hotspot – eMBB |
| --- | --- |
| Evaluation configuration | Configuration C |
| Channel model | InH_B |
| Subcarrier spacing | 60kHz |
| Symbols number per slot | 14 |
| Total transmit power per TRxP | Configuration C:21 dBm for 80 MHz bandwidth |
| TDD frame structure | DDDSU, S(10D,2G,2U) |
| RB number | Configuration C:105 |
| UE power class | 21 dBm |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC |
| TRxP number per site | 1 |
| TRxP boresight | - |
| Mechanic tilt | 180 deg in GCS (pointing to the ground) |
| Electronic tilt | 90 deg in LCS |
| Scheduling | PF |
| MIMO mode | SU-MIMO Maximum support 4 layers |
| UL Power control | P_0= -80， Alpha= 0.8 |
| BS receiver type | MMSE-IRC |
| UL CSI derivation | SRS(ideal) based, with delay |
| UL Codebook | NR 4Tx codebook |
| SRS transmission | Non-precoded SRS for codebook based |
| Waveform | OFDM |
| Maximum power reduction (MPR) | Outer RBs: 1dBm Inner RBs: 0dBm |
| Traffic model | Full buffer |
| Beam set at TRxP | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -1*pi/8, 1*pi/8, 3*pi/8] Zenith angle θj = [pi/4 3*pi/4] |
| Beam set at UE | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; |
| Overhead | PUCCH :1 slot with (3 PRB, 14 OS) and 1 slot with (3 PRB, 7 OS)and 2 slots with (1 PRB, 2 OS) SRS:2OS, 10 slots period DMRS :2/4/8/12 ports GP:2 symbols |