3GPP TSG RAN WG1 #97		R1-1906156

Reno, USA, 13th – 17th May, 2019

Source:	vivo

Title:	Evaluation on spectral efficiency in Rural-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our evaluation results on spectral efficiency in Rural-eMBB scenario for IMT-2020 self-evaluation.

- Evaluation on spectral efficiency

According to [1][2], evaluation on spectral efficiency involves 5th percentile user spectral efficiency and average spectral efficiency. The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. While average spectral efficiency is the aggregate throughput of all users divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. The minimum requirements for 5th percentile user spectral efficiency and average spectral efficiency for various test environments are summarized in Table1 and Table2 respectively.

Table1. 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |
| Dense Urban – eMBB | 0.225 | 0.15 |
| Rural – eMBB | 0.12 | 0.045 |

Table2. Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |
| Dense Urban – eMBB | 7.8 | 5.4 |
| Rural – eMBB | 3.3 | 1.6 |

- Evaluation results

In this section, the initial evaluation results on downlink 5th percentile user spectral efficiency and average spectral efficiency are summarized in Table3 and Table4. The detailed evaluation assumptions are listed in Annex A.

Table3. Downlink 5th percentile user spectral efficiency for Rural-eMBB (FDD, channel model B)

| Test environment | Antenna configuration | 5th percentile user spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- |
| Rural–eMBB (config A) | 16Tx/2Rx | 0.237 | 0.12 |
| Rural–eMBB (config B) | 32Tx/4Rx | 0.425 |  |

Table4. Downlink average spectral efficiency for Rural-eMBB (FDD, channel model B)

| Test environment | Antenna configuration | Average spectral efficiency (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Rural–eMBB (config A) | 16Tx/2Rx | 10.202 | 3.3 |
| Rural–eMBB (config B) | 32Tx/4Rx | 15.825 |  |

Observision1:

- For downlink Rural-eMBB scenario, NR can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

The initial evaluation results on uplink 5th percentile user spectral efficiency and average spectral efficiency are summarized in Table5 and Table6. The detailed evaluation assumptions are listed on Annex B.

Table5. Uplink 5th percentile user spectral efficiency for Rural-eMBB (700MHz, FDD, channel model B)

| Test environment | Antenna configuration | 5th percentile user spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- |
| Rural–eMBB (config A) | 1Tx/8Rx | 0.173 | 0.045 |
|  | 2Tx/8Rx | 0.211 |  |
| Rural–eMBB (config B) | 1Tx/32Rx | 0.176 |  |
|  | 4Tx/32Rx | 0.260 |  |

Table6. Uplink average spectral efficiency for Rural-eMBB (700MHz, FDD, channel model B)

| Test environment | Antenna configuration | Average spectral efficiency (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Rural–eMBB (config A) | 1Tx/8Rx | 4.778 | 1.6 |
|  | 2Tx/8Rx | 6.692 |  |
| Rural–eMBB (config B) | 1Tx/32Rx | 4.635 |  |
|  | 4Tx/32Rx | 8.545 |  |

Observision2:

- For uplink Rural-eMBB scenario, NR can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

- Conclusion

In this contribution, we provide our evaluation results on spectral efficiency in Rural-eMBB scenario for IMT-2020 self-evaluation with the following observations:

Observision1:

- For downlink Rural-eMBB scenario, NR can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Observision2:

- For uplink Rural-eMBB scenario, NR can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

- References

- Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, 2017/11

- Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, 2017/10

- Annex A: Downlink evaluation assumptions

Table7.Downlink evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Rural-eMBB Config A (700MHz); Rural-eMBB Config B (4GHz) |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Config A: For 16T: (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;2,4); (dH, dV)=(0.5, 0.8)λ Config B: For 32T: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ |
| MS antenna configurations | Config A: For 2R: (M,N,P,Mg,Ng; Mp,Np) = (1,1,2,1,1; 1,1) Config B: For 4R: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) |
| System bandwidth | 10MHz (52RBs) |
| UE attachment | Based on RSRP (formula as shown in Appendix 3 of RP-180524) from port 0 |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 120km/h for in car |
| Traffic model | Full buffer |
| Receiver | Non-ideal channel estimation and interference modeling MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions, Feedback delay is 2 ms |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Type II codebook,4 beams, WB+SB amplitude quantization, 8 PSK phase quantization Max 2 layers per UE Feedback delay is 2 ms |
| Transmission scheme | MU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput. |
| Overhead | PDCCH 2 symbol, TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |

- Annex B: Uplink evaluation assumptions

Table8.Uplink evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Rural-eMBB Config A (700MHz); Rural-eMBB Config B (4GHz) |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Config A: For 8R: (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1; 1,4) (dH, dV)=(0.5, 0.8)λ Config B: For 32R: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1; 2,8) (dH, dV)=(0.5, 0.8)λ |
| MS antenna configurations | Config A: For 1T: (M,N,P,Mg,Ng; Mp,Np)= (1,1,1,1,1; 1,1); For 2T: (M,N,P,Mg,Ng; Mp,Np)= (1,1,2,1,1; 1,2) (dH, dV)=( N/A, N/A)λ Config B: For 1T: (M, N, P, Mg, Ng; Mp, Np) = (1,1,1,1,1; 1,1) (dH, dV)=( N/A, N/A)λ; For 4T: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) dH, dV)=( 0.5, N/A)λ; |
| System bandwidth | 10MHz (52RBs) |
| UE attachment | Based on RSRP (formula as shown in Appendix 3 of RP-180524) from port 0 |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 120km/h for in car |
| Traffic model | Full buffer |
| Receiver | Non-ideal channel estimation and interference modeling MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions, Feedback delay is 2 ms |
| Feedback | Non-codebook For 1Tx:Up to 1 layers For 2Tx and 4Tx:Up to 2 layers Feedback delay is 2 ms |
| Transmission scheme | UL SU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput. |
| Overhead | 2 PRBs and 14 OS for the slots, DMRS Type 2, SRS 2 symbol per 5ms |