3GPP TSG RAN WG1 #97		                                                      R1-1906157

Reno, USA, May 13th – 17th, 2019

Source:	vivo

Title:	Evaluation on mobility in Rural-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our mobility evaluation results in Rural-eMBB scenario for IMT-2020 self-evaluation. The CDF of UL pre-processing SINR and UL spectral efficiency results are provided based on [1][2].

- Evaluation methodology

The mobility requirements of IMT-2020 [1] on spectral efficiency in Rural-eMBB scenario are listed in Table 1. The evaluation methodology in [2] involves first generating median (50th percentile) uplink SINR from system-level simulations. Then, link-level simulations are used to verify that the required normalized traffic channel link data rates can be reached at the median SINR values.

Table 1. Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link date rate (Bit/s/Hz) | Mobility(km/h) |
| --- | --- | --- |
| Rural-eMBB | 0.8 | 120 |

- Evaluation results

  - Pre-processing SINR CDF and median SINR

Based on the system level simulation assumptions in Appendix I, the preliminary UL pre-processing SINR results for Rural-eMBB with 700MHz and 4GHz carrier frequency are shown in Figure 1. The median pre-processing SINR value is given in Table 2.

Figure 1: UL pre-processing SINR distribution for Rural-eMBB with channel model B

Table 2. UL median pre-processing SINR

| Test environment | Configuration | Channel Model | 50%-tile UL pre-processing SINR(dB) |
| --- | --- | --- | --- |
| Rural-eMBB | Config A | Model B | 11.28 |
| Rural-eMBB | Config B | Model B | 5.36 |

  - UL spectral efficiency

Based on the pre-processing SINR provided in the Table 2 and link level simulation assumptions in Appendix II, the preliminary UL spectral efficiency evaluation results for mobility in Rural-eMBB scenario are list in Table 3.

Table 3. Mobility traffic channel link data rates

| Test environment | Configuration | Channel Model | Median UL SINR[dB] | ITU Requirement [bps/Hz] | Spectrum efficiency of NLOS | Spectrum efficiency of LOS | Fulfil the requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD |  |  |
| Rural-eMBB | Config A 700MHz | Model B | 11.28 | 0.8 | 2.75 | 3.46 | Y |
| Rural-eMBB | Config B 4GHz | Model B | 5.36 | 0.8 | 1.92 | 2.30 | Y |

Observision1:

- In Rural-eMBB scenario, NR can achieve the minimum requirements of mobility traffic channel link data rates of IMT-2020.

- Conclusion

In this contribution, we provide our evaluation results on mobility traffic channel link data rates in Rural-eMBB scenario for IMT-2020 self-evaluation with the following observations:

Observision1:

- In Rural-eMBB scenario, NR can achieve the minimum requirements of mobility traffic channel link data rates of IMT-2020.

- References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- Appendix I:  Simulation assumption of SLS part for mobility

The simulation assumption of system level part for mobility evaluation is listed in Table A1-1.

Table A1-1. Simulation assumptions in SLS

| Parameters | Value |
| --- | --- |
| Test environment | Rural-eMBB |
| Evaluation configuration | Configuration A/ Configuration B |
| Carrier frequency for evaluation | 700MHz/ 4GHz |
| Multiple access | OFDMA |
| Duplexing | FDD |
| Transmission scheme | SU-MIMO with rank 1 |
| Inter-site distance | 1732m |
| BS antenna height | 35 m |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| Number of antenna elements per TRxP | 32 Tx/Rx, (M,N,P,Mg,Ng) = (8,2,2,1,1), (dH,dV) = (0.5, 0.8)λ |
| Number of TXRU per TRxP | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) |
| Number of UE antenna elements | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) |
| Device deployment | 50% indoor, 50% outdoor (in-car), Randomly and uniformly distributed over the area |
| UE speeds of interest | Indoor users: 3 km/h Outdoor users (in-car): 120 km/h |
| Traffic model | Full buffer |
| Simulation bandwidth | 10 MHz |
| UE density | 10 UEs per TRxP |
| Channel model variant | Channel model B |
| UE antenna height | 1.5m |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to the horizontal direction) |
| Electronic tilt | 100° in LCS |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Polarized antenna model | Model-2 in TR36.873 |
| Power control parameters | For config. A: a = 0.8, P0 = -76 dBm For config. B: a = 0.6, P0 = -76 dBm |
| Numerology | 15 kHz SCS |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 |

- Appendix Ⅱ: Additional LLS assumption for Rural-eMBB

Table A1-2. The additional link level simulation assumption for Rural-eMBB

| Parameters | Value |
| --- | --- |
| Carrier frequency | 700MHz/ 4GHz |
| Waveform | CP-OFDM |
| Duplexing | FDD |
| Simulation bandwidth | 10 MHz |
| Link-level Channel model | CDL-iii |
|  | CDL-v |
| UE speed | 120 km/h |
| Subcarrier spacing | 15 kHz (700 MHz)/30 kHz (4 GHz) |
| Symbols number per slot | 14 |
| Antenna configuration at TRxP | 4R, (8,2,2,1,1;1,2) |
| Antenna configuration at UE | 1T, (1,1,1,1,1;1,1) |
| Transmission rank | 1 |
| TRxP receiver type | MMSE-IRC |
| Channel estimation | Real |
| Number of subcarriers per PRB | 12 |
| Data allocation | 14 symbol slots, with 12 RB allocated |
| Channel coding scheme | LDPC |
| Link adaptation | Yes |
| HARQ | Max 4 HARQ transmissions |
| DMRS configuration | 2 symbol DMRS(front loaded and one additional) |
| Other overhead | SRS: 2 symbols per 5 slots PUCCH:2 RBs in 10 MHz bandwidth |