3GPP TSG RAN WG1 #97		R1-1906166

Reno, USA, May 13th – 17th, 2019

Source:	vivo

Title:	Performance evaluation and observations for multi-DCI-based multi-TRP transmission

Agenda Item:	7.2.8.5

Document for:	Discussion and Decision

- Introductions

In RAN1 #94bis [1], the following agreements were achieved on performance evaluation for eMBB.

Agreement

For multi-TRP/panel performance evaluation:

  - For eMBB in FR1, 10MHz BW and 15kHz SCS are baseline.

  - For eMBB in FR1, 20MHz BW and 30kHz SCS are optional.

  - For eMBB in FR2, 80MHz BW and 120kHz SCS are baseline.

Agreement

For eMBB multi-TRP/panel performance evaluation, FTP traffic model 1 with packet size 0.5Mbytes as a baseline, and other traffic model is not precluded. RU=20/40/60% are baseline, and optional low RU (e.g. 5/10) can be considered.

Agreement

For eMBB multi-TRP/panel performance evaluation, MMSE IRC is the baseline, and advanced receiver is not precluded. Practical channel estimation and feedback model are used.

Agreement

- For eMBB multi-TRP performance evaluation, ideal and non-ideal backhaul are considered, the following delay values are assumed:

  - Ideal backhaul: 0ms

  - Non-ideal backhaul: 2ms, 5ms, 50ms(optional)

- Companies to provide the delay values used in their evaluations

Agreement:  SLS assumption for eMBB multi-TRP/panel enhancement

| Parameters | Dense urban (Macro Only) | Indoor hotspot |
| --- | --- | --- |
| Carrier frequency | 2GHz/4GHz is baseline (each company to choose 1 or more) 30GHz is optional | 4GHz is baseline, 30GHz is optional |
| Channel model | TR38.901 |  |
| TP antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) 16 ports: (M, N, P, Mg, Ng Mp, Np) = (8,4,2,1,1,2,4) (dH, dV) = (0.5, 0.8)λ for FR1 2 ports (8,8,2,1,1) and 8 ports (4,8,2,2,2) for 30GHz Other antenna configurations is not precluded (such as 32 ports) | 2 ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) for 4GHz 2 ports: (4,4,2,1,1) for 30GHz Other antenna configurations is not precluded. |
| UE antenna configuration | 4Rx Port: (Baseline) (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for FR1 For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 | 4Rx Port: (Baseline) (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for 4GHz For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 |
| Coordination assumptions | Each company to provide details on cluster size, coordination scheme, etc |  |

Agreement

Baseline scheme to evaluate eMBB multi-TRP enhancements is DPS or single TRP

- Each company to provide the details on backhaul delay, CSI reporting, transmission scheme, scheduling, etc.

In RAN1 #96 meeting [2], following agreements were achieved with some restrictions for FFS.

Agreement

For a UE supporting multiple-PDCCH based multi-TRP/panel transmission and each PDCCH schedules one PDSCH, at least for eMBB with non-ideal backhaul, support following restrictions:

- The UE may be scheduled with fully/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs with following restrictions:

  - The UE is not expected to assume different DMRS configuration with respect to actual number of front loaded DMRS symbol(s), the actual number of additional DMRS, the actual DMRS symbol location and DMRS configuration type if the UE may be scheduled with full/partially overlapping PDSCHs by multiple PDCCHs.

  - The UE is not expected to have more than one TCI index with DMRS ports within the same CDM group for fully/partially overlapped PDSCHs

  - Full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH.

  - The UE is expected to be scheduled with the same active BWP bandwidth and the same SCS if the UE is expected to receive multiple PDSCHs simultaneously at given symbols.

  - The number of active BWPs for a UE is 1 per CC

  - FFS: PDSCH mapping type from two co-scheduled PDSCHs

  - FFS: Alignment of PRG-level grid from multiple TRPs

  - FFS: How to ensure the same active BWP between multiple TRPs

  - Note that rate matching mechanisms (if need) to support multi-DCI based NCJT will be discussed separately.

In this paper we provide system-level simulation (SLS) results to compare multi-DCI-based multi-TRP transmission schemes, DPS and DPS+NCJT, with non-ideal backhaul.

- SLS evaluation

2.1 Simulation methodology

We conduct performance evaluation for eMBB in FR1 with 10MHz BW and 15kHz SCS, 4GHz carrier frequency. Multi-TRP transmission with non-ideal backhaul is evaluated such that independent scheduling is assumed in each TRP per cluster. DPS is the baseline. SU-MIMO is applied for both DPS and DPS+NC-JT cases.

For Indoor Hotspot scenario of multi-TRP transmission, four neighboring TRPs forming a cluster as shown in Figure 1. A UE with its associated TRP in the cluster measures the RSRP of all TRPs in the cluster and selects at most one candidate coordinating TRP, with the RSRP gap lower than a predefined threshold compared to the serving TRP.

Figure 1: TRP clustering for Indoor Hotspot

- DPS

The UE satisfying the threshold compares the estimated throughput among the two possible DPS transmissions by its associated TRP and coordinating TRP. And then, UE reports the CSI with maximal estimated throughput to its recommended transmitting TRP. As the number of Tx antenna ports per TRP is 2, rank 1 or 2 is reported for either TRP.

The scheduler per TRP in the cluster schedules one UE according to proportional fair algorithm without semi-statically configured resource allocation. With one optimal DPS CSI feedback, non-overlapping in time domain is achieved.

- DPS+NC-JT

The UE satisfying the threshold compares the estimated throughput among the two possible DPS transmissions, and one NC-JT from the two TRPs. And then, UE reports the CSI with maximal estimated throughput to its recommended transmitting TRP, i.e., one TRP for DPS transmission or both TRPs for NC-JT. If UE reports a NC-JT CSI, rank 1 or 2 is chosen per TRP to maximize the NC-JT estimated overall throughput.

The scheduler per TRP in the cluster schedules one UE according to proportional fair algorithm without semi-statically configured resource allocation. According to the non-ideal backhaul assumption, the scheduler of a TRP is not aware of the scheduling results of another TRP at the same time. With this dynamic independent scheduling, partially-overlapped PDSCHs can be achieved. One codeword per TRP is transmitted if the UE is scheduled as NC-JT.

For a UE receiving data from two TRPs in NC-JT manner, its equivalent channel is given by

He=H1, H2W100W2=H1W1, H2W2

where H1, H2 are channels from the two TRPs, and W1, W2 are the precoders of the two TRPs. Then the post-SINR per layer can be calculated assuming MIMO detection of the equivalent channel He.

As a results, if two TRPs happen to schedule same UE in one subband simultaneously, the transmission layers from two TRPs to the UE can be one out of (1, 1), (1, 2), (2, 1), and (2, 2) with total transmission layers being 2, 3, or 4 as the UE has 4 Rx antenna ports.

Other simulation assumptions can be found in the Appendix.

2.2 Performance evaluation results and discussion

We provide UPT comparison for FTP model 1 with RU for baseline DPS set to 4% and 9% for Indoor Hotspot. We set the same packet arrival rate (λ) for DPS+NCJT as that of DPS. The performance comparison are shown in Table 1 and Table 2. Considerable UPT gain can be observed at 5% and 95% UPT, and mean UPT as well. For Indoor Hotspot, which is not the typical interference-limited scenario as the interference level is very low from outer cell, not only TRP-edge UEs can take advantage of NCJT but also good UEs achieve almost a half higher throughput in low RU cases. The reason for the gain of good UEs with NJCT is that up to 4 transmission layers from two TRPs can be scheduled in NCJT while transmission layers are restricted to 1 or 2 for DPS.

Table 1: DPS+NC-JT vs. DPS for Indoor Hotspot with non-ideal backhaul (same λ, RU for DPS ≈ 4%)

|  | Mean UPT | 5% UPT | 50% UPT | 95% UPT |
| --- | --- | --- | --- | --- |
| DPS | 0.0% | 0.0% | 0.0% | 0.0% |
| DPS+NC-JT | 22.7% | 36.4% | 0.0% | 47.7% |

Table2: DPS+NC-JT vs. DPS for Indoor Hotspot with non-ideal backhaul (same λ, RU for DPS ≈ 9%)

|  | Mean UPT | 5% UPT | 50% UPT | 95% UPT |
| --- | --- | --- | --- | --- |
| DPS | 0.0% | 0.0% | 0.0% | 0.0% |
| DPS+NC-JT | 21.3% | 26.7% | 8.9% | 47.7% |

Observation: Considerable gain of DPS+NCJT can be observed at 5% and 95% UPT, and mean UPT as well with low packet arrival rate compared to DPS.

- Conclusion

In this contribution, we have the following observations according to our evaluation results for multi-TRP transmission with non-ideal backhaul.

Observation: Considerable gain of DPS+NCJT can be observed at 5% and 95% UPT, and mean UPT as well with low packet arrival rate compared to DPS alone.

References

- RAN1 #94bis Chairman’s Notes.

- RAN1 #96 Chairman's Notes.

Appendix

Table 3: SLS assumption for eMBB multi-TRP enhancement

| Parameters |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Indoor hotspot (InH) |
| Frequency Range |  | FR1 only, 4GHz |
| Inter-BS distance |  | 20m for InH |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at TRP |  | 2 Tx ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) for InH |
| Antenna setup and port layouts at UE |  | 4Rx Port: (M, N, P, Mg, Ng, Mp,Np) = (1,2,2,1,1,1,2) (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 23dBm for InH |
| BS antenna height |  | 3m for InH |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| Configuration for multi-TRP | Cluster | 4 neighboring TRPs for InH |
|  | Maximal number of coordinating TRPs | 2 |
|  | Backhaul assumption | Ideal |
| Transmission scheme |  | Baseline: DPS, transmission layer 1 or 2 per TRP Multi-TRP: dynamic switching between DPS and NCJT, transmission layer 1 or 2 per TRP |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms - Subband PMI, subband CQI - Type I codebook - Rank 1 or rank 2 per TRP - For DPS: DPS CSI of recommended TRP; for DPS+NC-JT: CSI of recommended transmission scheme: DPS or NCJT |
| Traffic model |  | FTP model 1 |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Ideal |