3GPP TSG RAN WG1 Meeting #97			R1-1906222

Reno, USA, May 13th – 17th, 2019

Source:	NTT DOCOMO, INC.

Title:	Updated evaluation results of rural-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

At the RAN1#94bis meeting, we present the TDD evaluation results of Rural-eMBB Config A/B/C under ideal channel estimation, including the DL/UL average spectral efficiency and the DL/UL 5th percentile user spectral efficiency. In this contribution, we present the updated evaluation results under non-ideal channel estimation.

- ITU Requirements

The minimum requirement for 5th percentile user spectral efficiency and average spectral efficiency for Rural-eMBB sceanrio is provided as follow [2].

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Rural – eMBB | 0.12 | 0.045 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Rural – eMBB | 3.3 | 1.6 |

- DL Performance

In this section, we provide the downlink results of Rural-eMBB Config A/B/C under non-ideal channel estimation. The detailed evaluation assumptions can be found in Annex.

The evaluation results of DL average spectral efficiency and DL 5th percentile spectral efficiency are summarized in Table 3.

TABLE 3

Average spectral efficiency and 5th percentile user spectral efficiency for Rural – eMBB Config A/B/C

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config A | 8T (8,4,2,1,1;1,4) 2R (1,1,2,1,1;1,1) | DSUUD | 5.024 | 3.3 | 0.145 | 0.12 |
|  |  | DDDSU | 4.880 | 3.3 | 0.137 | 0.12 |
| Rural – eMBB Config B | 16T (8,8,2,1,1;1,8) 4R (1,2,2,1,1;1,2) | DSUUD | 11.241 | 3.3 | 0.244 | 0.12 |
|  |  | DDDSU | 11.081 | 3.3 | 0.256 | 0.12 |
| Rural – eMBB Config C | 8T (8,4,2,1,1;1,4) 4R (2,2,2,1,1;1,2) | DSUUD | 9.180 | 3.3 | 0.234 | 0.12 |
|  |  | DDDSU | 9.204 | 3.3 | 0.237 | 0.12 |

Based on above evaluation results, we have following observation.

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Rural-eMBB configuration A, B and C.

- UL Performance

In this section, we provide the uplink results of Rural-eMBB Config A/B/C under non-ideal channel estimation. The detailed evaluation assumptions can be found in Annex.

The evaluation results of UL average spectral efficiency and UL 5th percentile spectral efficiency are summarized in Table 4.

TABLE 4

Average spectral efficiency and 5th percentile user spectral efficiency for Rural – eMBB Config A/B/C

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config A | 8R (8,4,2,1,1;1,4) 2T (1,1,2,1,1;1,1) | DSUUD | 5.331 | 1.6 | 0.142 | 0.045 |
|  |  | DDDSU | 4.532 | 1.6 | 0.131 | 0.045 |
| Rural – eMBB Config B | 16R (8,8,2,1,1;1,8) 4T (1,2,2,1,1;1,2) | DSUUD | 6.266 | 1.6 | 0.084 | 0.045 |
|  |  | DDDSU | 5.245 | 1.6 | 0.078 | 0.045 |
| Rural – eMBB Config C | 8R (8,4,2,1,1;1,4) 4T (2,2,2,1,1;1,2) | DSUUD | 4.556 | 1.6 | 0.087 | 0.045 |
|  |  | DDDSU | 3.791 | 1.6 | 0.077 | 0.045 |

Based on above evaluation results, we have following observation.

Observation 2: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Rural-eMBB configuration A, B and C.

- Summary

In this contribution, we provide the downlink and uplink performance on average spectral efficiency and 5th percentile user spectral efficiency of Rural-eMBB under non-ideal channel estimation. The observations are summarized as follow:

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Rural-eMBB configuration A, B and C.

Observation 2: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Rural-eMBB configuration A,B and C.

References

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, Feburary 2018.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, June 2017.

Annex: Evaluation assumptions

| Rural - eMBB | Config A | Config B |  | Config C |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 4 GHz |  | 700 MHz |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth |  |  |  |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) |  |  |  |
| Inter-site distance | 1732 m | 1732 m |  | 6000 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |  | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) |  | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°,90° polarization | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |  | 8Tx, (M,N,P,Mg,Ng) = (2,2,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°,90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) | 4TXRU (1-to-1 mapping) |  | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |  | 40% indoor, 40% outdoor (pedestrian), 20% outdoor (in-car) Randomly and uniformly distributed over the area |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |  | Indoor users: 3 km/h; Outdoor users (pedestrian): 3 km/h; Outdoor users (in-car): 30 km/h |
| Traffic model | Full buffer |  |  |  |
| UE density | 10 UEs per TRxP |  |  |  |
| UE antenna height | 1.5 m |  |  |  |
| Electronic tilt | 99° in LCS | 99° in LCS |  | 96° in LCS |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |  |  |  |
| Wrapping around method | Geographical distance based wrapping |  |  |  |
| Polarized antenna model | Model-2 in TR36.873 |  |  |  |
| scheduler | DL: MU PF (Max 12 layer) UL: SU PF |  |  |  |
| Subband number | 20 |  |  |  |
| Downlink overhead | SS/PBCH block |  | 1 SS/PBCH block per 20ms |  |
|  | PDCCH |  | 2 OFDM symbols per slot |  |
|  | DMRS (Type II) |  | 6, 12 ports |  |
|  | CSI-RS |  | 4, 8, 16, 24, 32 ports with periodicity of 5ms |  |
|  | TRS |  | 4 CSI-RS resource in 2 consecutive slots per 20ms, 50PRB |  |
|  | PT-RS |  | N.A. |  |
| Uplink overhead | PUCCH |  | 14 OS, 2PRB |  |
|  | DMRS (expect 2RBs for PUCCH) |  | 6, 12 ports |  |
|  | SRS(expect 2RBs for PUCCH) |  | 2 OFDM symbols per 5 slots |  |
|  | DL feedback |  | N.A. |  |
|  | PT-RS |  | N.A. |  |
| UE Receiver | MMSE-IRC |  |  |  |
| CSI feedback | periodically 5 slot feedback with 4 slot delay |  |  |  |
| MCS table | Non-ideal channel estimation based NR table |  |  |  |
| Uplink scheduler | SU PF |  |  |  |
| BS Receiver | MMSE_IRC |  |  |  |
| Uplink Power Control | Alpha = 0.6, P0 = -60dBm |  |  |  |
| MCS selection | MCS selection based on SE |  |  |  |