3GPP TSG RAN WG1 #97	R1-1906224

Reno, USA, May 13th – 17th, 2019

Source:	NTT DOCOMO, INC

Title:	Enhancements on multi-TRP/panel transmission

Agenda Item:	7.2.8.2

Document for: 	Discussion and Decision

Introduction

In the previous RAN1 meetings, multi-TRP/panel enhancements for eMBB and URLLC were discussed and following agreements were made [1-2].

Agreements made in RAN1#96bis meeting:

Agreement At least for eMBB with multi-DCI based multi-TRP/panel transmission, different PDSCH scrambling sequences can be supported for PDSCHs, and selection one from the following alternatives in RAN1#97: - Alt 1: enhance c_init, FFS detailed design in RAN1 97 - Alt 2: enhance RRC configurations to support multiple dataScramblingIdentityPDSCH Agreement For PDCCH monitoring and blind decoding for multi-DCI based multi-TRP/panel transmission, - Increase the maximal number of CORESETs per “PDCCH-config” up to N=[4, 5, or 6] subject to UE capability - Increase the maximal number of BD/CCE per slot per serving cell, subject to UE capability Agreement For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used - Support TDMed PUCCH transmission within a slot to convey, at least separate ACK/NACK only feedback, with separated HARQ-ACK codebook for two TRPs - FFS: Details on how this feature is supported in the specifications (for examples, introduction of restrictions and/or further enhancements) Above applies at least for FR1 Agreement For TDMed PUCCH transmission within a slot for separate ACK/NACK, study following alternatives for PUCCH resource configurations: - Alt 1: PUCCH resource groups can be explicitly configured by the NW. - All PUCCH resources configured within the first PUCCH resource group do not overlap in time with any PUCCH resources configured within the second PUCCH resource group, considering - how to support PUCCH resource groups composed with resources or resource sets - Alt 2: PUCCH resources can be configured by the NW to ensure TDM PUCCH resources among M-TRPs - PUCCH resource groups are not needed. - Alt 3: PUCCH resources configured by the NW may be overlapped among M-TRPs. Agreement Take into account following principles for single-PDCCH multi-TRP DMRS port indication: - Whether/how MU pairing cases between, e.g. UE1 from TRP1 and TRP 2 and UE 2 from TRP 1 and TRP 2, or UE1 from TRP1 and TRP 2 and UE 2 from TRP 1, is needed - Whether/how DMRS port indication using DMRS type 1 with 1 or 2 frontloaded symbols, and DMRS type 2 with 1 or 2 frontloaded symbols need to be enhanced Conclusion No consensus in RAN1 on the support enhancing codeword layer mapping, by which transmission layers from each TRP can be mapped to a separate codeword when the total number of layers is ≤4. Agreement For multi-TRP based URLLC, scheduled by single DCI, support scheme 3 and 4 agreed in email discussion [96-NR-09] - FFS any restrictions/modification of supporting scheme 3/4 for FR2 - For example, considering the number of beam switches within the slot, and the delay from scheduling DCI indicating beam switch to scheduled PDSCH - Note how to address M-TRP/panel based URLLC operation in FR2 can be discussed from RAN1 #98 Agreement For multi-TRP based URLLC, scheduled by single DCI, - Support scheme 1a as agreed in email discussion [96-NR-09] - FFS: Whether additional specification impact is necessary for URLLC - On the support of schemes 2a, 2b - Select one of the following: support 2a only, support 2b only, support both 2a and 2b, support none - To facilitate further comparisons among 2a, 2b and baseline to understand technical benefits and use cases, consider both SLS and LLS simulation results - Specification impact, and UE complexity need to be considered as well. - Companies are encouraged to provide simulation results for LLS using at least the following parameters - Pathloss delta between two TRPs: 0dB, 3dB, 6dB - Details on blockage to be provided by each company if any (for example, the probability that one out of 2 links is blocked is 5% or 10% with 10dB blockage loss for the blocked link)

Agreements made in RAN1#96 meeting:

Agreement For multi-DCI based multi-TRP/panel transmission, the total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to 2. Agreement For a UE supporting multiple-PDCCH based multi-TRP/panel transmission and each PDCCH schedules one PDSCH, at least for eMBB with non-ideal backhaul, support following restrictions: - The UE may be scheduled with fully/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs with following restrictions: - The UE is not expected to assume different DMRS configuration with respect to actual number of front loaded DMRS symbol(s), the actual number of additional DMRS, the actual DMRS symbol location and DMRS configuration type if the UE may be scheduled with full/partially overlapping PDSCHs by multiple PDCCHs. - The UE is not expected to have more than one TCI index with DMRS ports within the same CDM group for fully/partially overlapped PDSCHs - Full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH. - The UE is expected to be scheduled with the same active BWP bandwidth and the same SCS if the UE is expected to receive multiple PDSCHs simultaneously at given symbols. - The number of active BWPs for a UE is 1 per CC - FFS: PDSCH mapping type from two co-scheduled PDSCHs - FFS: Alignment of PRG-level grid from multiple TRPs - FFS: How to ensure the same active BWP between multiple TRPs - Note that rate matching mechanisms (if need) to support multi-DCI based NCJT will be discussed separately. Agreement For multiple-PDCCH based multi-TRP/panel transmission, rate matching, puncturing, and pre-emption mechanisms shall be studied/enhanced if need, e.g. ratematchpattern, DMRS ports, ZP/NZP CSI-RS, SSB, configured CORESET, lte-CRS-ToMatchAround, pre-emption indications. - to be discussed and down-selected in RAN1#96bis Agreement For TCI state configuration in order to enable one or two TCI states per a TCI code point, - MAC-CE enhancement to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2. - FFS whether increasing the number of bits of TCI field in DCI Include in LS to RAN2 Agreement To support multiple-PDCCH based multi-TRP/panel transmission with intra-cell (same cell ID) and inter-cell (different Cell IDs), following RRC configuration can be used to link multiple PDCCH/PDSCH pairs with multiple TRPs - one CORESET in a “PDCCH-config” corresponds to one TRP - FFS whether to increase the number of CORESETs per “PDCCH-config” more than 3 FFS: UE monitoring/decoding behavior for multiple PDCCHs. Include in LS to RAN2 Agreement For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used, - PUCCH resources conveying ACK/NACK feedback can be TDM with separated HARQ-ACK codebook. - FFS TDM within a slot - FFS: the format of PUCCH from multiple TRP shall be same or different For issues related to PUCCH resources, study including: - FFS: if PUCCH resources conveying ACK/NACK feedback are overlapped at time, whether predefined dropping rule is needed to drop ACK/NACK feedback. - FFS: how to handle ACK/NACK overlapping with CSI reporting for different TRPs - FFS: how to handle PUCCH overlapping with PUSCH at the time domain for different TRPs - FFS: whether the UE can assume simultaneous ACK/NACK transmission from multiple PUCCH resources, and associated details of configurations/indication/UE capability. Include in LS to RAN2 Conclusion of Email Discussion after RAN1#96: To facilitate further down-selection for one or more schemes in RAN1#96bis, schemes for multi-TRP based URLLC, scheduled by single DCI at least, are clarified as following: - Scheme 1 (SDM): n (n<=Ns) TCI states within the single slot, with overlapped time and frequency resource allocation - Scheme 1a: - Each transmission occasion is a layer or a set of layers of the same TB, with each layer or layer set is associated with one TCI and one set of DMRS port(s). - Single codeword with one RV is used across all spatial layers or layer sets. From the UE perspective, different coded bits are mapped to different layers or layer sets with the same mapping rule as in Rel-15. - Scheme 1b: - Each transmission occasion is a layer or a set of layers of the same TB, with each layer or layer set is associated with one TCI and one set of DMRS port(s). - Single codeword with one RV is used for each spatial layer or layer set. The RVs corresponding to each spatial layer or layer set can be the same or different. - FFS: codeword-to-layer mapping when total number of layers <= 4 - Scheme 1c: - One transmission occasion is one layer of the same TB with one DMRS port associated with multiple TCI state indices, or one layer of the same TB with multiple DMRS ports associated with multiple TCI state indices one by one. - Applying different MCS/modulation orders for different layers or layer sets can be discussed. - Scheme 2 (FDM): n (n<=Nf) TCI states within the single slot, with non-overlapped frequency resource allocation - Each non-overlapped frequency resource allocation is associated with one TCI state. - Same single/multiple DMRS port(s) are associated with all non-overlapped frequency resource allocations. - Scheme 2a: - Single codeword with one RV is used across full resource allocation. From UE perspective, the common RB mapping (codeword to layer mapping as in Rel-15) is applied across full resource allocation. - Scheme 2b: - Single codeword with one RV is used for each non-overlapped frequency resource allocation. The RVs corresponding to each non-overlapped frequency resource allocation can be the same or different. - Applying different MCS/modulation orders for different non-overlapped frequency resource allocations can be discussed. - Details of frequency resource allocation mechanism for FDM 2a/2b with regarding to allocation granularity, time domain allocation can be discussed. - Scheme 3 (TDM): n (n<=Nt1) TCI states within the single slot, with non-overlapped time resource allocation - Each transmission occasion of the TB has one TCI and one RV with the time granularity of mini-slot. - All transmission occasion (s) within the slot use a common MCS with same single or multiple DMRS port(s). - RV/TCI state can be same or different among transmission occasions. - FFS channel estimation interpolation across mini-slots with the same TCI index - Scheme 4 (TDM): n (n<=Nt2) TCI states with K (n<=K) different slots. - Each transmission occasion of the TB has one TCI and one RV. - All transmission occasion (s) across K slots use a common MCS with same single or multiple DMRS port(s) - RV/TCI state can be same or different among transmission occasions. - FFS channel estimation interpolation across slots with the same TCI index Note that M-TRP/panel based URLLC schemes shall be compared in terms of improved reliability, efficiency, and specification impact. Note: Support of number of layers per TRP may be discussed

In this contribution, we discuss the enhancements on multi-TRP/panel transmission in Rel-16, including the multi-TRP/panel enhancement for non-coherent joint transmission and multi-TRP enhancement for URLLC.

Multi-TRP/panel enhancement for non-coherent joint transmission

Single PDCCH based multi-TRP/Panel transmission

TCI state enhancement for multiple PDSCHs

The illustration of single PDCCH based multi-TRP/panel transmission is shown in Fig. 2-1.

Fig. 2-1 Single PDCCH based multi-TRP/panel transmission

For ideal backhaul or low latency backhaul, a single PDCCH can be used to dynamically indicate the related parameters for the transmission from different TRPs/panels. In last meeting, it has been agreed to support MAC-CE enhancement to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2. Whether increasing the number of bits of TCI field in DCI is still FFS and will be discussed in this meeting.

We think it is better to keep the Rel-15 DCI format to avoid additional specification effort. In addition, for 3-bit TCI code point, since MAC-CE can be enhanced to map one or two TCI states for a TCI code point, it is straightforward to support up to 16 TCI states to be activated by MAC CE enhancement. In that case, the flexibility of TCI state indication for multi-TRP/panel can be somehow increased even without increasing the number of bits for TCI field in DCI.

Proposal 2-1:

- For single PDCCH design for eMBB, up to 16 TCI states can be activated by MAC CE enhancement.

- For single PDCCH design for eMBB, the number of bits of TCI field in DCI is not increased.

Resource allocation for multiple PDSCHs

For multi-TRP/panel cases, multiple PDSCHs can be used to increase the transmission performance. For single PDCCH with idea backhaul, it can be assumed that the time and frequency between the TRPs are synchronized. In this case, the resource allocation of different PDSCHs from different TRPs can be non-overlapped, partially overlapped or completely overlapped.

For non-overlapped and partially overlapped cases, new DCI fields to indicate the time and frequency domain resources are needed, which will increase the DCI overhead. Besides, for partially overlapped case, the overlapped parts and non-overlapped parts may have different interference characteristics, it may increase the channel estimation and CSI feedback complexity. For completely overlapped case, no additional DCI fields are needed, and the interference characteristics will be similar for the allocated resources. Hence, we prefer to support completely overlapped frequency domain and time domain resource allocation for PDSCHs from different TRPs. For this purpose, regarding time domain resource allocation table, only one default or RRC configured time domain resource allocation table is applied for multiple TRPs/panels.

Proposal 2-2:

- For single PDCCH design for eMBB, frequency domain and time domain resources of PDSCHs from multiple TRPs/panels are completely overlapped.

  - Note: no additional DCI fields are needed to indicate the time/frequency domain resources

UCI enhancement

In the last RAN1 meeting, it was concluded to not support enhanced codeword to layer mapping when the total number of layers <=4. That is to say, only one CW is enabled for multiple TRPs/panels transmission when the total number of layers <=4. In this case, only one ACK/NACK bit will be generated for multiple panels/TRPs and the Rel.15 HARQ-ACK feedback can work well. However, when the total number of layers >4, two CWs can be enabled for multiple TRPs/panels transmission, in this case, how to feedback the ACK/NACK for different CWs from different TRPs/panels should be studied. In the NRAH#1901 meeting, separate ACK/NACK payload/feedback was already supported for multiple PDCCH based multi-TRP/panel transmission for eMBB, however, whether to support joint ACK/NACK payload/feedback was still FFS. For non-ideal backhaul with large latency, such separate feedback is necessary. However, for single-PDCCH based multiple-TRP/panel transmission with ideal backhaul, it is straightforward to report UCI feedback, e.g., A/N and CSI, for multiple TRPs/panels to one TRP/panel only. Then the UCI information can be shared with other TRPs/panels via backhaul. Therefore, joint ACK/NACK payload/feedback should be supported for single PDCCH based multiple TRPs/panels transmission.

Proposal 2-3:

- NR Rel-16 should support joint UCI (e.g., ACK/NACK and CSI) payload/feedback for received PDSCHs for single PDCCH based multiple-TRP/panel transmission for eMBB.

In order to support joint ACK/NACK payload/feedback, the corresponding HARQ-ACK codebook construction mechanism needs to be enhanced when the total number of layers >4.

Semi-static HARQ-ACK codebook

For semi-static HARQ-ACK codebook, additional rules on how to arrange the HARQ-ACK bits for different TRPs/panels need to be defined. In Rel.15, semi-static HARQ-ACK codebook is determined by two steps. The first step is to determine the candidate PDSCH reception occasions based on the HARQ-ACK timing values K1 and PDSCH time domain resource allocation table, etc. The second step is to determine the number of HARQ-ACK bits and HARQ-ACK bits order for each candidate PDSCH reception occasion based on the maximum number of codewords per CC, maximum number of CBGs per TB, and whether spatial bundling is configured or not. For multiple TRPs/panels transmission, generally, two alternatives can be considered:

- Alt1: For the first step, reuse the Rel.15 mechanism to determine the candidate PDSCH reception occasions for multiple TRPs/panels. For the second step, the number of HARQ-ACK bits and HARQ-ACK bits order for each candidate PDSCH reception occasion is determined based on the number of TPRs/panels per CC, maximum number of CWs per TRP/panel, maximum number of CBGs per TB, and whether spatial bundling across multiple TRPs/panels is configured or not.

- Alt.2: For each TRP/panel, reuse the Rel.15 mechanism to generate the HARQ-ACK sub-codebook; then concatenate the HARQ-ACK sub-codebook of each TRP/panel based on the TRP/panel index.

Fig.2-2: ACK/NACK payload/feedback based on Alt.1

Fig.2-3: ACK/NACK payload/feedback based on Alt.2

Fig.2-2 and Fig.2-3 shows the HARQ-ACK codebook for the above two alternatives, respectively. For simplicity, we assume there is only one PDSCH reception occasion in each slot. And the maximum number of codewords per TRP is assumed to be 1. For Alt.1, the RRC configured K1 timing values are assumed to be {1, 3} for both TRP1 and TRP2. In this case, the UE first determines the candidate PDSCH reception occasion for TRP1 and TRP2, i.e., candidate PDSCH reception occasion 1 and 2. Then based on the number of TRPs/panels and TRP/panel index, the UE determines the HARQ-ACK bits for each candidate PDSCH reception occasion. For PDSCH reception occasion 1, two HARQ-ACK bits are generated, one is for TRP1 and the other is for TRP2. The HARQ-ACK bits are ordered from low TRP/panel index, i.e., TRP1 to high TRP/panel index, i.e., TRP2. Same procedure is applied for PDSCH reception occasion 2. For Alt.2, the RRC configured K1 timing values are assumed to be {1, 3} for TRP1 and {3} for TRP2. In this case, the UE first generates HARQ-ACK sub-codebook for each TRP/panel based on Rel.15 mechanism. For TRP1, two PDSCH reception occasions are determined based on RRC configured K1 values, i.e., PDSCH reception occasion 1 and 2. Then for each PDSCH reception occasion for TRP 1, 1 ACK/NACK bit is generated. For TRP2, one PDSCH reception occasion is determined based on RRC configured K1 value for TRP2, i.e., PDSCH reception occasion 1. Then for the PDSCH reception occasion for TRP2, 1 ACK/NACK bit is generated. After generating the HARQ-ACK sub-codebook for each TRP, the HARQ-ACK sub-codebook is concatenated to form a HARQ-ACK codebook. The HARQ-ACK sub-codebook for TRP2 is appended on the HARQ-ACK sub-codebook for TRP1 as shown in Fig.2-3.

Based on the above analysis, Alt.1 is simple and can reuse the Rel.15 design as much as possible. However, it is only applicable when candidate PDSCH occasions are the same for different TRPs/panels; Alt.2 is more flexible and can be applicable when candidate PDSCH occasions are different for different TRPs/panels. The motivation to support different K1 values, etc. for different TRPs/panels is not clear. Therefore, it is preferred to reuse Rel.15 design as much as possible.

Dynamic HARQ-ACK codebook

For dynamic HARQ-ACK codebook, counter DAI and total DAI in the DL DCI are used to determine the HARQ-ACK payload and HARQ-ACK bits order. For single -PDCCH based multiple TRPs/panels transmission, the first issue needs to be considered is whether to additionally introduce new DCI filed to indicate counter DAI and total DAI for multiple TRPs/panels. As we discussed in section 2.1.1 and 2.1.3, introducing new DCI fields will increase the DL DCI overhead. On the other hand, this may also increase the blind decoding complexity at the UE side. Therefore, it is better to keep the Rel.15 DCI format. In this case, the DAI filed in single PDCCH would be applicable for multiple TRPs/panels.

The next question is how to count the DAI for multiple TRPs/panels. In general, there are two alternatives:

- Alt.1: DAI is counted jointly across TRPs/panels;

- Alt.2: DAI is counted separately for different TRPs/panels.

Fig.2-4 Joint DAI counting                      Fig.2-5: Separate DAI counting

Fig.2-4 and Fig.2-5 illustrate joint and separate DAI counting, respectively. For Alt.1, the counter DAI increment can be larger than 1 when more than one TRP/panel have PDSCH to be scheduled in a given PDCCH on a CC. The total DAI denotes the total number of PDSCHs scheduled up to current PDCCH monitoring occasion. For Alt.2, the counter DAI increment is always 1 as long as there is PDSCH scheduled in either of the TRPs/panels. It can be observed that Alt.1 is beneficial to identify the miss detection in TRP/panel domain. Alt.2 can relax the coordination among TRPs/panels. Since the typical scenario for single PDCCH is ideal backhaul, it is possible to perform dynamic coordination among multiple TRPs/panels. Therefore, jointly DAI counting should be supported for single PDCCH.

Proposal 2-4:

- To support joint ACK/NACK payload/feedback,

  - For semi-static HARQ-ACK codebook, Rel.15 design should be reused as much as possible.

    - Candidate PDSCH reception occasions for multiple TRPs/panels can be determined based on Rel.15 mechanism;

    - The HARQ-ACK payload and bits order for multiple TRPs can be determined based on the number of TRPs/panels and TRP/panel index

  - For dynamic HARQ-ACK codebook, the DAI filed in Rel.15 can be reused.

    - At least joint DAI counting across TRPs/panels should be supported.

DMRS enhancement

Regarding to DMRS enhancement for single PDCCH based multi-TRP/panel transmission, one main issue is how to design the DMRS port indication table. In the RAN1#96bis meeting, some design principles were discussed where one of the design principles is which layer combination should be supported for multi-TRP/panel transmission.

In our view, multiple TRP/panel is mainly applied for the UEs with small RSRP difference between two coordinated TRPs, in this case, similar/balanced layer combination from multiple TRPs/panels should be supported, i.e., (1,1), (2,1), (2,2) for <=4 layers and (3,3) (3,2), (4,3), (4,4) for >4 layers should be supported. It is noted that in Rel.15，for single CW, when the number of layers = 3, layer combination (1,2) is not supported. Basically, since the RSRP difference between two TRPs/panels should be small for multiple TRPs/panels, layer combination (1,2) would not provide much performance gain while this would need additional spec. change. If layer combination (1,2) is supported, the benefit should be justified. In addition, dynamic switching between multiple TRPs and single TRP should be supported, in this case, for <=4 layers, layer combination (1,0), (2,0), (3,0), (4,0), (0,1), (0,2), (0,3), (0,4) should also be supported. In the last RAN1 meeting, some companies propose to support unbalanced layer combinations e.g., (1,3), (3,1) for <=4 layers, in our view, this is not the typical configuration for NCJT. What’s more, as evaluated in contribution [3], supporting the unbalanced layer combination cannot provide much performance gain. However, this may introduce additional specification work.

Proposal 2-5:

- For single PDCCH based multi-TRP/panel DMRS port indication:

  - Support layer combinations (1,0), (2,0), (3,0), (4,0), (0,1), (0,2), (0,3), (0,4), (1,1), (2,1), (2,2) for <=4 layers;

  - Support layer combinations (3,3) (3,2), (4,3), (4,4) for >4 layers.

Multiple PDCCH based multi-TRP/Panel transmission

UE behavior for monitoring multiple PDCCHs

Number of CORESETs

The illustration of multiple PDCCH based multi-TRP/panel transmission is shown in Fig. 2-6.

Fig. 2-6 Multiple PDCCH based multi-TRP/panel transmission

In the last RAN1 meeting, it has been agreed to increase the maximal number of CORESETs per “PDCCH-config” up to N=[4, 5, or 6] subject to UE capability. However, the maximum number of CORESETs has not decided yet. Regarding to the maximum number of CORESET to be supported, there are following options:

- Option 1: increase the maximum number of CORESET per BWP to 4. In this option, one CORESET can be added for UE-specific data scheduling for the other TRP (and also for UE group common signalling for the other TRP).

- Option 2: increase the maximum number of CORESET per BWP to 5. Based on Option 1, separate CORESET configuration for UE-specific data scheduling and for UE group common signalling for the other TRP can be supported.

- Option 3: increase the maximum number of CORESET per BWP to 6. In this option, following configuration could be supported: 1 CORESET for broadcast, e.g., SI/Paging/…, for 1 serving TRP; 2 CORESETs for UE group common signaling, e.g., SFI/TPC/…, for 2 serving TRPs; 2 CORESETs for UE specific data scheduling for 2 serving TRPs; 1 CORESET for BFR for 1 serving TRP.

Considering the trade-off between configuration flexibility and UE complexity, at least 4 CORESET per BWP can be supported.

Proposal 2-6:

- For multiple PDCCH design for eMBB, support to increase the maximum number of CORESET per BWP for a UE to 4.

Resource allocation for multiple PDSCHs

PDSCH mapping type from two co-scheduled PDSCHs

In the last RAN1 meeting, resource allocation for multiple PDSCHs for multiple PDCCH based transmission was discussed and related agreements were made. PDSCH mapping type from two co-scheduled PDSCHs is still FFS. Considering that the required restriction of DMRS from multiple TRPs may be different for different PDSCH mapping types, it is simpler to support the same PDSCH mapping type as a starting point. At least PDSCH type A + PDSCH type A from multiple TRPs can be supported. Whether to support other PDSCH type combinations can be further discussed.

Proposal 2-7:

- For multiple PDCCH design for eMBB, support the same PDSCH mapping type (PDSCH Type A+ PDSCH Type A) for co-scheduled PDSCHs as starting point.

Active BWP for multiple TRPs

In Rel-15, two BWP switching methods, i.e., DCI-based BWP switching and timer-based BWP switching, are supported. For multiple PDCCH based multi-TRP/panel transmission, it has been agreed to schedule a UE on the same active BWP to receive multiple PDSCHs simultaneously. How to ensure the same active BWP for multiple TRPs should be discussed.

For ideal backhaul case, multiple TRPs can coordinate to schedule the same active BWP for a UE. One way to support DCI-based BWP switching is that UE is not expected to receive different UL or DL bandwidth part indicator from multiple TRPs/panels simultaneously if UE receives 2 DCI format 0_1 or 2 DCI format 1_1 at the same time. Another possible method is that, only one TRP is configured for DCI-based BWP switching and UE just follows UL or DL bandwidth part indication from this TRP. The bandwidth part indication from the other TRP can be not present in the DCI, or even present, UE will ignore the bandwidth part indication from the other TRP. Fig. 2-7 shows an example of DCI-based BWP switching. In Fig. 2-7, we assume TRP1 is configured with DCI-based BWP switching. Hence, UE just follows BWP indication from TRP1 and ignores the BWP indication information from TRP2. For timer-based BWP switching, a timer can be configured and maintained for multiple TRPs on a CC, and the timer can be restarted by either TRP by decoding related PDCCH or MAC PDU, as shown in Fig. 2-8.

Fig.2-7: DCI-based BWP switching

Fig.2-8: timer-based BWP switching

For non-ideal backhaul case, since it is difficult for multiple TRPs to coordinate timely to exchange the information for BWP switching, the simplest way is to not support DCI-based BWP switching or timer-based BWP switching. To support BWP switching in non-ideal backhaul case, frequent inter-TRP coordination should be avoided. A possible method is that, only one TRP can be configured with DCI-based or timer-based BWP switching, and UE just follows the configured TRP for BWP switching. In our view, the BWP switching would not happen frequently, therefore, it is feasible that the configured TRP shall make the BWP switching decision and notify the decision to the other TRP by Xn/backhaul signaling semi-statically to make sure the consistent active BWP operation among TRPs.

To support both ideal and non-ideal backhaul cases, whether supporting DCI-based or timer-based BWP switching or not can be configured by RRC signaling. In addition, only one TRP/panel configured with DCI-based or timer-based BWP switching can be a common solution for both ideal and non-ideal backhaul cases. Hence, we have following proposals.

Proposal 2-8:

- For multiple PDCCH design for eMBB, support DCI-based BWP switching, if configured.

  - UE assumes the same active BWP for multiple TRPs.

    - UE follows BWP indication from one TRP as specified or RRC configured, and ignores the BWP indication from another TRP.

- For multiple PDCCH design for eMBB, support timer-based BWP switching, if configured.

  - UE assumes the same active BWP for multiple TRPs.

    - UE follows timer-based switched BWP for one TRP as specified or RRC configured.

PDSCH scrambling

In the last RAN1 meeting, it was agreed that different PDSCH scrambling sequences can be supported for PDSCHs, while the detailed design is FFS. In Rel.15, the scrambling sequence generator is initialized by cinit=nRNTI*215+q*214+nID where nRNTI corresponds to the RNTI associated with the PDSCH transmission, q is the CW ID, nID is configured by RRC if the parameter dataScramblingIdentityPDSCH is configured, otherwise, nID =NIDcell . For multi-TRP/panel transmission, in order to support different PDSCH scrambling sequences, two alternatives were proposed in the last RAN1 meeting:

- Alt 1: enhance c_init, FFS detailed design in RAN1 97

- Alt 2: enhance RRC configurations to support multiple dataScramblingIdentityPDSCH

In our view, Alt.2 is one simple and effective way to support different PDSCH scrambling sequences for different PDSCHs. The only issue is when dataScramblingIdentityPDSCH is not configured, same cell ID will be used for different TRPs/panels for intra-cell ID scenario. In this case, same PDSCH scrambling sequences will be generated for different PDSCHs. However, this is not a big issue and it can be controlled by the network and the network can flexibly decide whether to configure the dataScramblingIdentityPDSCH based on the scenarios.

Proposal 2-9:

- For multiple PDCCH based multiple-TRPs/panels transmission, support different PDSCH scrambling sequences for PDSCHs where multiple dataScramblingIdentityPDSCH can be configured by RRC.

Rate matching enhancement

In the RAN1#96 meeting, it has been agreed that rate matching mechanisms (if need) to support multi-DCI based NCJT will be discussed separately. In Rel-15, UE’s PDSCH is mainly rate matched considering its serving cell’s resources. In multi-TRP/panel scenario, the discussion point is whether PDSCH rate matching of one TRP should consider the other TRP. We should first discuss the rate matching procedure of two eMBB PDSCHs from two TRPs for a UE. The rate matching procedure of other service, e.g., URLLC, should be deprioritized.

In Rel-15, rate matching pattern can be indicated by RRC signaling or DCI indication. For multiple PDCCH based multi-TRP/panel transmission with ideal backhaul, Rel-15 rate matching mechanism can be reused to indicate flexible rate matching pattern. The NW can consider any required rate matching pattern by RRC configuration signaling and DCI indication in ideal backhaul case.

For non-ideal backhaul, the situation will be different since it highly depends on the information exchange between TRPs via backhaul. Following options can be considered for non-ideal backhaul case.

- Option 1: The rate-matching/puncturing around the other TRP are configured/performed semi-statically

- Option 2: The rate-matching/puncturing around the other TRP are configured/performed dynamically

In Option 1, the rate-matched/punctured resources around the other TRP are configured semi-statically, and UE assumes the PDSCH is always rate-matched/punctured on the semi-statically configured resources. The required rate-matched/punctured resources can be semi-statically exchanged by TRPs via backhaul. But the resource utilization may be low if UE always assumes rate matching for some resources. In Option 2, rate-matching is preferred over puncturing to achieve better UE performance.

In Option 2, dynamic rate-matching/puncturing around the other TRP can provide higher resource utilization. However, frequent and fast information exchange between TRPs is required. To reduce the complexity of Option 2, puncturing is preferred over rate-matching so that gNB does not need to notify UE the dynamic resources for rate-matching.

For non-ideal backhaul, in addition to the explicit indication, UE can also assume PDSCH rate matching of one TRP around DMRS from another TRP to reduce inter-cell interference.

Proposal 2-10:

- For multiple PDCCH design for eMBB, rate-matching mechanism for eMBB should be firstly discussed and the rate-matching mechanism considering URLLC should be deprioritized.

- For rate-matching mechanism for multiple PDCCH design for eMBB,

  - For ideal backhaul, Rel-15 rate matching mechanism is baseline and can be reused.

  - For non-ideal backhaul, support at least one of following

    - The rate-matched resources around the other TRP are configured semi-statically

    - The puncturing of resources around the other TRP is performed dynamically

UCI enhancement

Joint A/N for multiple PDCCH design

As discussed above, for multiple PDCCH based multiple TRPs/panels with non-ideal backhaul and large latency, separate ACK/NACK payload/feedback is necessary. However, for multiple PDCCH based multiple TRPs/panels with ideal backhaul or non-ideal backhaul with very low latency, UCI feedback, e.g., A/N and CSI, for multiple TRPs/panels can be reported to one TRP/panel only. Then the UCI information can be shared with other TRPs/panels via backhaul.

Proposal 2-11:

- NR Rel-16 should support joint UCI (e.g., ACK/NACK and CSI) payload/feedback for received PDSCHs for multiple PDCCHs based multiple-TRP/panel transmission with ideal backhaul for eMBB.

In order to support joint ACK/NACK payload/feedback, the corresponding HARQ-ACK codebook construction mechanism needs to be enhanced.

For semi-static HARQ-ACK codebook, to determine how to arrange the HARQ-ACK bits for different TRPs/panels, same mechanism as single PDCCH discussed above can be applied.

For dynamic HARQ-ACK codebook, different from single-PDCCH based design, the PDSCH for one TRP/panel is scheduled with a corresponding PDCCH where the counter DAI and total DAI in the PDCCH can be used to determine the HARQ-ACK payload and HARQ-ACK bits order. Therefore, how to count the DAI in different PDCCHs corresponding to different TRPs/panels need to be discussed. Same as above discussion, in general, two alternatives can be considered:

- Alt.1: DAI is counted jointly across TRPs/panels;

- Alt.2: DAI is counted separately for different TRPs/panels.

Fig.2-9 Joint DAI counting                      Fig.2-10: Separate DAI counting

For Alt.1, counter DAIs and total DAIs in multiple PDCCHs scheduling PDSCHs for multiple TRPs/panels are counted jointly. As shown in Fig.2-9, counter DAI is counted first in ascending order of TRP/panel index and then in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index; total DAI denotes the total number of {TRP/panel, serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) associated with DCI format 1_0 or DCI format 1_1 is present, up to the current PDCCH monitoring occasion. For Alt.2, counter DAI and total DAI in each PDCCH scheduling PDSCHs for different TRPs/panels are counted separately. As shown in Fig.2-10, for TRP0, counter DAI is counted first in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index, the total DAI in PDCCH scheduling PDSCH for TRP0 denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) corresponding to TRP0 in which PDSCH reception(s) associated with DCI format 1_0 or DCI format 1_1 is present, up to the current PDCCH monitoring occasion. For TRP1, same principle is applied. Based on the above analysis, it can be observed that Alt.1 is beneficial to identify the miss detection in TRP/panel domain, however it requires tight coordination among multiple TRPs/panels since the DAI value in the PDCCH of one TRP is impacted by the scheduling of other TRPs. While Alt.2 doesn’t require such tight coordination among multiple TRPs since each PDCCH can decide its DAI value independently. For multiple PDCCH-based multiple TRPs/panels transmission, both ideal backhaul and non-ideal backhaul are supported. For ideal backhaul, Alt.1 is preferred since dynamic coordination among multiple TRPs/panels is possible. However, for non-ideal backhaul, it is difficult to perform dynamic coordination among multiple TRPs/panels, therefore, Alt.2 is preferred, otherwise, scheduling delay may be increased. To allow more flexibility, both Alt.1 and Alt.2 should be supported, which to be used can be configured by RRC signaling.

Proposal 2-12:

- For multiple PDCCH-based multiple TRPs/panels transmission, both joint and separate DAI counting across TRPs/panels should be supported:

  - Whether the DAI is counted jointly or separately can be configured by RRC signaling.

Separate A/N for multiple PDCCH design

In the last RAN1 meeting, it was agreed to support TDMed PUCCH transmission within a slot for separate A/N for multiple PDCCH design and three alternatives on the PUCCH resource configuration were discussed:

- Alt 1: PUCCH resource groups can be explicitly configured by the NW.

    - All PUCCH resources configured within the first PUCCH resource group do not overlap in time with any PUCCH resources configured within the second PUCCH resource group, considering

    - how to support PUCCH resource groups composed with resources or resource sets

- Alt 2: PUCCH resources can be configured by the NW to ensure TDM PUCCH resources among M-TRPs

    - PUCCH resource groups are not needed.

- Alt 3: PUCCH resources configured by the NW may be overlapped among M-TRPs.

The difference between Alt.1/2 and Alt.3 is whether the PUCCH resources configured by the NW can be overlapped or not. In our view, whether PUCCH resources can be configured to be overlapped or not depends on the backhaul condition. For ideal backhaul, for more efficient and flexible PUCCH resource utilization, the configured PUCCH resources can be overlapped. In this case, dynamic coordination among TRPs/panels are allowed, the TDMed PUCCH transmission can be ensured by dynamic indication of TDMed PUCCH resources in PRI field of DCI. However, for non-ideal backhaul, if PUCCH resources are configured to be overlapped among multiple TRPs/panels, then multiplexing/dropping rules need to be defined to handle the PUCCH transmission collision case for the UE which may cause significant specification efforts while the benefit is not clear. Therefore, for non-ideal backhaul, PUCCH resources configured by the NW should not be overlapped. Considering the significant specification effort, it is not expected to define multiplexing/dropping rules in Rel.16 for multiple TRPs/panels, i.e., Alt.3 above is not preferred. Regarding Alt.1 and Alt.2, there is no much difference except that Alt.1 requires additional specification work. Therefore, Alt.2 is preferred. In this case, semi-static coordination can be performed among different TRPs to ensure non-overlapped PUCCH resources for different TRPs/panels. For example, as shown in Fig.2-11 below, 8 PUCCH resources are configured within a PUCCH resource set where PUCCH resource #1~#4 are non-overlapped with PUCCH resource #5~#8 in time domain, in this case, PUCCH resources #1~#4 can be associated with TRP1 and PUCCH resources#5~#8 can be associated with TRP2.

Fig.2-11: PUCCH resources associated with different TRPs/panels (example of Alt.2)

Regarding separate ACK/NACK feedback, another issue is how to update/indicate the PUCCH spatialrelationinfo for PUCCH resources as different PUCCH spatialrelationinfo may be needed for transmission to different TRPs. In the multi-beam agenda, it was agreed to support simultaneous update/indication of a single spatial relation per group for PUCCH, however, the details of grouping need further study.

Agreement

Simultaneous update/indication of a single spatial relation per group of PUCCH is supported by using one MAC CE

- As a starting point, the group should correspond to all the PUCCHs in a BWP when a single active spatial relation is applied before and after activation

- If there is no consensus on the details of the grouping, only one group per BWP will be supported in Rel-16 which will correspond to all the PUCCHs in a BWP

Detailed design on the MAC CE is up to RAN2

In general, there are two alternatives for the grouping of PUCCH resources for PUCCH spatialrelationinfo update/indication:

- Alt.1: PUCCH resources grouping for PUCCH spatialrelationinfo update/indication is configured independently with PUCCH resources grouping for PUCCH resource allocation associated with different TRPs/panels;

- Alt.2: PUCCH resource grouping for PUCCH spatialrelationinfo update/indication are configured the same as PUCCH resources grouping for PUCCH resource allocation associated with different TRPs/panels.

For Alt.1, the no. of the PUCCH resources group for PUCCH spatialrelationinfo update/indication can be larger than the no. of PUCCH resources group for PUCCH resource allocation associated with multiple TRPs/panels. In this case, the PUCCH resources group associated with one TRP can be configured with multiple groups of PUCCH spatialrelationinfo which can be dynamically updated/indicated based on the PUCCH resource indication. For Alt.2, the no. of PUCCH resources group for PUCCH spatialrelationinfo update/indication is always the same as the no. of PUCCH resources group associated with multiple TRPs/panels where the PUCCH resources associated with one TRP is configured with only one PUCCH spatialrelationinfo which is semi-statically updated/indicated.

Besides, for both Alt.1 and Alt.2, a group of PUCCH resources for PUCCH spatialrelationinfo update/indication is not allowed to include PUCCH resources associated with different TRPs/panels.

Alt.1: Independent PUCCH resources grouping for PUCCH spatialrelationinfo update/indication

Alt.2: Joint PUCCH resources grouping for PUCCH spatialrelationinfo update/indication

Fig.2-12: PUCCH resources grouping for PUCCH spatialrationinfo update/indication

Comparing Alt.1 and Alt.2, Alt.2 can reduce the signaling overhead as much as possible while the PUCCH spatialrelationinfo update/indication is less flexible. On the other hand, Alt.1 can provide more flexibility for PUCCH spatialrelationinfo update/indication while the signaling overhead is larger. Considering that the main motivation of PUCCH spatialrelationinfo update/indication enhancement in Rel.16 is overhead reduction, Alt.2 is more preferred in which case the maximum number of groups for PUCCH spatialrelationinfo update/indication is 2.

Proposal 2-13:

- For separate ACK/NACK feedback, the network should ensure the TDMed PUCCH resources among multiple TRPs/panels.

  - The PUCCH resources grouping for PUCCH spatialrelationinfo update/indication is configured the same with PUCCH resources grouping for PUCCH resource allocation associated with different TRPs/panels.

FFS: the format of PUCCH from multiple TRP shall be same or different

In non-ideal backhaul case, the PUCCH resource configuration for multiple TRPs can be different for separate UCI feedback. Hence, the PUCCH resource and format to multiple TRPs determined by UE could be different. Even with the same RRC configured PUCCH resource configurations (except spatial relation information for PUCCH) for multiple TRPs, since multiple TRPs perform separate scheduling, the PUCCH resource and format to multiple TRPs determined by UE could still be different. Hence, there is no need to restrict the PUCCH format of multiple TRPs to be the same from flexibility perspective.

Proposal 2-14:

- For multiple PDCCH design for eMBB, the format of PUCCH of multiple TRPs can be different.

FFS: whether the UE can assume simultaneous ACK/NACK transmission from multiple PUCCH resources

Even though multi-panel UE is discussed in multi-beam Section for simultaneous UL transmission, the simultaneous transmission of multiple PUCCHs to multiple TRPs via different QCL is very challenging for UE. Therefore, we do not support simultaneous ACK/NACK transmission from multiple PUCCH resources via different QCL by UE in Rel-16.

Proposal 2-15:

- For multiple PDCCH design for eMBB, UE does not support simultaneous ACK/NACK transmission from multiple PUCCH resources via different QCL in Rel-16.

FFS: how to handle UL channel overlapping for different TRPs

Rel-15 defines some dropping/multiplexing rules when different UL channels from a UE collide at the same time for a serving cell. For multiple TRP scenario, if UE is not capable of simultaneous UL transmission via different TX antennas/panels, when different UL channels from different TRPs collide, multiplexing or dropping rule should also be defined. Generally, following two options can be considered.

- Option 1: UE selects the TRP/panel for UL transmission based on some predefined rules, and then re-use Rel-15 dropping/multiplexing rule for the selected TRP/panel if there are still multiple UL channels to be transmitted at the same time for the selected TRP/panel

- Option 2: Rel-15 rule is reused first within each TRP/panel, then TRP/panel is selected based on some predefined rules

For the two options above, Option 1 is preferred due to lower UE complexity. In Option 1, the predefined rule for selecting TRP/panel can be based on the priority of UL channel type and contents in the UL channel, similar as Rel-15 rule. The highest priority channel from each TRP/panel is compared first to decide the selected TRP/panel. If the overlapped UL channels from two TRPs/panels have the same priority with the same UL channel type and contents, the TRP/panel with the lowest TRP/panel ID, or with the earlier starting time of the UL channel can be selected.

Proposal 2-16:

- For multiple PDCCH design for eMBB, when UL channels from different TRPs collide,

  - UE selects the TRP/panel for UL transmission based on some predefined rules, and then re-use Rel-15 dropping/multiplexing rule for the selected TRP/panel if there are still multiple UL channels to be transmitted at the same time for the selected TRP/panel

    - The predefined rule is firstly based on the priority of UL channel type and contents in the UL channel like Rel-15 rule.

    - If the overlapped UL channels from two TRPs/panels have the same priority (e.g., with the same UL channel type and contents), the TRP/panel with the lowest TRP/panel ID or with the earlier starting time of the UL channel is selected

CSI measurement enhancement

To support joint transmission from multiple TRPs/panels, suitable Tx-Rx beam pair from each TRP/panel should be selected for high frequency. If beam management is performed for each TRP/panel individually, the selected beams from each TRP/panel individually do not consider inter-beam interference. In addition, the overhead of beam measurement and reporting would be high, and the latency of beam management procedure would be large. Therefore, a combination of multiple beams from multiple TRPs/panels (one beam from one TRP/panel) can be transmitted simultaneously for beam measurement and reporting. The new measurement and reporting of L1-SINR for a combination of multiple beams can be further studied, as discussed in our companion contribution [4].

Similarly, for CSI reporting in multiple TRPs/panels, inter-beam interference from multi-TRP/panel should be taken into account. NZP CSI-RS based interference measurement defined in NR Rel-15 can be reused for inter-beam interference measurement in multi-TRP/panel in NR Rel-16. However, how to configure CSI-RS from multiple TRPs and how to support multiple types of interference hypothesis measurement for a channel measurement should be further discussed.

To evaluate the performance of different transmission schemes and different CSI measurement methods, system-level simulation is performed for multi-panel transmission on 30 GHz. Detailed evaluation assumptions are shown in appendix Table A-1. Cell average spectral efficiency and 5% edge UE spectral efficiency are evaluated as the metrics. In the simulation, following four cases in Table 2-1 are evaluated with corresponding illustrations shown in Fig. 2-13.

Table 2-1: Simulation cases

| Case | Transmission scheme | CSI feedback | UE reception |
| --- | --- | --- | --- |
| 1 | Single-panel Tx for a UE | 1 CSI feedback for single-beam transmission per UE | 1 panel for reception at UE |
| 2 | Multi-panel Tx for a UE | 1 CSI feedback for multi-beam joint transmission per UE | 2 panel for reception at UE |
| 3 | Multi-panel Tx for a UE; | 3 CSI feedback (2 for single-beam transmission and 1 for multi-beam joint transmission) per UE | 2 panel for reception at UE |
| 4 | Multi-panel Tx for a UE; | 3 CSI feedback (Based on Case 3, 2 CSIs for single-beam transmission are updated considering the inter-beam interference from the other panel) per UE | 2 panel for reception at UE |

Fig. 2-13 Examples of four simulation cases.

For the four cases, the evaluation results of cell average spectral efficiency and 5% edge UE spectral efficiency are shown in Fig. 2-14. Compared with Case 1, Case 2 with multi-beam joint transmission for a UE can increase both cell average and cell edge performance in multi-panel scenario. The cell average performance gain is about 11%-16%, and the edge UE performance gain is about 22%-23%. Case 2 can be regarded as coherent joint transmission from multiple beams.

Comparing Case 2 and Case 3, it is observed that multiple types of CSI feedback in Case 3 increases cell average performance but reduces cell edge performance in multi-panel scenario. In Case 3, as the gNB can decide to schedule one UE with joint transmission from two panels simultaneously, or to schedule two different UEs with single-beam transmission, the performance of Case 3 should be better than Case 2 due to more flexible scheduling. However, when gNB schedules two different UEs for two panels, the beams from these two panels are selected dynamically based on scheduling, which means that the selected beams from two panels will have variable inter-panel/inter-beam interference. In that case, the reported CSI for single-beam transmission may be not aligned with the realistic CSI for data reception. Especially when the reported CSI of single-beam is high while the scheduled single-beam suffers large inter-panel interference from the selected beam from the other panel, the over-estimated MCS will make performance decrease. In the simulation, the edge UEs largely suffer from inter-panel interference. That’s why edge UE performance is decreased in Case 3. In Case 4, enhanced CSI feedback with inter-beam interference is considered based on Case 3. And Case 4 can further improve the cell edge and cell average performance.

Fig. 2-14 Performance of different transmission schemes with different CSI feedback methods.

Observation 2-1:

- Multiple CSI feedback considering inter-beam/inter-panel/inter-TRP interference measurement can increase both cell average and cell edge performance in multi-panel/TRP scenario.

Proposal 2-17:

- To support non-coherent joint transmission in multi-TRP/panel for eMBB, NR Rel-16 should study the CSI enhancement, including

  - Configuration and L1 measurement/reporting of a combination of multiple beams transmitted from multiple TRPs/panels simultaneously

  - Configuration and CSI measurement to support multiple types of interference hypothesis measurement from multiple TRPs/panels

HARQ process

For multiple PDSCH transmissions from multi-TRP in non-ideal backhaul, separate HARQ process number for multiple PDSCHs should be indicated. The first question is whether multiple HARQ entity should be used for different TRPs. Considering the impact to RAN2, the same HARQ entity should be supported, which is sufficient for ideal backhaul or low backhaul latency.

For the single HARQ entity for multiple TRPs, with Rel-15 DCI format, 16 HARQ processes can be supported for single PDCCH based multi-TRP/panel transmission, while the supported HARQ process number for multiple PDCCH based multi-TRP/panel transmission can be increased. For example, existing 4-bit HPN can be used to indicate 16 HARQ process number for each TRP, hence, totally 32 HARQ processes can be supported for 2-TRP case. Even for the same HPN from multiple DCIs, UE can distinguish the PDSCHs by different QCLs.

Proposal 2-18:

- For multi-panel/TRP transmission for eMBB,

  - Support the same HARQ entity for multi-panel/TRP transmission.

  - Support the increased HARQ process number per HARQ entity for multiple PDCCH based multi-panel/TRP transmission.

Multi-TRP enhancement for URLLC

In the last RAN1 meeting, following agreements were made for multiple TRP for URLLC:

Agreement For multi-TRP based URLLC, scheduled by single DCI, support scheme 3 and 4 agreed in email discussion [96-NR-09] - FFS any restrictions/modification of supporting scheme 3/4 for FR2 - For example, considering the number of beam switches within the slot, and the delay from scheduling DCI indicating beam switch to scheduled PDSCH Note how to address M-TRP/panel based URLLC operation in FR2 can be discussed from RAN1 #98 Agreement For multi-TRP based URLLC, scheduled by single DCI, - Support scheme 1a as agreed in email discussion [96-NR-09] - FFS: Whether additional specification impact is necessary for URLLC - On the support of schemes 2a, 2b - Select one of the following: support 2a only, support 2b only, support both 2a and 2b, support none - To facilitate further comparisons among 2a, 2b and baseline to understand technical benefits and use cases, consider both SLS and LLS simulation results - Specification impact, and UE complexity need to be considered as well. - Companies are encouraged to provide simulation results for LLS using at least the following parameters - Pathloss delta between two TRPs: 0dB, 3dB, 6dB - Details on blockage to be provided by each company if any (for example, the probability that one out of 2 links is blocked is 5% or 10% with 10dB blockage loss for the blocked link)

In this section, we present our views on multi-TRP enhancement for URLLC.

One TB from multiple TRPs

PDSCH repetition over multiple TRPs

In the last RAN1 meeting, it was agreed to support scheme 3 and 4 for single DCI based multi-TRP for URLLC, however, the details need further discussion.

For single DCI based multiple TRP for URLLC, scheme 3 and/or scheme 4 can be PDSCH repetitions across multiple TRPs scheduled by a DCI, in which each PDSCH repetition is associated with one QCL assumption/TCI state. The QCL assumptions/TCI states for PDSCH repetitions over multiple TRPs can be explicitly indicated to the UE by higher-layer configuration. For example, RRC signaling can configure the sequence of RS indexes, TCI states, or QCL assumptions, to be applied across repetitions. Once the UE is configured with the PDSCH repetitions across TRPs by higher-layer with a certain sequence of QCL assumptions/TCI states, the UE receives the PDSCH repetitions based on the configured sequence, where K is the repetition factor.

However, semi-static determination of QCL assumptions/TCI states across PDSCH repetitions is not flexible enough; depending on data type/size (e.g., eMBB data or URLLC data, initial transmission or re-transmission, HARQ process ID, CSI knowledge, etc.), appropriate QCL assumptions/TCI states are different. Therefore, the QCL assumptions/TCI states for PDSCH repetitions can be indicated by the scheduling DCI as shown in Table 3-1, i.e., RRC signaling configures more than one sequences of RS indexes, TCI-states, or QCL assumptions, and then the scheduling DCI indicates one of them to be applied across PDSCH repetitions.

Table 3-1: DCI indicates one sequence of RS indexes, TCI-states, or QCL assumptions

| DCI field value | TRPs for repetition (i.e., sequence of RS indexes, TCI-states, or QCL assumptions) |
| --- | --- |
| 00 | #0, #1, #2, #3 |
| 01 | #0, #1, #0, #1 |
| 10 | #0, #2, #0, #2 |
| 11 | #0, #0, #0, #0 |

Further, this can be combined with DCI indication of RV sequence for PDSCH repetition. In Rel.15, RV sequence for PDSCH repetition is fixed to {0, 2, 3, 1}, and the scheduling DCI can indicate the starting RV value among the four values. This RV sequence is useful to achieve coding gain after the repetitions are received and combined at the UE. However, use of multiple TRPs for PDSCH repetition may require different RV sequences. For example, if the gNB does not know which TRP is the best for the UE, each repetition among the K repetitions should be self-decodable (e.g., 0, 0, 0, 0) and the repetitions should be transmitted from multiple TRPs; while if the gNB knows which TRP is the best for the UE, all the K repetitions should be transmitted from the best TRP with as many RVs as possible (e.g., 0, 2, 3, 1). Hence, we propose to enhance RV sequence together with introducing QCL assumptions/TCI states for PDSCH repetitions. More specifically, for PDSCH repetition, RV sequence of {0, 3, 0, 3} and {0, 0, 0, 0} should be supported. For example, if the PDSCH repetition uses single TRP (QCL assumptions/TCI states {#0, #0, #0, #0}), the RV sequence {0, 2, 3, 1} can be used. While if the PDSCH repetition uses multiple TRPs (QCL assumptions/TCI states {#0, #1, #2, #3}), the RV sequence {0, 0, 0, 0} can be used. The field indicating QCL assumptions/TCI states can be either separate from, or jointly encoded with the RV indication field. Table 3-2 shows an example of the joint filed indication for precoder/QCL-cycling sequence and RV sequence.

Table 3-2: Example of joint field indication for QCL assumptions/TCI states and RV sequence

| DCI field value | TRPs for repetition | RV sequence | Note |
| --- | --- | --- | --- |
| 00 | #0, #1, #2, #3 | 0, 0, 0, 0 | gNB assumes at least one of them is received Each repetition should be self-decodable |
| 01 | #0, #1, #1, #0 | 0, 3, 0, 3 | gNB knows TRPs #0 and #1 are appropriate |
| 10 | #0, #2, #2, #0 | 0, 3, 0, 3 | gNB knows TRPs #0 and #2 are appropriate |
| 11 | #0, #0, #0, #0 | 0, 2, 3, 1 | gNB knows TRP #0 is appropriate Repetitions should maximize coding gain |

Proposal 3-1:

- For scheme 3 and scheme 4 with single DCI based multiple TRP for URLLC,

  - QCL assumptions/TCI states for PDSCH repetitions are indicated by DCI from multiple sequences of QCL assumptions/TCI states

    - Multiple sequences of QCL assumptions/TCI states are configured by higher layer.

  - Support RV sequences {0, 0, 0, 0} and {0, 3, 0, 3} for PDSCH repetitions

    - One of the RV sequences and starting RV value should be indicatable by the scheduling DCI

  - FFS whether the RV sequence and QCL assumptions/TCI states are jointly indicated by one field or separately indicated by different fields in the scheduling DCI

In Rel.15, PDSCH repetition is enabled only across multiple slots, where each repetition has the same start and length in each slot. In order to achieve spatial diversity gain by the multiple TRPs within a short time period, this should be scaled-down in time-domain. PDSCH mini-slot repetition is a promising scheme. In Rel.16 eURLLC SI，the mini-slot repetition schemes for PUSCH were discussed and details will be finalized during the eURLLC WI phase. In our view, same mechanism can be applied for PDSCH mini-slot repetition.

Proposal 3-2:

- Support mini-slot PDSCH repetition as a function of multi-TRP enhancement for URLLC

  - The details of mini-slot PUSCH repetition studied in eURLLC WI should be applied for mini-slot PDSCH repetition as well.

PUSCH repetition over multiple TRPs

The same enhancements should be enabled for PUSCH. For uplink, two solutions can be realized by the same mechanism in a unified manner:

For UL single-antenna transmission, different SRI can be applied across repetitions

For UL multi-antenna transmission, different precoders can be applied across repetitions

Both should be supported for PUSCH repetition. For dynamic grant PUSCH, the same solution for PDSCH repetition as presented in Section 3.1.1 should be enabled. For configured grant PUSCH, precoder/SRI-cycling across repetitions should also be supported. Note that RV sequences of {0, 0, 0, 0} and {0, 3, 0, 3} are already supported for configured grant PUSCH in Rel.15.

Proposal 3-3:

- Support precoder/SRI-cycling across repetitions for PUSCH repetition for both dynamic grant and configured grant

  - Precoders/SRIs for PUSCH repetitions are indicated by DCI from multiple sequences of precoders/SRIs

    - Multiple sequences of precoders/SRIs for PUSCH repetitions are configured by higher layer

- Support RV sequences {0, 0, 0, 0} and {0, 3, 0, 3} for PUSCH repetitions for dynamic grant

  - For dynamic grant, one of the RV sequences and starting RV value should be indicatable by the scheduling DCI

- FFS whether the RV sequence and precoders/SRIs are jointly indicated by one field or separately indicated by different fields in the scheduling DCI

Same as for PDSCH, PUSCH mini-slot repetition should be usable for this scheme. This should be a common solution as for Rel.16 URLLC PUSCH mini-slot repetition [5]. It is noted that details for mini-slot repetition will be finalized in eURLLC WI.

Proposal 3-4:

- Support mini-slot PUSCH repetition as a function of multi-TRP enhancement for URLLC

  - Details of mini-slot PUSCH repetition should be studied in eURLLC WI.

Number of TRPs for multi-TRP enhancement for URLLC

Assuming that the number of repetitions of a transport block in time-domain is up to 8, in theory, up to 8 TRPs can be used for one PDSCH or PUSCH with using single TRP at one time. On the other hand, using 8 TRPs for scheme 3 and/or scheme 4 implies that a PDSCH is received with 8 TCI-states or a PUSCH is transmitted with 8 spatial relation information. Considering the tradeoff between UE complexity increase and performance benefit, our suggestion is to support at least 4 TRPs for one PDSCH/PUSCH.

In order to confirm the performance benefit of more than 2 TRPs, we conducted link-level simulations. 4GHz and 30GHz with blockage are evaluated. It can be seen from Fig. 3-1 that using 4 TRPs offer non-negligible improvement in 4GHz, and significant improvement in 30GHz. Note that for a given carrier frequency, same time/frequency resource allocation for one PDSCH or PUSCH is assumed.

(a) PDSCH repetitions over 1, 2, and 4 TRPs (left: 4GHz, right: 30GHz).

(b) PUSCH repetitions for 1, 2, and 4 TRPs (left: 4GHz, right: 30GHz).

Fig. 3-1	Average BLER performances with different number of TRPs.

Table 3-3: Simulation assumptions for PDSCH BLER evaluation

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier-spacing | 30kHz | 120kHz |
| TBS | 32 Bytes |  |
| MCS (MCS table 3 is used) | For 1 TRP: MCS0 For 2 TRPs: MCS 3 For 4 TRPs: MCS 7 |  |
| Number of RBs | For 1 TRP: 51 RBs For 2 TRPs: 49 RBs For 4 TRPs: 47 RBs |  |
| Number of symbols | For 1 TRP: 8 symbols x 1 repetition For 2 TRPs: 4 symbols x 2 repetitions For 4 TRPs: 2 symbols x 4 repetitions |  |
| DMRS | DMRS configuration Type 1 For 1 TRP: one additional DMRS is inserted |  |
| PDSCH mapping Type | PDSCH mapping Type B |  |
| Channel estimation | MMSE |  |
| No. of BS antennas | 4 | 2 |
| No. of UE antennas | 4 | 2 |
| Channel model | TDL-C (DS=100ns) | CDL-A (DS=20ns) with blockage model A |
| UE speed | 3km/h |  |

Table 3-4: Simulation assumptions for PUSCH BLER evaluation

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier-spacing | 30kHz | 120kHz |
| TBS | 32 Bytes |  |
| MCS (MCS table 3 is used) | For 1 TRP: MCS2 For 2 TRPs: MCS 6 For 4 TRPs: MCS 9 |  |
| Number of RBs | For 1 TRP: 31 RBs For 2 TRPs: 31 RBs For 4 TRPs: 29 RBs |  |
| Number of symbols | For 1 TRP: 8 symbols x 1 repetition For 2 TRPs: 4 symbols x 2 repetitions For 4 TRPs: 2 symbols x 4 repetitions |  |
| DMRS | DMRS configuration Type 1 For 4 TRPs: FDM between DMRS and UL-SCH |  |
| PDSCH mapping Type | PUSCH mapping Type B |  |
| Frequency-hopping | Not used |  |
| Channel estimation | MMSE |  |
| No. of BS antennas | 4 | 2 |
| No. of UE antennas | 4 | 2 |
| Channel model | TDL-C (DS=100ns) | CDL-A (DS=20ns) with blockage model A |
| UE speed | 3km/h |  |

Proposal 3-5:

- Support at least up to 4 TRPs for multi-TRP for URLLC.

PDCCH repetition for multi-TRP enhancement for URLLC

With the PDSCH/PUSCH repetition over multiple TRPs, PDCCH should also be able to be transmitted by multiple TRPs. The simple solution is to allow transmitting the same DCI payload over multiple PDCCH candidates on one or more search space sets.

Figure 3-2 shows simulation results of PDCCH repetition over multiple TRPs at 4GHz and 30GHz carrier frequency. Simulation assumptions are provided in Table 3-6. At the receiver, two ways of handling repetitions are considered; one is soft-combining, the other is selection (i.e., decoded individually and if at least one is correctly decoded, the PDCCH is considered as successfully received). Here, we compare BLER performance of AL4x4 (repetition with soft-combining), AL4x4 (repetition with selection), AL8x2 (repetition with soft-combining), AL8x2 (repetition with selection), and AL16 (single PDCCH) on various channel models. The assumed channel models are TDL-D at 4GHz, CDL-A without blockage at 30GHz, and CDL-A with blockage (k=4) at 30GHz, which are denoted as case (a), case (b), and case (c), respectively.

The results show that the BLER performances of the repetitions using both AL4 and AL8 with soft-combining outperforms that of the single PDCCH with AL16 in all the cases. With lower BLER e.g. BLER=10-6, which is the target of URLLC, the gain gets bigger. Also, it is observed that the repetitions with selection can obtain better performance compared to the single PDCCH with AL16 at 30GHz. Especially focusing on case (c), the performances of the repetition with selection are better than that of the single PDCCH and the improvements get similar to those of the repetitions with soft-combining. This is because blockage, which is serious problem for FR2, is alleviated by repetition diversity gain. Therefore, for FR2, it is concluded that repetition over multiple TRPs with both of repetition ways can improve PDCCH reliability. However, for FR1 (case (a)), the repetition using AL4 with selection cannot achieve better performance than the single PDCCH with AL16, while the repetition using AL8 with selection can. This indicates that repetition with selection is not always effective for FR1. Thus, it is considered that repetition over multiple TRPs with soft-combining is a better candidate to improve PDCCH reliability. To facilitate understanding the improvements by soft-combining, the gains between each assumption and single PDCCH with AL16 for target BLER=10-6 are summarized in Table 3-5.

Table 3-5: Gains of PDCCH repetition.

(a) FR1: TDL-D at 4GHz

(a) FR1: TDL-D at 4GHz

(b) FR2: CDL-A at 30GHz

(b) FR2: CDL-A at 30GHz

(c) FR2: CDL-A with blockage at 30GHz

(c) FR2: CDL-A with blockage at 30GHz

Fig. 3-2	BLER performances of PDCCH repetitions.

As stated above, PDCCH repetition over multiple TRPs highly improves reliability. However, a concern regarding PDCCH soft-combining was mentioned from the UE implementation point of view in the past meetings. Alternative way to soft-combining multiple PDCCH candidates is to treat multiple PDCCH candidates over multiple CORESETs as one PDCCH candidate for blind decode. Now, each search space set is associated to one CORESET. It can be changed so that each SS set can be associated to more than one CORESETs. If a SS set is associated to multiple CORESETs, a PDCCH candidate in the SS set is across the associated multiple CORESETs. The UE performs PDCCH blind decodes using CCEs across multiple CORESETs as shown in Fig. 3-3.

Fig. 3-3		PDCCH candidates on a SS set which is associated to multiple CORESETs.

Proposal 3-6:

- Support either of the following:

  - PDCCH repetition over multiple CORESETs with soft-combining of PDCCH candidates, or;

  - Search space set is associated to multiple CORESETs, in which case each PDCCH candidate is composed of CCEs over multiple CORESETs.

Table 3-6: Link level simulation assumptions.

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Carrier Frequency |  | 4 GHz | 30 GHz |
| Sub-carrier Spacing |  | 30 kHz | 120 kHz |
| Modulation |  | QPSK |  |
| Channel Coding |  | Polar code |  |
| Aggregation Level |  | 4, 8. 16 |  |
| Control Resource Set (CORESET) configuration | Time-domain duration | 1 OFDM symbol | 2 OFDM symbol |
|  | CORESET Bandwidth | 40 MHz | 100 MHz |
|  | CCE-to-REG mapping | Interleaved (Interleaver row: 2) |  |
|  | REG-bundle size | 6 |  |
|  | Precoder granularity | REG-bundle |  |
|  | Resource mapping | Distributed transmission |  |
| Transmission Diversity Scheme |  | 1-port Precoder Cycling |  |
| DMRS density |  | 1/4; symbol #1, #5, #9 within each REG |  |
| Channel Model |  | TDL-D, Delay spread: 30 ns, UE speed: 3 km/h | CDL-A, Delay spread: 20 ns, UE speed: 3 km/h |
| gNB antenna configuration |  | 2Tx | 4Tx |
| UE antenna configuration |  | 2Rx | 2Rx |
| Channel Estimation |  | MMSE |  |
| Noise Estimation |  | Ideal |  |

PUCCH transmission for multi-TRP enhancement for URLLC

Any channels can be benefited from the multiple TRP transmissions. PUCCH repetition is not an exception. Similar as PDCCH, PDSCH, and dynamic/configured grant PUSCH, PUCCH repetition should be combined with multiple TRPs. More specifically, different spatial relation info across PUCCH repetitions should be supported. Further study is needed on how to indicate/configure the spatialrelationinfo for PUCCH for HARQ-ACK transmission to each TRP (or to each repetition).

In general, three options can be considered to indicate/configure the spatialrelationinfo for PUCCH for HARQ-ACK transmission to multiple TRPs:

- Alt.1: spatial relation info for PUCCH for HARQ-ACK transmission to multiple TRPs is configured by RRC

- Alt.2: spatial relation info for PUCCH for HARQ-ACK transmission to multiple TRPs is configured by RRC and MAC CE;

- Alt.3: spatial relation info for PUCCH for HARQ-ACK transmission for multiple TRPs is indicated by RRC and DCI;

Table 3-7, 3-8 and 3-9 illustrate the signaling configuration for each of the alternatives above. For Alt.1, spatial relation info for PUCCH transmission for each TRP (for each repetition) is configured by RRC signaling. Regardless of which PUCCH resource is determined by the ARI indicated in the DCI and/or CCE index of the PDCCH, the configured sequence of Spatialrelationinfo is applied across repetitions. Therefore, as shown in Table 3-7, for each set of PDSCH repetitions, same PUCCH resource can be indicated by PDCCH, but spatial relation info is different cross repetitions which is semi-statically fixed.

Table 3-7: Example of PUCCH resource indication with semi-statically configured Spatialrelationinfo sequence

| ARI field value | PUCCH resource | Spatialrelationinfo sequence |
| --- | --- | --- |
| 000 | Resource #a | Spatialrelationinfo ID#1 for 1st; Spatialrelationinfo ID#2 for 2nd; Spatialrelationinfo ID#3 for 3rd; Spatialrelationinfo ID#4 for 4th |
| 001 | Resource #b |  |
| 010 | Resource #c |  |
| … |  |  |

However, semi-static spatialrelationinfo sequence may not be flexible enough; depending on CSI knowledge at the gNB, the spatialrelationinfo sequences can be updated/indicated semi-statically by MAC CE or dynamically by DCI. For semi-static spatialrelationinfo update/indication, the same mechanism as Rel.15 can be used. For example, as shown in Table 3-8, a set of spatialrelationinfo sequences can be configured by RRC, and then MAC CE activates one spatialrelationinfo sequence from multiple spatialrelationinfo sequences configured by RRC. For dynamic spatialrelationinfo update/indication, either a separate filed in DCI or joint filed for indicating PUCCH resource can be considered. To reduce the signaling overhead, joint field for indicating the PUCCH resource and the spatialrelationinfo sequence is considered below. Table 3-9 gives an example.

Table 3-8: Spatialrelationinfo sequences configured by RRC

| Spatialrelationinfo sequence ID | Spatialrelationinfo sequence across PUCCH repetitions |
| --- | --- |
| 1 | {Spatialrelationinfo ID#1, Spatialrelationinfo ID#1, Spatialrelationinfo ID#1, Spatialrelationinfo ID#1} |
| 2 | {Spatialrelationinfo ID#1, Spatialrelationinfo ID#3, Spatialrelationinfo ID#1, Spatialrelationinfo ID#3} |
| 3 | {Spatialrelationinfo ID#2, Spatialrelationinfo ID#4, Spatialrelationinfo ID#2, Spatialrelationinfo ID#4} |
| … | … |
| M | {Spatialrelationinfo ID#1, Spatialrelationinfo ID#3, Spatialrelationinfo ID#2, Spatialrelationinfo ID#4} |

Table 3-9: Example of joint field indication for Spatialrelationinfo sequence and PUCCH resource

| ARI field value | PUCCH resource | Spatialrelationinfo sequence |
| --- | --- | --- |
| 000 | Resource #a | Spatialrelationinfo ID#1 for 1st; Spatialrelationinfo ID#2 for 2nd; Spatialrelationinfo ID#3 for 3rd; Spatialrelationinfo ID#4 for 4th |
| 001 | Resource #b | Spatialrelationinfo ID#3 for 1st; Spatialrelationinfo ID#1 for 2nd; Spatialrelationinfo ID#4 for 3rd; Spatialrelationinfo ID#2 for 4th |
| 010 | Resource #c | Spatialrelationinfo ID#1 for 1st; Spatialrelationinfo ID#1 for 2nd; Spatialrelationinfo ID#1 for 3rd; Spatialrelationinfo ID#1 for 4th |
| … |  |  |

Proposal 3-7:

- Conclude to support PUCCH repetition within a slot.

  - FFS details

- Support spatialrelationinfo/precoder-cycling across repetitions for PUCCH repetition. The following alternatives for spatialrelationinfo update/indication can be considered:

  - Alt.1: spatial relation info for PUCCH for HARQ-ACK transmission to multiple TRPs is configured by RRC

  - Alt.2: spatial relation info for PUCCH for HARQ-ACK transmission to multiple TRPs is configured by RRC and MAC CE;

  - Alt.3: spatial relation info for PUCCH for HARQ-ACK transmission for multiple TRPs is indicated by RRC and DCI.

Conclusion

In this contribution, we discussed the enhancements on multi-TRP/panel transmission in Rel-16. Based on the discussion, we made following observations and proposals.

For multi-TRP/panel enhancement for non-coherent joint transmission for eMBB,

Proposal 2-1:

- For single PDCCH design for eMBB, up to 16 TCI states can be activated by MAC CE enhancement.

- For single PDCCH design for eMBB, the number of bits of TCI field in DCI is not increased.

Proposal 2-2:

- For single PDCCH design for eMBB, frequency domain and time domain resources of PDSCHs from multiple TRPs/panels are completely overlapped.

  - Note: no additional DCI fields are needed to indicate the time/frequency domain resources

Proposal 2-3:

- NR Rel-16 should support joint UCI (e.g., ACK/NACK and CSI) payload/feedback for received PDSCHs for single PDCCH based multiple-TRP/panel transmission for eMBB.

Proposal 2-4:

- To support joint ACK/NACK payload/feedback,

  - For semi-static HARQ-ACK codebook, Rel.15 design should be reused as much as possible.

    - Candidate PDSCH reception occasions for multiple TRPs/panels can be determined based on Rel.15 mechanism;

    - The HARQ-ACK payload and bits order for multiple TRPs can be determined based on the number of TRPs/panels and TRP/panel index

  - For dynamic HARQ-ACK codebook, the DAI filed in Rel.15 can be reused.

    - At least joint DAI counting across TRPs/panels should be supported.

Proposal 2-5:

- For single PDCCH based multi-TRP DMRS port indication:

  - Support layer combinations (1,0), (2,0), (3,0), (4,0), (0,1), (0,2), (0,3), (0,4), (1,1), (2,1), (2,2) for <=4 layers;

  - Support layer combinations (3,3) (3,2), (4,3), (4,4) for >4 layers.

Proposal 2-6:

- For multiple PDCCH design for eMBB, support to increase the maximum number of CORESET per BWP for a UE to 4.

Proposal 2-7:

- For multiple PDCCH design for eMBB, support the same PDSCH mapping type (PDSCH Type A+ PDSCH Type A) for co-scheduled PDSCHs as starting point.

Proposal 2-8:

- For multiple PDCCH design for eMBB, support DCI-based BWP switching, if configured.

  - UE assumes the same active BWP for multiple TRPs.

    - UE follows BWP indication from one TRP as specified or RRC configured, and ignores the BWP indication from another TRP.

- For multiple PDCCH design for eMBB, support timer-based BWP switching, if configured.

  - UE assumes the same active BWP for multiple TRPs.

    - UE follows timer-based switched BWP for one TRP as specified or RRC configured.

Proposal 2-9:

- For multiple PDCCH based multiple-TRPs/panels transmission, support different PDSCH scrambling sequences for PDSCHs where multiple dataScramblingIdentityPDSCH can be configured by RRC.

Proposal 2-10:

- For multiple PDCCH design for eMBB, rate-matching mechanism for eMBB should be firstly discussed and the rate-matching mechanism considering URLLC should be deprioritized.

- For rate-matching mechanism for multiple PDCCH design for eMBB,

  - For ideal backhaul, Rel-15 rate matching mechanism is baseline and can be reused.

  - For non-ideal backhaul, support at least one of following

    - The rate-matched resources around the other TRP are configured semi-statically

    - The puncturing of resources around the other TRP is performed dynamically

Proposal 2-11:

- NR Rel-16 should support joint UCI (e.g., ACK/NACK and CSI) payload/feedback for received PDSCHs for multiple PDCCHs based multiple-TRP/panel transmission with ideal backhaul for eMBB.

Proposal 2-12:

- For multiple PDCCH-based multiple TRPs/panels transmission, both joint and separate DAI counting across TRPs/panels should be supported:

  - Whether the DAI is counted jointly or separately can be configured by RRC signaling.

Proposal 2-13:

- For separate ACK/NACK feedback, the network should ensure the TDMed PUCCH resources among multiple TRPs/panels.

  - The PUCCH resources grouping for PUCCH spatialrelationinfo update/indication is configured the same with PUCCH resources grouping for PUCCH resource allocation associated with different TRPs/panels.

Proposal 2-14:

- For multiple PDCCH design for eMBB, the format of PUCCH of multiple TRPs can be different.

Proposal 2-15:

- For multiple PDCCH design for eMBB, UE does not support simultaneous ACK/NACK transmission from multiple PUCCH resources via different QCL in Rel-16.

Proposal 2-16:

- For multiple PDCCH design for eMBB, when UL channels from different TRPs collide,

  - UE selects the TRP/panel for UL transmission based on some predefined rules, and then re-use Rel-15 dropping/multiplexing rule for the selected TRP/panel if there are still multiple UL channels to be transmitted at the same time for the selected TRP/panel

    - The predefined rule is firstly based on the priority of UL channel type and contents in the UL channel like Rel-15 rule.

    - If the overlapped UL channels from two TRPs/panels have the same priority (e.g., with the same UL channel type and contents), the TRP/panel with the lowest TRP/panel ID or with the earlier starting time of the UL channel is selected

Observation 2-1:

- Multiple CSI feedback considering inter-beam/inter-panel/inter-TRP interference measurement can increase both cell average and cell edge performance in multi-panel/TRP scenario.

Proposal 2-17:

- To support non-coherent joint transmission in multi-TRP/panel for eMBB, NR Rel-16 should study the CSI enhancement, including

  - Configuration and L1 measurement/reporting of a combination of multiple beams transmitted from multiple TRPs/panels simultaneously

  - Configuration and CSI measurement to support multiple types of interference hypothesis measurement from multiple TRPs/panels

Proposal 2-18:

- For multi-panel/TRP transmission for eMBB,

  - Support the same HARQ entity for multi-panel/TRP transmission.

  - Support the increased HARQ process number per HARQ entity for multiple PDCCH based multi-panel/TRP transmission.

For multi-TRP enhancement for URLLC,

Proposal 3-1:

- For scheme 3 and scheme 4 with single DCI based multiple TRP for URLLC,

  - QCL assumptions/TCI states for PDSCH repetitions are indicated by DCI from multiple sequences of QCL assumptions/TCI states

    - Multiple sequences of QCL assumptions/TCI states are configured by higher layer.

  - Support RV sequences {0, 0, 0, 0} and {0, 3, 0, 3} for PDSCH repetitions

    - One of the RV sequences and starting RV value should be indicatable by the scheduling DCI

  - FFS whether the RV sequence and QCL assumptions/TCI states are jointly indicated by one field or separately indicated by different fields in the scheduling DCI

Proposal 3-2:

- Support mini-slot PDSCH repetition as a function of multi-TRP enhancement for URLLC

  - The details of mini-slot PUSCH repetition studied in eURLLC WI should be applied for mini-slot PDSCH repetition as well.

Proposal 3-3:

- Support precoder/SRI-cycling across repetitions for PUSCH repetition for both dynamic grant and configured grant

  - Precoders/SRIs for PUSCH repetitions are indicated by DCI from multiple sequences of precoders/SRIs

    - Multiple sequences of precoders/SRIs for PUSCH repetitions are configured by higher layer

- Support RV sequences {0, 0, 0, 0} and {0, 3, 0, 3} for PUSCH repetitions for dynamic grant

  - For dynamic grant, one of the RV sequences and starting RV value should be indicatable by the scheduling DCI

- FFS whether the RV sequence and precoders/SRIs are jointly indicated by one field or separately indicated by different fields in the scheduling DCI

Proposal 3-4:

- Support mini-slot PUSCH repetition as a function of multi-TRP enhancement for URLLC

  - Details of mini-slot PUSCH repetition should be studied in eURLLC WI.

Proposal 3-5:

- Support at least up to 4 TRPs for multi-TRP for URLLC.

Proposal 3-6:

- Support either of the following:

  - PDCCH repetition over multiple CORESETs with soft-combining of PDCCH candidates, or;

  - Search space set is associated to multiple CORESETs, in which case each PDCCH candidate is composed of CCEs over multiple CORESETs.

Proposal 3-7:

- Conclude to support PUCCH repetition within a slot.

  - FFS details

- Support spatialrelationinfo/precoder-cycling across repetitions for PUCCH repetition. The following alternatives for spatialrelationinfo update/indication can be considered:

  - Alt.1: spatial relation info for PUCCH for HARQ-ACK transmission to multiple TRPs is configured by RRC

  - Alt.2: spatial relation info for PUCCH for HARQ-ACK transmission to multiple TRPs is configured by RRC and MAC CE;

  - Alt.3: spatial relation info for PUCCH for HARQ-ACK transmission for multiple TRPs is indicated by RRC and DCI.

- References

- 3GPP RAN1#96, chairman notes, Feb. 2019.

- 3GPP RAN1#96bis, chairman notes, Apr. 2019.

- 3GPP RAN1#96bis, R1-1905166, “NCJT performance with layer restriction between TRPs”, Ericsson, Apr. 2019.

- 3GPP RAN1 AH#1901, R1-1900979, “Discussion on multi-beam enhancement”, NTT DOCOMO, INC., Jan. 2019.

- 3GPP RAN1 #96, R1-1902806, “PUSCH enhancements for URLLC”, NTT DOCOMO, INC., Feb. 2019.

- Appendix

- Table A-1: Evaluation assumptions for multi-panel transmission

| Parameters | Values or assumptions |
| --- | --- |
| Scenario | Dense Urban Micro layer only |
| Carrier Frequency | 30 GHz |
| Mode | TDD |
| Bandwidth | 80MHz |
| Subcarrier Spacing | 120kHz |
| Channel Model | UMi in TR 38.901 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| ISD | 200m |
| BS Tx power | 39dBm |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |
| BS Antenna Configuration | Configuration 1: (M,N,P,Mg,Ng) = (4,8,2,1,2)(dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 0)λ Configuration 2: (M,N,P,Mg,Ng) = (4,8,2,2,2)(dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 0 degree, ΩUT,g = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration for UE | Ideal |
| Scheduling algorithm | PF |
| BS antenna height | 10m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 5dBi |
| UE receiver noise figure | 10dB |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Criteria for analog beam selection for serving TRxP | L1-RSRP |
| Traffic model | Full buffer |
| UE distribution | 80% indoor; 20% outdoor. 20 users per TRP |